3GPP TSG-RAN WG4 Meeting #117	R4-2522987

# Dallas, USA, Nov. 17 - 21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR on TC NR inter-frequency measurement for Redcap UEs with NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ph3-Perf |  |  |  |  |  | Date: |  |  | 2025-11-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on WF for Rel-19 NTN phase 3 (R4-2515056), the test cases for NR inter-frequency measurements for Redcap UEs with satellite access need to be introduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the test case for NR inter-frequency measurements for Redcap UEs with satellite access. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test configuration and procedure will be imcompleted. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new) A.20.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R4-2521336 |  |  |  |  |  |  |  |  |

------------ START OF CHANGE 1--------------

A.20.5.2	Inter-frequency Measurements

Table A.20.5.2.1.1-1: SA event triggered reporting tests without SSB index reading for FR1-FR1

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 3 | GSO, HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 4 | NGSO, HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| NOTE: If UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. |  |

A.20.5.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used with single gap for 2Rx RedCap UE with satellite access

A.20.5.2.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clauses 9.3E.4 and 9.3E.5.

The test environment in clause A.14.5.2.2.1 shall apply for 2RX RedCap UE except that:

-	Table A.14.5.2.2.1-1 is replaced with A.20.5.2.1.1-1, and

- 	Table A.14.5.2.2.1-2, Table A.14.5.2.2.1-3 shall apply to configuration 1,2,3,4.

Table A.20.5.2.1.1-1: SA event triggered reporting tests without SSB index reading for FR1-FR1

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 3 | GSO, HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 4 | NGSO, HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| NOTE: If UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. |  |

A.20.5.2.1.2	Test Requirements

The test requirement in clause A.14.5.2.2.2 shall apply for RedCap.

A.20.5.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used with single gap for 1Rx RedCap UE with satellite access

A.20.5.2.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clauses 9.3E.4 and 9.3E.5.

The test environment in clause A.14.5.2.2.1 shall apply for 1RX RedCap UE except that:

-	Table A.14.5.2.2.1-1 is replaced with A.20.5.2.1.1-1, and

- 	Table A.14.5.2.2.1-2 is replaced with A.20.5.2.2.1-2, and

- 	Table A.14.5.2.2.1-3 shall apply to configuration 1,2,3,4.

Table A.20.5.2.2.1-1: void

Table A.20.5.2.2.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| NR RF Channel Number |  | Config 1,2,3,4 | 1, 2 |  | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3,4 | NR Cell 1 (Pcell) |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3,4 | NR Cell 2 |  | NR Cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3,4 | 0 |  | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3,4 | 9 |  |  |
| A3-Offset | dB | Config 1,2,3,4 | -6 |  |  |
| Hysteresis | dB | Config 1,2,3,4 | 0 |  |  |
| CP length |  | Config 1,2,3,4 | Normal |  |  |
| TimeToTrigger | s | Config 1,2,3,4 | 0 |  |  |
| Filter coefficient |  | Config 1,2,3,4 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2,3,4 | DRX.1 | DRX. 7 | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  | Config 1,2,3,4 | 3 ms |  | Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
| T1 | s | Config 1,2,3,4 | 5 |  |  |
| T2 | s | Config 1,2,3,4 | 1.1 | 12 |  |

A.20.5.2.2.2	Test Requirements

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1080 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90 %.

In test 2 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 11520 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90 %.

In test 1 and 2 UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

A.20.5.2.3	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used with single gap for 2Rx RedCap UE with satellite access

A.20.5.2.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clauses 9.3E.4 and 9.3E.5.

The test environment in clause A.14.5.2.3.1 shall apply for 2RX RedCap UE except that:

-	Table A.14.5.2.3.1-1 is replaced with A.20.5.2.1.1-1, and

- 	Table A.14.5.2.3.1-2, Table A.14.5.2.3.1-3 shall apply to configuration 1,2,3,4.

A.20.5.2.3.2	Test Requirements

The test requirement in clause A.14.5.2.3.2 shall apply for RedCap.

A.20.5.2.4	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used with single gap for 1Rx RedCap UE with satellite access

A.20.5.2.4.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clauses 9.3E.4 and 9.3E.5.

The test environment in clause A.14.5.2.3.1 shall apply for 1RX RedCap UE except that:

-	Table A.14.5.2.3.1-1 is replaced with A.20.5.2.1.1-1, and

- 	Table A.14.5.2.3.1-2 is replaced with A.20.5.2.4.1-2, and

- 	Table A.14.5.2.3.1-3 shall apply to configuration 1,2,3,4.

Table A.20.5.2.4.1-1: void

Table A.20.5.2.4.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1 |  |
| NR RF Channel Number |  | Config 1,2,3,4 | 1, 2 | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3,4 | NR Cell 1 (Pcell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3,4 | NR Cell 2 | NR Cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3,4 | 0 | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3,4 | 9 |  |
| A3-Offset | dB | Config 1,2,3,4 | -6 |  |
| Hysteresis | dB | Config 1,2,3,4 | 0 |  |
| CP length |  | Config 1,2,3,4 | Normal |  |
| TimeToTrigger | s | Config 1,2,3,4 | 0 |  |
| Filter coefficient |  | Config 1,2,3,4 | 0 | L3 filtering is not used |
| DRX |  | Config 1,2,3,4 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1,2,3,4 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
| T1 | s | Config 1,2,3,4 | 5 |  |
| T2 | s | Config 1,2,3,4 | 1.4 |  |

A.20.5.2.4.2	Test Requirements

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1240 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90 %.

In test 1 UE is required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

A.20.5.2.5	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with two gaps in fully non-overlapped for 2Rx RedCap UE with satellite access

A.20.5.2.5.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clauses 9.3E.4 and 9.3E.5.

The test environment in clause A.14.5.2.4.1 shall apply for 2RX RedCap UE except that:

-	Table A.14.5.2.4.1-1 is replaced with A.20.5.2.1.1-1, and

- 	Table A.14.5.2.4.1-2, Table A.14.5.2.4.1-3 shall apply to configuration 1,2,3,4.

A.20.5.2.5.2	Test Requirements

The test requirement in clause A.14.5.2.4.2 shall apply for RedCap.

A.20.5.2.6	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with two gaps in fully non-overlapped for 1Rx RedCap UE with satellite access

A.20.5.2.6.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clauses 9.3E.4 and 9.3E.5.

The test environment in clause A.14.5.2.4.1 shall apply for 1RX RedCap UE except that:

-	Table A.14.5.2.4.1-1 is replaced with A.20.5.2.1.1-1, and

- 	Table A.14.5.2.4.1-2 is replaced with A.20.5.2.6.1-2, and

- 	Table A.14.5.2.4.1-3 shall apply to configuration 1,2,3,4.

Table A.20.5.2.6.1-1: void

Table A.20.5.2.6.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3,4 | 1, 2 | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3,4 | NR Cell 1 (Pcell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3,4 | NR Cell 2 and NR Cell 3 | NR Cell 2 and NR Cell 3 are on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3,4 | 0 for MeasGapId #1 0 for MeasGapId #2 | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3,4 | 9 for MeasGapId #1 19 for MeasGapId #2 |  |
| A3-Offset | dB | Config 1,2,3,4 | -6 |  |
| Hysteresis | dB | Config 1,2,3,4 | 0 |  |
| CP length |  | Config 1,2,3,4 | Normal |  |
| TimeToTrigger | s | Config 1,2,3,4 | 0 |  |
| Filter coefficient |  | Config 1,2,3,4 | 0 | L3 filtering is not used |
| DRX |  | Config 1,2,3,4 | OFF | DRX is not used |
| Time offset between serving and neighbour Cell 2,3 |  | Config 1,2,3,4 | 3 ms | Asynchronous cells. The timing of Cell 2 and Cell 3 is 3 ms later than the timing of Cell 1. |
| T1 | s | Config 1,2,3,4 | 5 |  |
| T2 | s | Config 1,2,3,4 | 1.2 |  |

A.20.5.2.6.2	Test Requirements

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1000 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90 %.

In test 1 UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

A.20.5.2.7	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with two gaps in partially partial overalpping for 2Rx RedCap UE with satellite access

A.20.5.2.7.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clauses 9.3E.4 and 9.3E.5.

The test environment in clause A.14.5.2.6.1 shall apply for 2RX RedCap UE except that:

-	Table A.14.5.2.6.1-1 is replaced with A.20.5.2.1.1-1, and

- 	Table A.14.5.2.6.1-2, Table A.14.5.2.6.1-3 shall apply to configuration 1,2,3,4.

A.20.5.2.7.2	Test Requirements

The test requirement in clause A.14.5.2.6.2 shall apply for RedCap.

A.20.5.2.8	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with two gaps in partially partial overalpping for 1Rx RedCap UE with satellite access

A.20.5.2.8.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clauses 9.3E.4 and 9.3E.5.

The test environment in clause A.14.5.2.6.1 shall apply for 1RX RedCap UE except that:

-	Table A.14.5.2.6.1-1 is replaced with A.20.5.2.1.1-1, and

- 	Table A.14.5.2.6.1-2 is replaced with A.20.5.2.8.1-2, and

- 	Table A.14.5.2.6.1-3 shall apply to configuration 1,2,3,4.

Table A.20.5.2.8.1-2: void

Table A.20.5.2.8.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3,4 | 1, 2 | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3,4 | NR Cell 1 (Pcell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3,4 | NR Cell 2 and NR Cell 3 | NR Cell 2 and NR Cell 3 are on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3,4 | 0 for MeasGapId #1 1 for MeasGapId #2 | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3,4 | 39 for MeasGapId #1 4 for MeasGapId #2 |  |
| A3-Offset | dB | Config 1,2,3,4 | -6 |  |
| Hysteresis | dB | Config 1,2,3,4 | 0 |  |
| CP length |  | Config 1,2,3,4 | Normal |  |
| TimeToTrigger | s | Config 1,2,3,4 | 0 |  |
| Filter coefficient |  | Config 1,2,3,4 | 0 | L3 filtering is not used |
| DRX |  | Config 1,2,3,4 | OFF | DRX is not used |
| Time offset between serving and neighbour Cell 1 |  | Config 1,2,3,4 | 3s | Synchronous. |
| Time offset between serving and neighbour Cell 2 |  | Config 1,2,3,4 | 5 ms | Asynchronous. The timing of Cell 3 is 5 ms later than the timing of Cell 1. |
| T1 | s | Config 1,2,3,4 | 5 |  |
| T2 | s | Config 1,2,3,4 | 1.6 |  |

A.20.5.2.8.2	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1440 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90 %.

The UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

A.20.5.2.9	Event triggered reporting test without gap under non-DRX for 2Rx RedCap UE with satellite access

A.20.5.2.9.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the inter-frequency cell search requirements in clauses 9.3E.7.

A.20.5.2.9.2	Test parameters

The test environment in clause A.14.5.2.7.1 shall apply for 2RX RedCap UE except that:

-	Table A.14.5.2.7.2-1 is replaced with A.20.5.2.1.1-1, and

- 	Table A.14.5.2.7.2-2, Table A.14.5.2.7.2-3 shall apply to configuration 1,2,3,4.

A.20.5.2.9.3	Test Requirements

The test requirement in clause A.14.5.2.7.3 shall apply for RedCap.

A.20.5.2.10	Event triggered reporting test without gap under non-DRX for 1Rx RedCap UE with satellite access

A.20.5.2.10.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the inter-frequency cell search requirements in clauses 9.3E.7.

A.20.5.2.10.2	Test parameters

The test environment in clause A.14.5.2.7.2 shall apply for 1RX RedCap UE except that:

-	Table A.14.5.2.7.2-1 is replaced with A.20.5.2.1.1-1, and

- 	Table A.14.5.2.7.2-2, Table A.14.5.2.7.2-3 shall apply to configuration 1,2,3,4.

A.20.5.2.10.3	Test Requirements

The test requirement in clause A.14.5.2.7.3 shall apply for RedCap.

A.20.5.2.11	Event triggered reporting tests without gap under DRX for 2Rx RedCap UE with satellite access

A.20.5.2.11.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the inter-frequency cell search requirements in clauses 9.3E.7.

A.20.5.2.11.2	Test parameters

The test environment in clause A.14.5.2.8.2 shall apply for 2RX RedCap UE except that:

-	Table A.14.5.2.8.2-1 is replaced with A.20.5.2.1.1-1, and

- 	Table A.14.5.2.8.2-2, Table A.14.5.2.8.2-3 shall apply to configuration 1,2,3,4.

A.20.5.2.11.3	Test Requirements

In test 1, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test. X=1280 for test configuration 2,4 and if UE indicates ‘n1’ for maxNumber-NGSO-SatellitesWithinOneSMTC, otherwise X=920.

In test 2, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Y ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test. Y=12800 for test configuration 2,4 and if UE indicates ‘n1’ for maxNumber-NGSO-SatellitesWithinOneSMTC, otherwise Y=6400.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90 %.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

A.20.5.2.12	Event triggered reporting tests without gap under DRX for 1Rx RedCap UE with satellite access

A.20.5.2.12.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the inter-frequency cell search requirements in clauses 9.3E.7.

A.20.5.2.12.2	Test parameters

The test environment in clause A.14.5.2.8.2 shall apply for 1RX RedCap UE except that:

-	Table A.14.5.2.8.2-1 is replaced with A.20.5.2.1.1-1, and

- 	Table A.14.5.2.8.2-2 is replaced with A.20.5.2.12.2-2, and

- 	Table A.14.5.2.8.2-3 shall apply to configuration 1,2,3,4.

Table A.20.5.2.12.2-2: void

Table A.20.5.2.12.2-2: General test parameters for inter-frequency event triggered reporting without gap for PCell in FR1 with DRX

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| Active cell |  | 1, 2, 3, 4 | Cell 1 |  |  |
| Neighbour cell |  | 1, 2, 3, 4 | Cell 2 and Cell 3 |  | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3, 4 | 1: Cell 1 2: Cell 2 and Cell 3 |  |  |
| SMTC configuration |  | 1, 2, 3, 4 | SMTC.2 |  |  |
| A3-Offset | dB | 1, 2, 3, 4 | -4.5 |  |  |
| CP length |  | 1, 2, 3, 4 | Normal |  |  |
| Hysteresis | dB | 1, 2, 3, 4 | 0 |  |  |
| Time To Trigger | s | 1, 2, 3, 4 | 0 |  |  |
| Filter coefficient |  | 1, 2, 3, 4 | 0 |  | L3 filtering is not used |
| DRX |  | 1, 2, 3, 4 | DRX.1 | DRX. 7 |  |
| Time offset between serving and neighbour cells |  | 1, 2, 3, 4 | 3 s |  | Synchronous cells |
| T1 | s | 1, 2, 3, 4 | 5 |  |  |
| T2 | s | 1, 2, 3, 4 | 5 | 16 |  |

A.20.5.2.12.3	Test Requirements

In test 1, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test. X=1520 for test configuration 2,4 and if UE indicates ‘n1’ for maxNumber-NGSO-SatellitesWithinOneSMTC, otherwise X=920.

In test 2, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Y ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test. Y=15360 for test configuration 2,4 and if UE indicates ‘n1’ for maxNumber-NGSO-SatellitesWithinOneSMTC, otherwise Y=7680.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90 %.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

------------ END OF CHANGE 1--------------