3GPP TSG RAN WG4 (Radio) Meeting #26	R4-030002

Madrid, Spain, 17 - 22 February, 2003

Title:		Approved Report of the 25th 3GPP TSG RAN WG4 meeting
		(Secaucus, NJ, US 11th -15th November, 2002)

Source:		3GPP support team

All the meeting documents can be found at:

ftp://ftp.3gpp.org/tsg_ran/WG4_Radio/TSGR4_25/

César Gutiérrez Miguélez

ETSI Mobile Competence Center

cesar.gutierrez@etsi.org

# Contents

Contents	2

1	Opening of the meeting	4

2	Approval of the agenda	4

3	Approval of meeting report	4

4	Letters / reports from other groups	4

5	Maintenance of Release 99, Release 4 and Release 5 specifications	10

5.1	25.941 - Document Structure	10

5.2	25.101 - UE Radio transmission and reception (FDD)	10

5.2.1	FDD UE HSDPA	11

5.2.1.1	QPSK only UEs	13

5.2.1.2	VRC Simulation Results	13

5.2.1.3	CQI definition and test methodology for VRC	13

5.3	25.102 - UE Radio transmission and reception (TDD)	14

5.3.1	TDD HSDPA	15

5.3.1.1	Fixed Reference Channel	15

5.3.1.2	Variable channel	15

5.4	25.104 - BTS Radio transmission and reception (FDD)	16

5.4.1	BS IPDL attenuation requirement	18

5.5	25.105 - BTS Radio transmission and reception (TDD)	18

5.6	25.123 - Support of RF parameters in Radio Resource Management (TDD)	20

5.7	25.133 - Support of RF parameters in Radio Resource Management (FDD)	20

5.7.1	IPDL issues concerning the UE	23

5.7.2	Cell Identification	24

5.7.3	Layer 3 filtering	25

5.8	25.141 - Base Station conformance testing (FDD)	26

5.9	25.142 - Base Station conformance testing (TDD)	28

5.10	25.113 - Base station EMC	29

5.11	34.124 - EMC requirements for mobile terminals and ancillary equipment	29

5.12	34.926 - Table of International requirements for Mobile terminals and ancillary equipment	30

5.13	25.942 - RF System scenarios	30

5.14	25.106 – Repeaters - Radio transmission and reception	30

5.15	25.143 – Repeaters conformance testing	31

5.16	25.956 – Repeaters technical report	33

5.17	25.943 – Deployment aspects	33

6	Release 6 Work Items	33

6.1	Improvement of Receiver Performance Requirements for FDD UE	33

6.2	FDD Base Station Classification	34

6.3	Work Items under other WG responsibility	37

7	Release 6 Study Items	37

7.1	Wide Band distribution Systems	37

7.2	FS for the deployment of UTRA in additional and diverse spectrum arrangement	37

8	Liaison and output to other groups	38

9	Revision of the Work Plan	38

10	Future meetings	38

11	Any other business	39

13	Close of Meeting	39

14	Email approval	39

14.1	Conclusion of the email approval	40

Annex A:	List of participants	41

Annex B:	List of documents	43

Annex C:	List of agreed CRs	53

# 1	Opening of the meeting

Howard Benn (chairman) opened the meeting at 9:00. and welcome delegates to New York. Cliff Bayle made the presentation on behalf of the host Spirent. He informed about the social event, a tour of New York City, and the meeting arrangements.

The chairman made the following IPR call:

The attention of the members of this Technical Specification Group is drawn to the fact that 3GPP Individual Members have the obligation under the IPR Policies of their respective Organizational Partners to inform their respective Organizational Partners of Essential IPRs they become aware of. The members take note that they are hereby invited: - to investigate in their company whether their company does own IPRs which are, or are likely to become Essential in respect of the work of the Technical Specification Group. - to notify the Chairman, or the Director-General of their respective Organizational Partners, of all potential IPRs that their company may own, by means of the IPR Statement and the Licensing declaration forms.

# 2	Approval of the agenda

R4-021397	Draft agenda meeting #25 (Chairman)

Edgar Fernandes (Motorola) requested that documents made available to the meeting should also be sent to the email reflector, for those interested but not being able to attend the meeting personally. The chairman agreed on the convenience of this procedure, but noted the difficulty to enforce it, since not everyone in the room had easy email access.

The chairman clarified that the agenda has changed slightly compared to the last meeting, HSDPA is quite mature now and it is covered within the Rel-5 Maintenance agenda point.

Decision: The agenda is approved

# 3	Approval of meeting report

R4-021398	Revised draft report meeting #24 (3GPP support)

No comments

Decision: The report is approved

# 4	Letters / reports from other groups

R4-021594	Report of TSG RAN/SA#17 (Vice chair)

The document summarizes the discussions in RAN and SA #17 of interest to WG4.

Torgny Palenius (Ericsson) clarified that the discussion on Layer 3 is not correctly reported, TSG RAN agreed that both solutions presented were acceptable in Rel99 and to review the decision for further Releases, hence no decision has been adopted for Rel-4.

Decision: The report is noted

R4-021399	Draft report TSG RAN #17 (3GPP support)

This report is presented for information. TSG RAN had requested all the WG to follow the actions below:

-	Concerning the CR file, it is requested that only the final revision is included, and not the intermediate revisions of revisions. This should be applied at all RAN WGs.

-	TSG RAN requested that particular care shall be taken in all Working Groups of  3GPP TSG RAN to the use of correct form for CR cover sheet and the way the different elements are filled. More particularly, for Release 99 the element provided for "Consequence if not approved" shall be very carefully written so as to provide a clear view at the plenary. Working Group chairs were requested to be particularly careful to the way the cover sheet is used and filled up before agreeing on CRs.

And WG4 in particular to take on the following:

-	to evaluate the different layer 3 filtering options and to conclude on which is the best solution and on which measurements it has to be used (sec. 7.2.2, doc RP-020667)

-	to ensure that there is a representative from a 3GPP member company attending COST and CTIA in order to report back to WG4 and RAN (sec. 8.7.7, doc. RP-020470).

Decision: The report is noted

R4-021427	LS on the completion of the FDD BS Classification Work Item 
(ETSI MSG, M-02-012)

ETSI MSG asks WG4 to agree the CRs on FDD BS Classification during this meeting and TSG RAN to produce Rel-6 versions of 25.104 and 25.141 in the December meeting, so that the FDD BS Classification can be included in the European Harmonised Standard (HS).

Ericsson noted that the CRs are ready and it is possible to finish most of the technical work in this meeting. Takaharu Nakamura (Fujitsu) questioned how CRs with square brackets could be used for a HS, and required clarification on which Release this LS refers to. It is clarified that it applies to Rel-6.

Han van Bussel (T-Mobile) questioned why the 3GPP procedure should be bypassed in this WI, and why the same rule shouldn't apply to the rest of the RAN4 WIs, like WDS or FDD UE Improvements. He suggested that ETSI MSG should produce the HS when the technical specification in 3GPP is ready, not force 3GPP to produce a specification when the technical work isn't finished. The chairman summarized that this discussion should take place in TSG RAN, WG4 should simply asses the technical aspect of the CRs presented.

Decision: The LS is noted

R4-021428	LS on the measurement uncertainties on the radiated emissions measurements  (ETSI MSG, M-02-016)

Ericsson comented that there are CRs to be presented covering this issue.

John Fenn (Samsung) noted that there is a special group in ETSI taking care of measurement uncertainties, and producing a standard that would be applicable to UMTS as well. He recommended not going ahead with the proposal from MSG, since it may happen that it is withdrawn within ETSI.

The chairman noted that, regardless of the discussions inside ETSI, nothing stops 3GPP to introduce the new uncertainties if they are considered correct.

Decision: The LS is noted

R4-021425	LS on the value of Maximum allowed UL TX power in case of correct behaviour at time-out test of Random Access (TSG T WG1 RF SWG, T1R020308)

T WG1/RF shows that the current test for "Correct behaviour at time-out during the random access" might result in fail for UEs in accordance with the specifications. T WG1/RF proposes to change the value of the "Maximum allowed UL TX power" in this test from 0 dBm to 33 dBm so that a UE measuring CPICH RSCP with a +6dB error (accuracy within the specified limits) doesn't reach the "Maximum allowed UL TX power" before transmitting the complete preamble cycle, a situation that will make the test fail. T WG1/RF asks WG4 to endorse this change.

The value of 33 dBm is questioned, DoCoMo agreed that a lower value can be used, there is no special concern about it. Nobuto Arai (DoCoMo) noted that the CRs for WG4 specifications are not presented yet, and asks the group to decide if a CR to 25.133 is required or a change to T1RF is sufficient. See R4-021651 in section 5.7 of this report.

Decision: The LS is noted

R4-021426	LS on the key concepts and parameters for BER/BLER Statistical Approach (UE conformance testing) (TSGT WG1 RF, T1R020310)

T WG1/RF informs RAN WG4 on the approach adopted for BER/BLER tests. No actions are required.

Moray Rumney (Agilent) presented this LS and asked the BS testing experts to look at the paper and discuss it together later.

Decision: The LS is noted

R4-021625	LS on application of Test Tolerances to RRM tests (TSG T WG1 RF SWG, T1R020352)

The LS highlights the impossibility to meet RAN WG4 requirements in the CELL_FACH reselection text when applying the test tolerances, and asks WG4 to study the issue and answer without delay.

Nadege Noisette (Orange) clarified that the main issue in T1RF is that when applying test tolerances with WG4 guidelines in the CELL_FACH cell-re-selection test case, the UE under test could not see any interfering cells because their CPICH_Ec/Io felt down to –29dB while the min reporting range is –24dB. This issue impacts the cell re-selection delay that is tested here. Orange had proposed at T1RF the proposition in table 2 which wasn't agreed. Finally it was suggested in T1/RF either to change the test requirement, and for instance considered that the level of the interfering cell also changes while the UE is moving, or not to apply any test tolerances.

Moray Rumney (Agilent) pointed that there actually two issues here. First, there is no common understanding of the requirements of 25.133 when writing the test, setting the test scenario and assuming perfect measurement equipment. And second, the tolerances should be discussed. Sari Korpela (Nokia) noted that the discussion on the test scenario shouldn't be reopened now, T1 has been working on this basis for a while. Moray apologized for raising the discussion again, but noted that the test scenario should try to emulate real network conditions. Sari noted that the objective of the test scenario is to test terminals, not networks.

It seems clear that further discussion is necessary, a response to T1 will be drafted in R4-021628

Decision: The LS is noted

R4-021606	LS on reference compressed mode pattern for signalling test case (TSG T1SIG SWG, T1S-020875)

This LS highlights a situation of overlapping transmission gap that disrupts the measurements. Mitsubishi commented it is possibly confusing to allow combinations of several gap patterns at this stage : TDD+FDD+GSM(3). No combinations have been discussed so far in 25.133 and that some inconsistency may be found with respect to the minimum gap density. It is noted that the proposal for T1SIG needs careful examination and it is recommended not to answer T1SIG right away.

Decision: The LS is noted

R4-021603	Response to LS on the definition of CQI (TSG RAN WG1, R1-02-1198)

This is a response to R4-021360. WG1 has studied and revised the definition of CQI as follows:

Based on an unrestricted observation interval, the UE shall report the highest tabulated CQI value for which a single HS-DSCH sub-frame formatted with the transport block size, number of HS-PDSCH codes and modulation corresponding to the reported or lower CQI value could be received in a 3-slot reference period ending 1 slot before the start of the first slot in which the reported CQI value is transmitted and for which the transport block error probability would not exceed 0.1.

Decision: The LS is noted, no response is required

R4-021630	LS response on uplink DPCCH transmission start timing in Hard Handover (TSG RAN WG1, R1-02-1188)

Panasonic will present a CR covering this issue.

Decision: The LS is noted

R4-021416	LS on TX diversity on radio links in the active set (TSG RAN WG1, R1-02-1159)

This LS is sent to WG2, cc WG4.

25.331 Rel5 provides signalling indicating the use or not of TX diversity per radio link basis. This wasn't available to the UE before, WG1 has corrected its specifications to require the UE to operate in TX diversity only in the links were its is used. WG1 proposes a small correction to 25.331 to fully align the specifications.

Edgar Fernandes (Motorola) noted that there are performance implications, WG4 needs to study the issue, even if the LS is only sent to WG4 as a copy.

Decision: The LS is noted

R4-021624	LS on TDD Reference Measurement Channels (TSG RAN WG1, R1-02-1423)

The LS asks WG 4 to update some figures and values in the reference channels used in 25.105. See R4-021700 in section 5.5 of this report for the CRs.

Decision: The LS is noted

R4-021417	LS on HS-PDSCH capability definition and QPSK-only UE categories (TSG RAN WG1, R1-02-1162)

This LS is sent to WG2 and cc WG4.

The definition of support of HS-PDSCH, and two recently approved QPSK UE categories, are missing in 25.306. WG1 asks WG2 to approved the CR correcting these issues.

No actions required from WG4

Decision: The LS is noted

R4-021418	LS on cell portion in beamforming (TSG RAN WG1, R1-02-1190)

This LS is sent to WG4 as response to the LS in R4-021378 asking for clarification on the definition of "cell portion". WG1 informs that a cell portion is a part of the cell covered by a specific beam, and it can have static and/or dynamic coverage.

Decision: The LS is noted

R4-021419	LS on beamforming enhancement WI (TSG RAN WG1, R1-021261)

This LS is sent to WG3 and WG4.

WG1 informs that it has finished the definition of the new (and modified) measurements to be used for RRM support when beamforming, and asks what is the status of the WI in WG3 and WG4.

R4-021605	TR 25.887 Beamforming enhancements (R1-021260) (3GPP Support)

This TR is referenced in the LS and it is presented as additional information.

Decision: The LS is noted

R4-021422	Response to LS on CPICH RSCP signalling range (R2-022212) (TSG RAN WG2, R2-022708)

This LS is sent to WG4 in response to the LS in R4-021339 that requested an extension of the reporting range of CPICH RSCP and and extension of the limit of Qrxlevmin from -115 dBm to -120 dBm. WG2 agrees to extend the range, but for backards compatibility reasons, the new values below -115 dBm will be as CPICH_RSCP_LEV_00 (-115 dBm) and Delta. The range for CCPCH RSCP is reviewed as well. WG2 asks WG4 to change the range tables in 25.123 and 25.133.

DoCoMo has produced the necessary CRs, see R4-021622 in section 5.5 of this report.

Decision: The LS is noted

R4-021420	LS on Reference configurations in TS 34.108 (TSG RAN WG2, R2-022204)

This LS is sent to T WG1, RAN WG4 and WG1.

WG2 has clarified 25.331 so the UE can make SIR target adjustments without having CRC in all transport formats in the TFCS. WG2 asks T WG1 to update the configuration of any of the RABs in a proposed list to a TFCS where all TF have CRC.

No actions required from WG4

Decision: The LS is noted

R4-021421	Response to LS (S4-020468) on Rate Adaptation of AMR Codec (TSG RAN WG2, R2-022423)

This LS is sent to SA WG4 cc RAN WG4.

The AMR rate selection behaviour is not specified in SA WG AMR specification, the issue is briefly covered the MAC specification and the requirements specification (25.133). WG2 believes that this issue should be handled in the AMR spec and not in the radio specs, and asks SA WG4 to reconsider the issue. WG2 ask also for clarification on the selection behaviour in case of rapidly degrading radio conditions, that would force a quick rate change of several steps.

R4-021423	Response LS on Rate Adaptation of AMR Codec (TSG SA WG4, S4-020604)

This LS is sent to RAN WG2 cc RAN WG4 in response to R2-022423

SA WG4 agrees to add some text to the AMR specification clarifying that the code modes are configured by the UTRAN and the UE shall select a mode compatible with the radio conditions according to 25.133.

Concerning the issue of a rapid change in radio conditions, SA WG4 requires that the codec mode is changed step by step to avoid disruptions in the decoding.

NEC questioned whether it is possible for the UE to do the step by step AMR mode change with the current UE requirements in 25.133. Nokia agreed that this is an issue to look over, but it is not particularly urgent since it affects Rel5. NEC suggests to send a LS to SA4 to note that RAN WG4 needs to study the issue, although Nokia recommended to look at it first.

Decision: R4-021421 and R4-021423 are noted

R4-021686	Response to LS on Rate Adaptation of AMR Codec (TSG RAN WG2, R2-023158)

This LS is sent to SA WG4, cc RAN WG4. This LS was received during the meeting.

No actions required from WG4

Decision: The LS is noted

R4-021691	LS(#2) on Rate Adaptation of AMR Codec (Response to R2-023158) (TSG SA WG4, S4-020720)

This LS is sent to RAN WG2, cc WG4. This LS was received during the meeting.

SA WG4 requires WG4 to revise/modify the requirement in 25.133 for Tadapt (the time it takes for higher layers to provide data to MAC in a new supported bitrate) to cope with the step by step change of AMR mode. This relates to the comment made by NEC before (R4-021423).

Sari Korpela (Nokia) noted that the solution proposed is not viable, a bigger change to the RRM procedures is required, affecting other specifications than 25.133. It is agreed to produce a response LS and send it as soon as possible so it can be seen by SA4 in its meeting this week (R4-021716).

Decision: The LS is noted

R4-021593	LS on coexistence Between IMT-2000 TDD and FDD Radio Interface Technologies Within the Frequency Range 2 500-2 690 MHz Operating in Adjacent Bands and in the Same Geographical Area (ITU-R Ad Hoc)

Nokia noted that sec.5 of the draft report on Mitigation is referring 3GPP specifications 25.104 25.105, it seems that ITU would require 3GPP to look after these parts in particular. Nokia notes that there are a few wrong statements and assumptions, so a response is required. However, the deadline from ITU allows WG4 to take more time to examine the issue, the response doesn't have to be produced during this meeting.

It is clarified that the answer has to go through TSG RAN.

Claudio Guerrini (Tilab/TIM) explained that he will send to the email reflector a proposal for a deadline for contributions to ITU-R.

Decision: The LS is noted

R4-021649	LS on Uplink reference measurement channels (TSG T WG1, T1-020891)

This LS is sent to RAN WG2 cc RAN WG4.

WG4 is asked to review the documents attached and answer to T1 if any concern is detected.

Decision: The LS is noted

The following table summarizes the LSs covered during the meeting:

| Tdoc | Title | Source | Source File | Decision | Response |
| --- | --- | --- | --- | --- | --- |
| R4-021416 | LS on TX diversity on radio links in the active set | TSG RAN WG1 | R1-02-1159 | Noted |  |
| R4-021417 | LS on HS-PDSCH capability definition and QPSK-only UE categories | TSG RAN WG1 | R1-02-1162 | Noted |  |
| R4-021418 | LS on cell portion in beamforming | TSG RAN WG1 | R1-02-1190 | Noted |  |
| R4-021419 | LS on beamforming enhancement WI | TSG RAN WG1 | R1-021261 | Noted |  |
| R4-021420 | LS on Reference configurations in TS 34.108 | TSG RAN WG2 | R2-022204 | Noted |  |
| R4-021422 | Response to LS on CPICH RSCP signalling range (R2-022212) | TSG RAN WG2 | R2-022708 | Noted |  |
| R4-021421 | Response to LS (S4-020468) on Rate Adaptation of AMR Codec | TSG RAN WG2 | R2-022423 | Noted | R4-021738 |
| R4-021423 | Response LS on Rate Adaptation of AMR Codec | TSG SA WG4 | S4-020604 | Noted |  |
| R4-021686 | Response to LS on Rate Adaptation of AMR Codec | TSG RAN WG2 | R2-023158 | Noted |  |
| R4-021691 | LS(#2) on Rate Adaptation of AMR Codec (Response to R2-023158) | TSG SA WG4 | S4-020720 | Noted |  |
| R4-021425 | LS on the value of Maximum allowed UL TX power in case of correct behaviour at time-out test of Random Access | TSG T WG1 RF SWG | T1R020308 | Noted |  |
| R4-021426 | LS on the key concepts and parameters for BER/BLER Statistical Approach (UE conformance testing) | TSGT WG1 RF | T1R020310 | Noted |  |
| R4-021427 | LS on the completion of the FDD BS Classification Work Item | ETSI MSG | M-02-012 | Noted |  |
| R4-021428 | LS on the measurement uncertainties on the radiated emissions measurements | ETSI MSG | M-02-016 | Noted |  |
| R4-021593 | LS on coexistence Between IMT-2000 TDD and FDD Radio Interface Technologies Within the Frequency Range 2 500-2 690 MHz Operating in Adjacent Bands and in the Same Geographical Area | ITU-R Ad Hoc |  | Noted |  |
| R4-021603 | Response to LS on the definition of CQI | TSG RAN WG1 | R1-02-1198 | Noted |  |
| R4-021606 | LS on reference compressed mode pattern for signalling test case | TSG T1SIG SWG | T1S-020875 | Noted |  |
| R4-021624 | LS on TDD Reference Measurement Channels | TSG RAN WG1 | R1-02-1423 | Noted |  |
| R4-021625 | LS on application of Test Tolerances to RRM tests | TSG T WG1 RF SWG | T1R020352 | Noted | R4-021739 R4-021740 |
| R4-021630 | LS response on uplink DPCCH transmission start timing in Hard Handover | TSG RAN WG1 | R1-02-1188 | Noted |  |
| R4-021649 | LS on Uplink reference measurement channels | TSG T WG1 | T1-020891 | Noted |  |

# 5	Maintenance of Release 99, Release 4 and Release 5 specifications

## 5.1	25.941 - Document Structure

No contributions

## 5.2	25.101 - UE Radio transmission and reception (FDD)

R4-021478	Discussion for Correction for TPC command combining test case 1 (Nokia)

Test in Section 8.7.2, ‘Combining of TPC commands from different radio links of different radio link sets’ was agreed based on an ideal case. In fact, due to additional noise coming from neighbour BS, the probability of detecting the sequence correctly is always less than 1. Nokia proposes to correct the minimum requirement to a 99% of the times instead of always.

Edgar Fernandes (Motorola) objected the implementation of the new requirement. It may imply that the test has to be run 100 times, or many times 100 times to get statistical information. Agilent noted that this is a clear case for WG4 to understand the difficulties of testing. Josef Blanz (Qualcomm) clarified that two issues need to be separated: The minimum requirement in 25.101 and the confidence level in the tests. After in house analysis Motorola found no further concern and agreed the CRs.

R4-021474	Correction for TPC combining test case 1 (CR 194 to 25.101 R99) (Nokia)

R4-021475	Correction for TPC combining test case 1 (CR 196 to 25.101 Rel-4) (Nokia)

R4-021476	Correction for TPC combining test case 1 (CR 195 to 25.101 Rel-5) (Nokia)

Decision: The CRs are agreed

R4-021469	Simulation results for minimum requirements of UE Phase Shift (Ericsson)

R4-021470	UE Phase Shift requirements (CR 193 to 25.101 Rel-5) (Ericsson)

Josef Blanz (Qualcomm) commented that there is no reference to channel conditions, so Josef questioned if it should be assumed that the requirements apply to all conditions. The rate allowed for 60 degrees phase shift seems too high, compared to the results presented.

Torgny Palenius (Ericsson) agreed to reduce the rate for the intermediate condition (30 to 60 Deg. discontinuity), and clarified that the requirements do apply to all channel conditions.

It is objected that the requirement shouldn't apply to compress mode cases, the gaps can be too long for the UE to meet the requirement with the previous and following timeslots.

Josef Blanz commented that the requirement should clearly specify the conditions where it has to be met, regarding channel and even the particular power control pattern. Otherwise it is always possible to find a combination of parameters and conditions that would make any UE produce a very high rate of phase change. Moray Rumnay (Agilent) agreed with this view, but warned that it would lead to requirements covering only some special cases. It seems that it is the way IS95 specifies this requirement.

Decision: The CR is revised

R4-021721	UE Phase Shift requirements (CR 193r1 to 25.101 Rel-5) (Ericsson)

Companies had requested more time to analyse the requirements, and since this is a Rel-5 topic, it was agreed to postpone the discussion to the next meeting. Moray Rumney (Agilent) questioned if it would be possible to reach an agreement on this issue for the next meeting, since it has been going for a time already. Josef Blanz (Qualcomm) noted that the measurement principles proposed by Agilent in its first papers are broadly agreed by the group, only the values are to be discussed.

Decision: The CR is not agreed

R4-021621	Interpretation of UE radio access parameter "Need for compressed Mode" when FALSE (Mitsubishi Electric Telecom Europe)

Josef Blanz(Qualcomm) agreed with proposal #2, although he could devise a situation when an operator sets an RRM policy in its network where universal compress mode is requested for all UE regardless of its capabilities. Josef suggested to draft a LS to WG2 asking for a decision, but not proposing a particular change to WG2 specification, that is up to WG2 to decide. He also noted that there is no agreement on WG4 on the interpretation of the parameter. Since the paper is also presented in WG2, the need for the LS is uncertain. After a long discussion, it was agreed not to send a LS.

As a conclusion, WG4 had two pending questions where a solution is expected in WG2:

-	Is the UE allowed to reject the "Need of compress Mode" configuration if it has declared that it doesn't support it (as stated in its UE capabilities)?

-	Shall the UE accept the "Need of compress Mode" configuration but reject its activation, if it has declared that it doesn't support it, when the network requests it to?

Decision: The document is noted

### 5.2.1	FDD UE HSDPA

R4-021538	TR25.890v1.2.0 (Motorola)

Jorma Kaikkonen (Nokia) rejected the addition of the MCC interpretation of DTX in section 6 as it is related to available HS-SCCH power issue in tests. Motorola agreed to remove that clarification. With the removal of that text, the TR is approved.

Decision: The report is noted

R4-021537	Correction to Specified TBS for HSDPA Reference Channels (CR 198 to 25.101 Rel-5) (Motorola)

Decision: The CR is agreed

R4-021429	Maximum received power at UE (CR 192 to 25.101 Rel-5) (Ericsson)

Motorola and Nokia agreed with the need for this requirement, but raised a number of comments. After off line discussions, the document is postponed to the next meeting.

Decision: The CR is rejected

R4-021607	UE maximum output power with HS-DPCCH (Nokia)

R4-021608	UE maximum output power with HS-DPCCH (CR 199 to 25.101 Rel-5) (Nokia)

Peak to Average Ratio is increased when HS-DPCCH is used due to the increase of number of codes used in the UE. This increase in PAR impacts the battery duration, the form factor and the cost of UE. Nokia proposes to reduce the maximum output power for HSDPA UEs, and the proposal is to increase the tolerances for the output power.

Josef Blanz (Qualcomm) objected that a PAR increase of 2.5 dB, as shown in the simulation, doesn't necessary map to a 2.5 dB in the maximum output power. Josef warned about changing anything related to max. output power levels, since that would put the 3GPP document in contradiction with current European regulation (ETSI Harmonised Standard).

Edgar Fernandes (Motorola) commented that this simulation had been performed before in WG1 but with very different results. Concerning the CR, Edgar questioned why not change the power level instead of the tolerances.

Han van Bussel (T-Mobile) asked for further clarification on how the relaxed maximum output power impacts the HSDPA service, and how this impact depends on deployment.

The chairman summarized that more simulation results are required, also the system level impact should be further explained. The CR cannot be approved in this meeting.

Decision: The CR is not agreed

R4-021639	HS-SCCH Performance  Revised Results (Motorola)

R4-021633	FRC with HS-SCCH impairments (Sony)

Decision: The documents are noted

R4-021673	FRC results with TBS adjustment (Motorola)

Decision: The document is noted

R4-021674	Results for HSDPA Fixed Reference Channels in Open Loop Transmit Diversity Mode (updated) (Motorola)

R4-021617	Simulation results for Open Loop and Closed Loop Mode 1 Transmit Diversity in HSDPA (Nokia)

Edgar Fernandes (Motorola) commented that the Open Loop and Closed Loop Mode 1 should be considered separately in the TR, he also questioned if the antenna verification is mandatory or not. Edgar objected that the simulations don't go further than PA3 propagation model.

Nokia clarified that antenna verification is not mandatory and it was presented only for simulation alignment purposes. It was agreed to remove it from the ideal assumptions.

Decision: The contribution is approved. The text will be added to TR25.890

R4-021634	Simulation results for FRC with STTD (Sony)

Decision: The document is noted

R4-021708	TR25.890v1.2.1 (Motorola)

The report is updated with the corrections proposed.

Decision: The report is approved

#### 5.2.1.1	QPSK only UEs

R4-021626	Fixed Reference Channels for FDD UE Categories 11 & 12 (Motorola)

R4-021627	Discussion of the requirements for QPSK only HS-DSCH categories (Nokia)

R4-021642	Relative performance of QPSK FRC proposals (Motorola)

R4-021620	Introduction of requirements for HSDPA UE categories 11 and 12 (CR 200 to 25.101 Rel-5) (Nokia)

Motorola and Nokia agreed to work off line to arrive to a common solution and a CR for the QPSK only requirements.

Decision: The documents are noted

R4-021709	Introduction of requirements for HSDPA UE categories 11 and 12 and open loop transmit diversity test (CR 200r1 to 25.101 Rel-5) (Nokia)

Qualcomm proposed to have a hybrid solution taking into account the advantages of the proposal from Motorola. Motorola raised some concerns but after off line discussions the CR was agreed

Decision: The CR is agreed

#### 5.2.1.2	VRC Simulation Results

The following simulation results for HSDPA Variable Reference Channel were presented:

| Tdoc | Title | Source |
| --- | --- | --- |
| R4-021453 | HSDPA: Variable Reference Channel | Ericsson |
| R4-021535 | Further Results for HSDPA Variable Reference Channels | Motorola |
| R4-021532 | VRC Simulation Results | Qualcomm |
| R4-021578 | Simulation Results of Variable Reference Channel | Panasonic |
| R4-021591 | Simulation results for HSDPA variable reference channels | Sony |
| R4-021619 | HSDPA Variable Reference Channel Simulation Results | Nokia |
| R4-021629 | HSDPA VRC simulation results | NEC |
| R4-021650 | Simulation result for HSDPA VRC | DoCoMo |
| R4-021412 | Simulation Results for HSDPA Variable Reference Channel | Lucent |

A document compiling and comparing this results will be produced.

R4-021693	Comparison of VRC results (Motorola et al.)

Decision: The document is noted

#### 5.2.1.3	CQI definition and test methodology for VRC

R4-021533	VRC Test Approach (Qualcomm)

The document shows that the current CQI reporting test for VRC doesn't meet the requirements of the CQI definition, recently modified by WG1. The CQI definition requires that the value reported corresponds to the channel conditions in the last 3 slots before the it is reported, and it seems that the current VRC test allows the UE to average and not report the value of that correct instant. Qualcomm proposes an alternate approach that would ensure that the UE reports correctly, since with the previous approach it could happen that a faulty UE behaves better in the test than a correct UE. One of the controversial points of the new approach is to use a fixed transport format instead of the NodeB actually varying the transport format based on the reported CQI.

Motorola argued against changing the test after 3 months of work in the VRC area, and noted that in many test cases already in place faulty UE could better pass the test than correct UE, but perform badly in the field; it is assumed that UEs are manufactured with the intention to work in real conditions and not only for passing tests.

R4-021596	VRC test methodology (Sony Corporation)

R4-021592	Test methodology for CQI (Sony )

These papers also shows that the current VRC test defined in 25.890 are not sufficient to distinguish UE reporting CQI incorrectly.

Two alternative test procedures are proposed:

-	Tests under static channel

-	Tests with fixed DL transport channel

Edgar Fernandes (Motorola) reminded that the HSDPA Work Item is due for conclusion in December, and accepting this new test approach will require to redo all the simulations and start the work like 3 months ago. He proposed to continue with the current test set, and consider the new proposal as an enhancement for Release 6.

Moray Rumney commented that from a testing point of view, it doesn't seem the best approach to base a CQI test in throughput, so it looks like the current test is mixing up two issues, throughput and CQI.

It was finally agreed to continue with the current tests, and evaluate a new methodology as a further improvement.

R4-021692	CQI working definition (Motorola, Sony)

Some comments were raised on the concept of median CQI values, Edgar Fernandes (Motorola) clarified that this is a proposal is intended to start the discussion for the VRC simulations, Edgar suggested to set up a small email group to progress on the work and to hold a physical ad hoc meeting around the end of January to speed up the work before WG4 meeting in February.

Decision: The document is noted

## 5.3	25.102 - UE Radio transmission and reception (TDD)

R4-021559	Introduction of Rel-5 clarifications and small corrections in Rel-4 (CR 129 to 25.102 Rel-4) (Siemens)

Decision: The CR is agreed

R4-021576	Name correction of logical and transport channels (CR 130 to 25.102 Rel-4) (Siemens)

R4-021577	Name correction of logical and transport channels (CR 131 to 25.102 Rel-5) (Siemens)

Decision: The CRs are agreed

### 5.3.1	TDD HSDPA

#### 5.3.1.1	Fixed Reference Channel

R4-021539	HSDPA simulation assumptions for 3,84 Mcps TDD option with fixed reference channels (InterDigital)

Frederic Charpentier (Siemens) pointed out that the IR combining is not considered in the simulation. If it is not included, this UE technique will not be tested. The chairman suggest to provide different simulations comparing Chase and IR combining to see if there is any difference in terms of performance. Joseph Levy (Interdigital) noted that preliminary analysis so far didn't show a substantial difference.

Decision: The assumptions are agreed

R4-021540	HSDPA simulation results for 3,84 Mcps TDD, Fixed Reference Channel (InterDigital)

R4-021541	Addition of HSDPA UE requirements for 3,84 Mcps TDD option for 16QAM and QPSK for fixed reference channels (CR 127 to 25.102 Rel-5) (InterDigital)

Frederic Charpentier (Siemens) presented some objections to the simulation results in R4-021540. Decision: The CR is revised

R4-021665	HSDPA simulation results for 3,84 Mcps TDD, Fixed Reference Channel (InterDigital)

R4-021666	Addition of HSDPA UE requirements for 3,84 Mcps TDD option for 16QAM and QPSK for fixed reference channels (CR 127r1 to 25.102 Rel-5) (InterDigital)

These simulation results and attached CR cover the concerns raised by Siemens

Decision: The CR is agreed

R4-021402	Simulation results for QPSK and 16QAM for 1,28 Mcps TDD option (Motorola)

These results are not included in R4-021601/R4-021602, which summarize the results from different companies (Samsung, Siemens, Panasonic). The results from Motorola are however in line with previous papers. Samsung will produce a revision of the comparison paper (R4-021601) and the CR (R4-021602) taking into account the results from Motorola.

R4-021600	HSDPA simulation results for 1.28 Mcps TDD option (Samsung)

R4-021604	Updated HSDPA simulation results for 1.28Mcps TDD option (Panasonic)

Decision :The simulations are noted

R4-021637	Comparison of HSDPA simulation results for 1.28 Mcps TDD option (Samsung, Siemens, Panasonic, Motorola)

R4-021638	HSDPA UE requirements for 1.28 Mcps TDD option for 16QAM and QPSK for fixed reference channels (CR 132r1 to 25.102 Rel-5) (Samsung, Siemens, Panasonic, Motorola)

These documents are the revisions of  R4-021601 and R4-021602.

Decision: The CR is agreed.

#### 5.3.1.2	Variable channel

R4-021563	HSDPA - Variable channel simulation assumptions (Siemens)

These assumptions are intended for 1.28 Mcps. Joseph Levy objected the assumption that the NodeB simulator should transmit with the last transport block size and the modulation scheme when it receives CQI=0. Frederic clarified that the situation is different for HCR and LCR, in any case Panasonic, Samsung and Siemens had reviewed off line the assumptions and agreed on the current paper.

Decision: R4-021563 is approved

R4-021542	HSDPA simulation assumptions for 3,84 Mcps TDD option with variable reference channels (InterDigital)

Frederic Charpentier (Siemens) objected that some these assumptions are implementation dependant. He commented that the CQI table is different for LCR and HCR, Joseph objected that this is not a problem since they are different systems.

Decision: The document is noted

R4-021667	HSDPA simulation assumptions for 3,84 Mcps TDD option with variable reference channels (InterDigital)

This document contains the assumptions agreed by Interdigital and Siemens.

Decision: The document is approved

R4-021543	Simulation Results for 3.84 Mcps TDD HSDPA with Variable Reference Channel (InterDigital)

R4-021544	Addition of HSDPA UE requirements for 3,84 Mcps TDD option for 16QAM and QPSK for variable reference channels (CR 128 to 25.102 Rel-5) (InterDigital)

There were some objections about the simulation assumptions in R4-021542, these two documents cannot be approved.

Decision: The documents are not approved

R4-021668	Simulation Results for 3.84 Mcps TDD HSDPA with Variable Reference Channel (InterDigital)

No comments

Decision: The results are noted

R4-021669	Addition of HSDPA UE requirements for 3,84 Mcps TDD option for 16QAM and QPSK for variable reference channels (CR 128r1 to 25.102 Rel-5) (InterDigital)

No comments

Decision: The CR is agreed

## 5.4	25.104 - BTS Radio transmission and reception (FDD)

R4-021447	FDD  GSM co-existence in the Same Geographic Area (CR 149 to 25.104 Rel-4) (Ericsson)

Ericsson clarified that no new co-existence simulation were done to derive these values, but existing co-existence and MCL values. Eric questioned the need for a new requirement in the UMTS spec since there is no rationale supporting it in the GSM specification.

Han van Bussel (T-Mobile) suggested to introduce this requirement in Rel99, if there is a real need for it, it doesn't make sense to delay to Rel4 since the networks are being deployed already based on Rel99 specifications. This was agreed, the CR cover will therefore be revised.

Decision: The CR is revised

R4-021658	FDD  GSM co-existence in the Same Geographic Area (CR 149r1 to 25.104 Rel-4) (Ericsson)

R4-021659	FDD  GSM co-existence in the Same Geographic Area (CR 167 to 25.104 R99) (Ericsson)

R4-021448	FDD  GSM co-existence in the Same Geographic Area (CR 153 to 25.104 Rel-5) (Ericsson)

It is agreed to send a small LS to GERAN informing of the work done in the area (R4-021720).

Decision: The CRs are agreed

R4-021449	FDD  GSM 850 / PCS 1900 co-existence in the Same Geographic Area (CR 150 to 25.104 Rel-5) (Ericsson)

Decision: The CR is agreed

R4-021551	Correction to table of regional requirements (CR 161 to 25.104 Rel-4) (Siemens)

R4-021552	Correction to table of regional requirements (CR 162 to 25.104 Rel-5) (Siemens)

Decision: The CRs are agreed

R4-021553	General Release 5 corrections (CR 163 to 25.104 Rel-5) (Siemens)

Decision: The CR is agreed

R4-021612	Proposal to correct Transmit Diversity in RAN4 specifications (Nortel Networks)

Eric Georgeaux (Nortel) clarified that there are only two antenna signals defined in WG1 specifications. If there are more antenna ports, it means that several ports are connected to the same antenna output and share the same signal.

The proposal is approved, it is agreed to introduce the correction in Rel-4 and Rel-5.

Decision: The document is agreed.

R4-021654	Clarification of TX diversity requirements (CR 166 to 25.104 Rel-5) (Nortel)

No comments.

Decision: The CR is agreed

R4-021505	Clarification of the W-CDMA interferer definition in BS requirements for ACS and blocking characteristics (CR 157 to 25.104 Rel-4) (Siemens)

Cliff Bailey (Spirent) required some time to examine it detail the implications of the change, it seems to affect the PAR ratio of the interfering signal. A revision was agreed off line.

Decision: The CR is revised

R4-021736	Clarification of the WCDMA interferer (Spirent)

This document is a proposal to agree upon before drafting the CRs. Cliff Baley (Spirent) suggested companies to study the paper and submit comments, the CRs will be made available on Monday 19th and discussion will take place until the Friday.

CRs are in documents R4-021744 to R4-021747

Decision: The document is noted

R4-021744	Clarification of the W-CDMA interferer definition in BS requirements for ACS and blocking characteristics (CR 157r2 to 25.104 Rel-4) (Spirent)

R4-021745	Clarification of the W-CDMA interferer definition in BS requirements for ACS and blocking characteristics (CR 158r2 to 25.104 Rel-5) (Spirent)

Decision: Email approval. See section 14.

### 5.4.1	BS IPDL attenuation requirement

R4-021546	IPDL attenuation simulation results (Siemens)

Michael Jeck (Ericsson) asked whether UE performance had been taken into account, since previous simulations showed that variations in the UE measurement capability have a big impact in the positioning accuracy. Frederic Charpentier (Siemens) clarified that only one value is used, CPICH_Ec/Io = -20 dB. Nokia commented that from the UE point of view there are some limitations in real radio environments on what we can do and on what we already are on the limit. Hence, the performance of the UE can’t always be improved but BS attenuation yes. If we can’t accept that, we should accept coverage limitations.

R4-021613	BS IPDL requirement (Nortel Networks)

Tuomo Säynäjäkangas (Nokia) objected that the area of the cell for which the IPDL requirement should be set is precisely in the vicinity of the BS. Eric Georgeaux (Nortel) noted that it has been traditionally assumed in WG4, for ACLR and UE ACS for example, that the UE is not close to the BS.

It seems that no agreement can be reached. So far, companies have expressed the following preferences:

| Nokia | - 45 dB |
| --- | --- |
| Ericsson | - 25 dB |
| Nortel | - 25 dB |
| Siemens | - 40 dB |

Given the bottleneck situation, after a year of discussion on this topic the interested companies haven't been able to reach consensus, the chairman decided to go for a vote. Nokia commented that they were happy with the Siemens compromise proposal of –40 dB. The question to be answered would be which of two values, -25 dB or -40 dB, should be used for the IPDL. Document R4-021640 contains the vote proposal by the chairman

R4-021640	IPDL vote document (Chairman)

After off line discussions the interested parties agreed a common proposal so the vote is finally cancelled. The following CRs are the results of this agreement, which proposes a value of -35 dB

Decision: The vote is cancelled

R4-021687	BS IPDL requirement (CR 159r1 to 25.104 Rel-4) (Siemens)

R4-021688	BS IPDL requirement (CR 160r1 to 25.104 Rel-5) (Siemens)

Decision: The CRs are agreed

## 5.5	25.105 - BTS Radio transmission and reception (TDD)

R4-021574	Name correction of logical and transport channels (CR 129 to 25.105 Rel-4) (Siemens)

R4-021575	Name correction of logical and transport channels (CR 130 to 25.105 Rel-5) (Siemens)

No comments

Decision: The CRs are agreed

R4-021560	Introduction of Rel-5 clarifications and small corrections in Rel-4 (CR 128 to 25.105 Rel-4) (Siemens)

The chairman noted that the figure introduced in section 6.1 is misleading, it seems that there are separate TX and RX filters, which is not the case in TDD. There is also different understanding in FDD and TDD on this area which need to be solved in the future, and new CRs may be required.

Decision: The CR is revised

R4-021645	Introduction of Rel-5 clarifications and small corrections in Rel-4 (CR 128r1 to 25.105 Rel-4) (Siemens)

Ericsson agreed with the CR, but recalled that there has to be further work to align the figures in FDD and TDD concerning the filters and the duplexers.

Decision: The CR is agreed

R4-021597	Spurious emission requirements for unsynchronized TDD operation (CR 131 to 25.105 R99) (Siemens)

R4-021598	Spurious emission requirements for unsynchronized TDD operation (CR 132 to 25.105 Rel-4) (Siemens)

R4-021599	Spurious emission requirements for unsynchronized TDD operation (CR 133 to 25.105 Rel-5) (Siemens)

Ericsson asked to cover also the case of a TDD BS interfering a TDD UE in an adjacent band. Joerg Gustrau (Siemens) suggested to approve the document as is and introduce the new case in the future.

Joerg clarified that the Rel4 and Rel5 are Cat F CRs, not Cat A, because of the additions for LCR and BS classification

Decision: The CRs are agreed

R4-021647	Correction of adjacent channel leakage power definition (CR 134 to 25.105 Rel-5) (Siemens)

No comments

Decision: The CR is agreed

R4-021700	Corrections to 3.84 Mcps TDD reference measurement channels (CR 135 to 25.105 R99) (IPWireless)

R4-021703	Corrections to 1.28 Mcps TDD reference measurement channels (CR 138 to 25.105 Rel-4) (IPWireless)

These CRs change 25.105 in answer to the LS from RAN WG1 (R4-021624)

It is noted that the cover sheet doesn't give enough rationale for introducing a Rel99 change, the "consequence if not approved" should go along the lines "the BS cannot be properly tested...". Also, the isolated impact analysis is missing.

Decision: The CRs are revised

R4-021722	Corrections to 3.84 Mcps TDD reference measurement channels (CR 135r1 to 25.105 R99) (IPWireless)

R4-021701	Corrections to 3.84 Mcps TDD reference measurement channels (CR 136 to 25.105 Rel-4) (IPWireless)

R4-021702	Corrections to 3.84 Mcps TDD reference measurement channels (CR 137 to 25.105 Rel-5) (IPWireless)

Decision: The CRs are agreed

R4-021723	Corrections to 1.28  Mcps TDD reference measurement channels (CR 138r1 to 25.105 Rel-4) (IPWireless)

R4-021704	Corrections to 1.28  Mcps TDD reference measurement channels (CR 139 to 25.105 Rel-5) (IPWireless)

Decision: The CRs are agreed

## 5.6	25.123 - Support of RF parameters in Radio Resource Management (TDD)

R4-021567	Handover Test Case Correction for 1.28Mcps TDD (CR 279 to 25.123 Rel-4) (Siemens)

R4-021568	Handover Test Case Correction for 1.28Mcps TDD (CR 280 to 25.123 Rel-5) (Siemens)

No comments

Decision: The CRs are agreed

R4-021569	Maximum allowed UL TX Power Correction for 1.28Mcps TDD (CR 281 to 25.123 Rel-4) (Siemens)

R4-021570	Maximum allowed UL TX Power Correction for 1.28Mcps TDD (CR 282 to 25.123 Rel-5) (Siemens)

No comments

Decision: The CRs are agreed

R4-021571	Corrections to Idle Mode Requirements and Test Cases for 1.28Mcps TDD (CR 283 to 25.123 Rel-4) (Siemens)

R4-021572	Corrections to Idle Mode Requirements and Test Cases for 1.28Mcps TDD (CR 284 to 25.123 Rel-5) (Siemens)

No comments

Decision: The CRs are agreed

R4-021573	P-CCPCH RSCP and CPICH RSCP signalling range extension (CR 285 to 25.123 Rel-5) (Siemens)

No comments

Decision: The CR is agreed

## 5.7	25.133 - Support of RF parameters in Radio Resource Management (FDD)

R4-021622	CPICH RSCP report mapping (CR 502 to 25.133 Rel-5) (NTT DoCoMo)

It is commented that the change in report mapping could have impact on interoperability, since would be terminals with a different range already available. It seems that RAN WG2 has studied the issue, and as shown in the LS in R4-021422, it was solved using a combination of the old range and a delta for the new range. It is recommended to add this clarification to the CR cover page.

Decision: The CR is revised

R4-021441	Correction of CPICH RSCP measurement reporting range (CR 475 to 25.133 Rel-5) (Ericsson)

The change would be included in the revision of the previous CR

Decision: The CR is rejected

R4-021663	CPICH RSCP report mapping (CR 502r1 to 25.133 Rel-5) (NTT DoCoMo)

No comments

Decision: The CR is agreed

R4-021442	Performance requirements for positioning in CELL_PCH and URA_PCH (CR 487 to 25.133 Rel-5) (Ericsson)

Sari Korpela (Nokia) thanked Ericsson for this paper on this long standing issue, but cannot accept the different proposed requirements for CELL_PCH and URA_PCH without any clear concept analysis. David Chouckroun raised a comment on different requirements for CELL_PCH, URA_PCH and CELL_DCH, CELL_FACH, and added that it might not be feasible to have different requirements for different Releases in the accuracy of this measurement. Torgny Palenius (Ericsson) clarified that there is a difference in performance in the different states, due to DRX cycle for example, and this has to be reflected in the requirement. Given the concerns raised, the chairman suggested that supporting papers to solve the issue should be presented in the future.

Decision: The CR is not agreed

R4-021458	Correction of Measurement Occasion Patterns for BSIC Reconfirmation (CR 478 to 25.133 R99) (Ericsson)

R4-021502	Correction of Verification times in CELL_FACH state for BSIC reconfirmation (CR 499 to 25.133 R99) (Mitsubishi)

The values proposed are different, Torgny Palenius (Ericsson) proposes to revise the simulation assumptions, which should be at the origin of the difference.

Decision: the CRs are revised

R4-021705	Correction of Measurement Occasion Patterns for BSIC Reconfirmation (CR 478r1 to 25.133 R99) (Ericsson, Mitsubishi)

R4-021706	Correction of Measurement Occasion Patterns for BSIC Reconfirmation (CR 489r1 to 25.133 Rel-4) (Ericsson, Mitsubishi)

R4-021707	Correction of Measurement Occasion Patterns for BSIC Reconfirmation (CR 479r1 to 25.133 Rel-5) (Ericsson, Mitsubishi)

Some companies requested clarification on how the final numbers had been chosen, from Ericsson's paper, Mitsubishi's paper or from new simulations? Mitsubishi clarified that the initial conditions of both simulators were not aligned, Mitsubishi was simulating a complete verification including initial BSIC identification whereas Ericsson concentrated on BSIC re-confirmation only. Mitsubishi and Ericsson finally concluded to go for Ericsson figures

Decision: The CR are agreed

R4-021471	UE Timer accuracy (CR 482 to 25.133 R99) (Ericsson)

R4-021472	UE Timer accuracy (CR 491 to 25.133 Rel-4) (Ericsson)

R4-021473	UE Timer accuracy (CR 483 to 25.133 Rel-5) (Ericsson)

Motorola notes that the timer requirements are already there for the RRM tests. The chairman noted that the test as defined is not testable, Torgny Palenius (Ericsson) agreed, but the intention is not to derive a test. Sari Korpela (Nokia) also noted that the accuracy value for time>4 secs doesn't make sense for some of the timers. T1 WG SIG has already a test for these timers and it would make sense to also have a core requirement in the specification but it is unclear whether the requirement should be added to a RAN4 or RAN2 specification. The chairman commented that there is a need for a general requirement for the accuracy of RRM timers, but warned about running into misalignment with WG2. Torgny clarified the new requirement shouldn't impact WG2 work, it has been produced by a WG2 expert.

Decision: The CRs are revised

R4-021713	UE Timer accuracy (CR 482r1 to 25.133 R99) (Ericsson)

R4-021714	UE Timer accuracy (CR 491r1 to 25.133 Rel-4) (Ericsson)

R4-021715	UE Timer accuracy (CR 483r1 to 25.133 Rel-5) (Ericsson)

No comments

Decision: The CRs are agreed

R4-021403	Correction of interruption time in FDD/FDD Hard Handover (CR 437r1 to 25.133 R99) (Panasonic)

R4-021404	Correction of interruption time in FDD/FDD Hard Handover (CR 438r1 to 25.133 Rel-4) (Panasonic)

R4-021405	Correction of interruption time in FDD/FDD Hard Handover (CR 439r1 to 25.133 Rel-5) (Panasonic)

No comments

Decision: The CRs are agreed

R4-021500	Total received power density definition for the BS (CR 498 to 25.133 Rel-5) (Nokia)

Moray Rumnay (Agilent) questioned what is the decision point for measuring this parameter.

Yannick Le Pezennec (Vodafone) noted that the corresponding CR in WG1 is for Rel-4.

Nokia and Agilent clarified that Io and Iob are to be interpreted as measured through an RRC filter at the chip decision points, although it was considered not essential to add the clarification to the specification. Nokia commented that the reference point is already approved in WG1. Measurement is defined within the bandwidth by the receiver pulse shaping filter but the reference point for the measurement is the Rx antenna connector.

Decision: The CR will be revised and a Rel-4 CR will be produced

R4-021644	Total received power density definition for the BS (CR 503 to 25.133 Rel-4) (Nokia)

R4-021643	Total received power density definition for the BS (CR 498r1 to 25.133 Rel-5) (Nokia)

Decision: The CRs are agreed

R4-021486	Correction of measurement and reporting capability requirements in CELL_DCH state in case of parallel measurements (CR 494 to 25.133 R99) (Ericsson)

Nokia and Motorola objected the deletion of the sentence in the second paragraph in section 8.3.2. It is less clear with the correction than before, it is recommended to add a sentence and to align with WG2 specification. It was objected also the need of the correction for Rel99, notably the cover page doesn't shown that it is essential for Rel99. After off line discussions, the CR is postponed for the next meeting.

Decision: The CR is not agreed

R4-021443	Correction of UE Transmitted Power requirements in case of Compressed Mode gaps (CR 476 to 25.133 R99) (Ericsson)

R4-021444	Correction of UE Transmitted Power requirements in case of Compressed Mode gaps (CR 488 to 25.133 Rel-4) (Ericsson)

R4-021445	Correction of UE Transmitted Power requirements in case of Compressed Mode gaps (CR 477 to 25.133 Rel-5) (Ericsson)

Edgar Fernandes (Motorola) noted that this CR has big implications in other areas. In particular, depending on the implementation of the L3 filter (linear or log) the response would be very much different. Also, Edgar noted that the behaviour of that filter is unclear when its input is no value.

It is suggested to allow some time for in house analysis and allow for an email approval.

Decision: Email approval. See section 14

R4-021651	Correction of UE  parameters for Random Access Test (CR 504 to 25.133 R99) (DoCoMo)

R4-021652	Correction of UE  parameters for Random Access Test (CR 505 to 25.133 Rel-4) (DoCoMo)

R4-021653	Correction of UE  parameters for Random Access Test (CR 506 to 25.133 Rel-5) (DoCoMo)

These CRs address the concerns raised by T1RF in the LS in R4-021425. It wasn't considered necessary to draft a LS back to T1RF, since that group systematically checks RAN WG4 CRs for impacts in the test specifications.

Decision: The CRs are agreed

R4-021717	Corrections to cell reselection test cases (CR 507 to 25.133 R99) (Nokia)

R4-021718	Corrections to cell reselection test cases (CR 508 to 25.133 Rel-4) (Nokia)

R4-021719	Corrections to cell reselection test cases (CR 509 to 25.133 Rel-5) (Nokia)

R4-021628	draft LS in response to the LS from T1  on application of Test Tolerances to RRM tests (Nokia)

Nadege Noisette (Orange) thanked Nokia for the quick response in this area and requested some time to let the T1 delegates analyse the proposal before approval. Sari Korpela (Nokia) pointed out that in the event the CR doesn't get approved, the LS shouldn't be sent either. Agilent and R&S objected this view, and suggested that the guidance in the LS should be sent regardless of the decision on the CR.

It is agreed to separate the guidance on test tolerances and the correction for this particular test case, the LS in R4-021628 is split in two, R4-021739 and R4.-021740. R4-021740 will be sent if the CRs are agreed, and R4-021739 (See section 8) will cover the general recommendations on test tolerances.

R4-021740	LS on application of Test Tolerances to intra-frequency cell reselection test cases (Nokia)

Decision: It is agreed to have an email approval of the CRs in R4-021717, R4-021718 and R4-021719 and the LS in R4-021740. See section 14.

### 5.7.1	IPDL issues concerning the UE

R4-021462	Required Window size for measurements using IPDL (Ericsson)

A number of comments were raised, although Ericsson was thanked for this clarifying paper. Mitsubishi commented that the impact of window search size is not assessed in terms of E-911 performance, and that the degradation of 40 chips window in rural outdoor in Figure 10 should not be 7 dB but around 2 dB only. The windows sizes proposed in the conclusion couldn't be agreed.

R4-021463	Required Window size for measurements using IPDL (CR 480 to 25.133 R99) (Ericsson)

R4-021564	IPDL performance requirements (CR 484r1 to 25.133 R99) (Nokia)

The proposal from Nokia differs from Ericsson on the different window sizes considered, Nokia proposes 80 chips as the largest value, against +160 chips. Also Ericsson doubles the measurement period for the 80 chips window size, which is supported by Mitsubishi.

After off line discussions, Nokia and Ericsson agreed to produce a revised common CR with the +160 chips cell size removed

Decision: The CRs are revised

Edgar Fernandes (Motorola) questioned what would be the impact on the performance requirements if IPDL gaps and compress mode gaps are run in parallel. The serving cell measurement performance of an UE not supporting this feature would be affected, and these measurements are required for basic operation of the system, unlike the IPDL which is for an additional feature (positioning). Does the UE have to take into account the IPDL gaps in addition to the compress mode gaps, or they are overlapping? It has been shown that the impact of IPDL on bit rate is small, but on Edgar's view the impact on RRM has not been sufficiently evaluated. Ericsson and Nokia believe that the IPDL would not affect significantly the measurement performance of the UE.

Motorola proposed to add an statement in the general requirements to ensure that the terminals not supporting IPDL do not see its performance degraded due to the interaction of compress mode and IPDL. Such a compromise was agreed by Nokia and Ericsson, although David Chokroun (Nortel) objected to add restrictions to the IPDL or compress mode, but it seems possible that an agreement on the statement is reached off line.

R4-021710	Required Window size for measurements using IPDL (CR 480r1 to 25.133 R99) (Ericsson)

Stefan Gustafsson (KPN) objected that the new sentence ("the UE shall be able to monitor up to 16 cells"), it seems to imply that one cell is enough. Torgny Palenius (Ericsson) noted that all the requirements in that section are stated that way. Claudio Guerrini (Tilab/TIM) noted that now the requirements for the BS power level in the idle period is -35 dB and not -45 dB, the parameter in the test should be aligned with the BS specification.

Volker Breuer (Siemens) showed some concern about the note of the performance of intra freq. measurements with IPDL active not having been studied. It was agreed that the issue has to be studied in the future by the group, and eventually the sentence would be removed after two meetings if no impact is detected.

Decision: The CR is revised

R4-021741	Required Window size for measurements using IPDL (CR 480r2 to 25.133 R99) (Ericsson)

R4-021742	Required Window size for measurements using IPDL (CR 490r2 to 25.133 Rel-4) (Ericsson)

R4-021743	Required Window size for measurements using IPDL (CR 481r2 to 25.133 Rel-5) (Ericsson)

Decision: The CRs are agreed

### 5.7.2	Cell Identification

R4-021641	Report of the cell identification evening session (NEC)

Tdocs R4-021408, R4-021409, R4-021579, R4-021580 and R4-021461 were shortly presented and discussed during this session.

The meeting led to the following conclusion:

1)	it was agreed by the participants that no problem had been identified by simulations so far on the intra-frequency cell identification

2)	there was no consensus on the way  of adding a general requirement limitation on transmission gap patterns (e.g. minimum gap density ) into TS 25.133.

3)	it was also agreed that the constraints to configure parallel transmission gap patterns for inter-RAT measurements had to be compliant with any potential transmission gap pattern limitation

The proposed way forward is :

1)	to agree on simulation assumptions as well as methodology

2)	to agree on a way forward for the introduction on transmission gap pattern limitation (e.g. the proposal in Tdoc R4-011409)

After presentation, it was extensively discussed whether there is a fundamental performance issue for inter-frequency measurements with compressed mode. Siemens and NEC stressed that identification requirements can be met in certain tuning of parameters only. Nokia and Ericsson replied that typical scenarios are still to be discussed by taking into account realistic deployment scenario from the network side, and that there are some risks excluding flexibility at this stage. Nokia also commented that they do not see need for presenting the simulation results of UE performance. They recognised that removal of some unlikely configurations has to be considered.

R4-021485	Fading test case for cell identification requirement (Nokia)

It is noted that different speeds should be considered also. Edgar Fernandes (Motorola) pointed out also that the 800ms figure coming from the general requirements might not be applicable to this particular situation, non-general values should be considered. Mitsubishi commented that some frequency error (ppm drift) has to be introduced in this test case to reflect realistic scenario where the 2 cells have not the same frequency sources and Doppler shifts.

R4-021580	On cell identification in multipath fading conditions (Siemens)

Information paper. It was felt that no problem is identified so far with regards to the intra-frequency cell measurements.

R4-021664	Simulation assumptions for inter FDD cell identification with compressed mode (Mitsubishi)

Siemens thanked Mitsubishi and wondered about the RF noise factor, noticed a typo in the ppm equation and insisted onto the minimum gap density which has to be set a bit lower. Mitsubishi clarified that the RF noise factor was taken for inclusion on top of the implementation margin. Nokia noticed that consideration of these assumptions needs more time, especially from network side. Nokia also commented that the proposed simulation assumptions include very implementation related parameters and there are also other means but simulations to evaluate the performance of the UE. Mitsubishi requested a deadline for the next meeting, particularly:

-	What is the lower bound of the gap density into the core requirements in Release 99?

-	What are the test cases to be included in Release 5 ?

It was agreed to further discuss this onto e-mail reflector.

Chairman concluded that, as Release 99 may be impacted by this topic, this will be reported to RAN plenary.

## 5.7.3	Layer 3 filtering

R4-021446	Discussion on linear or logarithmic L3 filtering (Ericsson)

KPN and Motorola objected figure 5, the reason the blue line and the green line are so separate is the filter coefficient chosen, if it is required that the filter response follows closely the non-filtered signal (green), the coefficients can be selected to allow for a faster response.

Josef Blanz (Qualcomm) commented figure 6, the delay of the log filter when the user is approaching to the cell is always bigger than that of the linear filter. The figures only go up to 30kmph, this effect would be more remarkable at higher speeds. Josef also noted that in a CDMA system is more important that a valid cell is added to the active set as soon as possible when it becomes selectable than a cell that becomes non valid is dropped from the active set list.

R4-021479	Unit of layer 3 filtering (Motorola)

Sari Korpela (Nokia) questioned what layer 1 filter is used, Motorola clarified that this simulation is using an ideal layer 1 filter, measurements are provided each 200 ms exactly.

R4-021484	L3 filtering (Nokia)

Nokia remarks in this contribution that GSM RSSI uses logarithmic filtering, and therefore when comparing GSM power levels to UTRA power level the same type of filtering should be used. Motorola objected that a faster filter response is required in UTRA.

Nokia notes that the main influence on the measurement coming out of the layer 3 filter is not the layer 3 filter itself, but the layer 1 filter; therefore the difference in the results between log or linear layer 3 is small.

Qualcomm warned that it may be concluded from the graphics in this paper that the linear filtering would provide a response around 4 dBs away from the actual average value, which is incorrect.

R4-021534	L3 Filtering (Qualcomm)

Josef Blanz (Qualcomm) remarked that the linear filter follows faster the real signal in the case there is an increasing ramp, which means that the cell is getting closer and eventually would get included in the active set. Josef noted that it is of highest importance that a cell gets into the active set as fast as possible.

Don Zelmer asked the UE manufacturers if any had real products and data taken from real life on the behaviour of these two filters.

The chairman summarized that the option left are 3: WG4 could recommend to make mandatory in the specification either logarithmic, linear or leave it free for the manufacturers to chose. He also reminded that the reason GSM selected logarithmic was because at that time floating point processing was not available to introduce in the UE.

Finally, no agreement was reached. The chairman will report it to TSG RAN and WG4 will continue to study the issue for at least another meeting.

## 5.8	25.141 - Base Station conformance testing (FDD)

R4-021450	FDD  GSM co-existence in the Same Geographic Area (CR 250 to 25.141 Rel-4) (Ericsson)

Similarly to the CR to 25.104 in R4-021447, it is agreed to introduce this change in Rel99 also. The CR cover page will be revised to change Category to A and a new Rel99 CR will be produced

Decision: The CR is revised

R4-021660	FDD  GSM co-existence in the Same Geographic Area (CR 250r1 to 25.141 Rel-4) (Ericsson)

R4-021661	FDD  GSM co-existence in the Same Geographic Area (CR 264 to 25.141 R99) (Ericsson)

R4-021451	FDD  GSM co-existence in the Same Geographic Area (CR 252 to 25.141 Rel-5) (Ericsson)

Decision: The CRs are agreed

R4-021452	FDD  GSM 850 / PCS 1900 co-existence in the Same Geographic Area (CR 251 to 25.141 Rel-5) (Ericsson)

No comments

Decision: The CR is agreed

R4-021507	Clarification of the W-CDMA interferer definition in BS conformance tests for ACS and blocking characteristics (CR 253 to 25.141 Rel-4) (Siemens)

See R4-021505.

Decision: The CR is not agreed

R4-021549	BS IPDL test (CR 255 to 25.141 Rel-4) (Siemens)

Due to the discussion on IPDL mask, the document cannot be agreed. This is the first presentation of a CR for the IPDL test.

Decision: The CR is revised

R4-021689	BS IPDL test (CR 255r1 to 25.141 Rel-4) (Siemens)

R4-021690	BS IPDL test (CR 256r1 to 25.141 Rel-5) (Siemens)

Decision: The CRs are agreed

R4-021554	General corrections to TS 25.141 (CR 257 to 25.141 Rel-4) (Siemens)

The change in table 4.1 requires an update of many references along the TS. A revision of the CR is required to cope with these updates.

Decision: The CR is revised

R4-021671	General corrections to TS 25.141 (CR 257r1 to 25.141 Rel-4) (Siemens)

R4-021672	General corrections to TS 25.141 (CR 258r1 to 25.141 Rel-5) (Siemens)

Decision: The CRs are agreed

R4-021556	General Release 5 corrections (CR 259 to 25.141 Rel-5) (Siemens)

No comments

Decision: The CR is agreed

R4-021557	Transmit intermodulation test correction (CR 260 to 25.141 Rel-4) (Siemens)

R4-021558	Transmit intermodulation test correction (CR 261 to 25.141 Rel-5) (Siemens)

Moray Rumnay (Agilent) noted The requirement "the width of the intermod product should be taken into account" is unclear; from a measurement point of view is not obvious how it has to be taken into account when such a wide bandwidth signal falls at the end of the inband test band. Part of the signal would be measured with the inband test procedure and part with the out of band procedure. After off line discussions, it was agreed to keep the CR as is and elaborate a more precise way of running the test in future meetings

Decision: The CRs are agreed

R4-021595	UTRA-FDD BS Receiver protection (Nortel Networks)

Ericsson commented that this proposal is raising many interesting points and issues to consider, and warns about approving CRs on this issue right away. It is proposed to have off line and e-mail discussions before the next meeting.

Decision: The document is noted

R4-021609	Addition of TX Diversity timing accuracy test (CR 263 to 25.141 Rel-5) (Agilent)

Thomas Maucksch (R&S) commented that the test is repeated for Low Med High frequency, it doesn't seem necessary since the result shouldn't vary much from one frequency to another. On the opposite other parameters, like power level, should be varied. Michael Jeck (Ericsson) noted that it might be necessary to test at various frequencies, filter phase response might be different. Finally, a new revision with a single frequency test will be provided.

Decision: The CR is revised

R4-021655	Addition of TX Diversity timing accuracy test (CR 263r1 to 25.141 Rel-5) (Agilent)

A short discussion followed the presentation, the CR was finally approved

Decision: The CR is agreed

R4-021401	UTRAN measurements SIR and  SIR error (CR 262 to 25.141 Rel-5) (Nokia)

The chairman questioned why the tolerances were set to 0 dB, at least measurement equipment tolerances should be considered. After off line discussions, it was agreed to postpone the discussion for the next meeting.

Decision: The document is postponed

R4-021406	Problem on PN9 seed setting in Test Model 5 (Lucent Technologies)

R4-021407	Correction on PN9 seed setting in Test Model 5 (CR 247 to 25.141 Rel-5) (Lucent, Panasonic, Nortel)

No comments

Decision: The CR is agreed

R4-021610	Draft TR 35.901 on Statistical theory of measurements (Agilent)

Moray Rumney (Agilent) made a brief presentation of this TR currently under development in T1RF. The document will be regularly presented to RAN WG4 in order to follow its progress.

Decision: The document is noted

R4-021746	Clarification of the W-CDMA interferer definition in BS conformance tests for ACS and blocking characteristics (CR 253r2 to 25.141 Rel-4) (Spirent)

R4-021747	Clarification of the W-CDMA interferer definition in BS conformance tests for ACS and blocking characteristics (CR 254r2 to 25.141 Rel-5) (Spirent)

Decision: Email approval. See section 14.

## 5.9	25.142 - Base Station conformance testing (TDD)

R4-021561	Introduction of Rel-5 clarifications and small corrections in Rel-4 (CR 147 to 25.142 Rel-4) (Siemens)

The CR will be revised together with R4-021560.

Decision: The CR is revised

R4-021646	Introduction of Rel-5 clarifications and small corrections in Rel-4 (CR 147r1 to 25.142 Rel-4) (Siemens)

Decision: The CR is agreed

R4-021582	Averaging period in ACLR test for 1.28 Mcps TDD option (CR 148 to 25.142
Rel-4) (Siemens)

R4-021583	Averaging period in ACLR test for 1.28 Mcps TDD option (CR 149 to 25.142 
Rel-5) (Siemens)

No comments.

Decision: The CRs are agreed

R4-021545	Correction of 16QAM EVM/PCDE testing for HSDPA for 3,84 Mcps TDD option (CR 146 to 25.142 Rel-5) (InterDigital)

It seems that in the FDD case it was decided not to correct the test, based on the fact that the PAR is not substantially different from the QPSK case. It is noted also that, if this correction is needed, it should apply to Low Chip Rate as well. It is clarified that the corresponding CR for Low Chip Rate was already agreed at the previous meeting.

It was remarked that these emission tests are transposed to the European HS, it is recommended to approve the changes, if needed, as quick as possible so they can be included in the second release of the HS, due for approval at ETSI in December 2002.

Decision: The CR is revised

R4-021670	Correction of 16QAM EVM/PCDE testing for HSDPA for 3,84 Mcps TDD option (CR 146r1 to 25.142 Rel-5) (InterDigital)

Decision: The CR is agreed

R4-021648	Correction of adjacent channel leakage power definition (CR 150 to 25.142 Rel-5) (Siemens)

Decision: The CR is agreed

R4-021724	Corrections to TDD 3.84Mcps Reference Measurement Channels (CR 151 to 25.142 R99) (IPWireless)

R4-021725	Corrections to TDD 3.84Mcps Reference Measurement Channels (CR 152 to 25.142 Rel-4) (IPWireless)

R4-021726	Corrections to TDD 3.84Mcps Reference Measurement Channels (CR 153 to 25.142 Rel-5) (IPWireless)

R4-021727	Corrections to TDD 1.28Mcps Reference Measurement Channels (CR 154 to 25.142 Rel-4) (IPWireless)

R4-021728	Corrections to TDD 1.28Mcps Reference Measurement Channels (CR 155 to 25.142 Rel-5) (IPWireless)

Decision: The CRs are agreed

## 5.10	25.113 - Base station EMC

R4-021454	New exclusion bands, requirements for telecommunication port and interpretation of measurement results (CR 018 to 25.113 Rel-4) (Ericsson, Nokia)

R4-021455	New exclusion bands, requirements for telecommunication port and interpretation of measurement results (CR 019 to 25.113 Rel-5) (Ericsson, Nokia)

A number of small corrections/comments were proposed. A revision is required for both Releases

Decision: The CRs are revised

R4-021656	New exclusion bands, requirements for telecommunication port and interpretation of measurement results (CR 018r1 to 25.113 Rel-4) (Ericsson, Nokia)

R4-021657	New exclusion bands, requirements for telecommunication port and interpretation of measurement results (CR 019r1 to 25.113 Rel-5) (Ericsson, Nokia)

Decision: The CRs are agreed

## 5.11	34.124 - EMC requirements for mobile terminals and ancillary equipment

R4-021562	Introduction of Rel-5 clarifications and small corrections in Rel-4 (CR 011 to 34.124 Rel-4) (Siemens)

All the changes in this CR are incorporated in R4-021456.

Decision: The CR is withdrawn

R4-021456	New exclusion bands and interpretation of measurement results (CR 009 to 34.124 Rel-4) (Ericsson, Nokia)

R4-021457	New exclusion bands and interpretation of measurement results (CR 010 to 34.124 Rel-5) (Ericsson, Nokia)

Decision: The CRs are agreed

## 5.12	34.926 - Table of International requirements for Mobile terminals and ancillary equipment

No contributions

## 5.13	25.942 - RF System scenarios

R4-021611	Optimizing code channel allocation in the presence of signal clipping (Agilent)

The document shows how the noise created by signal clipping falls to particular codes. This can be predicted, and then the majority of the energy due to clipping can be projected to unused codes to minimize the interference to used codes.

The document is presented for information.

## 5.14	25.106 – Repeaters - Radio transmission and reception

R4-021623	Repeater adjacent band gain issue (Orange)

Nadege Noisette (Orange) presented this document, showing scenarios where a repeater interferes operation in adjacent bands. Tilab/TIM shares the views of Orange on the need for and adjacent band gain requirement linked to the in band gain, and not only to the DL power. Alf Ahlstrom (Allgon) commented that he will provide a paper considering the same scenarios in the future, but in his view the situation is not as bad as depicted.

R4-021588	Out of band gain (Mikom/ Allgon)

R4-021586	Out of band gain (CR 017 to 25.106 Rel-4) (Mikom/ Allgon)

R4-021587	Out of band gain (CR 018 to 25.106 Rel-5) (Mikom/ Allgon)

No comments

Decision: The CRs are agreed

R4-021521	Input intermodulation: Correction of co-location and addition of co-existence (CR 011 to 25.106 Rel-4) (Mikom/ Allgon)

Michael Jeck (Ericsson) commented that the co-location requirements on table 11.2 don't need to cover the UE Rx bands, ie 876 - 915 MHz , 1710 - 1785 MHz . He also noted a typo in the second line in 11.3, that section refers to co-existence and not co-location. Otherwise, the CR is agreed. A revision will be provided with these corrections.

Decision: The CR is revised

R4-021529	Aligning of the requirement for "Output power" in extreme conditions with TS25.143 (CR 015 to 25.106 Rel-4) (Mikom/ Allgon)

R4-021530	Aligning of the requirement for "Output power" in extreme conditions with TS25.143 (CR 016 to 25.106 Rel-5) (Mikom/ Allgon)

No comments

Decision: The CRs are agreed

R4-021518	EVM Test: Change requirement for the use of HSDPA. (CR 010 to 25.106 Rel-5) (Mikom/ Allgon)

Han van Bussel (T-Mobile) suggested to introduce the change in Rel-4 also, since Rel-4 repeaters would be used together with Rel-5 networks in the future. This procedure is agreed. Johan Skold (Ericsson) asked for the rationale behind the values. This can be found in TR25.956. Johan also noted that the requirements for the protection of the GSM BS, approved earlier in this meeting for the BS, should be added to the repeaters specification also. Mikom agreed to produce the necessary CR in the future. The CR was finally agreed, but Michael Jeck (Ericsson) noted that the correctness of the change for 16QAM needs to be studied and if required, the issue will be reconsidered in the next meeting.

R4-021677	EVM Test: Change requirement for the use of HSDPA. (CR 19 to 25.106 Rel-4) (Mikom/ Allgon)

Decision: The CRs are agreed

R4-021675	Input intermodulation: Correction of co-location and addition of co-existence (CR 011r1 to 25.106 Rel-4) (Mikom/ Allgon)

R4-021676	Input intermodulation: Correction of co-location and addition of co-existence (CR 012r1 to 25.106 Rel-5) (Mikom/ Allgon)

Decision: The CRs are agreed

## 5.15	25.143 – Repeaters conformance testing

R4-021512	New test environment: Extreme power supply for output power test (CR 013 to 25.143 Rel-4) (Mikom/ Allgon)

Han van Bussel (T-Mobile) commented that the wording is not precise, a revision is required.

Decision: The CR is revised

R4-021678	New test environment: Extreme power supply for output power test (CR 013r1 to 25.143 Rel-4) (Mikom/ Allgon)

R4-021679	New test environment: Extreme power supply for output power test (CR 014r1 to 25.143 Rel-5) (Mikom/ Allgon)

Decision: The CRs are agreed

R4-021514	Addition of Repeater configuration (CR 015 to 25.143 Rel-4) (Mikom/ Allgon)

Han van Bussel (T-Mobile) commented that the use of "must" should be replaced by "shall" according to the drafting rules

Decision: The CR is revised

R4-021680	Addition of Repeater configuration (CR 015r1 to 25.143 Rel-4) (Mikom/ Allgon)

R4-021681	Addition of Repeater configuration (CR 016r1 to 25.143 Rel-5) (Mikom/ Allgon)

Decision: The CRs are agreed

R4-021516	Definition of the power to select the right table for the spectrum emission mask requirement. (CR 017 to 25.143 Rel-4) (Mikom/ Allgon)

R4-021517	Definition of the power to select the right table for the spectrum emission mask requirement. (CR 018 to 25.143 Rel-5) (Mikom/ Allgon)

Decision: The CRs are agreed

R4-021519	EVM Test: Change requirement for the use of HSDPA. (CR 019 to 25.143 Rel-5) (Mikom/ Allgon)

A Rel-4 CR is required, see R4-021518. It was commented that a 16QAM should be used for the EVM test, like test model 5 was introduced in the BS specification for the EVM test with 16QAM. It was discussed what the impact of the modulation would be on the test. Mikom agreed to add the reference to test case 5 in the repeaters specification, since the type of modulation of the signal is established in the test model.

Decision: The CR is agreed

R4-021682	EVM Test: Change requirement for the use of HSDPA. (CR 27 to 25.143 Rel-4) (Mikom/ Allgon)

Decision: The CR is agreed

R4-021520	EVM Test: Change from Test Model 4 to Test Model 1 (CR 020 to 25.143 Rel-5) (Mikom/ Allgon)

It was agreed to in the previous discussion to introduce test model 5 for this test in the future, so the need of this CR is unclear. Johan Skold (Ericsson) noted that test model 5 is only used in Release 5 for the BS.

Decision: The CR is agreed

R4-021685	EVM Test: Change from Test Model 4 to Test Model 1 (CR 28 to 25.143 Rel-4) (Mikom/ Allgon)

Decision: The CR is agreed

On the EVM test discussion, Michael Jeck (Ericsson) reminded that may be in the future the test case to be used will need to be revised.

R4-021523	Input intermodulation: Correction of co-location and addition of co-existence (CR 021 to 25.143 Rel-4) (Mikom/ Allgon)

See R4-021521.

Decision: The CR is revised

R4-021683	Input intermodulation: Correction of co-location and addition of co-existence (CR 021r1 to 25.143 Rel-4) (Mikom/ Allgon)

R4-021684	Input intermodulation: Correction of co-location and addition of co-existence (CR 022r1 to 25.143 Rel-5) (Mikom/ Allgon)

Decision: The CRs are agreed

R4-021527	Spurious emission: correction of the procedure (CR 023 to 25.143 Rel-4) (Mikom/ Allgon)

R4-021528	Spurious emission: correction of the procedure (CR 024 to 25.143 Rel-5) (Mikom/ Allgon)

No comments

Decision: The CRs are agreed

R4-021584	Out of band gain (CR 025 to 25.143 Rel-4) (Mikom/ Allgon)

R4-021585	Out of band gain (CR 026 to 25.143 Rel-5) (Mikom/ Allgon)

No comments

Decision: The CRs are agreed

## 5.16	25.956 – Repeaters technical report

No contributions

## 5.17	25.943 – Deployment aspects

No contributions

# 6	Release 6 Work Items

## 6.1	Improvement of Receiver Performance Requirements for FDD UE

R4-021511	Correction to Pilot Interference Mitigation Technical Report (CR 001 to 25.991 Rel-5) (Intel)

No comments

Decision: The CR is agreed

R4-021618	Simulation results for modified DCH demodulation tests in multi-path fading conditions (Nokia)

The paper provides simulation results for demodulation with power control activated, and gives a blueprint for the introduction of PC in the demodulation tests. Edgar Fernandes (Motorola) comments that this proposal includes values in square brackets, and those haven't been supported by simulations. He noted also that the values provided change compared to the results presented in the last meeting.

Shimon Moshavi (Intel) agreed on the convenience of introducing PC in some of the tests, but expressed concerns about the possible impact to overall testing time and about a 1 dB ambiguity inherent in the test as currently proposed.

R4-021489	Simulation results for DCH demodulation tests with power control (Ericsson)

The paper presents another set of simulation results.

Decision: The results are noted

R4-021662	UE performance requirements (Motorola)

Motorola questions what is the purpose of the WI, tighter/improved performance requirements for the UE demodulation, or performance requirements based on a realistic scenario (PC activated).

Motorola remarks the need of to agreed a set of simulation assumptions, there is a big degree of variability in the results due to different assumptions. The paper presents a number of issues to be considered when setting these assumptions.

Decision: the document is noted

The chairman commented that the WI Description Sheet doesn't describe clearly the scope of this WI, basically because at that time it wasn't clear what the work would be . He suggested the interested parties to come up with a new description of the intended work, so that there is a basis for the technical discussion. Also, a new TR will be produced to compile the simulations and the simulation assumptions. The chairman clarified that the closing time of Release 6 would be by the end of next year, so the group has plenty of time to conclude the work.

## 6.2	FDD Base Station Classification

R4-021490	Revised version 1.3.0 of TR 25.951 (Nokia)

Takaharu Nakamura (Fujitsu) asked to clarify that the inclusion of the BS Classification is for Release 6. This was agreed

Decision: The report is approved

R4-021491	Transmitter characteristics for FDD Local area BS class. (Nokia)

The chairman questioned if the same Output Power dynamic range is required for Local Area, since the output power is significantly less in a LA BS than in a normal BS. Ericsson agreed that this would be studied later.

T-Mobile objected the solution proposed for ACLR.

It was agreed to add the changes to the TR, except the ACLR section.

Decision: The document was approved

R4-021635	Proposal of maximum output power for Local area BS (DoCoMo)

The paper presents simulations results supporting the proposal of +20 dBm Max. Output Power for the Local Area BS. It was noted that the used macro-pico scenario is not in line with approved micro-pico scenario in 25.951. Further offline discussions are needed through the reflector to clarify this.

Decision: It is agreed to introduce these simulation results in the TR as a way of showing the current status of the discussions.

R4-021492	Receiver characteristics for FDD Local area BS class. (Nokia)

Michael Jeck (Ericsson) commented that the wanted signal in the intermodulation requirements has been kept the same for all the requirements derived before, there is no reason for having it 6 dB higher. Nokia agreed with that.

Nokia also clarified that only the micro-pico scenario has been considered, and not the macro-pico, because that was the agreed assumption as reflected in the TR.

Decision: The document is approved. Changes will be introduced in the TR

R4-021493	Performance requirements for FDD Local area BS class. (Nokia)

Yannick Li (Nortel) questioned if Uplink RX diversity could  be left out of the minimum requirement, as it is an optional feature. It is agreed to do so, but then new simulations are required to derive new figures for the minimum requirements. Takaharu Nakamura (Fujitsu) agreed that new simulations are required without diversity, but no clarification is required in the text in the requirements section. The chairman clarified that simulations comparing both cases should be presented to see how separate the results are. It is unclear whether this applies to Medium Area as well.

Han van Bussel (T-Mobile) raised the concern of the UE speed, since there might be some cases (a highway tunnel) were a Local Area BS is used for high speed mobiles.

Given the number of issues to be studied, it is agreed not to introduced the performance requirements in the TR yet.

Decision: The document is not approved

R4-021494	Changes in TS25.133 according to FDD Local area BS (Nokia)

Han van Bussel (T-Mobile) warned about how the current RRM implementation in the UEs would be affected by the new small cell area, the cell detection and active set update needs to be faster. This should be studied in the future.

Decision: It is agreed to add the text to the TR

R4-021497	Changes in TS25.133 according to FDD Local area BS (CR 497 to 25.133 Rel-6) (Nokia)

No comments.

Decision: The CR is agreed

R4-021430	Simulation results and scenarios for Medium Range BS in FDD mode (Ericsson, Nokia)

No comments.

Decision: The document is approved. Changes will be introduced in the TR

R4-021433	Dynamic range for Medium Range BS in FDD mode (Ericsson)

No comments.

Decision: The document is approved. Changes will be introduced in the TR

R4-021435	Proposal for Medium Range BS class output power (Ericsson)

Peter Muszynski (Nokia) objected that the value is derived from the very last scenario, and it gives no room for improvement. He would like to revise the value on the future. The chairman noted however that the paper can be introduced in the TR, as a way of showing the current status of the discussions.

Decision: The document is approved. Changes will be introduced in the TR

R4-021434	Co-location requirements for Medium Range and Local Area BS in FDD mode (Ericsson)

Some concerns were raised regarding the co-location with GSM BSs. It is preferred to study the issue further and not to add the text to the TR yet.

Decision: The document is not approved

R4-021436	Co-Siting, output power and dynamic range for the new BS classes (CR 147 to 25.104 Rel-6) (Ericsson)

R4-021437	Co-Siting, output power and dynamic range for the new BS classes (CR 248 to 25.141 Rel-6) (Ericsson)

It is clarified that the CR, if approved, will trigger the creation of the Release 6. T-Mobile objected the inclusion of a max. limit for the Wide Area BS, this is not under the scope of the Work Item.

It is generally objected that the values are not mature.

Decision: The CR is not agreed.

R4-021438	Introduction of Base Station Classes (CR 148 to 25.104 Rel-6) (Ericsson)

The CRs introduce the new BS classes definitions and the new requirements for Frequency error, Reference sensitivity level, Adjacent Channel Selectivity, Blocking and Intermodulation.

Johan Skold (Ericsson) commented that the intention is to produce a common CR including the contents of this and also the agreed TX requirements.

Decision: The CR is revised

R4-021439	Introduction of Base Station Classes (CR 249 to 25.141 Rel-6) (Ericsson)

A merge CR will be produced including these requirements and the agreed requirements in other CRs.

Decision: The CR is revised

R4-021495	Blocking scenarios for Medium Range BS in FDD mode (CR 010 to 25.942 Rel-6) (Ericsson and Nokia)

It is clarified that these scenarios are included in 25.942 after an explicit request from operators to have the BS classification background in 25.952

Decision: The CR is agreed

R4-021431	RRM requirement changes for FDD Base Station Classification (Ericsson)

No comments.

Decision: The document is approved. Changes will be introduced in the TR

R4-021432	RRM requirement changes for FDD Base Station Classification (CR 474 to 25.133 Rel-6) (Ericsson)

No comments.

Decision: The CR is agreed

R4-021694	Introduction of Base Station Classes (CR 148r1 to 25.104 Rel-6) (Ericsson)

R4-021695	Introduction of Base Station Classes (CR 249r1 to 25.141 Rel-6) (Ericsson)

Takaharu Nakamura (Fujitsu) explained that the maximum output power definitions for the new BS classes, still under discussion, might not be in compliance with the Japanese regulations. He noted that the issue will be studied and if necessary a paper will be presented at TSG RAN.

Decision: The CRs are agreed

The chairman reminded about the regulatory issues, both Japanese and from ETSI in Europe, but noted that WG4 should focus on the technical work and asses the level of completion. It seems that the work is very close to conclusion, and the chairman will report so to TSG RAN.

R4-021734	TR25.951 v1.4.0 (Nokia)

It is agreed to present the TR to TSG RAN for approval as version 2.0.0. If approved, version 6.0.0 will be created and the TR will be under the change control procedure.

Decision: The TR is approved

R4-021735	Status Report for FDD BS Classification WI (Nokia)

It was clarified that the CR to 25.133 will also be presented for approval at TSG RAN in December. The report was agreed, including the completion figure

Decision: The status report is approved

R4-021590	Low level RF interface on BS (Allgon)

Tuomo Säynäjäkangas (Nokia) commented that this is a very late stage on the BS classification work item to add such a new item, and it wasn't however included in the scope. Mike Vogel (Vodafone) expressed concerns on the impact of the BS performance of such an interface.

Stefan Gustafsson (KPN) supported the inclusion of this interface, whatever the administrative framework is. He believed that there is a lack of flexibility for the network deployment. Telefónica and Tilab/TIM also supported this view, and suggested that even if the radio parameters are defined later, the definition of the interface could be included in the BS classification.

Ericsson and Nortel agreed with Nokia that it is too late to have this interface as part of the BS Class WI.

Decision: The document is not approved

The chairman proposed as a way forward the creation of a new WI (or SI), since the low level interface is actually not a BS classification issue, but it involves many other considerations, the interconnection with the WDS for example. See R4-021737 in section 11 of this report.

## 6.3	Work Items under other WG responsibility

R4-021413	Joint 3GPP 3GPP2 Spatial Channel Model AHG Status Report (Lucent)

R4-021414	Spatial Channel Model Text Description (Lucent )

These documents are presented for information so WG4 is aware of the ongoing work of the Joint Ad Hoc group. No comments.

Decision: The documents are noted

R4-021636	Simulation results of CSICH when CA is active (Samsung Electronics.co.ltd)

GBT noted that it will present additional simulations in the next meeting.

Decision: The document is noted

# 7	Release 6 Study Items

## 7.1	Wide Band distribution Systems

No contributions

## 7.2	FS for the deployment of UTRA in additional and diverse spectrum arrangement

CWTS had raised concerns to contributions R4-021510 and R4-021616 presented under this agenda item, but since the CWTS delegation couldn't attend the meeting, they have requested that those papers are postponed for the next meeting when hopefully all the interested parties will participate. The group agreed to proceed that way.

R4-021440	TR 25.889 v1.1.1  (Ericsson)

No comments

Decision: The report is approved

R4-021501	Text Proposal for Section 7.4 of TR 25.889 V1.1.0 "Viable deployment of UTRA in additional and diverse spectrum arrangements" (Nokia)

Qualcomm and Motorola noted that the document contains much information and it is difficult to asses the impact right away, they requested some more time for evaluation.

Decision: It is not agreed to add the proposal to TR25.889

R4-021509	Proposed amendments to the current draft TR 25.889 V1.1.0 "Feasibility Study considering the viable deployment of UTRA in additional and diverse spectrum arrangements" (Siemens)

Decision: The document is approved

# 8	Liaison and output to other groups

R4-021716	LS on Rate Adaptation of AMR Codec (NEC)

Jean Francois Labal (NEC) noted the need for a small correction in Action sentence.

Decision: The LS is revised

R4-021738	LS on Rate Adaptation of AMR Codec (NEC)

Decision: The LS is approved

R4-021720	LS to GERAN on FDD/GSM co-existence (Ericsson)

Decision: The LS is approved

R4-021739	draft LS in response to the LS from T1  on application of Test Tolerances to RRM tests (Nokia)

Decision: The LS is approved

The following table summarizes the approved outgoing LSs:

| Tdoc | Title | LS To | LS Cc |
| --- | --- | --- | --- |
| R4-021720 | LS to GERAN on FDD/GSM co-existence | GERAN |  |
| R4-021738 | LS on Rate Adaptation of AMR Codec | SA WG4 | RAN WG2 |
| R4-021739 | draft LS in response to the LS from T1 on application of Test Tolerances to RRM tests | T1 RF |  |

# 9	Revision of the Work Plan

R4-021400	Work items under the responsibility of RAN WG4 (3GPP Support)

Modifications at RAN4 #24 are marked in bold and underlined.

| Name | Acronym | Start | Finish | % completed |
| --- | --- | --- | --- | --- |
| HSDPA RF Radio Transmission/ Reception, System Performance Requirements and Conformance Testing | HSDPA-RF | April-01 | Dec-02 | 95% |
| Base station classification | RInImp-BSClass | Aug-00 | Dec-02 |  |
| FDD Base station classification | RInImp-BSClass-FDD | Aug-00 | Dec-02 | 95% |
| FS on UTRA WideBand Distribution Systems | RInImp-WDS | March-01 | March-03 | 40% |
| FS for the viable deployment of UTRA in additional and diverse spectrum arrangements | RInImp-UMTSBands | Sep-01 | Dec-02 | 75% |
| Improving Receiver Performance Requirements for the FDD UE | RInImp-UERecPerf | March-02 | Dec-02 | ? |

# 10	Future meetings

RAN WG4 meetings:

| Meeting # | Date | Host | Location |
| --- | --- | --- | --- |
| 26 | 17 – 21 Feb 2003 | European Friends of 3GPP | Madrid, Spain |
| 27 | 19 - 23 May 2003 | European Friends of 3GPP | Paris, France |
| 28 | 18-22 August 2003 | CATT | China |
| 29 | 17 - 21 November 2003 | Qualcomm | San Diego, US |

TSG RAN meetings:

| Meeting # | Date | Host | Location |
| --- | --- | --- | --- |
| 18 | 03 - 06 December 2002 | North American Friends of 3GPP | New Orleans, US |
| 19 | 11 - 14 March 2003 | UK Friends of 3GPP | Birmingham, UK |
| 20 | 03 - 06 June 2003 | Nokia | Hämeenlinna, Finland |
| 21 | 16 - 19 September 2003 | Siemens | Berlin, Germany |
| 22 | 09 - 12 December 2003 | ARIB/TTC/NA Friends of 3GPP | Hawaii, US |
| 23 | 09 - 12 March 2004 |  | China |
| 24 | 01 - 04 June 2004 |  |  |
| 25 | 07 - 10 September 2004 |  | US |
| 26 | 07 - 10 December 2004 |  |  |

# 11	Any other business

R4-021396	UMTS 850 (Cingular Wireless LLC)

The documents presents a proposal to add to the bands covered by UTRA the 850 MHz band, currently used in ITU-2 (North America) for PCS operation. There were no objections, therefore WG4 endorses this WI for presentation to TSG RAN, where it will finally be approved.

Decision: The WI Description is endorsed

R4-021737	Proposal for a Study Item on an optional Low Level RF Interface (Tekmar et al.)

The documents follows up the discussion on document R4-021590. It seems unclear what the intention of the paper is, since the SI Description is not provided. Han van Bussel (T-Mobile) suggested study the pros and cons of the concept.

Decision: The document is noted

The chairman reminded that his period for chairmanship is due for February 2002. He noted if there is a candidature, he will not be able to present; but if it is not the case he will be pleased to continue. Discussions on the elections, and the group split in RAN and other TSGs, are expected to take place during the TSG meetings #18 in December.

# 13	Close of Meeting

The chairman closed the meeting at 12:00 on Friday 15th November. He thanked the participants for their assistance and the host for the social events and the organization.

# 14	Email approval

The following CRs and LS will be considered approved by the group if no comments/objections are received before Friday 22nd November 2002 17:00 CET. All these documents can be found in the 3GPP server at:

ftp://ftp.3gpp.org/tsg_ran/WG4_Radio/TSGR4_25/

R4-021744	Clarification of the W-CDMA interferer definition in BS requirements for ACS and blocking characteristics (CR 157r2 to 25.104 Rel-4) (Spirent)

R4-021745	Clarification of the W-CDMA interferer definition in BS requirements for ACS and blocking characteristics (CR 158r2 to 25.104 Rel-5) (Spirent)

R4-021746	Clarification of the W-CDMA interferer definition in BS conformance tests for ACS and blocking characteristics (CR 253r2 to 25.141 Rel-4) (Spirent)

R4-021747	Clarification of the W-CDMA interferer definition in BS conformance tests for ACS and blocking characteristics (CR 254r2 to 25.141 Rel-5) (Spirent)

R4-021443	Correction of UE Transmitted Power requirements in case of Compressed Mode gaps (CR 476 to 25.133 R99) (Ericsson)

R4-021444	Correction of UE Transmitted Power requirements in case of Compressed Mode gaps (CR 488 to 25.133 Rel-4) (Ericsson)

R4-021445	Correction of UE Transmitted Power requirements in case of Compressed Mode gaps (CR 477 to 25.133 Rel-5) (Ericsson)

R4-021717	Corrections to cell reselection test cases (CR 507 to 25.133 R99) (Nokia)

R4-021718	Corrections to cell reselection test cases (CR 508 to 25.133 Rel-4) (Nokia)

R4-021719	Corrections to cell reselection test cases (CR 509 to 25.133 Rel-5) (Nokia)

R4-021740	LS on application of Test Tolerances to intra-frequency cell reselection test cases (Nokia)

## 14.1	Conclusion of the email approval

R4-021744, R4-021745, R4-021746 and R4-021746 were not approved

R4-021443, R4-021444 and R4-021445 were approved

R4-021717, R4-021718, R4-021719 and R4-021740 were not approved

Annex A: List of participants

| Participant | Organization | Membership | Email |
| --- | --- | --- | --- |
| Mr. Alf Ahlström | ALLGON AB | 3GPPMEMBER (ETSI) | alf.ahlstrom@allgon.se |
| Mr. Nobuto Arai | NTT DoCoMo Inc. | 3GPPMEMBER (ARIB) | narai@cet.yrp.nttdocomo.co.jp |
| Mr. Mark Austin | DTI | 3GPPMEMBER (ETSI) | mark.austin@ra.gsi.gov.uk |
| Mr. Raja Bachu | MOTOROLA Ltd | 3GPPMEMBER (ETSI) | frb024@email.mot.com |
| Mr. Cliff Bailey | Spirent Communications | 3GPPMEMBER (ETSI) | cliff.bailey@spirentcom.com |
| Mr. David Bartlett | Cambridge Positioning Sytems | 3GPPMEMBER (ETSI) | david.bartlett@cursor-system.com |
| Mr. Martial Bellec | MITSUBISHI Electric Telecom | 3GPPMEMBER (ETSI) | MARTIAL.bellec@mef-rd.com |
| Dr. Howard Benn | MOTOROLA Ltd | 3GPPMEMBER (ETSI) | howard.benn@motorola.com |
| Dr. Josef Blanz | QUALCOMM EUROPE S.A.R.L. | 3GPPMEMBER (ETSI) | jblanz@qualcomm.com |
| Ing. Thomas Bohn | ALCATEL S.A. | 3GPPMEMBER (ETSI) | Thomas.Bohn@alcatel.de |
| Dr. Volker Breuer | SIEMENS AG | 3GPPMEMBER (ETSI) | volker.breuer@siemens.com |
| Mr. Thanh Bui | NEC Corporation | 3GPPMEMBER (ARIB) | thanhb@icpdd.neca.nec.com.au |
| Mr. Frédéric Charpentier | SIEMENS AG | 3GPPMEMBER (ETSI) | frederic.charpentier@siemens.com |
| Dr. Fang-Chen Cheng | Lucent Technologies | 3GPPMEMBER (ETSI) | fcc@lucent.com |
| Mr. David Choukroun | NORTEL NETWORKS (EUROPE) | 3GPPMEMBER (ETSI) | dchoukro@nortelnetworks.com |
| Mr. John B Fenn | SAMSUNG Electronics | 3GPPMEMBER (ETSI) | johnbfenn@aol.com |
| Mr. Edgar Fernandes | MOTOROLA Ltd | 3GPPMEMBER (ETSI) | edgar.fernandes@motorola.com |
| Mr. Stefan Fritze | SIEMENS AG | 3GPPMEMBER (ETSI) | stefan.fritze@siemens.com |
| Mr. Yutaka Fuke | NTT DoCoMo Inc. | 3GPPMEMBER (ARIB) | fuke@mlab.yrp.nttdocomo.co.jp |
| Mr. Andrea Garavaglia | ERICSSON L.M. | 3GPPMEMBER (ETSI) | andrea.garavaglia@eed.ericsson.se |
| Mr. Eric Georgeaux | NORTEL NETWORKS (EUROPE) | 3GPPMEMBER (ETSI) | eric.georgeaux@nortelnetworks.com |
| Mr. Gerhard Gerz | BMWi | 3GPPMEMBER (ETSI) | gerhard.gerz@regtp.de |
| Mr. Alessandro Guerrieri | TELECOM ITALIA S.p.A. | 3GPPMEMBER (ETSI) | aguerrieri@mail.tim.it |
| Dr. Claudio Guerrini | TELECOM ITALIA S.p.A. | 3GPPMEMBER (ETSI) | claudio.guerrini@tilab.com |
| Dr. Stefan Gustafsson | KPN N.V. | 3GPPMEMBER (ETSI) | n.a.s.gustafsson@kpn.com |
| Dr. Joerg Gustrau | SIEMENS AG | 3GPPMEMBER (ETSI) | joerg.gustrau@siemens.com |
| Mr. Cesar Gutierrez Miguelez | Mobile Competence Centre |  | cesar.gutierrez@etsi.fr |
| Mr. Paul Howard | IPWireless Inc. | 3GPPMEMBER (ETSI) | phoward@ipwireless.com |
| Mr. Howard Huang | Lucent Technologies | 3GPPMEMBER (T1) | hchuang@lucent.com |
| Mr. Sungoh Hwang | Samsung Electronics Co., Ltd | 3GPPMEMBER (TTA) | sungoh@samsung.com |
| Mr. Satoshi Imaizumi | Matsushita Communication | 3GPPMEMBER (ARIB) | Satoshi.Imaizumi@yrp.mci.mei.co.jp |
| Mr. Michael Jeck | ERICSSON L.M. | 3GPPMEMBER (ETSI) | michael.jeck@eed.ericsson.se |
| Mr. Jorma Kaikkonen | NOKIA Corporation | 3GPPMEMBER (ETSI) | jorma.kaikkonen@nokia.com |
| Mr. Emmanuel Kanterakis | GOLDEN BRIDGE TECHNOLOGY INC. | 3GPPMEMBER (ETSI) | ekanterakis@gbtwireless.com |
| Mr. Takeshi Kokubo | SONY Corporation | 3GPPMEMBER (ARIB) | kokubo@wtlab.sony.co.jp |
| Mr. Pasi Korento | NOKIA Corporation | 3GPPMEMBER (ETSI) | pasi.korento@nokia.com |
| Ms. Sari Korpela | NOKIA Corporation | 3GPPMEMBER (ETSI) | sari.k.korpela@nokia.com |
| Mr. Jean-François Labal | NEC Technologies (UK) LTD | 3GPPMEMBER (ETSI) | jean-francois.labal@mdc.nec.fr |
| Mr. Floyd Lazare | TTPCom Ltd | 3GPPMEMBER (ETSI) | floyd.lazare@ttpcom.com |
| Mr. Yannick Le Pezennec | VODAFONE LTD | 3GPPMEMBER (ETSI) | Yannick.LePezennec@vodafone.co.uk |
| Miss Hye-young Lee | Samsung Electronics Co., Ltd | 3GPPMEMBER (TTA) | isolde.lee@samsung.com |
| Mr. Joseph Levy | INTERDIGITAL COMMUNICATIONS | 3GPPMEMBER (ETSI) | joseph.levy@interdigital.com |
| Dr. Yannick Li | NORTEL NETWORKS (EUROPE) | 3GPPMEMBER (ETSI) | yli@nortelnetworks.com |
| Mr. Young seok Lim | Samsung Electronics Co., Ltd | 3GPPMEMBER (TTA) | ltyoung0@samsung.com |
| Mr. Masato Maeda | NTT DoCoMo Inc. | 3GPPMEMBER (ARIB) | maeda@cet.yrp.nttdocomo.co.jp |
| Mr. José Alberto Martín Ortega | TELEFONICA de España S.A. | 3GPPMEMBER (ETSI) | MARTIN_JA4@TSM.ES |
| Dr. Carlo Matarasso | TEKMAR Sistemi Srl | 3GPPMEMBER (ETSI) | carlo.matarasso@tekmar.it |
| Mr. Thomas Maucksch | ROHDE & SCHWARZ | 3GPPMEMBER (ETSI) | thomas.maucksch@rsd.rohde-schwarz.com |
| Mr. Michael McKernan | Spirent Communications | 3GPPMEMBER (ETSI) | michael.mckernan@spirentcom.com |
| Mr. Juha Meinilä | Elektrobit Ltd. | 3GPPMEMBER (ETSI) | Juha.Meinila@elektrobit.fi |
| Mr. Ralf Michanikl | MIKOM GmbH | 3GPPMEMBER (ETSI) | Ralf.Michanikl@mikom.com |
| Dr. Shimon Moshavi | Intel Sweden AB | 3GPPMEMBER (ETSI) | shimon.moshavi@intel.com |
| Mr. Peter Muszynski | NOKIA Corporation | 3GPPMEMBER (ETSI) | peter.muszynski@nokia.com |
| Mr. Katsuyoshi Naka | Matsushita Communication | 3GPPMEMBER (ARIB) | Katsuyoshi.Naka@yrp.mci.mei.co.jp |
| Mr. Takaharu Nakamura | Fujitsu Limited | 3GPPMEMBER (ARIB) | n.takaharu@jp.fujitsu.com |
| Dr. Man Hung Ng | Lucent Technologies N. S. UK | 3GPPMEMBER (ETSI) | ngm@lucent.com |
| Miss Nadege Noisette | ORANGE FRANCE | 3GPPMEMBER (ETSI) | nadege.noisette@francetelecom.com |
| Mr. Yoshiharu Ohsaki | Matsushita Communication | 3GPPMEMBER (ARIB) | yoshiharu.ohsaki@yrp.mci.mei.co.jp |
| Dr. Torgny Palenius | ERICSSON L.M. | 3GPPMEMBER (ETSI) | torgny.palenius@emp.ericsson.se |
| Mr. Markus Pettersson | NOKIA Corporation | 3GPPMEMBER (ETSI) | Markus.Pettersson@nokia.com |
| Mr. Jean Prudent | TDF | 3GPPMEMBER (ETSI) | jean.prudent@tdf.fr |
| Ms. Ghislaine Reybet-Degat | ALCATEL S.A. | 3GPPMEMBER (ETSI) | ghislaine.reybet-degat@alcatel.fr |
| Mr. Moray Rumney | AGILENT TECHNOLOGIES LTD | 3GPPMEMBER (ETSI) | moray_rumney@agilent.com |
| Mr. Tuomo Säynäjäkangas | NOKIA Corporation | 3GPPMEMBER (ETSI) | tuomo.saynajakangas@nokia.com |
| Mr. Donglin Shen | AT&T Wireless Services, Inc. | 3GPPMEMBER (T1) | donglin.shen@attws.com |
| Mr. Yoshitsugu Shimazu | NTT DoCoMo Inc. | 3GPPMEMBER (ARIB) | shimazu@wsp.yrp.nttdocomo.co.jp |
| Mr. Kenichiro Shinoi | Matsushita Communication | 3GPPMEMBER (ARIB) | Kenichiro.Shinoi@yrp.mci.mei.co.jp |
| Mr. Johan Sköld | ERICSSON L.M. | 3GPPMEMBER (ETSI) | johan.skold@era.ericsson.se |
| Mrs. Carolyn Taylor | Motorola Inc. | 3GPPMEMBER (T1) | carolyn.taylor@motorola.com |
| Mr. Ulf Tegth | TELIA AB | 3GPPMEMBER (ETSI) | Ulf.B.Tegth@telia.se |
| Mr. Han van Bussel | T-MOBILE DEUTSCHLAND | 3GPPMEMBER (ETSI) | han.van.bussel@t-mobile.de |
| Mr. Mike Vogel | Vodafone D2 GmbH | 3GPPMEMBER (ETSI) | mike.vogel@vodafone.com |
| Mr. Thierry Werling | WAVECOM | 3GPPMEMBER (ETSI) | thierry.werling@wavecom.com |
| Mr. Tim Wilkinson | IPWireless Inc. | 3GPPMEMBER (ETSI) | twilkinson@ipwireless.com |
| Mr. Eldad Zeira | INTERDIGITAL COMMUNICATIONS | 3GPPMEMBER (ETSI) | eldad.zeira@interdigital.com |
| Mr. Donald E. Zelmer | Cingular Wireless LLC | 3GPPMEMBER (T1) | don.zelmer@cingular.com |
| Mr. Yujian Zhang | SAMSUNG Electronics | 3GPPMEMBER (ETSI) | zyj@samsung.co.kr |

Annex B: List of documents

| Tdoc | Type | Title | Source | Decision |
| --- | --- | --- | --- | --- |
| R4-021396 | Approval | UMTS 850 | Cingular Wireless LLC | Approved |
| R4-021397 | Approval | Draft agenda meeting #25 | Chairman | Approved |
| R4-021398 | Approval | Revised draft report meeting #24 | 3GPP support | Approved |
| R4-021399 | Information | Draft report TSG RAN #17 | 3GPP support | Noted |
| R4-021400 | Discussion | Work items under the responsibility of RAN WG4 | 3GPP Support | Noted |
| R4-021401 | CR | UTRAN measurements SIR and SIR error | Nokia | Postponed |
| R4-021402 | Information | Simulation results for QPSK and 16QAM for 1,28 Mcps TDD option | Motorola | Noted |
| R4-021403 | CR | Correction of interruption time in FDD/FDD Hard Handover | Panasonic | Agreed |
| R4-021404 | CR | Correction of interruption time in FDD/FDD Hard Handover | Panasonic | Agreed |
| R4-021405 | CR | Correction of interruption time in FDD/FDD Hard Handover | Panasonic | Agreed |
| R4-021406 | Discussion | Problem on PN9 seed setting in Test Model 5 | Lucent Technologies | Noted |
| R4-021407 | CR | Correction on PN9 seed setting in Test Model 5 | Lucent, Panasonic, Nortel | Agreed |
| R4-021408 | Approval | Requirements on CM patterns for successful inter-frequency cell identification | NEC | Noted |
| R4-021409 | CR | Correction of requirements on CM patterns for inter-frequency cell identification | NEC | Not agreed |
| R4-021410 | CR | Correction of requirements on CM patterns for inter-frequency cell identification | NEC | Not agreed |
| R4-021411 | CR | Correction of requirements on CM patterns for inter-frequency cell identification | NEC | Not agreed |
| R4-021412 | Discussion | Simulation Results for HSDPA Variable Reference Channel | Lucent | Noted |
| R4-021413 | Information | Joint 3GPP 3GPP2 Spatial Channel Model AHG Status Report | Lucent | Noted |
| R4-021414 | Information | Spatial Channel Model Text Description | Lucent | Noted |
| R4-021415 | Information | Fading simulation results of CSICH when CA is active | Samsung Electronics.co.ltd | Revised in R4-021636 |
| R4-021416 | LS in | LS on TX diversity on radio links in the active set | TSG RAN WG1 | Noted |
| R4-021417 | LS in | LS on HS-PDSCH capability definition and QPSK-only UE categories | TSG RAN WG1 | Noted |
| R4-021418 | LS in | LS on cell portion in beamforming | TSG RAN WG1 | Noted |
| R4-021419 | LS in | LS on beamforming enhancement WI | TSG RAN WG1 | Noted |
| R4-021420 | LS in | LS on Reference configurations in TS 34.108 | TSG RAN WG2 | Noted |
| R4-021421 | LS in | Response to LS (S4-020468) on Rate Adaptation of AMR Codec | TSG RAN WG2 | Noted |
| R4-021422 | LS in | Response to LS on CPICH RSCP signalling range (R2-022212) | TSG RAN WG2 | Noted |
| R4-021423 | LS in | Response LS on Rate Adaptation of AMR Codec | TSG SA WG4 | Noted |
| R4-021424 | LS in | LS on application of Test Tolerances to RRM tests | TSG T WG1 RF SWG | Withdrawn |
| R4-021425 | LS in | LS on the value of Maximum allowed UL TX power in case of correct behaviour at time-out test of Random Access | TSG T WG1 RF SWG | Noted |
| R4-021426 | LS in | LS on the key concepts and parameters for BER/BLER Statistical Approach (UE conformance testing) | TSGT WG1 RF | Noted |
| R4-021427 | LS in | LS on the completion of the FDD BS Classification Work Item | ETSI MSG | Noted |
| R4-021428 | LS in | LS on the measurement uncertainties on the radiated emissions measurements | ETSI MSG | Noted |
| R4-021429 | CR | Maximum received power at UE | Ericsson | Not agreed |
| R4-021430 | Approval | Simulation results and scenarios for Medium Range BS in FDD mode | Ericsson, Nokia | Agreed |
| R4-021431 | Approval | RRM requirement changes for FDD Base Station Classification | Ericsson | Approved |
| R4-021432 | CR | RRM requirement changes for FDD Base Station Classification | Ericsson | Agreed |
| R4-021433 | Approval | Dynamic range for Medium Range BS in FDD mode | Ericsson | Approved |
| R4-021434 | Approval | Co-location requirements for Medium Range and Local Area BS in FDD mode | Ericsson | Not approved |
| R4-021435 | Approval | Proposal for Medium Range BS class output power | Ericsson | Approved |
| R4-021436 | CR | Co-Siting, output power and dynamic range for the new BS classes | Ericsson | Not agreed |
| R4-021437 | CR | Co-Siting, output power and dynamic range for the new BS classes | Ericsson | Not agreed |
| R4-021438 | CR | Introduction of Base Station Classes | Ericsson | Revised in R4-021694 |
| R4-021439 | CR | Introduction of Base Station Classes | Ericsson | Revised in R4-021695 |
| R4-021440 | Approval | TR 25.889 v1.1.1 | Ericsson | Approved |
| R4-021441 | CR | Correction of CPICH RSCP measurement reporting range | Ericsson | Not agreed |
| R4-021442 | CR | Perfomance requirements for positioning in Cell_PCH and URA_PCH | Ericsson | Not agreed |
| R4-021443 | CR | Correction of UE Transmitted Power requirements in case of Compressed Mode gaps | Ericsson | Email approval |
| R4-021444 | CR | Correction of UE Transmitted Power requirements in case of Compressed Mode gaps | Ericsson | Email approval |
| R4-021445 | CR | Correction of UE Transmitted Power requirements in case of Compressed Mode gaps | Ericsson | Email approval |
| R4-021446 | Discussion | Discussion on linear or logarithmic L3 filtering | Ericsson | Noted |
| R4-021447 | CR | FDD GSM co-existence in the Same Geographic Area | Ericsson | Revised in R4-021658 |
| R4-021448 | CR | FDD GSM co-existence in the Same Geographic Area | Ericsson | Agreed |
| R4-021449 | CR | FDD GSM 850 / PCS 1900 co-existence in the Same Geographic Area | Ericsson | Agreed |
| R4-021450 | CR | FDD GSM co-existence in the Same Geographic Area | Ericsson | Revised in R4-021660 |
| R4-021451 | CR | FDD GSM co-existence in the Same Geographic Area | Ericsson | Agreed |
| R4-021452 | CR | FDD GSM 850 / PCS 1900 co-existence in the Same Geographic Area | Ericsson | Agreed |
| R4-021453 | Approval | HSDPA: Variable Reference Channel | Ericsson | Noted |
| R4-021454 | CR | New exclusion bands, requirements for telecommunication port and interpretation of measurement results | Ericsson, Nokia | Revised in R4-021656 |
| R4-021455 | CR | New exclusion bands, requirements for telecommunication port and interpretation of measurement results | Ericsson, Nokia | Revised in R4-021657 |
| R4-021456 | CR | New exclusion bands and interpretation of measurement results | Ericsson, Nokia | Agreed |
| R4-021457 | CR | New exclusion bands and interpretation of measurement results | Ericsson, Nokia | Agreed |
| R4-021458 | CR | Correction of Measurement Occasion Patterns for BSIC Reconfirmation | Ericsson | Revised in R4-021705 |
| R4-021459 | CR | Correction of Measurement Occasion Patterns for BSIC Reconfirmation | Ericsson | Revised in R4-021706 |
| R4-021460 | CR | Correcction of Measurement Occasion Patterns for BSIC Reconfirmation | Ericsson | Revised in R4-021707 |
| R4-021461 | Discussion | On Cell Search Performance | Ericsson | Noted |
| R4-021462 | Approval | Required Window size for measurements using IPDL | Ericsson | Not approved |
| R4-021463 | CR | Required Window size for measurements using IPDL | Ericsson | Revised in R4-021710 |
| R4-021464 | CR | Required Window size for measurements using IPDL | Ericsson | Revised in R4-021711 |
| R4-021465 | CR | Required Window size for measurements using IPDL | Ericsson | Revised in R4-021712 |
| R4-021466 | CR | BS requirements for IPDL | Ericsson | Withdrawn |
| R4-021467 | CR | BS requirements for IPDL | Ericsson | Withdrawn |
| R4-021468 | CR | BS requirements for IPDL | Ericsson | Withdrawn |
| R4-021469 | Discussion | Simulation results for minimum requirements of UE Phase Shift | Ericsson | Noted |
| R4-021470 | CR | UE Phase Shift requirements | Ericsson | Revised in R4-021721 |
| R4-021471 | CR | UE Timer accuracy | Ericsson | Revised in R4-021713 |
| R4-021472 | CR | UE Timer accuracy | Ericsson | Revised in R4-021714 |
| R4-021473 | CR | UE Timer accuracy | Ericsson | Revised in R4-021715 |
| R4-021474 | CR | Correction for TPC combining test case 1 | Nokia | Agreed |
| R4-021475 | CR | Correction for TPC combining test case 1 | Nokia | Agreed |
| R4-021476 | CR | Correction for TPC combining test case 1 | Nokia | Agreed |
| R4-021477 | CR | Correction for TPC combining test case 1 | Nokia | Withdrawn |
| R4-021478 | Discussion | Discussion for Correction for TPC command combining test case 1 | Nokia | Noted |
| R4-021479 | Discussion | Unit of layer 3 filtering | Motorola | Noted |
| R4-021480 | CR | IPDL performance requirements | Nokia | Revised in R4-021564 |
| R4-021481 | CR | IPDL performance requirements | Nokia | Revised in R4-021565 |
| R4-021482 | CR | IPDL performance requirements | Nokia | Revised in R4-021566 |
| R4-021483 | CR | IPDL performance requirements | Nokia | Withdrawn |
| R4-021484 | Approval | L3 filtering | Nokia | Not approved |
| R4-021485 | Discussion | Fading test case for cell identification requirement | Nokia | Noted |
| R4-021486 | CR | Correction of measurement and reporting capability requirements in CELL_DCH state in case of parallel measurements | Ericsson | Not agreed |
| R4-021487 | CR | Correction of measurement and reporting capability requirements in CELL_DCH state in case of parallel measurements | Ericsson | Not agreed |
| R4-021488 | CR | Correction of measurement and reporting capability requirements in CELL_DCH state in case of parallel measurements | Ericsson | Not agreed |
| R4-021489 | Approval | Simulation results for DCH demodulation tests with power control | Ericsson | Noted |
| R4-021490 | Approval | Revised version 1.3.0 of TR 25.951 | Nokia | Approved |
| R4-021491 | Approval | Transmitter characteristics for FDD Local area BS class. | Nokia | Approved |
| R4-021492 | Approval | Receiver characteristics for FDD Local area BS class. | Nokia | Approved |
| R4-021493 | Approval | Performance requirements for FDD Local area BS class. | Nokia | Not approved |
| R4-021494 | Approval | Changes in TS25.133 according to FDD Local area BS | Nokia | Approved |
| R4-021495 | CR | Blocking scenarios for Medium Range BS in FDD mode | Ericsson and Nokia | Agreed |
| R4-021496 | Approval | Simulation results for Medium Range BS in FDD mode | Ericsson and Nokia | Withdrawn |
| R4-021497 | CR | Changes in TS25.133 according to FDD Local area BS | Nokia | Agreed |
| R4-021498 | CR | Node B requirements for IPDL | Nokia | Withdrawn |
| R4-021499 | CR | Node B requirements for IPDL | Nokia | Withdrawn |
| R4-021500 | CR | Total received power density definition for the BS | Nokia | Revised in R4-021643 |
| R4-021501 | Approval | Text Proposal for Section 7.4 of TR 25.889 V1.1.0 "Viable deployment of UTRA in additional and diverse spectrum arrangements" | Nokia | Not approved |
| R4-021502 | CR | Correction of Verification times in CELL_FACH state for BSIC reconfirmation | Mitsubishi | Not agreed |
| R4-021503 | CR | Correction of Verification times in CELL_FACH state for BSIC reconfirmation | Mitsubishi | Not agreed |
| R4-021504 | CR | Correction of Verification times in CELL_FACH state for BSIC reconfirmation | Mitsubishi | Not agreed |
| R4-021505 | CR | Clarification of the W-CDMA interferer definition in BS requirements for ACS and blocking characteristics | Siemens | Revised in R4-021696 |
| R4-021506 | CR | Clarification of the W-CDMA interferer definition in BS requirements for ACS and blocking characteristics | Siemens | Revised in R4-021697 |
| R4-021507 | CR | Clarification of the W-CDMA interferer definition in BS conformance tests for ACS and blocking characteristics | Siemens | Revised in R4-021698 |
| R4-021508 | CR | Clarification of the W-CDMA interferer definition in BS conformance tests for ACS and blocking characteristics | Siemens | Revised in R4-021699 |
| R4-021509 | Approval | Proposed amendments to the current draft TR 25.889 V1.1.0 "Feasibility Study considering the viable deployment of UTRA in additional and diverse spectrum arrangements" | Siemens | Approved |
| R4-021510 | Approval | Proposed new text for draft TR 25.889, clause 8 "Feasibility of UTRA TDD in the band 2500-2690 MHz" | Siemens | Postponed |
| R4-021511 | CR | Correction to Pilot Interference Mitigation Technical Report | Intel | Agreed |
| R4-021512 | CR | New test environment: Extreme power supply for output power test | Mikom/ Allgon | Revised in R4-021678 |
| R4-021513 | CR | New test environment: Extreme power supply for output power test | Mikom/ Allgon | Revised in R4-021679 |
| R4-021514 | CR | Addition of Repeater configuration | Mikom/ Allgon | Revised in R4-021680 |
| R4-021515 | CR | Addition of Repeater configuration | Mikom/ Allgon | Revised in R4-021681 |
| R4-021516 | CR | Definition of the power to select the right table for the spectrum emission mask requirement. | Mikom/ Allgon | Agreed |
| R4-021517 | CR | Definition of the power to select the right table for the spectrum emission mask requirement. | Mikom/ Allgon | Agreed |
| R4-021518 | CR | EVM Test: Change requirement for the use of HSDPA. | Mikom/ Allgon | Agreed |
| R4-021519 | CR | EVM Test: Change requirement for the use of HSDPA. | Mikom/ Allgon | Not agreed |
| R4-021520 | CR | EVM Test: Change from Test Model 4 to Test Model 1 | Mikom/ Allgon | Agreed |
| R4-021521 | CR | Input intermodulation: Correction of co-location and addition of co-existence | Mikom/ Allgon | Revised in R4-021675 |
| R4-021522 | CR | Input intermodulation: Correction of co-location and addition of co-existence | Mikom/ Allgon | Revised in R4-021676 |
| R4-021523 | CR | Input intermodulation: Correction of co-location and addition of co-existence | Mikom/ Allgon | Revised in R4-021683 |
| R4-021524 | CR | Input intermodulation: Correction of co-location and addition of co-existence | Mikom/ Allgon | Revised in R4-021684 |
| R4-021525 | CR | Spurious emission: correction of the procedure | Mikom/ Allgon | Withdrawn |
| R4-021526 | CR | Spurious emission: correction of the procedure | Mikom/ Allgon | Withdrawn |
| R4-021527 | CR | Spurious emission: correction of the procedure | Mikom/ Allgon | Agreed |
| R4-021528 | CR | Spurious emission: correction of the procedure | Mikom/ Allgon | Agreed |
| R4-021529 | CR | Aligning of the requirement for "Output power" in extreme conditions with TS25.143 | Mikom/ Allgon | Agreed |
| R4-021530 | CR | Aligning of the requirement for "Output power" in extreme conditions with TS25.143 | Mikom/ Allgon | Agreed |
| R4-021531 | Discussion | HS-SCCH Performance | Qualcomm | Withdrawn |
| R4-021532 | Information | VRC Simulation Results | Qualcomm | Noted |
| R4-021533 | Discussion | VRC Test Approach | Qualcomm | Noted |
| R4-021534 | Discussion | L3 Filtering | Qualcomm | Noted |
| R4-021535 | Approval | Further Results for HSDPA Variable Reference Channels | Motorola | Noted |
| R4-021536 | Approval | Results for HSDPA Fixed Reference Channels in Open Loop Transmit Diversity Mode | Motorola | Revised in R4-021674 |
| R4-021537 | CR | Correction to Specified TBS for HSDPA Reference Channels | Motorola | Agreed |
| R4-021538 | Approval | TR25.890v1.2.0 | Motorola | Revised in R4-021708 |
| R4-021539 | Approval | HSDPA simulation assumptions for 3,84 Mcps TDD option with fixed reference channels | InterDigital | Approved |
| R4-021540 | Approval | HSDPA simulation results for 3,84 Mcps TDD, Fixed Reference Channel | InterDigital | Revised in R4-021665 |
| R4-021541 | CR | Addition of HSDPA UE requirements for 3,84 Mcps TDD option for 16QAM and QPSK for fixed reference channels | InterDigital | Revised in R4-021666 |
| R4-021542 | Approval | HSDPA simulation assumptions for 3,84 Mcps TDD option with variable reference channels | InterDigital | Revised in R4-021667 |
| R4-021543 | Approval | Simulation Results for 3.84 Mcps TDD HSDPA with Variable Reference Channel | InterDigital | Revised in R4-021668 |
| R4-021544 | CR | Addition of HSDPA UE requirements for 3,84 Mcps TDD option for 16QAM and QPSK for variable reference channels | InterDigital | Revised in R4-021669 |
| R4-021545 | CR | Correction of 16QAM EVM/PCDE testing for HSDPA for 3,84 Mcps TDD option | InterDigital | Revised in R4-021670 |
| R4-021546 | Discussion | IPDL attenuation simulation results | Siemens | Noted |
| R4-021547 | CR | BS IPDL requirement | Siemens | Revised in R4-021687 |
| R4-021548 | CR | BS IPDL requirement | Siemens | Revised in R4-021688 |
| R4-021549 | CR | BS IPDL test | Siemens | Revised in R4-021689 |
| R4-021550 | CR | BS IPDL test | Siemens | Revised in R4-021690 |
| R4-021551 | CR | Correction to table of regional requirements | Siemens | Agreed |
| R4-021552 | CR | Correction to table of regional requirements | Siemens | Agreed |
| R4-021553 | CR | General Release 5 corrections | Siemens | Agreed |
| R4-021554 | CR | General corrections to TS 25.141 | Siemens | Revised in R4-021671 |
| R4-021555 | CR | General corrections to TS 25.141 | Siemens | Revised in R4-021672 |
| R4-021556 | CR | General Release 5 corrections | Siemens | Agreed |
| R4-021557 | CR | Transmit intermodulation test correction | Siemens | Agreed |
| R4-021558 | CR | Transmit intermodulation test correction | Siemens | Agreed |
| R4-021559 | CR | Introduction of Rel-5 clarifications and small corrections in Rel-4 | Siemens | Agreed |
| R4-021560 | CR | Introduction of Rel-5 clarifications and small corrections in Rel-4 | Siemens | Revised in R4-021645 |
| R4-021561 | CR | Introduction of Rel-5 clarifications and small corrections in Rel-4 | Siemens | Revised in R4-021646 |
| R4-021562 | CR | Introduction of Rel-5 clarifications and small corrections in Rel-4 | Siemens | Withdrawn |
| R4-021563 | Discussion | HSDPA - Variable channel simulation assumptions | Siemens | Not approved |
| R4-021564 | CR | IPDL performance requirements | Nokia | Not agreed |
| R4-021565 | CR | IPDL performance requirements | Nokia | Not agreed |
| R4-021566 | CR | IPDL performance requirements | Nokia | Not agreed |
| R4-021567 | CR | Handover Test Case Correction for 1.28Mcps TDD | Siemens | Agreed |
| R4-021568 | CR | Handover Test Case Correction for 1.28Mcps TDD | Siemens | Agreed |
| R4-021569 | CR | Maximum allowed UL TX Power Correction for 1.28Mcps TDD | Siemens | Agreed |
| R4-021570 | CR | Maximum allowed UL TX Power Correction for 1.28Mcps TDD | Siemens | Agreed |
| R4-021571 | CR | Corrections to Idle Mode Requirements and Test Cases for 1.28Mcps TDD | Siemens | Agreed |
| R4-021572 | CR | Corrections to Idle Mode Requirements and Test Cases for 1.28Mcps TDD | Siemens | Agreed |
| R4-021573 | CR | P-CCPCH RSCP and CPICH RSCP signalling range extension | Siemens | Agreed |
| R4-021574 | CR | Name correction of logical and transport channels | Siemens | Agreed |
| R4-021575 | CR | Name correction of logical and transport channels | Siemens | Agreed |
| R4-021576 | CR | Name correction of logical and transport channels | Siemens | Agreed |
| R4-021577 | CR | Name correction of logical and transport channels | Siemens | Agreed |
| R4-021578 | Discussion | Simulation Results of Variable Reference Channel | Panasonic | Noted |
| R4-021579 | Discussion | On cell identification in compressed mode | Siemens | Noted |
| R4-021580 | Discussion | On cell identification in multipath fading conditions | Siemens | Noted |
| R4-021581 | Discussion | System aspects of new cell identification side conditions | Siemens | Withdrawn |
| R4-021582 | CR | Averaging period in ACLR test for 1.28 Mcps TDD option | Siemens | Agreed |
| R4-021583 | CR | Averaging period in ACLR test for 1.28 Mcps TDD option | Siemens | Agreed |
| R4-021584 | CR | Out of band gain | Mikom/ Allgon | Agreed |
| R4-021585 | CR | Out of band gain | Mikom/ Allgon | Agreed |
| R4-021586 | CR | Out of band gain | Mikom/ Allgon | Agreed |
| R4-021587 | CR | Out of band gain | Mikom/ Allgon | Agreed |
| R4-021588 | Discussion | Out of band gain | Mikom/ Allgon | Approved |
| R4-021589 | Information | Out of band gain | Mikom/ Allgon | Noted |
| R4-021590 | Approval | Low level RF interface on BS | Allgon | Not approved |
| R4-021591 | Approval | Simulation results for HSDPA variable reference channels | Sony | Noted |
| R4-021592 | Discussion | Test methodology for CQI | Sony | Noted |
| R4-021593 | LS in | LS on coexistence Between IMT-2000 TDD and FDD Radio Interface Technologies Within the Frequency Range 2 500-2 690 MHz Operating in Adjacent Bands and in the Same Geographical Area | ITU-R Ad Hoc | Noted |
| R4-021594 | Information | Report of TSG RAN/SA#17 | Vice chair | Noted |
| R4-021595 | Discussion | UTRA-FDD BS Receiver protection | Nortel Networks | Not approved |
| R4-021596 | Discussion | VRC test methodology | Sony Corporation | Noted |
| R4-021597 | CR | Spurious emission requirements for unsynchronized TDD operation | Siemens | Agreed |
| R4-021598 | CR | Spurious emission requirements for unsynchronized TDD operation | Siemens | Agreed |
| R4-021599 | CR | Spurious emission requirements for unsynchronized TDD operation | Siemens | Agreed |
| R4-021600 | Discussion | HSDPA simulation results for 1.28 Mcps TDD option | Samsung | Noted |
| R4-021601 | Approval | Comparison of HSDPA simulation results for 1.28 Mcps TDD option | Samsung, Siemens, Panasonic | Revised in R4-021637 |
| R4-021602 | CR | HSDPA UE requirements for 1.28 Mcps TDD option for 16QAM and QPSK for fixed reference channels | Samsung, Siemens, Panasonic | Revised in R4-021638 |
| R4-021603 | LS in | Response to LS on the definition of CQI | TSG RAN WG1 | Noted |
| R4-021604 | Discussion | Updated HSDPA simulation results for 1.28Mcps TDD option | Panasonic | Noted |
| R4-021605 | Information | TR 25.887 Beamforming enhancements (R1-021260) | 3GPP Support | Noted |
| R4-021606 | LS in | LS on reference compressed mode pattern for signalling test case | TSG T1SIG SWG | Noted |
| R4-021607 | Discussion | UE maximum output power with HS-DPCCH | Nokia | Noted |
| R4-021608 | CR | UE maximum output power with HS-DPCCH | Nokia | Not agreed |
| R4-021609 | CR | Addition of TX Diversity timing accuracy test | Agilent | Revised in R4-021655 |
| R4-021610 | Information | Draft TR 35.901 on Statistical theory of measurements | Agilent | Noted |
| R4-021611 | Information | Optimizing code channel allocation in the presence of signal clipping | Agilent | Noted |
| R4-021612 | Approval | Proposal to correct Transmit Diversity in RAN4 specifications | Nortel Networks | Approved |
| R4-021613 | Discussion | BS IPDL requirement | Nortel Networks | Noted |
| R4-021614 | CR | BS IPDL requirement | Nortel Networks | Revised in R4-021631 |
| R4-021615 | CR | BS IPDL requirement | Nortel Networks | Revised in R4-021632 |
| R4-021616 | Approval | Text Proposal for Section 7.5 of TR 25.889 V1.1.0 "Viable deployment of UTRA in additional and diverse spectrum arrangements" | Nokia | Postponed |
| R4-021617 | Approval | Simulation results for Open Loop and Closed Loop Mode 1 Transmit Diversity in HSDPA | Nokia | Approved |
| R4-021618 | Discussion | Simulation results for modified DCH demodulation tests in multi-path fading conditions | Nokia | Noted |
| R4-021619 | Approval | HSDPA Variable Reference Channel Simulation Results | Nokia | Noted |
| R4-021620 | CR | Introduction of requirements for HSDPA UE categories 11 and 12 | Nokia | Revised in R4-021709 |
| R4-021621 | Approval | Interpretation of UE radio access parameter "Need for compressed Mode" when FALSE | Mitsubishi Electric Telecom Europe | Noted |
| R4-021622 | CR | CPICH RSCP report mapping | NTT DoCoMo | Revised in R4-021663 |
| R4-021623 | Discussion | Repeater adjacent band gain issue | Orange | Noted |
| R4-021624 | LS in | LS on TDD Reference Measurement Channels | TSG RAN WG1 | Noted |
| R4-021625 | LS in | LS on application of Test Tolerances to RRM tests | TSG T WG1 RF SWG | Noted |
| R4-021626 | Approval | Fixed Reference Channels for FDD UE Categories 11 & 12 | Motorola | Noted |
| R4-021627 | Discussion | Discussion of the requirements for QPSK only HS-DSCH categories | Nokia | Noted |
| R4-021628 | LS out | draft LS in response to the LS from T1 on application of Test Tolerances to RRM tests | Nokia | Revised in R4-021739 |
| R4-021629 | Discussion | HSDPA VRC simulation results | NEC | Noted |
| R4-021630 | LS in | LS response on uplink DPCCH transmission start timing in Hard Handover | TSG RAN WG1 | Noted |
| R4-021631 | CR | BS IPDL requirement | Nortel Networks | Withdrawn |
| R4-021632 | CR | BS IPDL requirement | Nortel Networks | Withdrawn |
| R4-021633 | Information | FRC with HS-SCCH impairments | Sony | Noted |
| R4-021634 | Approval | Simulation results for FRC with STTD | Sony | Noted |
| R4-021635 | Approval | Proposal of maximum output power for Local area BS | DoCoMo | Agreed |
| R4-021636 | Information | Simulation results of CSICH when CA is active | Samsung Electronics.co.ltd | Noted |
| R4-021637 | Approval | Comparison of HSDPA simulation results for 1.28 Mcps TDD option | Samsung, Siemens, Panasonic, Motorola | Noted |
| R4-021638 | CR | HSDPA UE requirements for 1.28 Mcps TDD option for 16QAM and QPSK for fixed reference channels | Samsung, Siemens, Panasonic, Motorola | Agreed |
| R4-021639 | Information | HS-SCCH Performance Revised Results | Motorola | Noted |
| R4-021640 | Approval | IPDL vote document | Chairman | Noted |
| R4-021641 | Information | Report of the cell identification evening session | NEC | Noted |
| R4-021642 | Information | Relative performance of QPSK FRC proposals | Motorola | Noted |
| R4-021643 | CR | Total received power density definition for the BS | Nokia | Agreed |
| R4-021644 | CR | Total received power density definition for the BS | Nokia | Agreed |
| R4-021645 | CR | Introduction of Rel-5 clarifications and small corrections in Rel-4 | Siemens | Agreed |
| R4-021646 | CR | Introduction of Rel-5 clarifications and small corrections in Rel-4 | Siemens | Agreed |
| R4-021647 | CR | Correction of adjacent channel leakage power definition | Siemens | Agreed |
| R4-021648 | CR | Correction of adjacent channel leakage power definition | Siemens | Agreed |
| R4-021649 | LS in | LS on Uplink reference measurement channels | TSG T WG1 | Noted |
| R4-021650 | Discussion | Simulation result for HSDPA VRC | DoCoMo | Noted |
| R4-021651 | CR | Correction of UE parameters for Random Access Test | DoCoMo | Agreed |
| R4-021652 | CR | Correction of UE parameters for Random Access Test | DoCoMo | Agreed |
| R4-021653 | CR | Correction of UE parameters for Random Access Test | DoCoMo | Agreed |
| R4-021654 | CR | Clarification of TX diversity requirements | Nortel | Agreed |
| R4-021655 | CR | Addition of TX Diversity timing accuracy test | Agilent | Agreed |
| R4-021656 | CR | New exclusion bands, requirements for telecommunication port and interpretation of measurement results | Ericsson, Nokia | Agreed |
| R4-021657 | CR | New exclusion bands, requirements for telecommunication port and interpretation of measurement results | Ericsson, Nokia | Agreed |
| R4-021658 | CR | FDD GSM co-existence in the Same Geographic Area | Ericsson | Agreed |
| R4-021659 | CR | FDD GSM co-existence in the Same Geographic Area | Ericsson | Agreed |
| R4-021660 | CR | FDD GSM co-existence in the Same Geographic Area | Ericsson | Agreed |
| R4-021661 | CR | FDD GSM co-existence in the Same Geographic Area | Ericsson | Agreed |
| R4-021662 | Discussion | UE performance requirements | Motorola | Noted |
| R4-021663 | CR | CPICH RSCP report mapping | NTT DoCoMo | Agreed |
| R4-021664 | Discussion | Simulation assumptions for inter FDD cell identification with compressed mode | Mitsubishi | Noted |
| R4-021665 | Approval | HSDPA simulation results for 3,84 Mcps TDD, Fixed Reference Channel | InterDigital | Noted |
| R4-021666 | CR | Addition of HSDPA UE requirements for 3,84 Mcps TDD option for 16QAM and QPSK for fixed reference channels | InterDigital | Agreed |
| R4-021667 | Approval | HSDPA simulation assumptions for 3,84 Mcps TDD option with variable reference channels | InterDigital | Approved |
| R4-021668 | Approval | Simulation Results for 3.84 Mcps TDD HSDPA with Variable Reference Channel | InterDigital | Noted |
| R4-021669 | CR | Addition of HSDPA UE requirements for 3,84 Mcps TDD option for 16QAM and QPSK for variable reference channels | InterDigital | Agreed |
| R4-021670 | CR | Correction of 16QAM EVM/PCDE testing for HSDPA for 3,84 Mcps TDD option | InterDigital | Agreed |
| R4-021671 | CR | General corrections to TS 25.141 | Siemens | Agreed |
| R4-021672 | CR | General corrections to TS 25.141 | Siemens | Agreed |
| R4-021673 | Approval | FRC results with TBS adjustment | Motorola | Noted |
| R4-021674 | Approval | Results for HSDPA Fixed Reference Channels in Open Loop Transmit Diversity Mode (updated) | Motorola | Noted |
| R4-021675 | CR | Input intermodulation: Correction of co-location and addition of co-existence | Mikom/ Allgon | Agreed |
| R4-021676 | CR | Input intermodulation: Correction of co-location and addition of co-existence | Mikom/ Allgon | Agreed |
| R4-021677 | CR | EVM Test: Change requirement for the use of HSDPA. | Mikom/ Allgon | Agreed |
| R4-021678 | CR | New test environment: Extreme power supply for output power test | Mikom/ Allgon | Agreed |
| R4-021679 | CR | New test environment: Extreme power supply for output power test | Mikom/ Allgon | Agreed |
| R4-021680 | CR | Addition of Repeater configuration | Mikom/ Allgon | Agreed |
| R4-021681 | CR | Addition of Repeater configuration | Mikom/ Allgon | Agreed |
| R4-021682 | CR | EVM Test: Change requirement for the use of HSDPA. | Mikom/ Allgon | Agreed |
| R4-021683 | CR | Input intermodulation: Correction of co-location and addition of co-existence | Mikom/ Allgon | Agreed |
| R4-021684 | CR | Input intermodulation: Correction of co-location and addition of co-existence | Mikom/ Allgon | Agreed |
| R4-021685 | CR | EVM Test: Change from Test Model 4 to Test Model 1 | Mikom/ Allgon | Agreed |
| R4-021686 | LS in | Response to LS on Rate Adaptation of AMR Codec | TSG RAN WG2 | Noted |
| R4-021687 | CR | BS IPDL requirement | Siemens | Agreed |
| R4-021688 | CR | BS IPDL requirement | Siemens | Agreed |
| R4-021689 | CR | BS IPDL test | Siemens | Agreed |
| R4-021690 | CR | BS IPDL test | Siemens | Agreed |
| R4-021691 | LS in | LS(#2) on Rate Adaptation of AMR Codec (Response to R2-023158) | TSG SA WG4 | Noted |
| R4-021692 | Discussion | CQI working definition | Motorola, Sony | Noted |
| R4-021693 | Information | Comparision of VRC results | Motorola et al. | Noted |
| R4-021694 | CR | Introduction of Base Station Classes | Ericsson | Agreed |
| R4-021695 | CR | Introduction of Base Station Classes | Ericsson | Agreed |
| R4-021696 | CR | Clarification of the W-CDMA interferer definition in BS requirements for ACS and blocking characteristics | Spirent | Revised in R4-021744 |
| R4-021697 | CR | Clarification of the W-CDMA interferer definition in BS requirements for ACS and blocking characteristics | Spirent | Revised in R4-021745 |
| R4-021698 | CR | Clarification of the W-CDMA interferer definition in BS conformance tests for ACS and blocking characteristics | Spirent | Revised in R4-021746 |
| R4-021699 | CR | Clarification of the W-CDMA interferer definition in BS conformance tests for ACS and blocking characteristics | Spirent | Revised in R4-021747 |
| R4-021700 | CR | Corrections to 3.84 Mcps TDD reference measurement channels | IPWireless | Revised in R4-021722 |
| R4-021701 | CR | Corrections to 3.84 TDD reference measurement channels | IPWireless | Agreed |
| R4-021702 | CR | Corrections to 3.84 Mcps TDD reference measurement channels | IPWireless | Agreed |
| R4-021703 | CR | Corrections to 1.28 Mcps TDD reference measurement channels | IPWireless | Revised in R4-021723 |
| R4-021704 | CR | Corrections to 1.28 Mcps TDD reference measurement channels | IPWireless | Agreed |
| R4-021705 | CR | Correction of Measurement Occasion Patterns for BSIC Reconfirmation | Ericsson, Mitsubishi | Agreed |
| R4-021706 | CR | Correction of Measurement Occasion Patterns for BSIC Reconfirmation | Ericsson, Mitsubishi | Agreed |
| R4-021707 | CR | Correcction of Measurement Occasion Patterns for BSIC Reconfirmation | Ericsson, Mitsubishi | Agreed |
| R4-021708 | Approval | TR25.890v1.2.0 | Motorola | Approved |
| R4-021709 | CR | Introduction of requirements for HSDPA UE categories 11 and 12 | Nokia | Agreed |
| R4-021710 | CR | Required Window size for measurements using IPDL | Ericsson | Revised in R4-021741 |
| R4-021711 | CR | Required Window size for measurements using IPDL | Ericsson | Revised in R4-021742 |
| R4-021712 | CR | Required Window size for measurements using IPDL | Ericsson | Revised in R4-021743 |
| R4-021713 | CR | UE Timer accuracy | Ericsson | Agreed |
| R4-021714 | CR | UE Timer accuracy | Ericsson | Agreed |
| R4-021715 | CR | UE Timer accuracy | Ericsson | Agreed |
| R4-021716 | LS out | LS on Rate Adaptation of AMR Codec | NEC | Revised in R4-021738 |
| R4-021717 | CR | Corrections to cell reselection test cases | Nokia | Email approval |
| R4-021718 | CR | Corrections to cell reselection test cases | Nokia | Email approval |
| R4-021719 | CR | Corrections to cell reselection test cases | Nokia | Email approval |
| R4-021720 | LS out | LS to GERAN on FDD/GSM co-existence | Ericsson | Approved |
| R4-021721 | CR | UE Phase Shift requirements | Ericsson | Not agreed |
| R4-021722 | CR | Corrections to 3.84 Mcps TDD reference measurement channels | IPWireless | Agreed |
| R4-021723 | CR | Corrections to 1.28 Mcps TDD reference measurement channels | IPWireless | Agreed |
| R4-021724 | CR | Corrections to TDD 3.84Mcps Reference Measurement Channels | IPWireless | Agreed |
| R4-021725 | CR | Corrections to TDD 3.84Mcps Reference Measurement Channels | IPWireless | Agreed |
| R4-021726 | CR | Corrections to TDD 3.84Mcps Reference Measurement Channels | IPWireless | Agreed |
| R4-021727 | CR | Corrections to TDD 1.28Mcps Reference Measurement Channels | IPWireless | Agreed |
| R4-021728 | CR | Corrections to TDD 1.28Mcps Reference Measurement Channels | IPWireless | Agreed |
| R4-021729 | CR | Corrections to TDD 3.84Mcps Reference Measurement Channels | IPWireless | Withdrawn |
| R4-021730 | CR | Corrections to TDD 3.84Mcps Reference Measurement Channels | IPWireless | Withdrawn |
| R4-021731 | CR | Corrections to TDD 3.84Mcps Reference Measurement Channels | IPWireless | Withdrawn |
| R4-021732 | CR | Corrections to TDD 1.28Mcps Reference Measurement Channels | IPWireless | Withdrawn |
| R4-021733 | CR | Corrections to TDD 1.28Mcps Reference Measurement Channels | IPWireless | Withdrawn |
| R4-021734 | Approval | TR25.951 v1.4.0 | Nokia | Approved |
| R4-021735 | Approval | Status Report for FDD BS Classification WI | Nokia | Approved |
| R4-021736 | Approval | Clarification of the WCDMA interferer | Spirent | Noted |
| R4-021737 | Approval | Proposal for a Study Item on an optional Low Level RF Interface | Tekmar | Not approved |
| R4-021738 | LS out | LS on Rate Adaptation of AMR Codec | NEC | Approved |
| R4-021739 | LS out | draft LS in response to the LS from T1 on application of Test Tolerances to RRM tests | Nokia | Approved |
| R4-021740 | LS out | LS on application of Test Tolerances to intra-frequency cell reselection test cases | Nokia | Email approval |
| R4-021741 | CR | Required Window size for measurements using IPDL | Ericsson | Agreed |
| R4-021742 | CR | Required Window size for measurements using IPDL | Ericsson | Agreed |
| R4-021743 | CR | Required Window size for measurements using IPDL | Ericsson | Agreed |
| R4-021744 | CR | Clarification of the W-CDMA interferer definition in BS requirements for ACS and blocking characteristics | Spirent | Email approval |
| R4-021745 | CR | Clarification of the W-CDMA interferer definition in BS requirements for ACS and blocking characteristics | Spirent | Email approval |
| R4-021746 | CR | Clarification of the W-CDMA interferer definition in BS conformance tests for ACS and blocking characteristics | Spirent | Email approval |
| R4-021747 | CR | Clarification of the W-CDMA interferer definition in BS conformance tests for ACS and blocking characteristics | Spirent | Email approval |

Annex C: List of agreed CRs

| Spec | CR | R | Cat | Rel | R4 Tdoc | Title | Work Item |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 25.101 | 194 |  | F | R99 | R4-021474 | Correction for TPC combining test case 1 | TEI |
| 25.101 | 195 |  | A | Rel-5 | R4-021476 | Correction for TPC combining test case 1 | TEI |
| 25.101 | 196 |  | A | Rel-4 | R4-021475 | Correction for TPC combining test case 1 | TEI |
| 25.101 | 198 |  | F | Rel-5 | R4-021537 | Correction to Specified TBS for HSDPA Reference Channels | HSDPA-RF |
| 25.101 | 200 | 1 | B | Rel-5 | R4-021709 | Introduction of requirements for HSDPA UE categories 11 and 12 | HSDPA-RF |
| 25.102 | 127 | 1 | B | Rel-5 | R4-021666 | Addition of HSDPA UE requirements for 3,84 Mcps TDD option for 16QAM and QPSK for fixed reference channels | HSDPA-RF |
| 25.102 | 128 | 1 | B | Rel-5 | R4-021669 | Addition of HSDPA UE requirements for 3,84 Mcps TDD option for 16QAM and QPSK for variable reference channels | HSDPA-RF |
| 25.102 | 129 |  | F | Rel-4 | R4-021559 | Introduction of Rel-5 clarifications and small corrections in Rel-4 | TEI4 |
| 25.102 | 130 |  | F | Rel-4 | R4-021576 | Name correction of logical and transport channels | TEI4 |
| 25.102 | 131 |  | A | Rel-5 | R4-021577 | Name correction of logical and transport channels | TEI4 |
| 25.102 | 132 | 1 | F | Rel-5 | R4-021638 | HSDPA UE requirements for 1.28 Mcps TDD option for 16QAM and QPSK for fixed reference channels | HSDPA-RF |
| 25.104 | 148 | 1 | B | Rel-6 | R4-021694 | Introduction of Base Station Classes | RInImp-BSClass-FDD |
| 25.104 | 149 | 1 | A | Rel-4 | R4-021658 | FDD GSM co-existence in the Same Geographic Area | TEI4 |
| 25.104 | 150 |  | F | Rel-5 | R4-021449 | FDD GSM 850 / PCS 1900 co-existence in the Same Geographic Area | TEI5 |
| 25.104 | 153 |  | A | Rel-5 | R4-021448 | FDD GSM co-existence in the Same Geographic Area | TEI4 |
| 25.104 | 159 | 1 | F | Rel-4 | R4-021687 | BS IPDL requirement | TEI4 |
| 25.104 | 160 | 1 | A | Rel-5 | R4-021688 | BS IPDL requirement | TEI4 |
| 25.104 | 161 |  | F | Rel-4 | R4-021551 | Correction to table of regional requirements | TEI4 |
| 25.104 | 162 |  | A | Rel-5 | R4-021552 | Correction to table of regional requirements | TEI4 |
| 25.104 | 163 |  | F | Rel-5 | R4-021553 | General Release 5 corrections | TEI5 |
| 25.104 | 166 |  | F | Rel-5 | R4-021654 | Clarification of TX diversity requirements | TEI5 |
| 25.104 | 167 |  | F | R99 | R4-021659 | FDD GSM co-existence in the Same Geographic Area | TEI4 |
| 25.105 | 128 | 1 | F | Rel-4 | R4-021645 | Introduction of Rel-5 clarifications and small corrections in Rel-4 | TEI4 |
| 25.105 | 129 |  | F | Rel-4 | R4-021574 | Name correction of logical and transport channels | TEI4 |
| 25.105 | 130 |  | A | Rel-5 | R4-021575 | Name correction of logical and transport channels | TEI4 |
| 25.105 | 131 |  | F | R99 | R4-021597 | Spurious emission requirements for unsynchronized TDD operation | TEI |
| 25.105 | 132 |  | F | Rel-4 | R4-021598 | Spurious emission requirements for unsynchronized TDD operation | TEI4 |
| 25.105 | 133 |  | F | Rel-5 | R4-021599 | Spurious emission requirements for unsynchronized TDD operation | TEI5 |
| 25.105 | 134 |  | F | Rel-5 | R4-021647 | Correction of adjacent channel leakage power definition | TEI5 |
| 25.105 | 135 | 1 | F | R99 | R4-021722 | Corrections to 3.84 Mcps TDD reference measurement channels | TEI |
| 25.105 | 136 |  | A | Rel-4 | R4-021701 | Corrections to 3.84 Mcps TDD reference measurement channels | TEI |
| 25.105 | 137 |  | A | Rel-5 | R4-021702 | Corrections to 3.84 Mcps TDD reference measurement channels | TEI |
| 25.105 | 138 | 1 | F | Rel-4 | R4-021723 | Corrections to 1.28 Mcps TDD reference measurement channels | LCRTDD-RF |
| 25.105 | 139 |  | A | Rel-5 | R4-021704 | Corrections to 1.28 Mcps TDD reference measurement channels | LCRTDD-RF |
| 25.106 | 010 |  | F | Rel-5 | R4-021518 | EVM Test: Change requirement for the use of HSDPA. | RInImp-REP |
| 25.106 | 011 | 1 | F | Rel-4 | R4-021675 | Input intermodulation: Correction of co-location and addition of co-existence | RInImp-REP |
| 25.106 | 012 | 1 | A | Rel-5 | R4-021676 | Input intermodulation: Correction of co-location and addition of co-existence | RInImp-REP |
| 25.106 | 015 |  | F | Rel-4 | R4-021529 | Aligning of the requirement for "Output power" in extreme conditions with TS25.143 | RInImp-REP |
| 25.106 | 016 |  | A | Rel-5 | R4-021530 | Aligning of the requirement for "Output power" in extreme conditions with TS25.143 | RInImp-REP |
| 25.106 | 017 |  | F | Rel-4 | R4-021586 | Out of band gain | RInImp-REP |
| 25.106 | 018 |  | A | Rel-5 | R4-021587 | Out of band gain | RInImp-REP |
| 25.106 | 019 |  | F | Rel-4 | R4-021677 | EVM Test: Change requirement for the use of HSDPA. | RInImp-REP |
| 25.113 | 018 | 1 | F | Rel-4 | R4-021656 | New exclusion bands, requirements for telecommunication port and interpretation of measurement results | TEI4 |
| 25.113 | 019 | 1 | A | Rel-5 | R4-021657 | New exclusion bands, requirements for telecommunication port and interpretation of measurement results | TEI4 |
| 25.123 | 279 |  | F | Rel-4 | R4-021567 | Handover Test Case Correction for 1.28Mcps TDD | LCRTDD-RF |
| 25.123 | 280 |  | A | Rel-5 | R4-021568 | Handover Test Case Correction for 1.28Mcps TDD | LCRTDD-RF |
| 25.123 | 281 |  | F | Rel-4 | R4-021569 | Maximum allowed UL TX Power Correction for 1.28Mcps TDD | LCRTDD-RF |
| 25.123 | 282 |  | A | Rel-5 | R4-021570 | Maximum allowed UL TX Power Correction for 1.28Mcps TDD | LCRTDD-RF |
| 25.123 | 283 |  | F | Rel-4 | R4-021571 | Corrections to Idle Mode Requirements and Test Cases for 1.28Mcps TDD | LCRTDD-RF |
| 25.123 | 284 |  | A | Rel-5 | R4-021572 | Corrections to Idle Mode Requirements and Test Cases for 1.28Mcps TDD | LCRTDD-RF |
| 25.123 | 285 |  | C | Rel-5 | R4-021573 | P-CCPCH RSCP and CPICH RSCP signalling range extension | LCRTDD-RF |
| 25.133 | 437 | 1 | F | R99 | R4-021403 | Correction of interruption time in FDD/FDD Hard Handover | TEI |
| 25.133 | 438 | 1 | A | Rel-4 | R4-021404 | Correction of interruption time in FDD/FDD Hard Handover | TEI |
| 25.133 | 439 | 1 | A | Rel-5 | R4-021405 | Correction of interruption time in FDD/FDD Hard Handover | TEI |
| 25.133 | 474 |  | B | Rel-6 | R4-021432 | RRM requirement changes for FDD Base Station Classification | RInImp-BSClass-FDD |
| 25.133 | 478 | 1 | F | R99 | R4-021705 | Correction of Measurement Occasion Patterns for BSIC Reconfirmation | TEI |
| 25.133 | 479 | 1 | A | Rel-5 | R4-021707 | Correcction of Measurement Occasion Patterns for BSIC Reconfirmation | TEI |
| 25.133 | 480 | 2 | F | R99 | R4-021741 | Required Window size for measurements using IPDL | TEI |
| 25.133 | 481 | 2 | A | Rel-5 | R4-021743 | Required Window size for measurements using IPDL | TEI |
| 25.133 | 482 | 1 | F | R99 | R4-021713 | UE Timer accuracy | TEI |
| 25.133 | 483 | 1 | A | Rel-5 | R4-021715 | UE Timer accuracy | TEI |
| 25.133 | 489 | 1 | A | Rel-4 | R4-021706 | Correction of Measurement Occasion Patterns for BSIC Reconfirmation | TEI |
| 25.133 | 490 | 2 | A | Rel-4 | R4-021742 | Required Window size for measurements using IPDL | TEI |
| 25.133 | 491 | 1 | A | Rel-4 | R4-021714 | UE Timer accuracy | TEI |
| 25.133 | 497 |  | B | Rel-6 | R4-021497 | Changes in TS25.133 according to FDD Local area BS | RInImp-BSClass-FDD |
| 25.133 | 498 | 1 | A | Rel-5 | R4-021643 | Total received power density definition for the BS | TEI |
| 25.133 | 502 | 1 | F | Rel-5 | R4-021663 | CPICH RSCP report mapping | TEI5 |
| 25.133 | 503 |  | F | Rel-4 | R4-021644 | Total received power density definition for the BS | TEI4 |
| 25.133 | 504 |  | F | R99 | R4-021651 | Correction of UE parameters for Random Access Test | TEI |
| 25.133 | 505 |  | A | Rel-4 | R4-021652 | Correction of UE parameters for Random Access Test | TEI |
| 25.133 | 506 |  | A | Rel-5 | R4-021653 | Correction of UE parameters for Random Access Test | TEI |
| 25.141 | 247 |  | F | Rel-5 | R4-021407 | Correction on PN9 seed setting in Test Model 5 | HSDPA-RF |
| 25.141 | 249 | 1 | B | Rel-6 | R4-021695 | Introduction of Base Station Classes | RInImp-BSClass-FDD |
| 25.141 | 250 | 1 | A | Rel-4 | R4-021660 | FDD GSM co-existence in the Same Geographic Area | TEI4 |
| 25.141 | 251 |  | F | Rel-5 | R4-021452 | FDD GSM 850 / PCS 1900 co-existence in the Same Geographic Area | TEI5 |
| 25.141 | 252 |  | A | Rel-5 | R4-021451 | FDD GSM co-existence in the Same Geographic Area | TEI4 |
| 25.141 | 255 | 1 | F | Rel-4 | R4-021689 | BS IPDL test | TEI4 |
| 25.141 | 256 | 1 | A | Rel-5 | R4-021690 | BS IPDL test | TEI4 |
| 25.141 | 257 | 1 | F | Rel-4 | R4-021671 | General corrections to TS 25.141 | TEI4 |
| 25.141 | 258 | 1 | A | Rel-5 | R4-021672 | General corrections to TS 25.141 | TEI4 |
| 25.141 | 259 |  | F | Rel-5 | R4-021556 | General Release 5 corrections | TEI5 |
| 25.141 | 260 |  | F | Rel-4 | R4-021557 | Transmit intermodulation test correction | TEI4 |
| 25.141 | 261 |  | A | Rel-5 | R4-021558 | Transmit intermodulation test correction | TEI4 |
| 25.141 | 263 | 1 | F | Rel-5 | R4-021655 | Addition of TX Diversity timing accuracy test | RinImp |
| 25.141 | 264 |  | F | R99 | R4-021661 | FDD GSM co-existence in the Same Geographic Area | TEI4 |
| 25.142 | 146 | 1 | F | Rel-5 | R4-021670 | Correction of 16QAM EVM/PCDE testing for HSDPA for 3,84 Mcps TDD option | HSDPA-RF |
| 25.142 | 147 | 1 | F | Rel-4 | R4-021646 | Introduction of Rel-5 clarifications and small corrections in Rel-4 | TEI4 |
| 25.142 | 148 |  | F | Rel-4 | R4-021582 | Averaging period in ACLR test for 1.28 Mcps TDD option | LCRTDD-RF |
| 25.142 | 149 |  | A | Rel-5 | R4-021583 | Averaging period in ACLR test for 1.28 Mcps TDD option | LCRTDD-RF |
| 25.142 | 150 |  | F | Rel-5 | R4-021648 | Correction of adjacent channel leakage power definition | TEI5 |
| 25.142 | 151 |  | F | R99 | R4-021724 | Corrections to TDD 3.84Mcps Reference Measurement Channels | TEI4 |
| 25.142 | 152 |  | A | Rel-4 | R4-021725 | Corrections to TDD 3.84Mcps Reference Measurement Channels | TEI4 |
| 25.142 | 153 |  | A | Rel-5 | R4-021726 | Corrections to TDD 3.84Mcps Reference Measurement Channels | TEI4 |
| 25.142 | 154 |  | F | Rel-4 | R4-021727 | Corrections to TDD 1.28Mcps Reference Measurement Channels | LCRTDD-RF |
| 25.142 | 155 |  | A | Rel-5 | R4-021728 | Corrections to TDD 1.28Mcps Reference Measurement Channels | LCRTDD-RF |
| 25.143 | 013 | 1 | F | Rel-4 | R4-021678 | New test environment: Extreme power supply for output power test | RInImp-REP |
| 25.143 | 014 | 1 | A | Rel-5 | R4-021679 | New test environment: Extreme power supply for output power test | RInImp-REP |
| 25.143 | 015 | 1 | F | Rel-4 | R4-021680 | Addition of Repeater configuration | RInImp-REP |
| 25.143 | 016 | 1 | A | Rel-5 | R4-021681 | Addition of Repeater configuration | RInImp-REP |
| 25.143 | 017 |  | F | Rel-4 | R4-021516 | Definition of the power to select the right table for the spectrum emission mask requirement. | RInImp-REP |
| 25.143 | 018 |  | A | Rel-5 | R4-021517 | Definition of the power to select the right table for the spectrum emission mask requirement. | RInImp-REP |
| 25.143 | 020 |  | F | Rel-5 | R4-021520 | EVM Test: Change from Test Model 4 to Test Model 1 | RInImp-REP |
| 25.143 | 021 | 1 | F | Rel-4 | R4-021683 | Input intermodulation: Correction of co-location and addition of co-existence | RInImp-REP |
| 25.143 | 022 | 1 | A | Rel-5 | R4-021684 | Input intermodulation: Correction of co-location and addition of co-existence | RInImp-REP |
| 25.143 | 023 |  | F | Rel-4 | R4-021527 | Spurious emission: correction of the procedure | RInImp-REP |
| 25.143 | 024 |  | A | Rel-5 | R4-021528 | Spurious emission: correction of the procedure | RInImp-REP |
| 25.143 | 025 |  | F | Rel-4 | R4-021584 | Out of band gain | RInImp-REP |
| 25.143 | 026 |  | A | Rel-5 | R4-021585 | Out of band gain | RInImp-REP |
| 25.143 | 027 |  | F | Rel-4 | R4-021682 | EVM Test: Change requirement for the use of HSDPA. | RInImp-REP |
| 25.143 | 028 |  | F | Rel-4 | R4-021685 | EVM Test: Change from Test Model 4 to Test Model 1 | RInImp-REP |
| 25.942 | 010 |  | B | Rel-6 | R4-021495 | Blocking scenarios for Medium Range BS in FDD mode | RInImp-BSClass-FDD |
| 25.991 | 001 |  | F | Rel-5 | R4-021511 | Correction to Pilot Interference Mitigation Technical Report | RInImp-UERecPerf |
| 34.124 | 009 |  | F | Rel-4 | R4-021456 | New exclusion bands and interpretation of measurement results | TEI4 |
| 34.124 | 010 |  | A | Rel-5 | R4-021457 | New exclusion bands and interpretation of measurement results | TEI4 |