3GPP TSG-RAN WG4 #50bis	R4-091335

March 23-27, 2009

Seoul, Korea

Agenda item:	7.5, FDD HeNB RF Requirements

Source: 	Qualcomm Europe

Title: 	Considerations on HeNBs

Document for:	Discussion

# 1	Introduction

Home eNodeBs (HeNBs) or femto cells are a useful mechanism of improving coverage and capacity in LTE networks. It is well understood that HeNBs have several differences with macro eNBs. For example, HeNBs have lower power, may use a consumer backhaul, may be autonomously deployed in an unplanned manner and may make use of Closed Subscriber Group (CSG) restricting some UEs from accessing them. Additionally, the radio propagation conditions could be different from those of macro eNBs.

In this document, we present some additional issues that are specific to LTE femto cells (HeNBs), and recommend that RAN4 consider these in its study of HeNB RF requirements.

# 2	Discussion

## 2.1 Interference Conditions and Mitigation

In LTE, different powers can be used for various control and data channels.  This in turn causes different interference levels on different subcarriers. The impact of such interference on neighboring carriers should be investigated, since there should be no negative impact especially if the adjacent carrier is owned by a different operator. Note that HeNBs should detect the presence of both LTE and WCDMA systems used by other operators on neighboring carriers, and choose their power accordingly [1].

Furthermore, the impact of such interference within the carrier on various control and data channels is different. The impact on all of these channels should be studied carefully when analyzing different proposals. For example, reservation of some frequency resources could lead to better performance for the PDSCH channel. However, it may not lead to better performance for the control channels. Also, different types of deployment scenarios (co-channel deployment of HeNBs and macros, open access HeNBs, CSG HeNBs, pico cells) etc should be considered.

## 2.2 Multiple Transmit Antennas

Multiple TX antennas may be present in HeNBs. This can be used for spatial interference avoidance using Cooperative Beamforming (CBF). In Cooperative Beamforming, a HeNB2 forms a beam that improves the SINR for UE1 being served by HeNB1. The beam can be chosen in two different ways. In the first case, the beam could be chosen based on direct channel measurements by HeNB2. For example, this could be done in TDD by measuring the uplink channel of UE1 by monitoring its Sounding Reference Signal (SRS.) In the second case, UE reports about a preferred beam direction could be considered. Further studies on UE performance, as well as backhaul support for such CBF mechanisms are FFS.

## 2.3 Measurement Reports

There are some issues with LTE UEs in reporting and making use of pilot strength measurements. In particular, the use of RSRP by LTE UEs in idle mode can create outage. For example, a UE may go deep into the coverage of a CSG HeNB even while the RSRP measurement from a macro eNB is above a threshold (that would have been okay in a macro-only deployment.) This issue would not occur if RSRQ of the serving sector would be very low in the coverage of a CSG HeNB. Further details of this may be found in [2].

## 2.4 Synchronization

It is clear that synchronization is required for TDD HeNBs, and hence techniques that enable synchronization should be considered for such HeNBs.  It should also be noted that there are several benefits for synchronization in FDD HeNBs. A detailed list of such benefits can be found in [3]. Furthermore, asynchronous Rel 9 HeNBs can prevent synchronization of LTE-A HeNBs deployed in the same carrier since a LTE-A HeNB may only see the asynchronous Rel 9 HeNB, and hence can’t synchronize to other synchronous LTE-A cells. Therefore it is highly desirable to support synchronization techniques in LTE Rel 9.

# 3	Conclusion

In this contribution, we presented some considerations for the study of HeNBs. We request RAN4 to evaluate these carefully in the course of the FDD and TDD HeNB work items.

# References

[1] TS 25.104 v8.5.0

[2] R4-091332, “RSRQ Measurement in Idle Mode,” Qualcomm Europe.

[3] R1-091471, “Advantages of and Requirements for synchronous network operation for LTE Rel 9 and LTE-A,” Qualcomm Europe.