3GPP TSG-RAN4 Meeting #53	 R4-094429

# Jeju, Korea, November 9 – 13, 2009

| CR-Form-v9.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 401 | rev |  | Current version: | 8.7.0 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | A-MPR for NS_07 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Europe |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE-RF |  |  |  |  |  |  | Date: |  | 10/30/2009 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-8 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) |  |
| Reason for change: |  | As described in R4-093121, recent measured PA data indicates that spurious requirements imposed under the network signaled NS_07 condition may not be met reliably for certain RB allocations. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Table 6.2.4-2 has been modified to introduce an additional column in Region B where the L_CRB is greater than or equal to 12, but less than 18. In this case, an A-MPR value of 2 dB is allowed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Overly tight requirement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  | TS 36.521-1 |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## << Unchanged sections omitted >>

### 6.2.4	UE Maximum Output Power with additional requirements

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction A-MPR is allowed for the output power as specified in Table 6.2.2-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

For UE Power Class 3 the specific requirements and identified sub-clauses are specified in Table 6.2.4-1 along with the allowed A-MPR values that may be used to meet these requirements. The allowed A-MPR values specified below in Table 6.2.4.-1 and 6.2.4-2 are in addition to the allowed MPR requirements specified in clause 6.2.3.

Table 6.2.4-1: Additional Maximum Power Reduction (A-MPR) / Spectrum Emission requirements

| Network Signalling value | Requirements (sub-clause) | E-UTRA Band | Channel bandwidth (MHz) | Resources Blocks | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 | NA | NA | NA | NA | NA |
| NS_03 | 6.6.2.2.1 | 2, 4,10, 35, 36 | 3 | >5 | ≤ 1 |
|  | 6.6.2.2.1 | 2, 4,10, 35,36 | 5 | >6 | ≤ 1 |
|  | 6.6.2.2.1 | 2, 4,10, 35,36 | 10 | >6 | ≤ 1 |
|  | 6.6.2.2.1 | 2, 4,10,35,36 | 15 | >8 | ≤ 1 |
|  | 6.6.2.2.1 | 2, 4,10,35, 36 | 20 | >10 | ≤ 1 |
| NS_04 | 6.6.2.2.2 | TBD | TBD | TBD |  |
| NS_05 | 6.6.3.3.1 | 1 | 10,15,20 | ≥ 50 for QPSK | ≤ 1 |
| NS_06 | 6.6.2.2.3 | 12, 13, 14, 17 | 1.4, 3, 5, 10 | n/a | n/a |
| NS_07 | 6.6.2.2.3 6.6.3.3.2 | 13 | 10 | Table 6.2.4-2 | Table 6.2.4-2 |
| .. |  |  |  |  |  |
| NS_32 | - | - | - | - | - |

Table 6.2.4-2: A-MPR for “NS07”

|  | Region A |  | Region B |  |  | Region C |
| --- | --- | --- | --- | --- | --- | --- |
| RB_start1 |  | 0 - 12 |  |  | 13 – 18 |  | 19 – 42 |  | 43 – 49 |  |
| L_CRB2 [RBs] |  | 6-8 | 1 to 5 and 9-50 |  | ≥8 |  | ≥12 and < 18 | ≥18 | ≤2 |  |
| A-MPR [dB] |  | 8 | 12 |  | 12 |  | 2 | 6 | 3 |  |
| Note 1 RB_start indicates the lowest RB index of transmitted resource blocks 2 L_CRB is the length of a contiguous resource block allocation 3 For intra-subframe frequency hopping between two regions, notes 1 and 2 apply on a per slot basis. 4 For intra-subframe frequency hopping between two regions, the larger A-MPR value of the two regions may be applied for both slots in the subframe. |  |  |  |  |  |  |

For the UE maximum output power modified by A-MPR, the power limits specified in subclause 6.2.5 apply.

## << Unchanged sections omitted >>