3GPP TSG RAN WG4 Meeting #58AH  	                                                                R4-112140

Shanghai, China, 11th - 15th April 2011

### Agenda Item:	5.5.5

### Source:	InterDigital

### Title:	Simulation results for UL-MIMO PUSCH performance requirements

Document for:	Discussion

# Introduction

This document presents LTE Rel10 UL-MIMO PUSCH ideal simulation results according to the simulation assumptions in [1] agreed in RAN4 #58 meeting

# Simulation Results

Table 2.1  1.4 MHz Channel Bandwidth

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and Correlation Matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | 2 | Normal | EPA 5Hz Low | [A3-2] | 70% | 1.8 |
|  |  |  |  | A4-3 | 70% | 14.8 |
|  |  |  |  | A5-2 | 70% | 21.8 |
|  | 4 | Normal | EPA 5Hz Low | [A3-2] | 70% | -3.2 |
|  |  |  |  | A4-3 | 70% | 8.4 |
|  |  |  |  | A5-2 | 70% | 15.3 |

Table 2.2  3 MHz Channel Bandwidth

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and Correlation Matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | 2 | Normal | EPA 5Hz Low | [A3-3] | 70% | 1.6 |
|  |  |  |  | A4-4 | 70% | 14.8 |
|  |  |  |  | A5-3 | 70% | 21.8 |
|  | 4 | Normal | EPA 5Hz Low | [A3-3] | 70% | -3.2 |
|  |  |  |  | A4-4 | 70% | 8.4 |
|  |  |  |  | A5-3 | 70% | 15.6 |

Table 2.3  5 MHz Channel Bandwidth

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and Correlation Matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | 2 | Normal | EPA 5Hz Low | [A3-4]* | 70% | 1.6 |
|  |  |  |  | A4-5 | 70% | 16.4 |
|  |  |  |  | A5-4 | 70% | 23.9 |
|  | 4 | Normal | EPA 5Hz Low | [A3-4]* | 70% | -3.6 |
|  |  |  |  | A4-5 | 70% | 8.8 |
|  |  |  |  | A5-4 | 70% | 15.8 |

Table 2.4  10 MHz Channel Bandwidth

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and Correlation Matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | 2 | Normal | EPA 5Hz Low | [A3-5]* | 70% | 2.6 |
|  |  |  |  | A4-6 | 70% | 16.5 |
|  |  |  |  | A5-5 | 70% | 24.2 |
|  | 4 | Normal | EPA 5Hz Low | [A3-5]* | 70% | -3 |
|  |  |  |  | A4-6 | 70% | 8.8 |
|  |  |  |  | A5-5 | 70% | 15.9 |

Table 2.5  15 MHz Channel Bandwidth

| Number of TX antennas |  | Number of RX antennas | Cyclic prefix | Propagation conditions and Correlation Matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 2 |  | Normal | EPA 5Hz Low | [A3-6]* | 70% | 1.9 |
|  |  |  |  |  | A4-7 | 70% | 17.8 |
|  |  |  |  |  | A5-6 | 70% | 25.3 |
|  | 4 |  | Normal | EPA 5Hz Low | [A3-6]* | 70% | -3.4 |
|  |  |  |  |  | A4-7 | 70% | 9.5 |
|  |  |  |  |  | A5-6 | 70% | 16.3 |

Table 2.6  20 MHz Channel Bandwidth

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and Correlation Matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | 2 | Normal | EPA 5Hz Low | [A3-7]* | 70% | 3.2 |
|  |  |  |  | A4-8 | 70% | 17.9 |
|  |  |  |  | A5-7 | 70% | 26.7 |
|  | 4 | Normal | EPA 5Hz Low | [A3-7]* | 70% | -2.7 |
|  |  |  |  | A4-8 | 70% | 9.6 |
|  |  |  |  | A5-7 | 70% | 17.8 |

# Conclusion

This document presents LTE Rel10 UL-MIMO PUSCH ideal simulation results according to the simulation assumptions in [1].

# References

R4-111626, “PUSCH simulation assumption for UL MIMO”, Huawei, Ericsson, ST-Ericsson, Nokia Siemens Networks ,ZTE, CATT, HiSilicon