3GPP TSG-RAN WG4 #60AH  	 R4-115101

Zhuhai, China, 10 – 14 October, 2011

Source:	ZTE

Title:	E-UTRA FDD Carrier Aggregation Test Case

Agenda item:	6.2.3

Document for:	Discussion

# 1	Introduction

The RRM core requirements related to the Rel-10 carrier aggregation work item have been completed. In document [1] a list of CA test case scenarios were agreed to be introduced in Rel-10.  The Phase I CA RRM test cases consist of 6 tests that include: Event triggered reporting on deactivating SCells with DRX (FDD), Event triggered reporting on deactivating SCell with DRX (TDD), Absolute and Relative RSRP Accuracies in CA (FDD), Absolute and Relative RSRP Accuracies in CA (TDD), Absolute and Relative RSRQ Accuracies in CA (FDD), and Absolute and Relative RSRQ Accuracies in CA (TDD). In this paper we provided a draft of test case FDD: Absolute and Relative RSRQ Accuracies in CA.

# 2	Discussion

The proposed test case to verify the RRM core requirements for FDD: Absolute and Relative RSRQ Accuracies in CA is provided below.

# 3	Conclusion

In conclusion, we have provided the test case to verify the core requirements in section 9.1.11 for the newly introduced in Rel-10 CA RRM requirements concerning the RSRQ measurement accuracy.

# Reference

[1] 	R4-114849, “List of RRM Test Cases for Carrier Aggregation,” Ericsson, ST-Ericsson, Alcatel-Lucent,

NTT DOCOMO, CATT

[2]	R4-113216, “RSTD Accuracy Requirements for Carrier Aggregation”, Alcatel-Lucent, RIM, Qualcomm,

Ericsson, ST-Ericsson, Huawei, Verizon

<Start of proposal for section A.9.2>

A.9.2.5 	FDD Carrier Aggregation Case

#### A.9.2.5.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in Section 9.1.11.

#### A.9.2.5.2	Test parameters

In this test case the PCell and the SCell are on different carrier frequencies. There are three cells used in this test case. Both RSRQ absolute and relative accuracy requirements in carrier aggregation are tested by using test parameters in Table A.9.2.5.2-1 and A.9.2.5.2-2.  In all tests, Cell 1 is the PCell, Cell 2 is the SCell and Cell 3 is on the Secondary Component Carrier (SCC). The SCC is configured and activated.

Table A.9.2.5.2-1: General test parameters for RSRQ FDD Carrier Aggregation

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PCell |  | Cell 1 | Cell 1 is configured on Primary Component Carrier (PCC) |
| SCells |  | Cell 2 and Cell 3 | Cells are configured on Secondary Component Carrier (SCC) |
| E-UTRA RF Channel Number |  | F1, F2 | F1 is the primary carrier. F2 is the secondary carrier |
| DRX |  | OFF | Non-DRX test |
| Time offset between cells | s | 3 | Synchronous cells |

Table A.9.2.5.2-2: RSRQ FDD Carrier Aggregation test parameters

| Parameter |  |  | Unit | Test 1 |  |  |  |  |  | Test 2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 |  | Cell 2 |  | Cell 3 |  | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| E-UTRA RF Channel Number |  |  |  | F1 |  | F2 |  | F2 |  | F1 |  | F2 |  | F2 |  |
| BWchannel_CA |  |  | MHz | 10 |  | 10 |  | 10 |  | 10 |  | 10 |  | 10 |  |
| Gap Pattern Id |  |  |  | 0 |  | - |  | - |  | 0 |  | - |  | - |  |
| Measurement bandwidth |  |  |  |  |  | [22—27] |  |  |  |  |  | [22—27] |  |  |  |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  |  |  |  |  | R.0 FDD |  | - |  | - |  | R.0 FDD |  | - |  | - |
| PDSCH allocation |  |  |  |  |  | [13—36] |  | - |  | - |  | [13—36] |  | - |  | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  |  |  |  | R.6 FDD |  |  |  |  |  | R.6 FDD |  |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and A.3.2.1.2 (OP.2 FDD) |  |  |  |  |  | OP.1 FDD |  | OP.2 FDD |  | OP.2 FDD |  | OP.1 FDD |  | OP.2 FDD |  | OP.2 FDD |
| PBCH_RA |  |  | dB | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  |
| PBCH_RB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  | Bands 1, 4, 6, 10, 11, 18, 19, 21, 23 and 24 | dBm/15 kHz | [-104.70] |  | [-104.70] |  | [-104.70] |  | [-119.50] |  | [-119.50] |  | [-119.50] |  |
|  |  | Bands 2, 5 and 7 |  |  |  |  |  |  |  | [-117.50] |  | [-117.50] |  | [-17.50] |  |
|  |  | Band 25 |  |  |  |  |  |  |  | [-116] |  | [-116] |  | [-116] |  |
|  |  | Bands 3, 8, 12, 13, 14, 17, 20 and 22 |  |  |  |  |  |  |  | [-116.50] |  | [-116.50] |  | [-116.50] |  |
|  |  | Band 9 |  |  |  |  |  |  |  | [-118.50] |  | [-118.50] |  | [-118.50] |  |
|  |  |  | dB | [-4.0] |  | [-4.0] |  | [-4.0] |  | [-4.0] |  | [-4.0] |  | [-4.0] |  |
| RSRPNote3 |  | Bands 1, 4, 6, 10, 11, 18 ,19, 21, 23 and 24 | dBm/15 kHz | [-108.70] |  | [-108.70] |  | [-108.70] |  | [-123.50] |  | [-123.50] |  | [-123.50] |  |
|  |  | Bands 2, 5 and 7 |  |  |  |  |  |  |  | [-121.50] |  | [-121.50] |  | [-121.50] |  |
|  |  | Band 25 |  |  |  |  |  |  |  | [-120.0] |  | [-120.0] |  | [-120.0] |  |
|  |  | Bands 3, 8, 12, 13, 14, 17, 20 and 22 |  |  |  |  |  |  |  | [-120.50] |  | [-120.50] |  | [-120.50] |  |
|  |  | Band 9 |  |  |  |  |  |  |  | [-122.50] |  | [-122.50] |  | [-122.50] |  |
| RSRQNote3 |  | Bands 1, 4, 6, 10, 11, 18 ,19, 21, 23 and 24 | dB | [-16.25] |  | [-16.25] |  | [-16.25] |  | [-16.25] |  | [-16.25] |  | [-16.25] |  |
|  |  | Bands 2, 5, 7 and 25 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Bands 3, 8, 12, 13, 14, 17, 20 and 22 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Band 9 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IoNote3 |  | Bands 1, 4, 6, 10, 11, 18 ,19, 21, 23 and 24 | dBm/9 MHz | [-75.46] |  | [-75.46] |  | [-75.46] |  | [-90.26] |  | [-90.26] |  | [-90.26] |  |
|  |  | Bands 2, 5 and 7 |  |  |  |  |  |  |  | [-88.26] |  | [-88.26] |  | [-88.26] |  |
|  |  | Band 25 |  |  |  |  |  |  |  | [-86.76] |  | [-86.76] |  | [-86.76] |  |
|  |  | Bands 3, 8, 12, 13, 14, 17, 20 and 22 |  |  |  |  |  |  |  | [-87.26] |  | [-87.26] |  | [-87.26] |  |
|  |  | Band 9 |  |  |  |  |  |  |  | [-89.26] |  | [-89.26] |  | [-89.26] |  |
|  |  |  | dB | [-4.0] |  | [-4.0] |  | [-4.0] |  | [-4.0] |  | [-4.0] |  | [-4.0] |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRQ, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### A.9.2.5.3	Test Requirements

The RSRQ measurement accuracy shall fulfil the requirements in Section 9.1.11.

<End of proposal for section A.9.2>