3GPP TSG-RAN WG4 Meeting #62	R4-120123

Dresden, Germany, 6 – 10 February, 2012

Source:	TeliaSonera AB

Title:	TP for TR ab.cde (inter-band CA): 1UL/2DL 2UL/2DL IMD study for B3 + B7 CA

Agenda item:	6.11.1 and 6.11.2

Document for:	Approval

# Introduction

The grouping of the inter-band CA combinations in low-high and low-low or high-high bands with large or small band separation in order to understand possible diplexer/quadplexer losses was presented in [1] (Feb 2010). After years of discussion how to group the many inter-band CA scenarios, considering also intermodulation products [2], it was agreed in the Zhuhai #60bis meeting (Oct 2011) to have for all low-high band combinations without harmonic problems (Class A1 with 1UL) 0 dB REFSENS relaxation and 0.3 dB relaxation for PCMAX_L [3]. We can note that this follows the relaxation of the MOP tolerance for low-high bands in DB-DC-HSDPA, see reference [4] (Feb 2011) and the generic low-high inter-band CA scenario with band 1 + 5, [5]. For all the other combinations (including 2ULs) no simple way to group the combinations could be agreed and it was decided to consider them case-by-case.

Noting the above  the current structure of the inter-band CA TR can be questioned as from an IMD point of view the groups with Class A2 to A4 can’t give any help. This was also noticed in the last meeting and from the discussion there we can understand that some vendor(s) like to have a reference architecture for inter-band CA and for that reason the Class A1 to A4 may be still relevant (see meeting minutes at R4-115683)

In this input all the “lower order“ IMD products for 1UL/2DL and 2UL/2DL are calculated for B3 + B7 CA. The structure can be also used in general for other inter-band CA combinations. Furthermore some headings of the TR have been slightly changed in order to adapt to the fact that Class A2 to A4 are not really related to IMD as mentioned above.

# IMD products in general

For two tones the following IMD products can be found causing possible desense or interference to the own or other bands:

- 2nd order: 2f1, 2f2, f1+f2, |f1-f2|

- 3rd order: |2f1-f2|, |2f2-f1|, 2f1+f2, 2f2+f1, 3f1, 3f2

The harmonics with 2f1, 3f1, etc. are of interest to the UE with 1UL. For three tones we can find additional 3rd order products at:

- 3rd order: |f1-f2+f3|, |f1+f2-f3|, |f1-f2-f3|

Which as indicated in [6] could cause desense to the own band or interference to neighbour bands if sharing the same antenna BS system.  To the own band the problem exists if the duplex gap in “Band A” < the transmission BW in “Band B”.

We note that the IMD products above do not give any information of the magnitude for the IMD. For CA combinations as stretched in reference [7], [8] where intermodulation products may fall into the own band or in other close by receive bands a more detailed analysis on the distortion (magnitude) and filter attenuations may be needed. The frequency range where the IMD may cause problems depends on the used transmission bandwidth (e.g. 10 MHz or 20 MHz) and RB location/allocation. The analysis below considers the whole band which gives the worst case range.

# References

[1]	R4-100317, “LTE-Advanced Rel-10 inter-band carrier aggregation”, TeliaSonera, 3GPP TSG-RAN WG4 Meeting #54, San Francisco, USA, 22nd – 26th February 2010.

[2]	R4-113290, “TP for TR 36.807: handling of inter-band CA scenarios”, Ericsson, 3GPP TSG-RAN WG4 Meeting #59, Barcelona, Spain, 9 – 13 May 2011.

[3]	R4-115502, “Way forward on interband insertion loss”, Nokia, 3GPP TSG-RAN WG4 Meeting #60bis, Zhuhai, P.R.China, Oct 10-14, 2011.

[4]	R4-111402, “Introduction of Tx core requirements for DB-DC-HSDPA and dual band 4C-HSDPA for I/VIII and I/V band combinations”, Qualcomm, 3GPP TSG-RAN WG4 Meeting #58, Taipei, Taiwan, 21st – 25th Feb 2011

[5]	R4-113233, “Minutes and agreements of inter – band CA band combinations frame work Ad-Hoc”, Nokia, et al, 3GPP TSG-RAN WG4 Meeting #59, Barcelona, Spain, 9 – 13 May 2011.

[6]	R4-115523, “Harmonics and Intermodulation Products caused by LTE Advanced Carrier Aggregation of Band Combination (3 + 7)”, Alcatel-Lucent, 3GPP TSG-RAN WG4 Meeting #61,San Francisco, US, 14 – 18 November, 2011

[7]	R4-112600, “Harmonics and inter-band carrier aggregation”, Ericsson, 3GPP TSG-RAN WG4 Meeting #59, Barcelona, Spain, 9 – 13 May 2011.

[8]	R4-115296, “Considerations of Inter-band combinations with IM2 problem”, Renesas, 3GPP TSG-RAN WG4 Meeting #60bis, Zhuhai, P.R. of China, Oct 10th - 14th , 2011.

# Text proposal for inter-band carrier aggregation TR ab.cde V0.1.0 (Release 11)

----- Unchanged sections omitted -----

----- Start of TP -----

## 6.3		Class A3. Low-low or high-high band combinations

This chapter shall include all inter band CA combinations which can be included in the group low-low or high-high combinations without transmitter harmonics.

### 6.3.1	LTE-Advanced Carrier Aggregation of Band 3 and Band 7

Table 6.3.1-1: Inter-band non-contiguous CA

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) band |  |  |  | Downlink (DL) band |  |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | Channel BW (MHz) | BS transmit / UE receive |  |  | Channel BW (MHz) |  |
|  |  | FUL_low – FUL_high |  |  |  | FUL_low – FUL_high |  |  |  |  |
| CA_3-7 | 3 | 1710 MHz | – | 1785 MHz | 5, 10, 15, 20 (note 1) | 1805 MHz | – | 1880 MHz | 5, 10, 15, 20 | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 10, 15, 20 (note 1) | 2620 MHz | – | 2690 MHz | 10, 15, 20 |  |
| NOTE 1: The first part of the WI considers only one uplink component carrier to be used in any of the two frequency bands at any time. |  |  |  |  |  |  |  |  |  |  |

#### 6.3.1.1		List of specific combination issues

##### 6.3.1.1.1	Channel bandwidths per operating band for CA

Table 6.3.1.1.1-1: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| CA operating / channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| CA_3A-7A | 3 |  |  | Yes | Yes | Yes | Yes |
|  | 7 |  |  |  | Yes | Yes | Yes |

##### 6.3.1.1.2	Co-existence studies for 1UL/2DL

Table 6.3.1.1.2-1 gives the intermodulation products for band 3 + band 7 CA with 2DLs. For the 3-tone IMD analysis the maximum transmission BW as defined in Table 6.3.1.1.1-1 is considered. None of the intermodulation products fall into the own receive bands.  Considering bands in the same geographical area we observe that the BS distortion could fall into the BS receive bands of Band 8, 20, 22, 42 and 43.

With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters the IMDs generated within the band 8, 20, 22, 38, 42 and 43 receiver should be well below the receiver noise floor eliminating the possibility of receiver desensitization. Provided that the Bands 3 and 7 BS transmitters should not share the same antenna with Band 8, 20, 22, 38, 42 or 43 BS receiver.

Table 6.3.1.1.2-1: 2DLs B3 + B7 IMD products

| BS DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 1805 | 1880 | 2620 | 2690 |
| 2nd order harmonics frequency range (MHz) | 3610 to 3760 |  | 5240 to 5380 |  |
| 3rd order harmonics frequency range (MHz) | 5415 to 5640 |  | 7860 to 8070 |  |
| Two-tone 2nd order IMD products | (f2_low – f1_high) | (f2_high – f1_low) | (f2_low + f1_low) | (f2_high + f1_high) |
| IMD frequency range (MHz) | 740 to 885 |  | 4425 to 4570 |  |
| Two-tone 3rd order IMD products | (2*f1_low – f2_high) | (2*f1_high – f2_low) | (2*f2_low – f1_high) | (2* f2_high – f1_low) |
| IMD frequency range (MHz) | 920 to 1140 |  | 3360 to 3575 |  |
| Three-tone 3rd order IMD products | (f1_low – max BW f2) | (f1_high + max BW f2) | (f2_low – max BW f1) | (f2_high + max BW f1) |
| IMD frequency range (MHz) | 1785 to 1860 |  | 2600 to 2670 |  |

Table 6.3.1.1.2-2 gives the intermodulation products for band 3 + band 7 CA with 1UL. None of the intermodulation products fall into the own receive bands. For the UE the distortion could fall into the UE receive bands for Band 22 and 43. As a UE does not simultaneous operate in B3 + B7 and any other band this should be not a problem in case the UE supports these bands . These bands are also rather far away from band 3 and band 7 and we can assume that the UE filter can effectively attenuate these IMD products.

Table 6.3.1.1.2-2: 1UL B3 + B7 IMD products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 2500 | 2570 |
| 2nd order harmonics frequency range (MHz) | 3420 to 3570 |  | 5000 to 5140 |  |
| 3rd order harmonics frequency range (MHz) | 5130 to 5355 |  | 7500 to 7710 |  |

##### 6.3.1.1.3	Co-existence studies for 2UL/2DL

Table 6.3.1.1.2-1 gives the intermodulation products for band 3 + band 7 CA with 2ULs. . For the 3-tone IMD analysis the maximum transmission BW as defined in Table 6.3.1.1.1-1 is considered. None of the intermodulation products fall into the own receive bands.  Considering bands in the same geographical area we observe that the UE distortion could fall into the UE receive bands of Band 8, 20 and 42. As a UE does not simultaneous operate in B3 + B7 and any other band this should be not a problem in case the UE supports these bands . These bands are also rather far away from band 3 and band 7 and we can assume that the UE filter can effectively attenuate these IMD products.

We can conclude that for 2UL/2DL for band 3 + band 7 CA no additional relaxations in output power (A-MPR) and/or REFSENS or filter duplexer and diplexer/quadplexer requirements with respect to the 1UL/2DL due to IMD is needed.

Table 6.3.1.1.3-1: 2ULs B3 + B7 IMD products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 2500 | 2570 |
| Two-tone 2nd order IMD products | (f2_low – f1_high) | (f2_high – f1_low) | (f2_low + f1_low) | (f2_high + f1_high) |
| IMD frequency range (MHz) | 715 to 860 |  | 4210 to 4355 |  |
| Two-tone 3rd order IMD products | (2*f1_low – f2_high) | (2*f1_high – f2_low) | (2*f2_low – f1_high) | (2* f2_high – f1_low) |
| IMD frequency range (MHz) | 850 to 1070 |  | 3215 to 3430 |  |
| Three-tone 3rd order IMD products | (f1_low – max BW f2) | (f1_high + max BW f2) | (f2_low – max BW f1) | (f2_high + max BW f1) |
| IMD frequency range (MHz) | 1690 to 1765 |  | 2480 to 2550 |  |

##### 6.3.1.1.3	∆TIB and ∆RIB values

The following ILs for ETC combining band 3 + 7 were reported:

Table 6.3.1.1.3-1: Reported ILs for band 3 + 7 diplexer and quadplexers for ETC

| E-UTRA bands | IL (dB) | IL (dB) | IL (dB) |
| --- | --- | --- | --- |
| 3 | 0.93 | 0.85 | 0.5 |
| 7 | 1.08 | 0.8 | 0.4 |

For the reported IL values there was no or only marginal difference for the Tx/Rx paths reported. With the average IL given as:

Table 6.3.1.1.3-2: Average Tx and Rx IL for combining band 3 and band 7 for ETC

| Inter-band CA Configuration | E-UTRA Band | Tx IL [dB] | Rx IL [dB] |
| --- | --- | --- | --- |
|  | 3 | 0.76 | 0.76 |
|  | 7 | 0.76 | 0.76 |

For two simultaneous DL and one UL the TIB,c and RIB values are given in the tables below:

Table 6.3.1.1.3-3: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_3A-7A | 3 | FFS |
|  | 7 | FFS |

# Table 6.3.1.1.3-4: ΔRIB

| Inter-band CA Configuration | E-UTRA Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_3A-7A | 3 | FFS |
|  | 7 | FFS |

----- End of TP -----