TSG-RAN4 Meeting #62	R4-120577

# Dresden, Germany, 6-10 February 2012

Source:	Ericsson, ST-Ericsson

Title:	Phase II CA RRM test case scenario 1 (FDD): Event-Triggered Reporting on SCell with PCell interruption

Agenda item:	4.2.3

Document for:	Discussion

# Introduction

In RAN4 meeting #60bis the CA RRM test cases to be included in Phase II were agreed, see [1]. After the meeting the work has been split between interested companies, with one or more companies providing drafts for each test case for discussions and alignment during RAN4 #60bis. In this paper we provide a draft on test case 1: FDD Event triggered reporting on deactivated Scell and interruption probability (<0.5%) without DRX.

# Discussion

The purpose of the test case is to verify the UE performance regarding cell identification time and event evaluation time for the CA scenario where a SCell is configured but deactivated, in static propagation conditions (AWGN). Moreover it is verified that the applicable interruption probability – in this case less than 0.5% - is fulfilled.

The test is based on three synchronous cells distributed on two carriers: PCell on primary component carrier, and SCell and a neighbour cell on secondary component carrier. The UE is configured with event triggered reporting for the following event: A6 (Neighbour becomes better than SCell).

The test is split into two time intervals T1 and T2. During T1only PCell and SCell are transmitting. At beginning of T2 cell 3 starts transmitting on the secondary component carrier with the same transmission power as used by SCell. Event A6 is been configured with an offset value, thus the Event A6 shall be detected and an event-triggered measurement report be sent to the network within 20×measCycleSCell from beginning of T2.

The offset for Event A6 needs to take into account that the allowed measurement tolerance for intra-frequency relative RSRP accuracy is ±2 dB under normal conditions provided that Ês/Iot  ≥ -3dB. This is taken into account by using an offset for Event A6 that means that if the neighbour cell measured RSRP becomes stronger than -4dB relative to the measured RSRP of the SCell, the event shall be detected. This means that there is a margin of 2dB to the tolerance that in worst case makes one of the cells appear to be 2 dB stronger than the other.

The requirement is to detect and report event A6 within the required time in at least 90% of the cases while at the same time having less than 0.5% dropped ACK/NACKs since start of T1. If a larger fraction of ACK/NACKs have been dropped, the test is considered to have failed. To verify the ACK/NACK dropping probability the UE shall be scheduled on PCell continuously throughout the test i.e. from the start of T1 until the measurement report is received during T2.

A draft of the test case proposed to be included in 36.133 [2] is provided in the Appendix.

# Conclusion

A draft of the Phase II CA RRM test case 1: “FDD Event triggered reporting on deactivated Scell and interruption probability (<0.5%) without DRX” has been presented and is proposed to be included in 36.133. Please see the Appendix for the test case.

##### References

- R4-115255, “Updated List of RRM Test Cases for Carrier Aggregation”, Ericsson, ST-Ericsson, Alcatel-Lucent, NTT DOCOMO, CATT.

- 3GPP TS 36.133 V10.5.0

##### Appendix – Proposed test case

###### A.8.16		E-UTRAN Carrier Aggregation Measurements

## A.8.16.x1		E-UTRAN FDD Event-Triggered Reporting on Deactivated SCell with PCell Interruption

### A.8.16.x1.1		Event-Triggered Reporting on Deactivated SCell with PCell Interruption in Non-DRX

#### A.8.16.x1.1.1		Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects event A6 (Neighbour becomes better than SCell) defined in [2] within the requirements stated in section 8.3.3.2.1 while at the same time fulfilling the requirement on interruption rate.

The test parameters are given in Tables A.8.16.x1.1.1-1 and A.8.16.x1.1.1-2 below. It is indicated to the UE in the measurement control information that event-triggered reporting with Event A6 is used. The test consists of two successive time periods, with duration of T1 and T2, respectively. During T1 the UE shall not have any information on cell 3. Immediately at beginning of T2 the transmission power of cell 3 is increased to same level as for cell 2, and due to usage of an offset this shall result in reporting of Event A6. PDCCH indicating a new transmission on the PCell shall be sent continuously to ensure that the UE sends ACK/NACKs throughout the test.

Table A.8.16.x1.1.1-1: General test parameters for E-UTRAN FDD event-triggered reporting on configured but deactivated SCell with PCell interruption in non-DRX

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| PDSCH parameters |  |  | DL Reference Measurement Channel R.0 FDD | As specified in section A.3.1.1.1 |
| PCFICH/PDCCH/PHICH parameters |  |  | DL Reference Measurement Channel R.6 FDD | As specified in section A.3.1.2.1 |
| E-UTRA RF Channel Number |  |  | 1, 2 | Two radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| Neighbour cell |  |  | Cell 3 | Neighbour cell to be identified on RF channel number 2. |
| Channel Bandwidth (BWchannel) |  | MHz | 10 | Channel bandwidth for cells on primary and secondary component carriers |
| CP length |  |  | Normal |  |
| Max time offset between any two cells |  |  | 3 s | Synchronous cells on primary and secondary component carriers. |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | 4 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in section 9.1.11.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | S | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle (measCycleSCell) |  | Ms | 1280 |  |
| T1 |  | S | 5 | During this time the UE shall be aware of cells 1 and 2 but not cell 3. |
| T2 |  | S | ≤30 | UE should report Event A6 within 25.6s (20×measCycleSCell) |

Table A.8.16.x1.1.1-2: Cell specific test parameters for E-UTRAN FDD event triggered reporting on configured but deactivated SCell with PCell interruption in non-DRX

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  | 2 |  |
| BWchannel | MHz | 10 |  | 10 |  | 10 |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and in A.3.2.1.2 (OP.2 FDD) |  | OP.1 FDD |  | OP.2 FDD |  | OP.2 FDD |  |
| PBCH_RA | dB | 0 |  | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -98 |  | -98 |  |  |  |
| RSRP Note 3 | dBm/15 kHz | -82 | -82 | -82 | -82 | -infinity | -82 |
| Ês/Iot | dB | 16 | 16 | 16 | -0.11 | -infinity | -0.11 |
| SCH_RP Note 3 | dBm/15 kHz | -82 | -82 | -82 | -82 | -infinity | -82 |
| Ês/Noc | dB | 16 | 16 | 16 | 16 | -infinity | 16 |
| Propagation Condition |  | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

#### A.8.16.x1.1.2		Test Requirements

The UE shall send one Event A6 triggered measurement report with a measurement reporting delay of less than 25.6s (20×measCycleSCell) from the beginning of time T2.

The UE shall not send event triggered measurement reports as long as the reporting criterion is not fulfilled.

The UE shall be scheduled on PCell continuously throughout the test and from the start of T1 until the measurement report is received during T2, at least 99.5% of all ACK/NACKs shall be detected as being transmitted by the UE.

For a test to be considered successful requirements on both event detection and percentage of transmitted ACK/NACKs have to be fulfilled simultaneously.

The rate of successful tests during repeated tests shall be at least 90%.

NOTE 1: 	The actual overall delays measured in the tests may be up to 2×TTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

NOTE 2: 	In order to calculate the rate of correct events the system simulator shall verify that it has received correct Event A6 measurement report