3GPP TSG-RAN WG4 Meeting #62                                                          R4-120919

Dresden, Germany, 6-9 February 2012                           (revised from R4-120710)

Agenda item:            8.3

Source:	ST-Ericsson/Ericsson

Title:	Link level simulation results for advanced receiver

Document for:	          Discussion

# Introduction

In the last meeting the discussion on advanced receiver started. A way forward document was agreed in [1]. The way forward document covers the following aspects:

By 23rd January 2012

- Provide DIP table

RAN4 #62

- Provide the link level simulation results based on conditional median DIP [24]

- Provide the link level simulation results based on the “DIP table for weighted average throughput gain study” table provided on 23rd January 2012 by interested companies

  - Typical DIP scenario based on the average throughput gain will be defined.

- Provide the system level simulation results by interested companies

- Finalize TR

The link level simulation results should be provided by considering the following baseline conditions:

- Synchronized network (baseilne)

  - Number of interfing cells: 2 cells

  - Conditional median DIP values:

    - DIP1 = -2.8 dB, DIP2 = -7.3 dB on 0 dB geometry.

    - DIP1 = -3.1 dB, DIP2 = -5.4 dB on -3dB geometry.

Table 1: Initial simulation assumptions for the evaluation of link-level evaluation.

| Parameter | Scenario 1 (CRS based) | Scenario 2 (DM-RS based) |
| --- | --- | --- |
| Carrier frequency | 2 GHz |  |
| System bandwidth | 10 MHz |  |
| Transmission mode on Serving cell | TM6 | TM9 with 1-layer transmission |
| Transmission mode on interference cell | TM4 | TM9 |
| MIMO configuration | 2x2 and low correlation | 4x2 and low correlation |
| Channel model and Doppler frequency for target and interference cells | EVA, 3km/h, Use different channel seed for between cells |  |
| CRS configuration | 2 CRS ports with planning (non-colliding) |  |
| CSI-RS configuration | None | 4 CSI-RS ports, and 5 msec periodicity |
| MCS for target signal | Fixed MCS as follow: #10, #11, #12 for SINR = 0 dB, and #7, #8 ,#9 for SINR= -3 dB as baseline, and outer-loop link adaptation by interested companies |  |
| PMI for target signal | Follow wideband PMI as baseline Fixed wideband PMI by interested companies |  |
| H-ARQ | 8 HARQ processes and max 4 transmissions |  |
| Feedback periodicity for target signal | Feedback periodicity: 5 msec Feedback delay: 8 msec | Feedback periodicity: 5 msec Feedback delay: 8 msec |
| MCS/ PMI transmission guranuality and Number of transmission ranks for interfernce signals (% of rank-1 and % of rank-2) | Randomly changing per sub-band from subframe to subframe as baseline. Randomly changing per sub-band per 10 msec periodicity by interested companies Frequency granularity is 6 RBs |  |
|  | 80% for rank-1 and 20% for rank-2 | 70% for rank-1 and 30% for rank-2 |
| PCFICH | CFI = 2 |  |
| PCFICH/PDCCH detection | Not considered |  |
| Resource allocation | 50 RBs |  |
| Cyclic prefix | Normal |  |
| Simulation length | 10000 sub-frames at minimum |  |

Note: Other transmission modes are not precluded in WI.

In Section 2 we provide simulation results for the baseline assumptions mentioned in the table above and for synchronous case (Case 1). Case 2 as well as the conditions which are labeled ”to be provided by interested companies” are not included here. These additional conditions were not sufficiently discussed and analyzed during previous meetings and not agreed by the group to be necessary/representative. Hence we think that it can be questionable whether the results associated to these conditions can be taken into account to derive average gains of the advanced receiver. Different conditions w.r.t to the baseline conditions which were agreed by the group can be discussed, if needed, during the work item phase.

# Results

## Throughput results with median DIP

The following table (Table 2) provides the throughput results with the conditional median DIP.

Table 2. Throughput simulation results with MMSE and IRC receivers and gains with conditional median DIP

| Geometry | MCS |  | Scenario1 |  |  | Scenario2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MMSE | MMSE-IRC | IRC/MMSE gain | MMSE | MMSE-IRC | IRC/MMSE gain |
|  |  | Throughput (bps) | Throughput (bps) |  | Throughput (bps) | Throughput (bps) |  |
| G=0dB | 10 | 5386234,372 | 5809497,789 | 7,86% | 4937420,302 | 5379129,246 | 8,95% |
|  | 11 | 5256874,975 | 5956366,131 | 13,31% | 4608486,332 | 5215879,196 | 13,18% |
|  | 12 | 4365449,347 | 5342912,864 | 22,39% | 4007983,92 | 4943568,844 | 23,34% |
| Average gain |  |  |  | 14,52% |  |  | 15,16% |
| G=-3dB | 7 | 3712880,503 | 4053348,442 | 9,17% | 3030181,005 | 3386641,106 | 11,76% |
|  | 8 | 3748229,447 | 4242115,879 | 13,18% | 3008562,01 | 3514149,749 | 16,80% |
|  | 9 | 3323517,99 | 4086786,533 | 22,97% | 2447345,93 | 3209112,06 | 31,13% |
| Average gain |  |  |  | 15,10% |  |  | 19,90% |

## Throughput results with DIP table for weighted average throughput gain study

The following tables (Table 3 and 4) provide the DIP table for the weighted average throughput gain study after averaging among all the companies for 0 and -3dB geometry level. The Annex provides the DIP tables provided by Ericsson/ST-Ericsson.

Table 3. DIP table for 0dB geometry

| # | DIP1 | DIP2 | DIP3 | DIP4 | DIP5 | DIP6 | DIP7 | DIP8 | DIP9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | [dB] | [dB] | [dB] | [dB] | [dB] | [dB] | [dB] | [dB] | [dB] |
| 1 | -5.8756 | -7.0712 | -9.2161 | -11.6383 | -13.2606 | -14.4786 | -15.4275 | -16.4086 | -17.2367 |
| 2 | -4.8707 | -6.2968 | -9.8932 | -12.4335 | -13.9319 | -15.1298 | -16.1461 | -17.1757 | -17.9721 |
| 3 | -4.4235 | -6.0674 | -10.4683 | -12.6669 | -14.3042 | -15.5219 | -16.5624 | -17.5677 | -18.3642 |
| 4 | -4.0946 | -5.7811 | -10.9230 | -13.0898 | -14.7880 | -15.9233 | -16.9939 | -18.0285 | -19.0689 |
| 5 | -3.8324 | -5.8516 | -11.4141 | -13.2123 | -15.0501 | -16.0650 | -17.1284 | -18.1029 | -19.1504 |
| 6 | -3.6164 | -5.7613 | -11.5225 | -13.6073 | -15.4317 | -16.5767 | -17.7026 | -18.6865 | -19.8633 |
| 7 | -3.4257 | -5.6410 | -11.9433 | -14.1211 | -15.8476 | -16.7291 | -17.9870 | -19.0622 | -20.1216 |
| 8 | -3.2545 | -5.1860 | -12.7269 | -14.7114 | -16.6441 | -17.2220 | -18.4128 | -19.4727 | -20.6932 |
| 9 | -3.1187 | -4.9768 | -13.6190 | -15.6438 | -17.4545 | -18.5337 | -19.8016 | -20.9741 | -22.1226 |
| 10 | -3.0035 | -4.3905 | -15.9347 | -17.8697 | -19.9738 | -24.1105 | -25.2075 | -26.1956 | -28.3577 |
| 11 | -2.8813 | -5.0915 | -14.5968 | -16.4632 | -18.4665 | -20.8711 | -21.9710 | -22.8627 | -25.5805 |
| 12 | -2.6167 | -7.2098 | -12.4779 | -14.1822 | -16.2392 | -17.3941 | -18.7131 | -19.7297 | -21.2875 |
| 13 | -2.3314 | -7.5707 | -12.7063 | -14.4677 | -16.4213 | -17.5312 | -19.0203 | -20.2314 | -21.7555 |
| 14 | -2.0561 | -8.2463 | -12.8626 | -14.7272 | -16.5210 | -17.6566 | -19.2153 | -20.5094 | -22.0448 |
| 15 | -1.7830 | -8.9602 | -12.9834 | -14.8263 | -16.8133 | -17.6834 | -19.6412 | -21.1566 | -22.5336 |
| 16 | -1.5155 | -9.7449 | -13.2458 | -15.2033 | -16.9709 | -18.0138 | -20.4783 | -22.1490 | -23.7060 |
| 17 | -1.2429 | -10.7098 | -13.6667 | -15.6233 | -17.1317 | -18.7696 | -21.6967 | -23.5683 | -24.9530 |
| 18 | -0.8743 | -11.5208 | -14.4146 | -17.5961 | -19.3167 | -22.3218 | -24.5771 | -26.0987 | -27.2738 |
| 19 | -0.4148 | -13.3836 | -17.5367 | -22.5611 | -24.7592 | -27.7245 | -29.1161 | -30.2893 | -31.5747 |
| 20 | -0.1714 | -15.6040 | -24.1766 | -29.2237 | -31.8375 | -34.0250 | -35.2416 | -36.4086 | -37.9132 |

Table 4. DIP table for -3dB geometry

| # | DIP1 | DIP2 | DIP3 | DIP4 | DIP5 | DIP6 | DIP7 | DIP8 | DIP9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | [dB] | [dB] | [dB] | [dB] | [dB] | [dB] | [dB] | [dB] | [dB] |
| 1 | -5.2285 | -6.6182 | -9.5564 | -12.0561 | -13.5444 | -14.9076 | -16.1124 | -17.2245 | -18.0498 |
| 2 | -4.0092 | -5.8471 | -10.7553 | -12.9172 | -14.9012 | -16.8757 | -18.0942 | -19.0422 | -20.0232 |
| 3 | -3.5414 | -4.4647 | -13.2615 | -15.9211 | -17.4415 | -20.6641 | -21.9310 | -23.2256 | -23.3233 |
| 4 | -3.3712 | -4.0386 | -14.1881 | -16.8942 | -18.3063 | -22.4200 | -23.8520 | -25.0706 | -24.8780 |
| 5 | -3.2222 | -3.9973 | -15.2041 | -17.7331 | -19.4534 | -24.1810 | -25.3379 | -26.7208 | -26.7289 |
| 6 | -3.1078 | -3.5986 | -18.8374 | -21.6149 | -23.4891 | -30.3468 | -31.6637 | -32.6386 | -33.2279 |
| 7 | -3.0660 | -3.3460 | -20.6528 | -23.7117 | -25.4209 | -32.6381 | -33.5548 | -34.6981 | -36.7913 |
| 8 | -3.0426 | -3.3358 | -21.8975 | -25.9997 | -27.6287 | -34.3431 | -35.5411 | -36.5680 | -37.7810 |
| 9 | -3.0127 | -3.5442 | -20.4491 | -24.7029 | -26.1773 | -30.6464 | -31.8486 | -33.1919 | -33.4181 |
| 10 | -2.9655 | -3.4608 | -20.7371 | -23.6312 | -25.7816 | -27.4733 | -28.2479 | -29.3706 | -28.9740 |
| 11 | -2.8678 | -4.1277 | -17.5126 | -20.1599 | -21.6615 | -22.4232 | -23.5480 | -24.5886 | -25.7378 |
| 12 | -2.6047 | -5.6851 | -13.9475 | -16.0074 | -17.6255 | -17.9079 | -19.1133 | -20.1178 | -21.2491 |
| 13 | -2.3330 | -5.9009 | -14.2919 | -16.3024 | -18.2590 | -18.2057 | -19.4189 | -20.3595 | -21.8157 |
| 14 | -2.0844 | -6.4156 | -14.7056 | -16.4982 | -18.4206 | -18.3878 | -19.5630 | -20.5906 | -22.0139 |
| 15 | -1.8617 | -6.6742 | -15.0511 | -16.8358 | -18.8130 | -18.6399 | -20.0420 | -21.2698 | -22.8031 |
| 16 | -1.6581 | -7.0132 | -15.6023 | -17.2217 | -19.0901 | -18.7732 | -20.7126 | -22.1964 | -23.8335 |
| 17 | -1.4470 | -7.8461 | -15.3880 | -17.1916 | -18.8891 | -19.3352 | -21.7793 | -23.4463 | -25.2927 |
| 18 | -1.2095 | -8.7584 | -15.3769 | -17.5211 | -19.0383 | -20.6533 | -23.6806 | -25.4556 | -27.1489 |
| 19 | -0.8701 | -10.1265 | -16.2263 | -19.7819 | -21.6125 | -26.0967 | -27.9890 | -29.2449 | -30.9404 |
| 20 | -0.6180 | -12.4646 | -18.5602 | -20.6644 | -22.3917 | -33.0207 | -34.3076 | -35.4357 | -36.8072 |

Table 5 and 6 provides ST-Ericsson/Ericsson throughput results for the DIP table averaged among all the companies for scenario 1.

Table 6. Throughput simulation results with averaged DIP table for G=-3dB, Scenario 1

| DIP Profile # |  | MCS-7 |  |  | MCS-8 |  |  | MCS-9 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MMSE | MMSE-IRC | Gain (%) | MMSE | MMSE-IRC | Gain (%) | MMSE | MMSE-IRC | Gain (%) |
| 1 | 3556657,1 | 3718397,3 | 4,55% | 3504908,7 | 3745448,1 | 6,86% | 2968049 | 3357182,2 | 13,11% |
| 2 | 3652791,5 | 3902115,2 | 6,83% | 3642319,7 | 4033996,9 | 10,75% | 3186611,2 | 3757488,7 | 17,91% |
| 3 | 3753127,3 | 4176414,9 | 11,28% | 3804525,9 | 4360855 | 14,62% | 3416739,5 | 4241897,7 | 24,15% |
| 4 | 3813207,4 | 4364056,5 | 14,45% | 3883061,5 | 4535587,9 | 16,80% | 3540470,3 | 4434912,6 | 25,26% |
| 5 | 3840079 | 4410355,7 | 14,85% | 3913441,1 | 4595933 | 17,44% | 3585783,1 | 4546075 | 26,78% |
| 6 | 3907266,8 | 4621402,3 | 18,28% | 3988959,9 | 4760068,4 | 19,33% | 3673056,7 | 4785173,8 | 30,28% |
| 7 | 3955139,1 | 4782553,3 | 20,92% | 4055629,7 | 4865922,4 | 19,98% | 3777548,6 | 4958982,2 | 31,28% |
| 8 | 3962099 | 4805257 | 21,28% | 4051985 | 4880489,9 | 20,45% | 3799481,8 | 4982607,6 | 31,14% |
| 9 | 3923201,1 | 4703036,6 | 19,88% | 4025698,4 | 4808234,8 | 19,44% | 3750325,3 | 4867322 | 29,78% |
| 10 | 3957949 | 4786884,7 | 20,94% | 4054048,5 | 4873424,6 | 20,21% | 3784400,1 | 4960461,1 | 31,08% |
| 11 | 3882098,3 | 4549831,5 | 17,20% | 3959398,3 | 4685278,8 | 18,33% | 3662311,6 | 4675786 | 27,67% |
| 12 | 3801864,5 | 4275885,8 | 12,47% | 3874156,8 | 4505555,4 | 16,30% | 3532896,3 | 4387792,7 | 24,20% |
| 13 | 3843460,3 | 4396426,9 | 14,39% | 3921840,8 | 4588901,7 | 17,01% | 3586374,7 | 4541096,8 | 26,62% |
| 14 | 3856757,2 | 4461833,4 | 15,69% | 3953600,2 | 4635711 | 17,25% | 3643467,1 | 4601376,6 | 26,29% |
| 15 | 3899995,5 | 4562843,7 | 17,00% | 3994230,3 | 4731976,5 | 18,47% | 3705044,8 | 4758624,4 | 28,44% |
| 16 | 3937181,4 | 4689679,2 | 19,11% | 4048026,3 | 4825527,8 | 19,21% | 3742373,2 | 4869030,5 | 30,11% |
| 17 | 3951957,2 | 4751159,8 | 20,22% | 4065532,2 | 4873436 | 19,87% | 3812231,2 | 4967246,3 | 30,30% |
| 18 | 4027747,4 | 4893203,5 | 21,49% | 4120438,5 | 4994485,4 | 21,21% | 3866465,9 | 5114783,5 | 32,29% |
| 19 | 4120077 | 5220511,4 | 26,71% | 4230138,6 | 5213100,7 | 23,24% | 4041868 | 5434288,2 | 34,45% |
| 20 | 4199333,8 | 5567955 | 32,59% | 4337865 | 5410285,5 | 24,72% | 4159898,1 | 5692593,5 | 36,84% |
| AWGN | 3193666,9 | 3157388,5 | -1,14% | 2904664,3 | 2847246,2 | -1,98% | 2262283,5 | 2198142,4 | -2,84% |
| 1 strong cell, no AWGN | 4635934 | 7175572,4 | 54,78% | 4718249,6 | 7025205 | 48,89% | 4683179 | 6880595,9 | 46,92% |
| Average gain |  |  | 17,51% |  |  | 18,08% |  |  | 27,90% |

Table 7. Throughput simulation results with averaged DIP table for G=0dB, Scenario 1

| DIP Profile # |  | MCS-10 |  |  | MCS-11 |  |  | MCS-12 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MMSE | MMSE-IRC | Gain (%) | MMSE | MMSE-IRC | Gain (%) | MMSE | MMSE-IRC | Gain (%) |
| 1 | 5152553,2 | 5333763,4 | 3,52% | 4939415,6 | 5233184,9 | 5,95% | 3927156,3 | 4373817,6 | 11,37% |
| 2 | 5244575,2 | 5497988,4 | 4,83% | 5094775,2 | 5515109,2 | 8,25% | 4097860,3 | 4704297,4 | 14,80% |
| 3 | 5291714,5 | 5618761,2 | 6,18% | 5144137,1 | 5652701,3 | 9,89% | 4241744 | 4894413,5 | 15,39% |
| 4 | 5357595,6 | 5717458,4 | 6,72% | 5204274,6 | 5819707,2 | 11,83% | 4237669,7 | 5101935,9 | 20,39% |
| 5 | 5375683 | 5770937 | 7,35% | 5249568,8 | 5870106,3 | 11,82% | 4341773,2 | 5227583,4 | 20,40% |
| 6 | 5422133,2 | 5845224,8 | 7,80% | 5268793,9 | 5950129,4 | 12,93% | 4382911,6 | 5419961,7 | 23,66% |
| 7 | 5457429,7 | 5930718,4 | 8,67% | 5344476,8 | 6072312,4 | 13,62% | 4458174,4 | 5496210,6 | 23,28% |
| 8 | 5524052,2 | 6058502,2 | 9,67% | 5421679,6 | 6260649,9 | 15,47% | 4590114 | 5773901,8 | 25,79% |
| 9 | 5553181,2 | 6159567,9 | 10,92% | 5488127,6 | 6369100,7 | 16,05% | 4716401,5 | 5956389,9 | 26,29% |
| 10 | 5646021,7 | 6336317,2 | 12,23% | 5613077,8 | 6592271,2 | 17,44% | 4897903,6 | 6231525 | 27,23% |
| 11 | 5576023,1 | 6225501,2 | 11,65% | 5529816,2 | 6438819,5 | 16,44% | 4780918,3 | 5980360,1 | 25,09% |
| 12 | 5529573 | 6031000,4 | 9,07% | 5428828,6 | 6223793,7 | 14,64% | 4661480,9 | 5731539,3 | 22,96% |
| 13 | 5572087,1 | 6128931,3 | 9,99% | 5492918,3 | 6364111,4 | 15,86% | 4747939,9 | 5919653,5 | 24,68% |
| 14 | 5620044,9 | 6245974,6 | 11,14% | 5581966,1 | 6493307 | 16,33% | 4834931 | 6136292,1 | 26,92% |
| 15 | 5671232,2 | 6369434,4 | 12,31% | 5665301,6 | 6695691 | 18,19% | 4929416 | 6344082,4 | 28,70% |
| 16 | 5759318,2 | 6541394,4 | 13,58% | 5748038,9 | 6900747,4 | 20,05% | 5101720 | 6602659,4 | 29,42% |
| 17 | 5848205,3 | 6754206,5 | 15,49% | 5892350,4 | 7119011,8 | 20,82% | 5242273,6 | 6930259,2 | 32,20% |
| 18 | 6034859,2 | 7128143,6 | 18,12% | 6090331,7 | 7571568,5 | 24,32% | 5568031,5 | 7446319,2 | 33,73% |
| 19 | 6281348,3 | 7732532,5 | 23,10% | 6399646,4 | 8307673,6 | 29,81% | 5910909,3 | 8371076,1 | 41,62% |
| 20 | 6484315,7 | 8344965 | 28,69% | 6667681,5 | 8771299,3 | 31,55% | 6283163,7 | 9050160,1 | 44,04% |
| AWGN | 4721925,4 | 4667887 | -1,14% | 4090434,3 | 4046544,4 | -1,07% | 2800209 | 2600902,5 | -7,12% |
| 1 strong cell, no AWGN | 6629049,9 | 9110658,2 | 37,44% | 6812307,1 | 9242074,1 | 35,67% | 6499270,3 | 9628705,1 | 48,15% |
| Average gain |  |  | 11,55% |  |  | 16,56% |  |  | 25,90% |

In the table we have also included 2 scenarios for comparison,

- AWGN where we have set the Dip table definition as follows:

| # | DIP1 | DIP2 | DIP3 | DIP4 | DIP5 | DIP6 | DIP7 | DIP8 | DIP9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | [dB] | [dB] | [dB] | [dB] | [dB] | [dB] | [dB] | [dB] | [dB] |
| 1 | -50 | -50 | -50 | -50 | -50 | -50 | -50 | -50 | -50 |

- 1 Strong cell interference with the following DIP setup

| # | DIP1 | DIP2 | DIP3 | DIP4 | DIP5 | DIP6 | DIP7 | DIP8 | DIP9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | [dB] | [dB] | [dB] | [dB] | [dB] | [dB] | [dB] | [dB] | [dB] |
| 1 | -0.001 | -50 | -50 | -50 | -50 | -50 | -50 | -50 | -50 |

Figure 1 and Figure 2 show the throughput behaviour for Scenario 1 Geometry level 0 and -3dB.

Figure 1. Scenario 1, 0dB

Figure 2. Scenario 1, -3dB

Table 7 and 8 provide ST-Ericsson/Ericsson throughput results for the DIP table averaged among all the companies for scenario 2.

Table 7. Throughput simulation results with averaged DIP table for G=-3dB, Scenario 2

| DIP Profile # |  | MCS-7 |  |  | MCS-8 |  |  | MCS-9 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MMSE | MMSE-IRC | Gain (%) | MMSE | MMSE-IRC | Gain (%) | MMSE | MMSE-IRC | Gain (%) |
| 1 | 2932238,5 | 3081240,9 | 5,08% | 2821894,9 | 3070602 | 8,81% | 2252743,8 | 2542513,4 | 12,86% |
| 2 | 2967657,6 | 3227204,7 | 8,75% | 2908098,3 | 3318535,9 | 14,11% | 2355629,8 | 2865791,6 | 21,66% |
| 3 | 3043358 | 3416984,3 | 12,28% | 3013088 | 3571774,5 | 18,54% | 2463931,3 | 3281358 | 33,18% |
| 4 | 3090013,9 | 3516392,3 | 13,80% | 3056241,8 | 3718744,3 | 21,68% | 2545336,9 | 3487529,4 | 37,02% |
| 5 | 3089055,1 | 3568094,3 | 15,51% | 3095877,4 | 3753857,9 | 21,25% | 2582620,7 | 3546953,6 | 37,34% |
| 6 | 3136896,9 | 3677083 | 17,22% | 3122193,6 | 3891904,3 | 24,65% | 2649349,2 | 3760201,6 | 41,93% |
| 7 | 3155258 | 3763521 | 19,28% | 3158476,8 | 3943585,3 | 24,86% | 2702074,4 | 3872607,6 | 43,32% |
| 8 | 3160961,3 | 3784726,8 | 19,73% | 3167659,3 | 3954170,5 | 24,83% | 2709576,8 | 3903341,6 | 44,06% |
| 9 | 3150014,1 | 3731490,3 | 18,46% | 3137719,2 | 3921440 | 24,98% | 2676853 | 3805408,3 | 42,16% |
| 10 | 3163445,2 | 3768162,2 | 19,12% | 3168155,8 | 3951983,5 | 24,74% | 2666545,1 | 3885205 | 45,70% |
| 11 | 3125776,9 | 3637167,5 | 16,36% | 3113027,5 | 3831247,8 | 23,07% | 2623032,1 | 3672262,4 | 40,00% |
| 12 | 3074105,6 | 3496446 | 13,74% | 3054659,5 | 3674670,6 | 20,30% | 2552199,7 | 3436974,3 | 34,67% |
| 13 | 3086155 | 3561456,5 | 15,40% | 3086915,6 | 3747247,6 | 21,39% | 2596322,6 | 3534565,3 | 36,14% |
| 14 | 3110284,2 | 3591037,7 | 15,46% | 3088140,1 | 3789698,3 | 22,72% | 2602015,7 | 3611286,6 | 38,79% |
| 15 | 3125625,7 | 3656737,1 | 16,99% | 3136867,7 | 3849281 | 22,71% | 2693361 | 3743505,3 | 38,99% |
| 16 | 3161462,7 | 3726431,1 | 17,87% | 3143482,2 | 3908215,3 | 24,33% | 2658821 | 3847169,9 | 44,69% |
| 17 | 3170069,1 | 3781425,6 | 19,29% | 3157731,1 | 3978099,5 | 25,98% | 2727707,6 | 3934816,5 | 44,25% |
| 18 | 3193422,9 | 3870385,3 | 21,20% | 3208978,7 | 4046123 | 26,09% | 2767561,5 | 4049238,6 | 46,31% |
| 19 | 3251569,5 | 4085050,8 | 25,63% | 3273271,6 | 4242944,7 | 29,62% | 2896492,1 | 4311980,1 | 48,87% |
| 20 | 3289675,9 | 4303652,9 | 30,82% | 3348261,9 | 4399860,3 | 31,41% | 2999666,1 | 4530794,6 | 51,04% |
| AWGN | 2917304 | 2882222,3 | -1,20% | 2787819,7 | 2735721,8 | -1,87% | 2106222,5 | 2042908,2 | -3,01% |
| 1 strong cell, no AWGN | 3529893,6 | 5242131 | 48,51% | 3600169,4 | 5213017,5 | 44,80% | 3381888,8 | 5363805,1 | 58,60% |
| Average gain |  |  | 17,10% |  |  | 22,80% |  |  | 39,15% |

Table 8. Throughput simulation results with averaged DIP table for G=0dB, Scenario 2

| DIP Profile # |  | MCS-10 |  |  | MCS-11 |  |  | MCS-12 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MMSE | MMSE-IRC | Gain (%) | MMSE | MMSE-IRC | Gain (%) | MMSE | MMSE-IRC | Gain (%) |
| 1 | 4813534 | 4932335,3 | 2,47% | 4336875,4 | 4633275,4 | 6,83% | 3680949,7 | 3964024,1 | 7,69% |
| 2 | 4862893,1 | 5084489,1 | 4,56% | 4459392,5 | 4817981,4 | 8,04% | 3760809 | 4275278,4 | 13,68% |
| 3 | 4880502 | 5155730,7 | 5,64% | 4502302,3 | 4947371,3 | 9,89% | 3806146,7 | 4470652,3 | 17,46% |
| 4 | 4909749,6 | 5241014,2 | 6,75% | 4538767,3 | 5028355,2 | 10,79% | 3874097,5 | 4613343,7 | 19,08% |
| 5 | 4941663,5 | 5294846,1 | 7,15% | 4579605,3 | 5082159,9 | 10,97% | 3894596 | 4677813,1 | 20,11% |
| 6 | 4963007,8 | 5339158,5 | 7,58% | 4568487,1 | 5153215,4 | 12,80% | 4018529,6 | 4829504,5 | 20,18% |
| 7 | 4993809,4 | 5400390,8 | 8,14% | 4632520,3 | 5222817,8 | 12,74% | 4017498,5 | 4957131,7 | 23,39% |
| 8 | 5021515,3 | 5499526,3 | 9,52% | 4685397,3 | 5340438,2 | 13,98% | 4098086,4 | 5170197 | 26,16% |
| 9 | 5057187,9 | 5559843 | 9,94% | 4710895,7 | 5430879,4 | 15,28% | 4146338,7 | 5227584,9 | 26,08% |
| 10 | 5123413,9 | 5710225,5 | 11,45% | 4817564,3 | 5591948,4 | 16,07% | 4244098,5 | 5530531,7 | 30,31% |
| 11 | 5083711,3 | 5627628,9 | 10,70% | 4752741,1 | 5474584,3 | 15,19% | 4213993 | 5376677,4 | 27,59% |
| 12 | 5002297,5 | 5499116,6 | 9,93% | 4683252,7 | 5357017,8 | 14,39% | 4124077,4 | 5170100,5 | 25,36% |
| 13 | 5048539,4 | 5587246,6 | 10,67% | 4780015,1 | 5456526,4 | 14,15% | 4146697,5 | 5302759,8 | 27,88% |
| 14 | 5074056,1 | 5662875,5 | 11,60% | 4787967,2 | 5555660 | 16,03% | 4251101,5 | 5472273,4 | 28,73% |
| 15 | 5117917,1 | 5762305,9 | 12,59% | 4830207,8 | 5677510,9 | 17,54% | 4339706,5 | 5639327,6 | 29,95% |
| 16 | 5192269,5 | 5879239,1 | 13,23% | 4881278,3 | 5826158,8 | 19,36% | 4447411,1 | 5847926,6 | 31,49% |
| 17 | 5241509,2 | 6024588,4 | 14,94% | 4936609,2 | 6002103,5 | 21,58% | 4603168,8 | 6081082,4 | 32,11% |
| 18 | 5343668,3 | 6284705,8 | 17,61% | 5107786,9 | 6329796,4 | 23,92% | 4793529,6 | 6448341,7 | 34,52% |
| 19 | 5524688,2 | 6670106,4 | 20,73% | 5301349,7 | 6850671,9 | 29,23% | 5062364,8 | 7040531,7 | 39,08% |
| 20 | 5603846,7 | 6979677,5 | 24,55% | 5485894,1 | 7246814,7 | 32,10% | 5308116,6 | 7459545,7 | 40,53% |
| AWGN | 4613314,2 | 4590136,1 | -0,50% | 4218554,6 | 4121415,1 | -2,30% | 3213741,7 | 3010778,9 | -6,32% |
| 1 strong cell, no AWGN | 5722883,5 | 7273968,5 | 27,10% | 5597381,7 | 7519258,3 | 34,34% | 5447408 | 7827762,8 | 43,70% |
| Average gain |  |  | 10,99% |  |  | 16,04% |  |  | 26,07% |

Figure 3 and Figure 4 show the throughput behaviour for Scenario 2 and geometry level 0 and -3dB.

Figure 3. Scenario 2, 0dB

Figure 4. Scenario 2, -3dB

# Conclusions

In this paper we have provided simulation results for the conditional median DIP and for the averaged DIP table. We propose to take these results into account for the computation of average gains of advanced receiver algorithms.

Moreover Case 2 as well as the conditions which are labeled ”to be provided by interested companies” were not sufficiently discussed during the meeting and were not agreed by the group to be considered as baseline. Hence we think that it can be questionable whether the results associated to these conditions can be taken into account to derive average gains of the advanced receiver. Different conditions w.r.t to the baseline conditions which were agreed by the group can be discussed, if needed, during the work item phase.

# References

[1]	R4-116304, “Evaluation methodologies and Simulation assumptions for Enhanced performance requirements for LTE UE SI (Revision 2)”, NTT DOCOMO, ST-Ericsson, Ericsson, Renesas Mobile Europe Ltd.

# Annex

DIP tables provided by Ericsson/ST-Ericsson.

Table 9 Ericsson/ST-Ericsson DIP table for -3dB

| # | DIP1 | DIP2 | DIP3 | DIP4 | DIP5 | DIP6 | DIP7 | DIP8 | DIP9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | [dB] | [dB] | [dB] | [dB] | [dB] | [dB] | [dB] | [dB] | [dB] |
| 1 | -5.8677 | -7.1428 | -8.6273 | -10.4515 | -12.3388 | -14.0114 | -15.5336 | -16.7787 | -17.9149 |
| 2 | -4.4906 | -6.1856 | -8.2153 | -11.1464 | -14.5904 | -17.1839 | -18.963 | -20.0864 | -21.2066 |
| 3 | -4.1803 | -6.2904 | -9.8567 | -12.2766 | -14.4571 | -15.8563 | -17.0488 | -18.4436 | -19.4269 |
| 4 | -3.8679 | -6.4528 | -9.7748 | -12.5505 | -14.8304 | -16.5126 | -17.9345 | -19.2204 | -19.9653 |
| 5 | -3.4573 | -6.3029 | -10.2647 | -13.9225 | -15.7244 | -17.7837 | -18.988 | -20.4838 | -21.5411 |
| 6 | -3.0801 | -5.0333 | -18.6087 | -22.9157 | -25.4422 | -27.4121 | -28.6306 | -29.6289 | -30.5853 |
| 7 | -3.0207 | -3.031 | -31.3366 | -34.3722 | -35.908 | -37.6706 | -38.9567 | -40.019 | -41.2796 |
| 8 | -2.9806 | -4.4785 | -24.7736 | -28.0989 | -30.619 | -32.0424 | -33.5203 | -34.7522 | -35.4484 |
| 9 | -2.8252 | -6.0065 | -17.1215 | -20.6644 | -22.368 | -23.9755 | -25.2726 | -26.7533 | -27.8506 |
| 10 | -2.5302 | -7.1095 | -13.0451 | -16.3515 | -18.4141 | -19.964 | -20.7369 | -21.8691 | -23.0109 |
| 11 | -2.192 | -7.0968 | -14.5731 | -17.7746 | -20.3808 | -21.8042 | -23.0922 | -24.4226 | -25.7011 |
| 12 | -1.8896 | -7.7448 | -14.2922 | -17.5761 | -20.1096 | -22.114 | -23.5282 | -24.6956 | -25.8253 |
| 13 | -1.6585 | -8.267 | -13.9422 | -17.1478 | -19.5727 | -21.5038 | -23.0277 | -24.0357 | -25.0588 |
| 14 | -1.4246 | -9.2622 | -14.8 | -17.8028 | -19.8359 | -21.7787 | -22.8837 | -24.0097 | -25.1998 |
| 15 | -1.2146 | -9.5896 | -15.8045 | -18.491 | -20.3519 | -21.8102 | -23.2596 | -24.558 | -25.5568 |
| 16 | -1.0321 | -10.1727 | -16.9482 | -19.9313 | -21.6003 | -23.1282 | -24.6996 | -25.9659 | -26.8643 |
| 17 | -0.8434 | -10.6448 | -17.7268 | -20.6128 | -22.9563 | -24.6595 | -26.2547 | -28.2719 | -29.5016 |
| 18 | -0.6314 | -11.7499 | -19.3821 | -22.3893 | -23.9848 | -25.2969 | -26.8858 | -28.0669 | -28.8963 |
| 19 | -0.3875 | -13.8403 | -20.9325 | -24.3568 | -26.2844 | -27.5411 | -28.9403 | -29.8351 | -30.9223 |
| 20 | -0.1607 | -17.5633 | -24.9932 | -27.8219 | -30.4978 | -32.2575 | -33.6094 | -34.8986 | -36.069 |

Table 10 Ericsson/ST-Ericsson DIP table for -0dB

| # | DIP1 | DIP2 | DIP3 | DIP4 | DIP5 | DIP6 | DIP7 | DIP8 | DIP9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | [dB] | [dB] | [dB] | [dB] | [dB] | [dB] | [dB] | [dB] | [dB] |
| 1 | -6.213 | -7.1383 | -8.9969 | -10.9965 | -13.1147 | -14.6155 | -15.8355 | -16.7794 | -17.7265 |
| 2 | -5.1159 | -6.861 | -9.4339 | -11.3914 | -13.961 | -15.1213 | -16.6035 | -17.6962 | -18.5537 |
| 3 | -4.6711 | -6.408 | -10.0659 | -12.2611 | -14.1322 | -15.4957 | -16.8239 | -17.8502 | -18.8451 |
| 4 | -4.2915 | -6.1979 | -10.2094 | -12.6791 | -14.7442 | -16.0228 | -17.1936 | -18.8606 | -20.0437 |
| 5 | -4.0541 | -6.17 | -11.2617 | -13.1923 | -14.5144 | -15.7216 | -16.8918 | -18.2625 | -19.5565 |
| 6 | -3.8426 | -6.055 | -11.0147 | -13.2864 | -15.0134 | -16.4658 | -17.7555 | -18.8063 | -20.188 |
| 7 | -3.6123 | -6.1555 | -11.0628 | -14.005 | -15.6856 | -17.1201 | -18.4592 | -19.617 | -20.5712 |
| 8 | -3.3881 | -6.1193 | -11.5838 | -14.0054 | -15.6612 | -17.3465 | -18.6023 | -19.866 | -21.0943 |
| 9 | -3.2015 | -6.7318 | -11.9764 | -14.3366 | -16.187 | -17.7134 | -18.9575 | -20.4128 | -21.3968 |
| 10 | -3.0543 | -5.0764 | -19.8148 | -22.9153 | -24.4543 | -26.168 | -27.2281 | -28.6871 | -29.416 |
| 11 | -2.9497 | -5.7946 | -18.1387 | -20.7744 | -22.8762 | -24.6453 | -25.6338 | -26.8546 | -27.8943 |
| 12 | -2.7043 | -7.0489 | -15.8326 | -18.3487 | -19.8285 | -21.5027 | -22.939 | -23.9216 | -25.0896 |
| 13 | -2.4317 | -7.2002 | -15.3419 | -18.2996 | -20.1298 | -21.5246 | -22.7268 | -23.8021 | -24.7903 |
| 14 | -2.1417 | -7.505 | -16.1587 | -18.7568 | -20.4297 | -21.7792 | -23.434 | -24.4833 | -25.3289 |
| 15 | -1.9022 | -8.1877 | -15.9649 | -18.9637 | -20.711 | -22.0019 | -23.1935 | -24.5648 | -25.7809 |
| 16 | -1.6126 | -8.9666 | -16.8318 | -19.2871 | -21.1167 | -22.6883 | -24.2588 | -25.5367 | -26.8926 |
| 17 | -1.2746 | -10.0438 | -16.2709 | -18.2937 | -20.0613 | -21.6552 | -23.3034 | -24.5813 | -25.5787 |
| 18 | -0.8848 | -11.6853 | -16.9014 | -18.9897 | -20.5568 | -22.4786 | -24.2277 | -25.6468 | -26.9273 |
| 19 | -0.4505 | -13.2655 | -19.2315 | -24.3682 | -26.3203 | -27.5637 | -29.0276 | -30.3315 | -31.2689 |
| 20 | -0.1614 | -16.2675 | -25.5136 | -29.4815 | -31.8823 | -33.7171 | -34.8286 | -36.088 | -36.8849 |