3GPP TSG-RAN WG4 Meeting #63 	R4-122751

# Prague, Czech Republic, 21 – 25 May 2012

| CR-Form-v9.9 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 1323 | rev |  | Current version: | 8.17.0 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handover Dealy from E-UTRAN to cdma2000 1X |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, VIA Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE-RF |  |  |  |  |  |  | Date: |  | 21/05/2012 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-8 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change: |  | The air interface handover delay requirement from E-UTRAN to cdma2000 1X specified in TS 36.133 is too short compared with the existing cdma2000 1x intra-RAT hard handover. Figure 1 shows the handover delay from E-UTRAN to cdma2000 1X. Figure 1 Handover from E-UTRAN to cdma2000 1X From Figure 1, it is shown that the handover from E-UTRAN to cdma2000 1X is similar to cdma20001x intra-RAT hard handoff, where the UE receives Universal Handoff Direction Message (UHDM) from one Base Station (BS) and sends Handoff Completion Message to another BS. Therefore, the requirement for inter-RAT air interface handover delay should not be less than intra-RAT air interface hard handover delay. As specified in 3GPP2 C.S0005 section 2.6.6.2.5.2, if the handoff message (Extended Handoff Direction Message, General Handoff Direction Message or Universal Handoff Direction Message) specifies a hard handoff without return on failure, the mobile station shall send the Handoff Completion Message within T73m seconds after the action time of the received handoff message. According to Table D-1 of C.S0005, T73m is defined as 300ms. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modify air interface handover delay requirement from E-UTRAN to cdma2000 1X from 200ms to 300ms to align with the existing cdma2000 1x intra-RAT hard handover delay requirement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Air interface handover delay requirement from E-UTRAN to cdma2000 1X will not be correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X | X | Test specifications |  |  | TS 36.521-3 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< start of changes >

### 5.4.2	E-UTRAN – cdma2000 1X Handover

##### 5.4.2.1	Introduction

The handover procedure from E-UTRAN to cdma2000 1X is initiated when E-UTRAN sends handover command to the UE through dedicated RRC signalling.

##### 5.4.2.1.1	Handover delay

The handover delay (Dhandover) is defined as the sum of the RRC procedure delay, which is 230 ms and the interruption time specified in section 5.4.2.1.2.

NOTE:	Handover delay requirement is equivalent to cdma2000 1x intra-RAT hard handover requirement T73m as specified in [15].

When the UE receives a RRC message implying handover to cdma2000 1X, the UE shall be ready to start the transmission of the new reverse control channel in cdma2000 1X within Dhandover from the end of the last E-UTRAN TTI containing the RRC command.

##### 5.4.2.1.2	Interruption time

The interruption time is the time between the end of the last TTI containing the RRC command on the E-UTRAN PDSCH and the time the UE starts transmission of the reverse control channel in cdma2000 1X, excluding the RRC procedure delay. The interruption time depends on whether the target cell is known to the UE or not.

NOTE:	The interruption time specified here does not include the time for the UE to determine the target cell is of sufficient signal quality as specified in [15].

A cdma2000 1X cell is known if it has been measured by the UE during the last 5 seconds otherwise it is unknown. Under the reference conditions specified in sub-clause 4.2.1 of [14], the interruption time shall be less than Tinterrupt:

Tinterrupt = TIU + 40 + 10*KC*SWK + 10*OC*SWO ms

Where:

TIU	It is the interruption uncertainty when changing the timing from the E-UTRAN to the new cdma2000 1X cell. TIU can be up to one cdma2000 1X frame (20 ms).

SWK 	is  SWK = where srch_win_k is the number of cdma2000 1x chips indicated by the search window for known target cdma2000 1x cells in the message

SWO 	is  SWO = where srch_win_o is the number of cdma2000 1x chips indicated by the search window for unknown target cdma2000 1x cells in the message

KC 	It is the number of known target cdma2000 1X cells in the message, and

OC 	It is the number of unknown target cdma2000 1X cells in the message.

< end of changes >