3GPP TSG RAN WG4 Meeting #63                                                    R4-123426

Prague, Czech Republic, May 21th – 25th , 2012

Source: 	Broadcom Corporation

Title: 	Link level performance results for MMSE-IRC receiver

Agenda Item: 	6.25.2

Document for: 	Discussion

# Introduction

In RAN4 #61, link level assumptions of enhanced performance requirement for LTE UE study item were agreed in which the advanced receiver is an IRC receiver, based on correlation matrix estimation approximation [1]. The results of this study item were captured in [2]. During the RAN#55 meeting the work item for interference rejection was approved [3].

In the study item phase, the following transmission modes were agreed as simulation assumptions [1]:

- CRS based transmission: TM6 as serving cell and TM4 as interfering cells

- CSI-RS based transmission: TM9 with single-layer as serving cell and TM9 with single/dual-layer as interfering cells

However, other transmission modes were not precluded. Since TM2/TM3 could perform better than TM4 in mid/high UE speed environments and some operators have already deployed these transmission modes, it was agreed to add the following transmission modes to the simulation assumptions [4]:

- CRS based transmission: TM2 as serving cell and TM3 as interfering cells

This contribution presents link level simulation results characterizing the gains of advanced receivers over MMSE implementations for Scenario 1-1 (TM2), based on the agreed simulation assumptions in [4].

# Simulation Assumptions

Table 1 summarizes the different simulation assumptions which were agreed in [4] for the CRS-based scenario 1-1.

Table 1: Simulation assumptions for link level performance evaluation

| Parameter | Scenario 1-1 (TM2) |
| --- | --- |
| Carrier frequency | 2 GHz |
| System bandwidth | 10 MHz |
| Transmission mode in serving cell | TM2 |
| Transmission mode in interfering cells | TM3 |
| MIMO configuration | 2x2, medium correlation |
| Channel model and Doppler frequency for target and interfering cells | EVA5 (also higher velocities can be considered in additions) Use different channel seed for between cells |
| Number of interfering cells | 1 & 2 to be considered |
| Geometry | G=-2.5dB and G=0dB |
| DIP values | At G=-2.5dB: DIP1= -1.73dB and DIP2=-8.66dB At G=0dB: DIP1=-2.0561dB and DIP2=-8.2463dB |
| CRS configuration | 2 CRS ports with planning (non-colliding) |
| CSI-RS configuration | None |
| MCS for target signal | Fixed MCS as follows: #10, #11 for G=0dB, and #7, #8 for G=-2.5 dB as baseline |
| PMI for target signal | N/A |
| HARQ | 8 HARQ processes and max 4 transmissions |
| Feedback periodicity for target signal | Feedback periodicity: 5 msec Feedback delay: 8 msec |
| PMI granularity and rank of interfering signals (% of rank-1 and % of rank-2) | Randomly changing per sub-band from subframe to subframe as baseline. Randomly changing per sub-band per 10 msec periodicity by interested companies Frequency granularity is 6 PRBs |
|  | [80% rank-1,20% rank-2] |
| Modulation in interfering cells | Fixed modulation order: QPSK or 16QAM |
| PCFICH | CFI = 2 |
| PCFICH/PDCCH detection | Not considered |
| Resource allocation | 50 PRBs |
| Cyclic prefix | Normal |
| Simulation length | 10000 sub-frames at minimum |

In addition to the configuration described in Table 1, the following assumptions were used for the link level performance simulations presented in this paper:

- Network configuration: Synchronous

- Channel Estimation: Ideal

- Modulation in interfering cells: QPSK & 16QAM

- Feedback periodicity for target signal: N/A

# Discussion

The transmission scheme here is the Alamouti (space-frequency block code) transmission scheme. The system model is given by

where matrix u represents the contribution of the interference plus noise. We assume the interference is correlated across the receive antennas and is uncorrelated across the channel uses. Rewriting the above equation, we have

or equivalently

The following two UE receivers can be defined for evaluation:

- Release 8 Baseline MMSE Receiver:  For Release 8 baseline receiver, the MMSE receiver can be expressed as follow:

WMMSE=H*R-1,

where

R=HH*+σI+N2I,

where σI+N2 denotes the interference plus noise power.

- RAN4 Enhanced MMSE-IRC Receiver:

MMSE-IRC has been the baseline assumption for the enhanced performance requirement for LTE UE receiver study item. The MMSE-IRC receiver for the above channel model can be written as

WMMSE-IRC=H*R-1,

where

R=Eyy*,

denotes the spatial correlation matrix of the total received signal.

# Simulation Results for conditional median DIP profiles

The TM2/TM3 results for the conditional median DIP profiles for one interferer with an MMSE receiver as well as the advanced MMSE-IRC receiver are given in Table 2.

Table 2: Link level throughput (Mbps) in TM2 scenario with one TM3 interferers

|  |  |  | MMSE | MMSE-IRC | Improvement |
| --- | --- | --- | --- | --- | --- |
| 4QAM | G = -2.5 dB | MCS#7 | 3.836 | 4.453 | 16.08 % |
|  |  | MCS#8 | 3.712 | 4.42 | 19.07 % |
|  | G = 0 dB | MCS#10 | 5.105 | 5.291 | 3.64 % |
|  |  | MCS#11 | 5.136 | 5.302 | 3.23 % |
| 16QAM | G = -2.5 dB | MCS#7 | 3.805 | 4.460 | 17.21 % |
|  |  | MCS#8 | 3.7244 | 4.235 | 13.71 % |
|  | G = 0 dB | MCS#10 | 5.076 | 5.284 | 4.10 % |
|  |  | MCS#11 | 5.159 | 5.321 | 3.14 % |

The TM2/TM3 results for the conditional median DIP profiles for two interferers with an MMSE receiver as well as the advanced MMSE-IRC receiver are given in Table 3.

Table 3: Link level throughput (Mbps) in TM2 scenario with two TM3 interferers

|  |  |  | MMSE | MMSE-IRC | Improvement |
| --- | --- | --- | --- | --- | --- |
| 4QAM | G = -2.5 dB | MCS#7 | 3.832 | 4.589 | 19.75 % |
|  |  | MCS#8 | 3.726 | 4.425 | 18.76 % |
|  | G = 0 dB | MCS#10 | 5.061 | 5.455 | 7.79 % |
|  |  | MCS#11 | 5.136 | 5.545 | 7.96 % |
| 16QAM | G = -2.5 dB | MCS#7 | 3.795 | 4.560 | 20.16 % |
|  |  | MCS#8 | 3.737 | 4.382 | 17.26 % |
|  | G = 0 dB | MCS#10 | 5.038 | 5.451 | 8.20 % |
|  |  | MCS#11 | 5.146 | 5.498 | 6.84 % |

# Conclusion

This contribution shows the link simulation results for TM2/TM3 scenario for testing MMSE-IRC receiver.

# References

- R4-116304, “Evaluation methodologies and Simulation assumptions for Enhanced performance requirements for LTE UE SI”, NTT DOCOMO.

- TR 36.829 v1.0.0, “Enhanced performance requirement for LTE UE Study Item”

- RP-120382, “New WI: Improved minimum performance requirements for E-UTRA: interference rejection”

- R4-122201, “Simulation assumptions for enhanced receiver”, Renesas Mobile Europe Ltd.