3GPP TSG RAN WG4 Meeting #63	R4-122651

Prague, Czech Republic, 21-25 May, 2012

Agenda Item:	4.2.3

Source: 	Ericsson, ST-Ericsson

Title: 	Inter-frequency and Inter-RAT Requirements for Measurements without Measurement Gaps

Document for:	Discussion

# Introduction

Since RAN4#60bis requirements for inter-frequency measurements for UEs that are capable of conducting such measurements without measurement gaps have been discussed [1][2]. No agreement was reached and the topic was further discussed at RAN4#61 (e.g. [3][4]), resulting in a Way Forward agreement [5] where the following was settled:

- Measurement accuracy: existing inter-frequency requirements apply

  - Inter-frequency RSRP accuracy requirements in 36.133 section 9.1.3 [6]

  - Inter-frequency RSRQ accuracy requirements in 36.133 section 9.1.3

- Side conditions: existing inter-frequency requirements apply

  - RSRP, RSRP Ês/Iot, SCH_RP and SCH Ês/Iot as specified in 36.133 Annex B.2.3

  - Additional RSRP side conditions as specified in 36.133 section 9.1

- Number of cells: existing inter-frequency requirements apply

  - Minimum requirement 4 monitored cells per carrier as specified in 36.133 section 8.1.2.3

- Number of monitored carriers: existing inter-frequency requirements apply

  - Number of carriers as specified in 36.133 section 8.1.2.1.1

- Cell identification delay and L1 measurement period: identification times and measurement periods are scaled by the total number of carriers monitored by the UE (with or without gaps)

  - Scaling by Nfreq which is specified in 36.133 section 8.1.2.1.1

The outstanding issues were related to cell identification and L1 measurement period, and particularly the following

- Cell identification delay and L1 measurement period: The cell identification and L1 measurement period are FSS. There are following options:

  - Option 1: intra-frequency requirements apply

  - Option 2: inter-frequency requirements (based on 40ms gap periodicity) apply

  - Option 3: FFS

This issue was further discussed at RAN4#62 where we argued for Option 1 as reasonable from system perspective although it would result in a somewhat increased processing load on the UE side [7]. However no agreement was reached then. In RAN4#62bis there were discussions about the increased UE processing load and power consumption in case measurement period is based on intra-frequency requirements.

In this contribution we propose a solution based on previous Option 2, but extend it to also include inter-RAT. The requirements are applicable for any UE which is capable of measuring without gaps i.e. not limited to CA UE.

# Proposal on Cell Identification and L1 Measurement delays

Taking the concerns about increased UE processing load into account it is proposed that a UE that can carry out inter-frequency and inter-RAT measurements without measurement gaps is to fulfil the same requirements as had Gap Pattern Id #0 been used (6 ms gap length, 40 ms gap periodicity). Particularly requirements on detection time that are based on how much time a UE can spend on intra-frequency carrier and/or inter-frequency carrier shall be identical to those for a UE that requires measurement gaps and which has been configured with Gap Pattern Id #0. However we propose that the requirements are applicable for all UE capability capable of performing inter-frequency and inter-RAT measurements without gaps. In other words the requirements without gaps should not be limited to carrier aggregation capable UE.

Proposal 1: Measurement requirements for a UE that is capable of carrying out inter-frequency and inter-RAT measurements without gaps shall in all respects be identical to those for a UE that requires measurement gaps and has been configured with Gap Pattern Id#0.

Since a generic requirement is proposed, there is also a need for removal of consistencies in the current revision of the specification [6]. The inconsistencies are the following:

- For UE UTRA FDD CPICH measurement capability (8.1.2.4.1.1.2), it is stated:

If the UE does not need measurement gaps to perform UTRA FDD measurements, the measurement period for UTRA FDD measurements is 480 ms.

- For Identification of new UTRA TDD cell (8.1.2.4.3.1.1), it is stated:

If the UE does not require transmit gap to perform inter-RAT UTRA TDD measurements, the UE shall be able to identify a new detectable inter-RAT UTRA TDD cell belonging to the monitored set within 5000 ms.

- For Enhanced UTRA TDD cell identification requirements (8.1.2.4.3.1.1a), it is stated:

If the UE does not require transmit gap to perform inter-RAT UTRA TDD measurements, the UE shall be able to identify a new detectable inter-RAT UTRA TDD cell belonging to the monitored set within 500 ms.

- For UE UTRA TDD P-CCPCH RSCP measurement capability (8.1.2.4.3.1.2), it is stated:

If the UE does not need measurement gaps to perform UTRA TDD measurements, the measurement period for UTRA TDD measurements is 480 ms.

- For E-UTRAN FDD-cdma2000 1xRTT measurements when no DRX is used (8.1.2.4.9.1A), it is stated:

If the UE does not need measurement gaps to perform CDMA2000 1xRTT Pilot Strength measurements, the measurement period is given by

.

- E-UTRAN FDD - cdma2000 1xRTT measurements for SON ANR ( 8.1.2.4.15.1.1), it is stated:

If the UE does not need measurement gaps to perform CDMA2000 1xRTT Pilot Strength measurements, the measurement period is given by

All of those inconsistencies are proposed to be removed as a consequence of accepting Proposal 1. It shall be noted that the requirements are not tightened hence it shall not pose a problem for existing UE implementations.

Proposal 2: Inconsistencies by means of UE requirements that differ between a UE that is requiring gaps for inter-frequency and inter-RAT measurements and which is configured with Gap Pattern Id#0, and a UE that can carry out such measurements without gaps, shall be removed.

Our proposals are captured in the two CRs: [8] and [9].

# Conclusions

We are proposing the following

Proposal 1: Measurement requirements for a UE that is capable of carrying out inter-frequency and inter-RAT measurements without gaps shall in all respects be identical to those for a UE that requires measurement gaps and has been configured with Gap Pattern Id#0.

Proposal 2: Inconsistencies by means of UE requirements that differ between a UE that is requiring gaps for inter-frequency and inter-RAT measurements and which is configured with Gap Pattern Id#0, and a UE that can carry out such measurements without gaps, shall be removed.

Our proposals are captured in the two CRs: [8] and [9].

# References

- R4-115292 “Introduction of UE Measurement Capability for non configured frequencies without measurement gaps”, NTT DOCOMO, Qualcomm.

- R4-115024 “Inter-frequency Measurement Requirements on Non-Configured Frequencies without Measurement Gaps”, Ericsson, ST-Ericsson.

- R4-115697 “Further Analysis of Inter-frequency Requirements for Measurements without Gaps”, ST-Ericsson, Ericsson

- R4-115963 “Considerations on measurement performance without measurement gaps”, Renesas Mobile Europe Ltd.

- R4-116151 “Way forward on requirements for inter-frequency measurements without gaps”, NTT DoCoMo, Qualcomm Inc., Ericsson, ST-Ericsson, Renesas Mobile Europe Ltd., ZTE.

- 3GPP TS 36.133

- R4-120579, “Analysis of Inter-frequency Requirements for Measurements without Measurement Gaps”, Ericsson, ST-Ericsson

- R4-122654, “Inter-Frequency and inter-RAT Requirements for Measurements without Measurement Gaps – Rel-10”, Ericsson, ST-Ericsson

- R4-122656, “Inter-Frequency and inter-RAT Requirements for Measurements without Measurement Gaps – Rel-11”, Ericsson, ST-Ericsson