3GPP TSG-RAN WG4 meeting #64bis 	R4-125810

Santa Rosa, USA, 8 – 12 October, 2012

Source:	Motorola Mobility

Title:	Determination of Pcmax for scenarios with different TAGs

Agenda item:	6.8.1

Document for:	Discussion/Decision

# Introduction

There have been several LSs from RAN1 to RAN4 on the topic of handling the overlap between subframes of different component carriers for the UL CA + multiple timing advance (MTA) scenario [1],[2],[3],[4]. RAN4 has provided some views in [5], but several outstanding issues still remain.

In this document we provide our view on open issues related to UL CA + MTA scenario, and present some solutions for addressing them.

# Discussion

## ON/OFF Time Masks and Transient Period for UL CA

In [1], RAN1 asked RAN4 - “How would the transient period be defined when there are multiple TAGs”.

In [5], RAN4 replied – “RAN WG4 has not considered modifying the transient period definitions because of multiple TAGs”.

In [2], RAN1 replied saying – “RAN1 notes that, according to current agreements, the maximum offset between TAGs may be up to 30us and therefore the period during which this UE power adjustment is applied could exceed the currently defined transient period”, and has requested RAN4 to “consider adjusting the transient period accordingly”.

Transient period is currently defined in 36.101 [6] Section 6.3.4 (see Figures 6.3.4.1-1, 6.3.4.2-1, 6.3.4.2.2-1, 6.3.4.2.2-2, 6.3.4.3-1, 6.3.4.4-1, 6.3.4.4-2, 6.3.4.4-3, 6.3.4.4-4) as part of discussion on UE ON/OFF time mask. When the UE is configured with UL CA, how the transient period shown in these figures applies to each individual component carrier (CC) is currently not specified (at least for all the explicit physical channel and signal cases). We think RAN4 should first consider clarifying the applicability of all the time masks (and consequently transient periods) for UL CA with single TA in 36.101 before considering RAN1’s request to adjust the transient period for MTA.

Section 6.3.4A in 36.101 describes how the general output power ON/OFF mask (in figure 6.3.4.1-1) is applicable for intra-band contiguous carrier aggregation. However, the applicability of slot/subframe boundary time masks, PRACH/SRS time masks and PUCCH/PUSCH/SRS time masks for UL CA is currently not defined. Some discussion on the applicability of other time masks to UL CA occurred in RAN4 #59 [7] and RAN4 #59AH [8] but there was no decision to update the text in 36.101. Following the principles in [7], [8] and the current text in 36.807 Section 6.3.4 [9], we propose that RAN4 adopt the following definitions for time periods with ON and OFF power requirements and transient periods for UL CA

Proposal 1:

For UL CA, RAN4 should adopt the following definitions for time periods with ON and OFF power requirements

- For a UE configured with UL CA, ON power requirements apply for a CC, according to the time masks shown in section 6.3.4, only when none of the UL CCs have a transient period according to the time masks shown in section 6.3.4.

- For a UE configured with UL CA, OFF power requirements apply for a CC, according to the time masks shown in section 6.3.4, only when all the UL CCs have an OFF power requirement according to the time masks shown in section 6.3.4.

The consequences of adopting the above definitions are the following

- If at least one of the UL CCs has a transient period during a given time duration (based on time masks shown in section 6.3.4), all the other UL CCs are also treated as having a transient period during that time duration (this aspect need not be explicitly stated in the specification).

- For the MTA scenarios, the overlap period between transmissions in subframe n of a first CC and subframe n+1 of a second CC, falls within the combined transient period of all the CCs (as indicated in ‘1’ above) as long as the time offset difference between the CCs is less than <40us. This addresses the issue raised by RAN1 (in [2]) about adjusting transient periods to account for up to 30us time difference between CCs.

- Given the ON power requirements do not apply for the partial overlap period between 2CCs (since the overlap is part of the combined transient period (as indicated in ‘1’ above), a new definition of Pcmax for the partial overlap period between two CCs (as requested by RAN1 in [4]) is not required.

Figures 1, 2, 3 show how the ON/OFF power requirements apply when proposal 1 is used, for the cases of,, and respectively where is the difference in timing advance between two CCs (CC1 and CC2).

Figure 1 – Illustration of ON/OFF requirements for UL-CA when no time difference between CCs

Figure 2 – Illustration of ON/OFF requirements for UL-CA when

Figure 3 – Illustration of ON/OFF requirements for UL-CA when  is between 20us and 30us

# Conclusions

We propose the following

- Proposal 1: For UL CA, RAN4 should specify the following definitions for time periods with ON and OFF power requirements in TS 36.101

  - For a UE configured with UL CA, ON power requirements apply for a CC, according to the time masks shown in section 6.3.4, only when none of the UL CCs have a transient period according to the time masks shown in section 6.3.4.

  - For a UE configured with UL CA, OFF power requirements apply for a CC, according to the time masks shown in section 6.3.4, only when all the UL CCs have an OFF power requirement according to the time masks shown in section 6.3.4.

- Proposal 2: RAN4 should respond to RAN1 LSs [2],[4], indicating that RAN4 has agreed on Proposal 1, and given the agreement,

  - RAN4 definitions for time periods with ON and OFF power requirements address the issue raised by RAN1 in [2] about adjusting transient periods to account for up to 30us time difference between CCs

  - RAN4 does not see a need for defining Pcmax for the partial overlap period between two CCs

# References

- R4-121991 (R1-120946) - “LS on simultaneous transmission of PUSCH/PUCCH/SRS for multiple TA”, RAN WG1, RAN4 #62bis, March 2012, Jeju, South Korea.

- R4-124730 (R1-123067) – “LS on simultaneous transmission of PUSCH/PUCCH/SRS for multiple TA”, RAN WG1, RAN4 #64, Aug 2012, Qingdao, China.

- R4-125854 (R1-124027) – “LS on parallel transmission of SRS and PUSCH/PUCCH for multiple TAGs”, RAN WG1, RAN4 #64bis, Oct 2012, San Diego, USA.

- R4-125855 (R1-124028) – “LS on the value of Pcmax for the partial overlap period between different TAGs”, RAN WG1, RAN4 #64bis, Oct 2012, San Diego, USA.

- R4-122226 – “Reply LS on simultaneous transmission of PUSCH/PUCCH/SRS for multiple TA”, RAN WG4, RAN4 #62bis, March 2012, Jeju, South Korea.

- 3GPP TS 36.101 V11.2.0 (2012-09)

- R4-112940 – “Discussion on LTE-CA UE time mask requirements”, CATT, RAN4 #59, May 2011, Barcelona, Spain.

- R4-113563 – “Further Discussion on CA UE time mask requirements”, CATT, RAN4 #59AH, June/July 2011, Bucharest, Romania.

- 3GPP TR 36.807 v10.0.0 (2012-07)