TSG-RAN Working Group 4 (Radio) meeting #65	R4-126751

New Orleans, Louisiana, 12-16 November 2012

Source:	TruePosition

Title:	Discussion on 36.111 Section 6 Measurement Time Requirements

Agenda item:	6.16.1

Document for:	Discussion

1	Introduction

In this contribution we discuss the LMU’s UL RTOA measurement time requirements.

2	Discussion

LMUs consist of an RF front end that converts the UE transmissions to baseband for sampling and digital signal processing resources for the measurement of a target UE’s transmission UL RTOA.  When a LMU is requested to perform a measurement the LMU will complete the two step process shown in the simplified block diagram below.  Note that the LMU can have K instances of the Acquisition block and L instances of the Delay Processing block.  For the purpose of specifying the UL RTOA measurement time requirements we assume that the LMU’s Acquisition block can perform multiple acquisitions for different UE carriers that are in the same RF band with an equal number of delay processing blocks so that K = L and that all parallel UL RTOA have the same SRS periodicity and number of SRS transmissions.

Figure 1 Simplified LMU Block Diagram

The diagram above suggests that the allocation of a time budget for LMU UL RTOA measurements be made into two parts, the time it takes to acquire or collect the sampled UE SRS transmissions and the time it takes for the LMU to compute the UL RTOA from the collected samples in the delay processing block.  The time to acquire the sampled UE SRS transmissions will depend on the SRS periodicity, TSRS in ms, and the number of expected UE SRS transmissions, M, so that the time to collect the sampled UE transmissions, TAcq-SRS, is

TAcq-SRS = TSRS × (M-1) ms

The time to process the collected samples is dependent on the number of UE SRS transmissions, M, and the speed of the delay processing hardware.  Since the capabilities of the delay processing hardware is dependent on the implementation it is appropriate to set a maximum time requirement for the delay processing block instead of using an expression dependent on M.  Let the maximum time to compute the UL RTOA from the acquired UE SRS transmissions as = 5000 ms and the time required to perform one UL RTOA, TUL_RTOA, measurement is then

TUL_RTOA = TAcq-SRS + ms

When the LMU is capable of processing L UL RTOA measurements in parallel the time required to process up to L measurements at any time will remain the same since those L measurements will not have to wait for acquisition and processing resources to become available.  Accounting for the availability of K acquisition and L processing resources, where K equals L, the modified expression for the UL RTOA measurement time for N parallel measurements becomes

ms,

or alternatively

ms

Proposal 1:  State the time required for the LMU to acquire and process UL RTOA measurements to be

ms

3	Summary

In this contribution we discussed the LMU’s UL RTOA measurement time requirements.  To account for the LMU’s capability in performing N measurements in parallel the following is proposed:

Proposal 1:  State the time required for the LMU to acquire and process UL RTOA measurements to be

ms