TSG-RAN Working Group 4 (Radio) meeting #65	R4-126759

New Orleans, Louisiana, 12-16 November 2012

Source:	TruePosition

Title:	Discussion of LS on Uplink Positioning Parameters

Agenda item:	6.16.1

Document for:	Discussion

1	Introduction

RAN 3 submitted an LS, R3-122373, to RAN 4 asking RAN 4 to validate the need for certain IEs used by the SLmAP protocol as described in 3GPP TS 36.459, and confirm type, range and units for some IEs.  This contribution addresses these requests and discusses the usage of these IEs by the LMU and E-SMLC.

2	Discussion

In figure 1 we have an LMU at the serving and adjacent cells and the UE is in the serving cell.  The Expected Propagation Delay is the propagation delay between the LMU at the serving cell and the LMU at the adjacent cell.  In this scenario we expect the propagation delay to be twice the cell radius.  Which LMUs participate in a UE’s location measurement is related to the LMU’s measurement sensitivity and processing gain, the interference level at the LMUs and how far the participating LMUs are from the serving cell.  For the larger cell size spacing the interference from adjacent cells is expected to be lower than it would be for more closely spaced cells so the LMUs’ sensitivity and processing gain will determine the maximum separation between the LMU at the serving cell and the participating LMUs.  A cell range of 50 km is possible in a rural scenario operating at 700 MHz and the eNB antenna is mounted high enough.  Since the propagation delay is twice the cell radius the upper value of the Expected Propagation Delay IE should be able to accommodate the maximum delay equivalent to 100 km or approximately 340000 ns.

Figure 1 LMUs at Serving and Adjacent Cells

Expressing the propagation delay in ns provides more precision than is necessary.  Since the LMU samples and operates on UE transmissions and the sample interval is a multiple of Ts it would be more appropriate to express the propagation delay in integer multiples of Ts.

Proposal 1:  the Expected Propagation Delay IE shall be represented as:

| Expected Propagation Delay IE |  |
| --- | --- |
| Type | INTEGER |
| Units | Ts |
| Minimum Value | 30 |
| Maximum Value | 10500 |

The Delay Uncertainty IE is the uncertainty as shown in Figure 1.  For a maximum cell size of 50 km the uncertainty should not be larger than the expected propagation delay.  Therefore the upper value of the Delay Uncertainty IE should be able to accommodate the maximum delay equivalent to 100 km or approximately 340000 ns. The Delay Uncertainty IE can also be expressed in units of TS.

Proposal 2:  the Delay Uncertainty IE shall be represented as:

| Delay Uncertainty IE |  |
| --- | --- |
| Type | INTEGER |
| Units | Ts |
| Minimum Value | 30 |
| Maximum Value | 10500 |

In figure 2 we have an LMU at the serving cell and the adjacent cells.  In this scenario the UE is not in the serving cell resulting in the case where the UE is closer to an adjacent cell than the serving cell so that for the LMUs that are closer to the UE than the LMU at the serving cell the UL RTOA will be negative.  Therefore, the range of the UL RTOA IE will include negative values.  The RTOA value returned by the LMU will be bounded by the sum of the expected propagation delay and ± half the uncertainty, for which using their maximum values we have the range equivalent to 100 ± 50 km.  It is unlikely that an UL RTOA value equivalent to 150 km would ever be encountered but if we limit the range of the UL RTOA to ± 100 km there should not be any need to change the range of the UL RTOA IE in the future.

LMUs use a time base locked to a GNSS so that the typical timing accuracy of an LMU is 15 ns RMS or better.  For that reason the precision of the UL RTOA IE  should be smaller than Ts but larger than a ns therefore the units of

Figure 2 UE in an Adjacent Cell

Proposal 3:  the UL RTOA IE shall be represented as:

| UL RTOA IE |  |
| --- | --- |
| Type | INTEGER |
| Units | 0.25 Ts |
| Minimum Value | -42000 |
| Maximum Value | 42000 |

The UL RTOA Metric IE is the quality indicator used by the E-SMLC to weight the measured UL RTOA value in the UE’s location solution.  Since the UE’s UL RTOA is found through correlation against the ideal reconstructed SRS sequence an appropriate metric is the correlation coefficient.

Proposal 4:  the UL RTOA Metric IE shall be represented as a unsigned 16-bit integer with 65535 equivalent to a correlation coefficient of 1.0:

| UL RTOA Metric IE |  |
| --- | --- |
| Type | INTEGER |
| Units | Represents the correlation coefficient |
| Minimum Value | 0 |
| Maximum Value | 65535 |

- In the LMU Information IE (Sec. 9.2.8):

  - Type, range and units for the set of measurement-related LMU capabilities (ListofRFBands IE, FreqBandIndicator IE, NumberCC IE, NumberULRTOA IE) signaled in the SLm SETUP REQUEST. RAN4 should also confirm the usage of the NumberULRTOA IE and FreqBandIndicator IE.

The FreqBandIndicator IE is used by the LMU during the setup procedure to inform the E-SMLC which RF bands the LMU is capable of operating on.  The E-SMLC needs to know which bands the LMU is capable of operating on so that it doesn’t try to task an LMU for a UL RTOA measurement that the LMU does not support.  In a scenario where there are LMUs with a mix of RF band support, the E-SMLC can appropriately choose which LMUs to task for a UL RTOA measurement in order to minimize the resulting position dilution of precision.  Expressing the supported bands as a BITSTRING(64) would be the most efficient representation and is large enough to accommodate the likely maximum number of  RF Bands supported by E-UTRA.

Proposal 5:  the FreqBandIndicator IE shall be represented as a BITSTRING(64). A band is shown as supported by the LMU by setting bit(band-1)or otherwise clearing that bit.

The NumberULRTOA IE is used by the LMU to indicate to the E-SMLC how many parallel, not necessarily simultaneous, UL RTOA measurements it can support.  The E-SMLC can use this information to manage the scheduling of LMU UL RTOA measurements.  In the scenario where the LMUs are highly loaded the E-SMLC will be able to appropriately choose which LMUs to task for a UL RTOA measurement in order to minimize the resulting position dilution of precision.  We propose that the maximum value of the NumberULRTOA IE be extensible with an initial value of 16 so that the range of the NumberULRTOA IE will be from 1 to 16.

Proposal 6:  the NumberULRTOA IE shall be represented as an extensible INTEGER with the range from 1 to 16.

| NumberULRTOA IE |  |
| --- | --- |
| Type | INTEGER |
| Units | Represents number of parallel UL RTOA measurements an LMU can process |
| Range | 1..16,... |

The NumberCC IE is used by the LMU to indicate how many CA combined carriers it supports.  Unlike the NumberULRTOA IE the NumberCC IE refers the number of concurrent or simultaneous UE frequency carriers the LMU can support.  The E-SMLC will use the Number CC IE to appropriately select which carrier or set of carriers the LMU will use to process an UL RTOA measurement in order to maximize the LMU’s processing gain when the UE’s SRS transmissions are not equally allocated across the CA carriers.  We propose that the maximum value of the NumberCC be extensible with an initial value of 2 so that the range of the NumberULRTOA IE will be from 1 to 2.

Proposal 7:  the NumberCC IE shall be represented as an extensible INTEGER with the range from 1 to 16.

| NumberCC IE |  |
| --- | --- |
| Type | INTEGER |
| Units | Represents number of concurrent CA carriers the LMU can process for a UL RTOA measurement |
| Range | 1..2,... |

3	Summary

In this contribution we base the following proposals on the above discussion:

Proposal 1:  the Expected Propagation Delay IE shall be represented as:

| Expected Propagation Delay IE |  |
| --- | --- |
| Type | INTEGER |
| Units | Ts |
| Minimum Value | 30 |
| Maximum Value | 10500 |

Proposal 2:  the Delay Uncertainty IE shall be represented as:

| Delay Uncertainty IE |  |
| --- | --- |
| Type | INTEGER |
| Units | Ts |
| Minimum Value | 30 |
| Maximum Value | 10500 |

Proposal 3:  the UL RTOA IE shall be represented as:

| UL RTOA IE |  |
| --- | --- |
| Type | INTEGER |
| Units | 0.25 Ts |
| Minimum Value | -42000 |
| Maximum Value | 42000 |

Proposal 4:  the UL RTOA Metric IE shall be represented as a unsigned 16-bit integer with 65535 equivalent to a correlation coefficient of 1.0:

| UL RTOA Metric IE |  |
| --- | --- |
| Type | INTEGER |
| Units | Represents the correlation coefficient |
| Minimum Value | 0 |
| Maximum Value | 65535 |

Proposal 5:  the FreqBandIndicator IE shall be represented as a BITSTRING(64). A band is shown as supported by the LMU by setting bit(band-1)or otherwise clearing that bit.

Proposal 6:  the NumberULRTOA IE shall be represented as an extensible INTEGER with the range from 1 to 16.

| NumberULRTOA IE |  |
| --- | --- |
| Type | INTEGER |
| Units | Represents number of parallel UL RTOA measurements an LMU can process |
| Range | 1..16,... |

Proposal 7:  the NumberCC IE shall be represented as an extensible INTEGER with the range from 1 to 2.

| NumberCC IE |  |
| --- | --- |
| Type | INTEGER |
| Units | Represents number of concurrent CA carriers the LMU can process for a UL RTOA measurement |
| Range | 1..2,... |