3GPP TSG RAN WG4 Meeting #66BIS	    					R4-131245

Chicago, USA, April 15th-19th 2013

Agenda Item: 	5.4

Source: 		Broadcom Corporation

Title: 		TM3 demodulation performance for high Doppler Tests

Document for: 		Discussion

# Introduction

RAN4 is evaluating to introduce high Doppler test cases for TM3 Rank 2 PDSCH demodulation performance (to cover higher speeds for high frequency bands [1]).

In RAN4#66, the way forward [2] has been agreed, which invited interested companies to present simulation results and evaluate the feasibility of introducing the high Doppler test cases.

The FRC that is being considered:

- For Doppler frequency, consider 200Hz

- For channel model, consider EVA 2x2 low correlation channel

- For MCS, consider 64-QAM ½

- Consider both FDD and TDD cases

- TM : TM3 rank 2

- FRC:  R.35 FDD and R.35 TDD

- SNR test points : 4:1:26 dB

In this contribution, we provide our simulation results for the above defined FRC and outline our conclusions.

# Simulation Results

In this section, based on the agreed FRC, we provide simulation results of the link level throughput.  For completeness, the simulation setup and the simulation results are presented. The simulation parameters are similar to the tests of 8.2.1.3.1 of [8] for FDD and tests of 8.2.23.1 of [8] for TDD.

## FDD

### Simulation Setup

| Parameter |  | Unit | Setting |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| PDSCH transmission mode |  |  | 3 |
| Precoding Type |  |  | Large Delay CDD |
| Propagation Condition |  |  | EVA |
| Correlation matrix and antenna config. |  |  | 2x2 Low |
| Channel Doppler |  | Hz | 200 |
| Reference Channel |  |  | R.35 FDD |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 |
| # HARQ Process |  |  | 8 |
| Maximum number of HARQ transmissions |  |  | 4 |
| Number of OFDM symbols for PDCCH |  | Symbols | 2 |
| Cyclic Prefix |  |  | Normal |
| SNR Test Points |  | dB | 4:1:26 |
| Max Throughput |  | Mbps | 35.424 |

### Simulation Results

## TDD

### Simulation Setup

| Parameter |  | Unit | Setting |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| PDSCH transmission mode |  |  | 3 |
| Precoding Type |  |  | Large Delay CDD |
| Propagation Condition |  |  | EVA |
| Correlation matrix and antenna config. |  |  | 2x2 Low |
| Channel Doppler |  | Hz | 200 |
| Reference Channel |  |  | R.35 TDD |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 |
| # HARQ Process |  |  | 7 |
| Maximum number of HARQ transmissions |  |  | 4 |
| Number of OFDM symbols for PDCCH |  | Symbols | 2 |
| Cyclic Prefix |  |  | Normal |
| Uplink downlink configuration |  |  | 1 |
| Special subframe configuration |  |  | 4 |
| SNR Test Points |  | dB | 4:1:26 |
| Max Throughput |  | Mbps | 14.276 |

### Simulation Results

## Observations

Based on the simulation analysis we have done, we observe:

- Observation 1: Simulation results suggest that we can achieve peak throughput within the SNR range considered. It is feasible to have high Doppler tests for TM3 Rank 2 demodulation with the agreed FRCs as outlined in [2].

- Observation 2: We can achieve 70% of the max throughput at 18.3dB SNR for FDD and at 18.0dB SNR for TDD.

Based on the observations, we draw the following conclusions

- Proposal 1: We propose to use the FRC as outlined in [2] for testing TM3 at high Doppler: EVA LOW CORR 2x2, TM3 Rank 2, R.35 FDD and R.35 TDD

- Proposal 2: We propose the passing criteria to be 70% of the maximum throughput at [18.3dB + Implementation Margin] SNR for FDD and [18.0dB + Implementation Margin] for TDD.

# Conclusions

Observations:

- Observation 1: Simulation results suggest that we can achieve peak throughput within the SNR range considered. It is feasible to have high Doppler tests for TM3 Rank 2 demodulation with the agreed FRCs as outlined in [2].

- Observation 2: We can achieve 70% of the max throughput at 18.3dB SNR for FDD and at 18.0dB SNR for TDD.

Proposals:

- Proposal 1: We propose to use the FRC as outlined in [2] for testing TM3 at high Doppler: EVA LOW CORR 2x2, TM3 Rank 2, R.35 FDD and R.35 TDD

- Proposal 2: We propose the passing criteria to be 70% of the maximum throughput at [18.3dB + Implementation Margin] SNR for FDD and [18.0dB + Implementation Margin] for TDD.

# References

- R4-126804, “Way forward on UE demodulation performance for high frequency band under high speed scenario”, Huawei, HiSilicon, Ericsson, ST-Ericsson, Verizon

- R4-130812, “Way forward on high Doppler FRC test”, Qualcomm, Ericsson/ST-Ericsson, Renesas, Verizon Wireless, Huawei/HiSilicon, Fujitsu

- R4-130072, “Demodulation test for high CINR region of high Doppler channel”, Qualcomm

- R4-130095, “UE demodulation performance in high Doppler scenarios”, Renesas

- R4-130224, “Necessity of UE demodulation requirement under high Doppler spread”, Huawei, HiSilicon

- R4-130471, “Consideration on need for new demodulation test under high Doppler condition”, Fujitsu

- R4-130519, “High Doppler Test”, Ericsson, ST-Ericsson

- TS 36.101, “3GPP TS  36.101”