TSG-RAN Working Group 4 (Radio) meeting #66bis	R4-131250

Chicago IL, 15 -19 April 2013

Source:	TruePosition

Title:	TS 36.111 LMU RF Simulation Assumptions

Agenda item:	6.8.1.1

Document for:	Discussion and Approval

1	Introduction

During RAN 4 #64 in Qingdao, China, a way forward was agreed upon that included running UE-LMU link level simulations.  This contribution proposes the parameters to be used for the UE-LMU link level simulations that will support the study of LMU’s the receiver RF characteristics [1].

2	Discussion

The purpose of having requirements for the receiver characteristics as in section 5 of TS 36.111[1] is to ensure that the performance of the LMU RF receiver is adequate to support the successful measurements of an UE’s UL RTOA.  Simulation parameters for the following receiver characteristics will be discussed:

- Sensitivity

- Dynamic Range

- In-channel selectivity

- Adjacent channel selectivity

- Co-location with other base stations

- Intermodulation

- False alarm rate

For all of the above characteristics except the in-channel selectivity the simulations’ wanted signal will be the SRS allocated over the full channel bandwidth as described in Table 2-2.  For the in-channel selectivity requirement the simulation’s wanted signal will be allocated per table 2-3.

Table 2-1 lists the receiver characteristics used for the simulations.

| Table 2-1. Simulation receiver characteristics |  |
| --- | --- |
| Gain | 40 dB |
| Noise Figure | 5 dB |
| Third-order intercept (IIP3) | -25 dBm |
| Out-of-band RF Attenuation | 90 dB |
| False Alarm Rate | 0.5% |

Table 2-2 lists the SRS configuration parameters used for the simulations except for in-channel selectivity.

| Table 2-2. Simulation SRS Configuration Parameters |  |  |
| --- | --- | --- |
| Group hopping | disabled |  |
| Sequence hopping | disabled |  |
| NIDcell | 1 |  |
|  | 0 |  |
| cyclicShift | 0 |  |
| SRS-Bandwidth | 0 |  |
| SRS Bandwidth Configuration | Channel BW (MHz) | Csrs |
|  | 1.4 | 7 |
|  | 3 | 5 |
|  | 5 | 3 |
|  | 10 | 0 |
|  | 15 | 0 |
|  | 20 | 0 |

Table 2-3 lists the SRS configuration parameters used for in-channel selectivity.

| Table 2-3. Simulation SRS Configuration Parameters for In-Channel Selectivity |  |  |
| --- | --- | --- |
| Group hopping | disabled |  |
| Sequence hopping | disabled |  |
| NIDcell | 1 |  |
|  | 0 |  |
| cyclicShift | 0 |  |
| SRS-Bandwidth | Channel BW (MHz) | Bsrs |
|  | 1.4 | - |
|  | 3 | 0 |
|  | 5 | 0 |
|  | 10 | 1 |
|  | 15 | 1 |
|  | 20 | 1 |
| SRS Bandwidth Configuration | Channel BW (MHz) | Csrs |
|  | 1.4 | - |
|  | 3 | 7 |
|  | 5 | 6 |
|  | 10 | 0 |
|  | 15 | 0 |
|  | 20 | 3 |

2.1	Received SINR

The LMU’s processing gain, Gp, using the UE’s transmitted SRS can be estimated as a function of the number of SRS sub-carriers and the number of SRS transmissions using the following equation

Gp=10×log10MscSRS+5×log10NtrxSRS  dB

Where MscSRS is the number of SRS sub-carriers and NtrxSRS is the number of SRS transmissions.

Table 2.1-1 shows the expected processing gain for the given number of resource blocks allocated for SRS transmissions per 36.211[2] that will be used in the simulations with 100 SRS transmissions and MscSRS=NRBSRS×6.

| Table 2.1-1 Processing Gain for SRS Detection |  |  |
| --- | --- | --- |
| Number of SRS Resource Blocks | Number of SRS Transmissions | Processing Gain (dB) |
| 4 | 100 | 23.8 |
| 12 | 100 | 28.6 |
| 20 | 100 | 30.8 |
| 48 | 100 | 34.6 |
| 72 | 100 | 36.4 |
| 96 | 100 | 37.6 |

The SINR simulation will vary the received SINR from -7 to +7 dB from the values in Table 2.1-1 to create the SINR detection curve for each of the possible number of resource blocks allocated for 100 transmissions of the SRS.  Simulations for the receiver characteristics described in the following sections will use the SINR results to set the simulated receiver input power levels.

2.2	Reference Sensitivity and the False Alarm Rate

The reference sensitivity is the minimum mean received power level into one antenna port where there is sufficient SINR to detect the transmitted SRS at least 90% of the time.  The reference sensitivity (REFSENS) can be calculated using [1]:

REFSENS=kTB+NF+SINR  (dBm)

where kTB is the thermal noise level in dBm over the configured channel bandwidth, NF is the receiver’s noise figure in dB, SINR is the signal to interference plus noise ratio for 90% detection of the SRS and IM is the implementation margin.  The reference sensitivity simulation will vary the received power level from -7 to +7 dB from kTB+NF+SINR to create the reference sensitivity detection curve as per Table 2.2-1.  The SINR for each channel bandwidth is from the simulation described in section 2.1.  To simulate the false alarm rate the simulation will be repeated with the wanted signal turned off.

| Table 2.2-1. Reference Sensitivity |  |  |  |  |
| --- | --- | --- | --- | --- |
| Channel BW (MHz) | kTB (dBm) | NF (dB) | SRS RB Allocation | Number of SRS Transmissions |
| 1.4 | -113.7 | 5 | 4 | 100 |
| 3 | -109.7 | 5 | 12 | 100 |
| 5 | -107.5 | 5 | 20 | 100 |
| 10 | -104.5 | 5 | 48 | 100 |
| 15 | -102.7 | 5 | 72 | 100 |
| 20 | -101.4 | 5 | 96 | 100 |

2.3 	Dynamic Range

The dynamic range requirement is a measure of the capability of the receiver to receive a wanted signal in the presence of an interfering signal inside the received channel bandwidth where there is sufficient SINR to detect the transmitted SRS at least 90% of the time.  The power level of the interferer signal for the dynamic range can be calculated using:

Pinteferer=kTB+NF+35   (dBm)

and the power level of the wanted signal is:

Pwanted= Pinterferer+SINR  (dBm)

where kTB is the thermal noise level in dBm over the configured channel bandwidth, NF is the receiver’s noise figure in dB, SINR is the signal to interference plus noise ratio for 90% detection of the SRS and IM is the implementation margin.  The dynamic range simulation will vary the received power level from -7 to +7 dB from (Pinterferer + SINR) to create the dynamic range detection curve as per Table 2.3-1.  The SINR for each channel bandwidth is from the SINR simulation results as described in section 2.1.

| Table 2.3-1. Dynamic Range |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Channel BW (MHz) | kTB (dBm) | NF (dB) | Interfering signal mean power [dBm] / BWConfig | Type of interfering signal | SRS RB Allocation | Number of SRS Transmissions |
| 1.4 | -113.7 | 5 | -73.7 | AWGN | 4 | 100 |
| 3 | -109.7 | 5 | -69.7 | AWGN | 12 | 100 |
| 5 | -107.5 | 5 | -67.5 | AWGN | 20 | 100 |
| 10 | -104.5 | 5 | -64.5 | AWGN | 48 | 100 |
| 15 | -102.7 | 5 | -62.7 | AWGN | 72 | 100 |
| 20 | -101.4 | 5 | -61.4 | AWGN | 96 | 100 |

2.4 	In-channel Selectivity

The in-channel requirement is a measure of the capability of the receiver to receive a wanted signal in the presence of an interfering signal inside the same received channel bandwidth where there is sufficient SINR to detect the transmitted SRS.   For this requirement the interfering signal will be same E-UTRA signal and at the same received mean power level as used for the In-channel selectivity requirement for the eNodeB per 36.104, Appendix C [3], with the exception for the 1.4 MHz channel where there isn’t sufficient room for the interferer.  The mean power level of the wanted signal will be

Pwanted=kTB+NF+SINR+D   (dBm)

or (REFSENS + D)  where D is the allowed desensitization for detection of the wanted signal. The wanted signal will be the SRS configured as shown in Table 2.4-1 where the value of the SINR corresponds to the number of resource blocks allocated for the SRS.

| Table 2.4-1. In-channel Selectivity |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Channel BW (MHz) | kTB (dBm) | NF (dB) | Interfering signal mean power [dBm] | Type of interfering signal | SRS RB Allocation | Number of SRS Transmissions |
| 3 | -109.7 | 5 | -84 | 3 MHz E-UTRA signal, 4 RBs | 4 | 100 |
| 5 | -107.5 | 5 | -81 | 5 MHz E-UTRA signal, 8 RBs | 8 | 100 |
| 10 | -104.5 | 5 | -77 | 10 MHz E-UTRA signal, 20 RBs | 20 | 100 |
| 15 | -102.7 | 5 | -77 | 15 MHz E-UTRA signal, 24 RBs | 24 | 100 |
| 20 | -101.4 | 5 | -77 | 20 MHz E-UTRA signal, 24 RBs | 24 | 100 |

2.5 	Adjacent channel selectivity and narrow-band blocking

The adjacent channel requirement is a measure of the capability of the receiver to receive a wanted signal in the presence of an interfering signal in the adjacent channel where there is sufficient SINR to detect the transmitted SRS.   For the adjacent channel requirement the interferer in the adjacent channel will be identical to and time aligned to the wanted signal.   The mean power level of the wanted signal will be

Pwanted=kTB+NF+SINR+D   (dBm)

or (REFSENS + D) where D is the allowed desensitization for detection of the wanted signal and the value of the SINR corresponds to the number of resource blocks allocated for the SRS.  The REFSENS for each channel bandwidth is from the sensitivity simulation described in section 2.2.  The simulation will vary the wanted power –D to +7 dB to create the adjacent channel selectivity detection curve.

For the narrow-band blocking requirement the interfering signal will be same E-UTRA signal and at the same received mean power level and frequency offsets as used for the narrow-band requirement for the eNodeB per TS 36.104, Table 7.5.1-2 [3].

| Table 2.5-1. Adjacent Channel Selectivity |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Channel BW (MHz) | Wanted signal mean power [dBm] | NF (dB) | Interfering signal mean power [dBm] | Type of interfering signal | SRS RB Allocation | Number of SRS Transmissions |
| 1.4 | REFSENS + 11dB | 5 | -52 | 1.4 MHz E-UTRA signal, 4 RBs | 4 | 100 |
| 3 | REFSENS + 8dB | 5 | -52 | 3 MHz E-UTRA signal, 4 RBs | 12 | 100 |
| 5 | REFSENS + 6dB | 5 | -52 | 5 MHz E-UTRA signal, 20 RBs | 20 | 100 |
| 10 | REFSENS + 6dB | 5 | -52 | 10 MHz E-UTRA signal, 24 RBs | 48 | 100 |
| 15 | REFSENS + 6dB | 5 | -52 | 10 MHz E-UTRA signal, 24 RBs | 72 | 100 |
| 20 | REFSENS + 6dB | 5 | -52 | 10 MHz E-UTRA signal, 24 RBs | 96 | 100 |

| Table 2.5-2. Narrow-band Blocking |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Channel BW (MHz) | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Type of interfering signal | SRS RB Allocation | Number of SRS Transmissions |
| 1.4 | REFSENS + 6dB | -49 | 1.4 MHz E-UTRA signal, 1 RB | 4 | 100 |
| 3 | REFSENS + 6dB | -49 | 3 MHz E-UTRA signal, 1 RB | 12 | 100 |
| 5 | REFSENS + 6dB | -49 | 5 MHz E-UTRA signal, 1 RB | 20 | 100 |
| 10 | REFSENS + 6dB | -49 | 10 MHz E-UTRA signal, 1 RB | 48 | 100 |
| 15 | REFSENS + 6dB | -49 | 15 MHz E-UTRA signal, 1 RB | 72 | 100 |
| 20 | REFSENS + 6dB | -49 | 20 MHz E-UTRA signal, 1 RB | 96 | 100 |

2.6 	Co-location with other base stations

The blocking requirement is a measure of the capability of the receiver to receive a wanted signal in the presence of an unwanted interfering signal from another base station where there is sufficient SINR to detect the wanted signal.   The wanted signal is the wide-band SRS and the mean power level of the wanted signal will be

Pwanted=kTB+NF+SINR+D   (dBm)

or (REFSENS + D) where D is the allowed desensitization for detection of the wanted signal and the value of the SINR corresponds to the number of resource blocks allocated for the SRS.  The REFSENS for each channel bandwidth is from the sensitivity simulation described in section 2.2.  The out-of-band interferer is the other base station’s downlink transmissions that will be simulated as a CW tone at high power, +16 dBm.   The simulation will assume that the RF front-end filter has at least the out-of-band attenuation as given in Table 2-1 so that the receiver’s AGC is not active and that the resulting third order product of the attenuated  interferer falls into the wanted signal’s bandwidth.

| Table 2.6-1. Co-location Blocking Simulation Parameters |  |  |  |  |
| --- | --- | --- | --- | --- |
| Channel Bandwidth [MHz] | Allocated SRS Resource Blocks | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Type of interfering signal |
| 1.4 | 4 | REFSENS + 6 dB | +16 | CW |
| 3 | 12 | REFSENS + 6 dB | +16 | CW |
| 5 | 24 | REFSENS + 6 dB | +16 | CW |
| 10 | 48 | REFSENS + 6 dB | +16 | CW |
| 15 | 72 | REFSENS + 6 dB | +16 | CW |
| 20 | 96 | REFSENS + 6 dB | +16 | CW |

2.6 	Receiver Intermodulation

The receiver intermodulation requirement is a measure of the capability of the receiver to detect the wanted signal in the presence of two interfering signals which have a relationship such that intermodulation products created in the receiver fall within the bandwidth of the wanted signal where there is sufficient SINR to detect the wanted signal.   For the intermodulation simulation the E-UTRA interferer is listed in Table 2-6.1.   The second interfere is a CW tone.  The frequency offsets for both interferers relative to the wanted signal are also listed in Table 2.6-1.  The mean power level of the wanted signal will be

Pwanted=kTB+NF+SINR+D  (dBm)

or (REFSENS + D) where D is the allowed desensitization for detection of the wanted signal and the value of the SINR corresponds to the number of resource blocks allocated for the SRS.  The REFSENS for each channel bandwidth is from the sensitivity simulation described in section 2.2.  The simulation will vary the wanted power –D to +7 dB to create the intermodulation detection curve.

| Table 2.6-1. Intermodulation Parameters |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Channel Bandwidth (MHz) | Type of wanted signal | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Interfering signal center frequency from the center frequency of the wanted signal [MHz] | Type of interfering signal |
| 1.4 | SRS, 4 RBs | REFSENS + 6dB | -52 | ±1.4 | CW |
|  |  |  |  | ±2.8 | 1.4 MHz E-UTRA signal, 4 RB |
| 3 | SRS, 12 RBs | REFSENS + 6dB | -52 | ±3 | CW |
|  |  |  |  | ±6 | 3 MHz E-UTRA signal, 8 RB |
| 5 | SRS, 20 RBs | REFSENS + 6dB | -52 | ±5 | CW |
|  |  |  |  | ±10 | 5 MHz E-UTRA signal, 20 RB |
| 10 | SRS, 48 RBs | REFSENS + 6dB | -52 | ±10 | CW |
|  |  |  |  | ±20 | 10 MHz E-UTRA signal, 24 RB |
| 15 | SRS, 72 RBs | REFSENS + 6dB | -52 | ±12.5 | CW |
|  |  |  |  | ±25 | 10 MHz E-UTRA signal, 24 RB |
| 20 | SRS, 96 RBs | REFSENS + 6dB | -52 | ±15 | CW |
|  |  |  |  | ±30 | 10 MHz E-UTRA signal, 24 RB |

3	Summary

This contribution discussed a suite of simulations that can to be used for the study of the LMU’s RF characteristics.

- Sensitivity

- Dynamic Range

- In-channel selectivity

- Adjacent channel selectivity

- Blocking characteristics

- Inter-modulation

- False alarm rate

4	References

1.	3GPP TS 36.111 “Location Measurement Unit (LMU) performance specification”

2.	3GPP TS 36.211 “Physical Channel and Modulation”

3.	3GPP TS 36.104 “Base Station (BS) radio transmission and reception”