3GPP TSG-RAN WG4 Meeting #66bis	 R4-131253

Chicago, IL  15 – 19 April, 2013

Agenda Item:	6.8.1.2

Source: 	TruePosition, Andrew Corporation

Title: 	UL RTOA Measurement Time Requirements for FDD and TDD

Document for:	Discussion and Approval

# Introduction

This contribution provides a discussion of the UL RTOA Measurement Time Requirements for FDD and TDD.

# Discussion

In [1] it was agreed that for FDD the UL RTOA Measurement Time requirement would be specified using the following equation

where TSRS is the SRS periodicity in ms according to the srsConfiguration IE received by LMU from E-SMLC via SLmAP [3], M is the number of SRS measurement occasions (depends on the applicable bandwidth), n is the number of requested UL RTOA measurements per carrier, N is the minimum number of UL RTOA measurements per uplink carrier frequency for different UEs that can be measured in parallel andis a margin in ms to account for any additional time needed to produce the measurement, e.g. the time necessary for sampling and processing.  The start time for the UL RTOA measurement is the time of the first SRS transmission.

With fixed the measurement time is dependent on the SRS periodicity, TSRS, and the number of SRS transmissions M.  In 5.5.3.2 of [2] the following equation is given to count the number of SRS transmissions for FDD and TDD:

where nSRS is the count of SRS transmissions, ns is the slot number, nf is the frame number, NSP is the number of  downlink to uplink switch points is a radio frame,  is SRS subframe offset defined in Table 8.2-2 of [3] and  is the maximum value of  for the configuration of SRS subframe offset.  According to the equation for nSRS the only difference between the count of SRS transmissions for FDD and TDD is when the SRS periodicity is 2 ms.

The figure below shows a count of SRS transmissions over time for SRS periodicity of 2 ms for both FDD and TDD with NSP set to 2 (for TDD switch periodicity of 5 ms) and 1 (for TDD switch periodicity of 10 ms).

As shown in the plot the rate at which the SRS is transmitted for TDD with SRS periodicity of 2 ms depends on the downlink to uplink switch-point periodicity.  For the case when NSP equals 2 the SRS transmission rate is 0.8 times the SRS transmission rate for FDD and when NSP equals 1 the SRS transmission rate is 0.4 times the SRS transmission rate for FDD.  This result suggests that we can modify the the existing equation for the UL RTOA measurement time to account for the case of TDD with SRS periodicity of 2 ms by including an appropriate multiplier that depends on the frame structure and the SRS periodicity.

The impact of the ratio of Toffset to Toffset_max on the UL RTOA measurement time for the case of TDD with 2 ms SRS periodicity depends if M, the number of SRS transmissions, is odd or even because according to 8.2-2 [3] there are two subframe offset values for Toffset per half frame.  To account for the ratio of Toffset to Toffset_max we can add the difference in time between the two subframe offset values to the measurement time equation.  The two Toffset values are set by the SRS configuration index ISRS according to table 8.2-2 [3].  By adding the offset L we also have to adjust the total count M of SRS transmissions used in the measurement equation.

Proposal 1: the UL RTOA measurement time for TDD shall be calculated using

where K is a multiplier for 2 ms SRS periodicity in TDD,

K=2.5/NSP,   for 2ms SRS periodicity1,           otherwise                     ,

the offset L accounts for the two values of Toffset for  2ms SRS periodicity in TDD,

L=Madj2Toffset_max-Toffset_min,   2 ms SRS periodicity0,                                                  otherwise,

and Madj is the adjustment to the count of SRS transmissions to account for the addition of the offset L,

Madj=2-M mod 2,   2 ms SRS periodicity1,               otherwise

# Conclusion

This contribution discussed and proposed a modification to the calculation of the UL RTOA measurement time to account for the difference in the rate of SRS transmissions for SRS periodicity of 2 ms and all other SRS periodicities in TDD.

Proposal 1: the UL RTOA measurement time for TDD shall be calculated using

where K is a multiplier for 2 ms SRS periodicity in TDD,

K=2.5/NSP,   for 2ms SRS periodicity1,           otherwise                     ,

the offset L accounts for the two values of Toffset for  2ms SRS periodicity in TDD,

L=Madj2Toffset_max-Toffset_min,   2 ms SRS periodicity0,                                                  otherwise,

and Madj is the adjustment to the count of SRS transmissions to account for the addition of the offset L,

Madj=2-M mod 2,   2 ms SRS periodicity1,               otherwise

# References

[1]	3GPP TS 36.111 “Location Measurement Unit (LMU) performance specification”

[2] 3GPP TS 36.211 “Physical channels and modulation”

[3] 3GPP TS 36.213 “Physical layers procedures”