3GPP TSG-RAN WG4 Meeting #66bis	R4-131348

Chicago, IL, USA, April 15 – 19, 2013

Agenda item:	7.10.1, 7.11.1

Source: 	MediaTek Inc.

Title: 	Formulation of intra-band non-contiguous CA UL intermodulation products overlapping with its own DL carriers

Document for:	Discussion

## 1	Introduction

Criteria for intra-band non-contiguous CA UL intermodulation products overlapping with its own DL carriers had been formulated for LTE_CA_NC_B3 up to 7th order in RAN4 #65 meeting [1]. In this contribution, a generalized formula for the same overlapping criteria is derived which can be applied to any band and extended to any higher-order intermodulation products.

## 2	Discussion

Criteria for intra-band non-contiguous CA UL intermodulation products overlapping with its own DL carriers had been profoundly studied and presented in RAN4 #65 meeting [1], where the formula were derived specifically for LTE_CA_NC_B3 and the intermodulation products were limited up to 7th order. Considering that more bands could be introduced with intra-band non-contiguous CA requirement and intermodulation products up to 11th order or higher might need to be considered to address self-interference issue [2], it would be beneficial to derive a generalized formula which can be applied to any band and extended to any higher-order intermodulation products.

Here we start the analysis by considering the worst case scenario where the intermodulation products would align with its own downlink carriers. In this situation, the carrier can be treated as a single tone as illustrated in Figure 2-1.

Figure 2-1 Tone representation of intra-band non-contiguous CA with 2 UL CC

Where a few parameters are defined as below,

fULCC1: Uplink CA carrier 1

fULCC2: Uplink CA carrier 2

fDLCC1: Downlink CA carrier 1

fDLCC2: Downlink CA carrier 2

FBW: Bandwidth of the operation band

FDUP: FDD duplex frequency distance

FGAP: Frequency gap between two CA carriers

The frequency allocation for two-tone intermodulation products can then be depicted as in Figure 2-2.

Figure 2-2 Frequency allocation for two-tone intermodulation products

fIMD3R and fIMD3L are the frequency location for 3rd order intermodulation product on the higher frequency side (right side to main tones) and the lower frequency side (left side to main tones), respectively. The same definition also applies to 5th order, 7th order, and further higher order intermodulation products. In this analysis, we only focused on right side intermodulation products as most FDD DL bands are on the higher frequency side.

From Figure 2-1 and Figure 2-2, it can be seen that for fIMD3R to align with fDLCC1, the following condition needs to be met,

FDUP = 2*FGAP or FGAP = FDUP /2        (1)

And for fIMD3R to align with fDLCC2, the following condition needs to be met,

FGAP = FDUP        (2)

Similarly, for fIMD5R to align with fDLCC1, the following condition needs to be met,

FGAP = FDUP /3        (3)

And for fIMD5R to align with fDLCC2, the following condition needs to be met,

FGAP = FDUP /2        (4)

Notice that for 5th order intermodulation product, in addition to fIMD5R, it also has a component located at the same frequency as fIMD3R since a “square” of one carrier would generate a DC component which multiplies the remaining 3rd order permutation would have an intermodulation component aligned with fIMD3R. The two aligned intermodulation components can only be differentiated if the main carriers have envelope modulation where the 5th order component would have wider bandwidth than 3rd order component. Similarly, for 7th order intermodulation product, in addition to fIMD7R, it also has a component located at the same frequency as fIMD5R and a component located at the same frequency as fIMD3R. The same scenario is applied to further higher order nonlinear terms.

Based on this systemic pattern, a generalized formula for UL intermodulation products aligning with its DL carriers can be deduced which is summarized in Table 2-1.

| IMD Order | Criteria for IMD alignment with |  |
| --- | --- | --- |
|  | CC1 | CC2 |
| 3rd | FGAP = FDUP/2 | FGAP = FDUP |
| 5th | FGAP = FDUP/3 | FGAP = FDUP/2 |
|  | FGAP = FDUP/2 | FGAP = FDUP |
| 7th | FGAP = FDUP/4 | FGAP = FDUP/3 |
|  | FGAP = FDUP/3 | FGAP = FDUP/2 |
|  | FGAP = FDUP/2 | FGAP = FDUP |
| … | … | … |
| (2N+1)th N > 3 | FGAP = FDUP/(N+1) | FGAP = FDUP/N |
|  | FGAP = FDUP/N | FGAP = FDUP/(N-1) |
|  | FGAP = FDUP/(N-1) | … |
|  | … | FGAP = FDUP/2 |
|  | FGAP = FDUP/2 | FGAP = FDUP |

Table 2-1 Criteria for UL intermodulation products aligning with its DL carriers

Now let’s turn to the more general situation where both component carriers have finite channel bandwidth as shown in Figure 2-3.

Figure 2-3 A generalized intra-band non-contiguous CA model for UE self-interference analysis

The parameters as noted in Figure 2-3 can all be self-explained without further text description. Following the analogy of two-tone intermodulation analysis, it can be seen that fU1L and fU2H will define the intermodulation products upper frequency boundary fIMD3RH, and fU1H and fU2L will define the lower frequency boundary fIMD3RL, as illustrated in Figure 2-4 (3rd order IMD in this example). From Figure 2-3 and Figure 2-4, it can be seen that for 3rd order intermodulation component to overlap with DLCC1, the following criteria has to be met,

2*(FBW – (c+d)) > FDUP and 2*(FBW – (c+d) – (a+b)) < FDUP        (5)

which can also be rearranged as,

FBW – (FDUP/2) – (a+b) < c+d < FBW – (FDUP/2)        (6)

Figure 2-4 3rd order intermodulation product generated by ULCC1 and ULCC2

Similarly, for 3rd order intermodulation component to overlap with DLCC2, the following criteria has to be met,

FBW – (c+d) > FDUP – b and FBW – (c+d) – (a+b) < FDUP + b        (7)

which can also be rearranged as,

FBW – FDUP – (a+b) – b < c+d < FBW – FDUP + b        (8)

For 5th order primary intermodulation component (at higher frequency) to overlap with DLCC1, the criteria can be derived based on equations (3) and (6) as,

FBW – (FDUP/3) – (a+b) < c+d < FBW – (FDUP/3)        (9)

For 5th order secondary intermodulation component (at the same frequency as 3rd order component) to overlap with DLCC1, the criteria can be derived from equation (6) by extending both upper and lower ranges with (b/2) which accounts for the bandwidth extension (by +/- b as compared to 3rd order component) through a “square” operation of ULCC2. This results in,

FBW – (FDUP/2) – (a+b) – (b/2) < c+d < FBW – (FDUP/2) + (b/2)        (10)

Similarly, for 5th order primary intermodulation component (at higher frequency) to overlap with DLCC2, the criteria can be derived based on equations (4) and (8) as,

FBW – (FDUP/2) – (a+b) – (b/2) < c+d < FBW – (FDUP/2) + (b/2)        (11)

For 5th order secondary intermodulation component to overlap with DLCC2, the criteria can be derived from equation (8) by extending both upper and lower ranges with b, and that is,

FBW – FDUP – (a+b) – 2b < c+d < FBW – FDUP + 2b        (12)

As intermodulation products have a fixed frequency spacing between adjacent components (FGAP as noted in Figure 2-2), the overlapping criteria for higher order intermodulation products can be easily deduced from a systematic pattern which was implicitly described in the above analysis.

Table 2-2 summarizes the generalized formula which can be applied to any band and any order of intermodulation. Moreover, this generalized formula can also be adopted for base-station intra-band non-contiguous CA self-interference analyses.

| IMD Order | Criteria for IMD overlapping with |  |
| --- | --- | --- |
|  | CC1 | CC2 |
| 3rd | FBW – (FDUP/2) – (a+b) < c+d < FBW – (FDUP/2) | FBW – FDUP – (a+b) – b < c+d < FBW – FDUP + b |
| 5th | FBW – (FDUP/3) – (a+b) < c+d < FBW – (FDUP/3) | FBW – (FDUP/2) – (a+b) – (b/2) < c+d < FBW – (FDUP/2) + (b/2) |
|  | FBW – (FDUP/2) – (a+b) – (b/2) < c+d < FBW – (FDUP/2) + (b/2) | FBW – FDUP – (a+b) – 2b < c+d < FBW – FDUP + 2b |
| 7th | FBW – (FDUP/4) – (a+b) < c+d < FBW – (FDUP/4) | FBW – (FDUP/3) – (a+b) – (b/3) < c+d < FBW – (FDUP/3) + (b/3) |
|  | FBW – (FDUP/3) – (a+b) – (b/3) < c+d < FBW – (FDUP/3) + (b/3) | FBW – (FDUP/2) – (a+b) – b < c+d < FBW – (FDUP/2) + b |
|  | FBW – (FDUP/2) – (a+b) – b < c+d < FBW – (FDUP/2) + b | FBW – FDUP – (a+b) – 3b < c+d < FBW – FDUP + 3b |
| 9th | FBW – (FDUP/5) – (a+b) < c+d < FBW – (FDUP/5) | FBW – (FDUP/4) – (a+b) – (b/4) < c+d < FBW – (FDUP/4) + (b/4) |
|  | FBW – (FDUP/4) – (a+b) – (b/4) < c+d < FBW – (FDUP/4) + (b/4) | FBW – (FDUP/3) – (a+b) – (2b/3) < c+d < FBW – (FDUP/3) + (2b/3) |
|  | FBW – (FDUP/3) – (a+b) – (2b/3) < c+d < FBW – (FDUP/3) + (2b/3) | FBW – (FDUP/2) – (a+b) – (3b/2) < c+d < FBW – (FDUP/2) + (3b/2) |
|  | FBW – (FDUP/2) – (a+b) – (3b/2) < c+d < FBW – (FDUP/2) + (3b/2) | FBW – FDUP – (a+b) – 4b < c+d < FBW – FDUP + 4b |
| 11th | FBW – (FDUP/6) – (a+b) < c+d < FBW – (FDUP/6) | FBW – (FDUP/5) – (a+b) – (b/5) < c+d < FBW – (FDUP/5) + (b/5) |
|  | FBW – (FDUP/5) – (a+b) – (b/5) < c+d < FBW – (FDUP/5) + (b/5) | FBW – (FDUP/4) – (a+b) – (b/2) < c+d < FBW – (FDUP/4) + (b/2) |
|  | FBW – (FDUP/4) – (a+b) – (b/2) < c+d < FBW – (FDUP/4) + (b/2) | FBW – (FDUP/3) – (a+b) – b < c+d < FBW – (FDUP/3) + b |
|  | FBW – (FDUP/3) – (a+b) – b < c+d < FBW – (FDUP/3) + b | FBW – (FDUP/2) – (a+b) – 2b < c+d < FBW – (FDUP/2) + 2b |
|  | FBW – (FDUP/2) – (a+b) – 2b < c+d < FBW – (FDUP/2) + 2b | FBW – FDUP – (a+b) – 5b < c+d < FBW – FDUP + 5b |
| … | … | … |
| (2N+1)th N > 5 | FBW – (FDUP/(N+1)) – (a+b) < c+d < FBW – (FDUP/(N+1)) | FBW – (FDUP/N) – (a+b) – (b/N) < c+d < FBW – (FDUP/N) + (b/N) |
|  | FBW – (FDUP/N) – (a+b) – (b/N) < c+d < FBW – (FDUP/N) + (b/N) | FBW – (FDUP/(N-1)) – (a+b) – (2b/(N-1)) < c+d < FBW – (FDUP/(N-1)) + (2b/(N-1)) |
|  | FBW – (FDUP/(N-1)) – (a+b) – (2b/(N-1)) < c+d < FBW – (FDUP/(N-1)) + (2b/(N-1)) | FBW – (FDUP/(N-2)) – (a+b) – (3b/(N-2)) < c+d < FBW – (FDUP/(N-2)) + (3b/(N-2)) |
|  | FBW – (FDUP/(N-2)) – (a+b) – (3b/(N-2)) < c+d < FBW – (FDUP/(N-2)) + (3b/(N-2)) | FBW – (FDUP/(N-3)) – (a+b) – (4b/(N-3)) < c+d < FBW – (FDUP/(N-3)) + (4b/(N-3)) |
|  | FBW – (FDUP/(N-3)) – (a+b) – (4b/(N-3)) < c+d < FBW – (FDUP/(N-3)) + (4b/(N-3)) | … |
|  | … | FBW – (FDUP/2) – (a+b) – (N-1)b/2 < c+d < FBW – (FDUP/2) + (N-1)b/2 |
|  | FBW – (FDUP/2) – (a+b) – (N-1)b/2 < c+d < FBW – (FDUP/2) + (N-1)b/2 | FBW – FDUP – (a+b) – Nb < c+d < FBW – FDUP + Nb |

Table 2-2 Formula for the criteria of UL intermodulation products overlapping with its DL carriers

## 3	Conclusion

In this paper, a generalized formula for the criteria of intra-band non-contiguous CA UL intermodulation products overlapping with its DL carriers has been derived. The formula can be applied to any band and any order of intermodulation which can also be adopted for base-station self-interference analysis.

## 4	References

- R4-126235, “Harmonics and intermodulation products caused by intra-band non-contiguous CA_3 UE”, Intel Corporation

- R4-131349, “Measurement of intermodulation products caused by intra-band non-contiguous CA for UE which supports 2UL CC”, MediaTek Inc.