3GPP TSG RAN WG4 Meeting #66BIS	R4-131383

- Chicago, IL, U. S., 15th – 19st April, 2013

- Agenda Item:	7.19

- Source:	LG Electronics

- Title:	UE-to-UE coexistence study with Harmonics and IMD analysis for 2UL inter-band CA_1A-5A UE

Document for:	Approval

- Introduction

In the RAN Plenary #59 meetings, the study for the LTE-A dual uplink inter-band CA is approved. And the skeleton of technical report 36.8xx was discussed in e-mail reflectors [1].

The objective of work item on dual uplink inter-band CA is to gather the relevant background information and studies in order to address Dual Uplink Inter-band Carrier Aggregation requirements according to the CA class1 to 5.

In this contribution, we provide the harmonics and IMD analysis for B1+B5 CA_1A-5A UE with 2ULs. And then we will further study for spurious emission for UE-to-UE coexistence requirements.

- Study for UE-to-UE coexistence for 2UL Band 1 + 5 inter-band CA

In this section, we provide UE-to-UE coexistence requirements for the protected bands in cross-region between Band 1 and Band 5. As you know that operating Band 1 and Band 5 are deployed in some countries (Brazil, South Korea, Australia and New Zealand and others) so, we think that the protected bands restricted to protect B1,3,5,7,8,26,27,28,38,40 and 43 in both Band and Band 5 are deployed regions[2].

Table 1: 2UL inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) band |  |  |  | Downlink (DL) band |  |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | Channel BW (MHz) | BS transmit / UE receive |  |  | Channel BW (MHz) |  |
|  |  | FUL_low – FUL_high |  |  |  | FUL_low – FUL_high |  |  |  |  |
| CA_1-5 | 1 | 1920 MHz | – | 1980 MHz | 10 | 2110 MHz | – | 2170 MHz | 10 | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 10 | 869 MHz | – | 894 MHz | 10 |  |

Table 1 show the operating bands of the CA_1A-5A UE.

Band 1 UE is operated for Europe and/or Asia regions, and Band 5 UE is deployed for North/South America regions. The protected bands of each region are displayed in the table 2 and 3.

Table 2: Protected bands for Band 1 UE

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | Note |
| 1 | E-UTRA Band 1, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 38, 40, 42, 43, 44 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | Frequency range (B 39) | 1880 |  | 1895 | -40 | 1 | 15,27 |
|  | Frequency range (B 33/39) | 1895 |  | 1915 | -15.5 | 5 | 15,27 |
|  | Frequency range (B 33/39) | 1915 |  | 1920 | +1.6 | 5 | 15,27 |
|  | Frequency range (PHS Band) | 1884.5 | - | 1915.7 | -41 | 0.3 | 6, 8, 15 |
|  | Frequency range (B 9) | 1839.9 | - | 1879.9 | -50 | 1 | 15 |

Table 3: Protected bands for Band 5 UE

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | Note |
| 5 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 22, 23, 24, 25, 28, 29,42, 43 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |

For the Band 27, if conventional Band 5 UE satisfies the SEM requirement and then SE for UE-to-UE co-existence is ignored.

We analyze IMD and harmonics from 2UL inter-band CA 1A-5A UE in figure 1~4. Figure 1 show the adjacent operating band from Band 5 in South Korea.

Figure 1 Spectrum allocations around Band5 in Korea

Figure 2 show the adjacent operating band from Band 1 in South Korea.

Figure 2 Spectrum allocations around Band1 in Korea

Figure 3 Spectrum allocations around Band5 in Brazil and Australia

Figure 4 Spectrum allocations around Band1 in Brazil and Australia

From the above proposal1 and figure 1~4, we can see the protected band for CA 1A-5A UE that are combined bands in the same areas where both Band 1 and Band 5 are operated. So we analyze the legacy UE-to-UE coexistence requirements for 1 UL CA case as below.

- Co-existence issues in Asia and South America

- For the case of Band1 UL (1920-1980MHz) are used :

- B3 DL (1805-1880MHz) : 40MHz GAP case  -50dBm/MHz

- B40 (2300-2400MHz) : 320MHz GAP case  -50dBm/MHz

- Others cases (B1/5/7/8/26/27/28/38/43) : No IMD  -50dBm/MHz

- For the case of Band5 UL (824-849MHz) are used:

  - B8 DL (925-960MHz) : 76MHz GAP case  -50dBm/MHz    (no requirements)

  - B26 DL (859-894MHz) : 10MHz GAP case  -27 dBm/MHz (legacy requirements)

  - B27 DL (852-869MHz) : 3MHz GAP case  include SEM area  ignore SE

  - B28 DL (758-803MHz) : 21MHz GAP case  -50dBm/MHz (So tight  if necessary, can be changed)

  - Other cases (B1/3/5/7/38/40/43) : No IMD  -50dBm/MHz

For the analysis of 2UL inter-band CA 1A-5A UE, we should study and analyze the inter-modulation products and harmonics products when both Band 1 and Band 5 with 2ULs are transmitting.

The combination of Band 1 and Band 5 has harmonics frequencies that are far away from the components receive and transmit frequencies, in the UL and in the DL of the CA combination, as shown in table 5. So there was no harmonic issue except Band 41. In the conventional SE for UE-to-UE coexistence requirements, the 3rd harmonics of Band 5 UE already fall into B41 DL band, but exceptions for SE measurements are permitted if there is at least one individual RB within the transmission bandwidth for which the 2nd, 3rd or 4th harmonic totally or partially overlaps the measurement bandwidth (MBW) [3].  And also, Band 41 is not defined as protected band when both Band1 and Band5 are operating.

Table 5: 2 UL B1 + B5 harmonic products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 824 | 849 |
| 2nd order harmonics frequency range (MHz) | 3840 to 3960 |  | 1648 to 1698 |  |
| 3rd order harmonics frequency range (MHz) | 5760 to 5940 |  | 2472 to 2547 |  |

Table 6 gives the intermodulation products for CA_1A-5A UE with 2 ULs. None of the IMD products fall into the own CA_1A-5A UE receive bands.  But, the 3-tone 3rd IMD products may fall into the Band 27 DL band, and also 2-tone 3rd IMD products may have impact on the Band 43 DL band. So we need to study the IMD level from CA 1A-5A UE to Band 27 DL bands, but Band 43 is far away from Band1 and Band 5, so we can assume that the UE front-end filter can effectively attenuate these IMD products for Band 43.

Table 6: 2 ULs B1 + B5 IMD products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 824 | 849 |
| Two-tone 2nd order IMD products | ¦f2_low – f1_high¦ | ¦f2_high – f1_low¦ | ¦f2_low + f1_low¦ | ¦f2_high + f1_high¦ |
| IMD frequency range (MHz) | 1071 to 1156 |  | 2744 to 2829 |  |
| Two-tone 3rd order IMD products | ¦2*f1_low – f2_high¦ | ¦2*f1_high – f2_low¦ | ¦2*f2_low – f1_high¦ | ¦2* f2_high – f1_low¦ |
| IMD frequency range (MHz) | 2991 to 3136 |  | 222 to 332 |  |
| Three-tone 3rd order IMD products | (f1_low – max BW f2) | (f1_high + max BW f2) | (f2_low – max BW f1) | (f2_high + max BW f1) |
| IMD frequency range (MHz) | 1910 to 1990 |  | 814 to 859 |  |
| Two-tone 3rd order IMD products | (2*f1_low +f2_low) | (2*f1_high + f2_high) | (2*f2_low + f1_low) | (2* f2_high + f1_high) |
| IMD frequency range (MHz) | 4664 to 4809 |  | 3568 to 3678 |  |

From the analysis, we propose the SE for UE-to-UE coexistence requirements for CA_1A-5A UE with 2ULs as below

- Co-existence issues in Asia and South America

- For 2ULs within Band1 UL (1920-1980MHz) + Band 5 UL (824-849MHz):

  - B26 DL (859-894MHz) : 10MHz GAP case  -27 dBm/MHz (legacy requirements)

  - B27 DL (852-869MHz) : 3MHz GAP case  include SEM area  if satisfy the SEM requirements can be ignored SE requirements, but if do not satisfy the SEM, need to relax the SE or SEM requirements.

  - B28 DL (758-803MHz) : 21MHz GAP case  current requirements with -50dBm/MHz, but if necessary, can be relaxed to -37dBm/MHz or -40dBm/MHz

  - Other cases (B1/3/5/7/8/38/40/43) : No IMD  -50dBm/MHz

- Conclusions

In this contribution we provided the harmonics and IMD analysis for B1+B5 CA_1A-5A UE with 2ULs. And also we initiated further discussion for SE requirements of Band 26, 27 and 28 from CA_1A-5A UE.

- Reference

- TR 36.8xx v0.0.1

- R4-131416, “Protected Band for B1+B5 2 UP LINK CA,” LG U+

- TS 36.101 v11.4.0

- TR 36.850 v11.0.0

- TR36.811 v10.0.0