3GPP TSG-RAN4 Meeting #67	 R4-132115

# Fukuoka, Japan, 20-24 May, 2013

Source: 	SONY

Title: 	TRP/TRS Performance Requirements for LEE

Agenda Item: 	5.7

Document for:	Approval

# 1		Introduction R4-125719

Recently sets of minimum/maximum, average and recommended TRP/TRS values were proposed for release 11 work item on “UE Over The Air (OTA) conformance testing methodology – Laptop Mounted Equipment (LME) Free Space test” [1-2]. In order to reach consensus we propose TRP/TRS requirements based on recent [3] and previous [4-6] measurements.

# 2	Proposed TRP/TRS values

Below we propose TRP/TRS requirements for minimum/maximum, average and recommended values.

Table 2.1:  TRP minimum performance requirement for LEE devices in data transfer position

| Operating band | Power Class 3 |  |
| --- | --- | --- |
|  | Power (dBm) |  |
|  | Average | Min |
| I | 19 | 17 |
| VIII | 17 | 15 |

Table 2.2: TRP recommended performance for LEE devices in data transfer position

| Operating band | Power Class 3 |
| --- | --- |
|  | Power (dBm) |
|  | Average |
| I | 20 |
| VIII | 20 |

Table 2.3: TRS minimum performance requirement for LEE devices in data transfer position

| Operating Band | Unit | <REFÎor> |  |
| --- | --- | --- | --- |
|  |  | Average | Max |
| I | dBm/3.84 MHz | -103 | -101 |
| VIII | dBm/3.84 MHz | -101 | -99 |

Table 2.4: TRS recommended performance requirement for LEE devices in data transfer position

| Operating Band | Unit | <REFÎor> |
| --- | --- | --- |
|  |  | Average |
| I | dBm/3.84 MHz | -104 |
| VIII | dBm/3.84 MHz | -104 |

# 3	Conclusions

We have presented proposal for TRP/TRS performance requirements for Bands I and VIII.

# 4	Reference

[1] R4-126205. “TRP performance requirements for LEE (Laptop Embedded Equipment) devices”, Vodafone, Orange, Telecom Italia, RAN4#65, Nov 2012

[2] R4-126206, “TRS performance requirements for LEE (Laptop Embedded Equipment) devices”, Vodafone, Orange, Telecom Italia, RAN4#65, Nov 2012

[3] R4-130991, “TRP/TRS Performance Measurement Data from 5 Laptop Types”, Sony, RAN4#66-bis, Chicago, USA, April 2013

[4] R4-115568, “TRP/TRS requirement values for LEE (Laptop Embedded Equipment) devices”, Ericsson, RAN4#61, November 2011

[5] R4-123268, “TRP and TRS measurements for LEE devices”, Orange, RAN4#63, May 2012

[6] R4-123066, “LEE TRP TRS measurement results”, Vodafone Group, RAN4#63, May 201