3GPP TSG-RAN WG4 Meeting #67	 R4-132333

Fukuoka Japan,  20 - 25 May, 2013

Agenda Item:	6.8.1.1

Source: 	TruePosition, Andrew Corporation

Title: 	Architecture discussion for NBPS

Document for:	Approval

# Introduction

During the discussion of a proposed antenna sharing network for NBPS in RAN4#66bis, it was agreed to discuss the RF architecture for the deployment of LMUs.  This contribution illustrates deployment alternatives and defines the test ports for use in testing to the requirements in 36.104, clauses 6 and 7, and 36.111 clause 5.

# Discussion

In [1], an architecture block diagram for the LMU sharing the BS antenna was proposed showing a duplexer to separate the base station’s transmit and receive signals at the antenna connector.  The receive signal was then split to the LMU and the base station.  The base station side of the splitter is connected to another duplexer to recombine the transmit and receive signals at the base station’s antenna connector.  In [2], Table 5.1 lists a port “C” that is otherwise not identified as to its location in the antenna sharing network.  Defining the test ports in this manner does not conform to the RF requirements for the base station as defined in TS 36.104 [3] clauses 6 and 7.  Alternatively, the antenna sharing network architectures and test port definitions in this contribution will allow testing of the base station for conformance to the RF characteristics requirements of TS 36.104 clauses 6 and 7 and the LMU for conformance to the RF requirements in TS 36.111 [4] clause 5.

Figure 1 shows the general LMU deployment architecture that from the LMU’s perspective could be either with a separate antenna or sharing the antenna with the operator’s RAN equipment.  This diagram shows that the test port used to determine conformance to the requirements of TS 36.111 clause 5 is determined by the presence of any equipment external to the LMU’s receiver in the LMU’s RF path.

LMU

Test port A

Test port B

External

device

e.g.

RX filter

(if any

)

External

LNA

(if any)

From

antenna connector



Figure 1.  LMU receiver test ports

Base station architectures where the output of the base station’s own receiver LNA is accessible allows for the antenna sharing network shown below in Figure 2 where a directional coupler is inserted into the BS receiver RF path after the BS LNA and the LMU receives the coupled RF output of the LNA.  The test port for the base station is the same as it was before the addition of the coupler and LMU.  The test port for the LMU is the same physical test port (in Figure 1 labelled as “BS A” and “BS B” respectively) as is used for the base station per TS 36.104.

Figure 2.  LMU antenna sharing with access to the BS LNA

When the BS provides a receiver access port the antenna sharing architecture is shown in Figure 3.  The test port for the base station is the same as it was before the addition of the LMU.  The test port for the LMU is the same physical test port, “A” or “B” (in Figure 1 labelled as “BS A” and “BS B” respectively), as is used for the base station, per TS 36.104.

Figure 3.  LMU with Rx access port at BS

For base station architectures that do not provide access to the base station’s LNA output, an antenna sharing combiner is required as shown in Figure 4.  Antenna sharing combiners are already in use by operators to share antennas with different air interfaces, such as GSM, UMTS and LTE, to share dual band antennas or to allow different operators to share the same antenna.  Since the antenna sharing combiner of Figure 4 is installed before the base station’s antenna connector, the test port for the base station per TS 36.104 clauses 6 and 7 is “B” (labelled as “BS B”).  Since the base station and LMU share a part of the RF path the test port for the LMU is also the same physical port “B” that is used for the base station per TS 36.104.

Proposal 1:  For antenna sharing the LMU test port is the same test port as is used for the base station.

Note that antenna sharing combiners are used for other purposes besides the sharing the antennas with LMUs and that the use of antenna sharing combiners does not impact the requirements in TS 36.104 clauses 6 and 7 other than to move the conformance test port to the antenna side of the combiner.  Similarly the use of an antenna sharing combiner should not impact the LMU RF requirements in TS 36.111 clause 5.

Proposal 2: The use of antenna sharing combiners in LMU deployments does not impact the LMU RF requirements in TS 36.111 clause 5.

Figure 4.  LMU with antenna sharing combiner

Proposal 3:  It was agreed during RAN 4 #66bis to add text to TS 36.111 clause 4 to define the LMU integrated into the BS as class 1, the LMU that shares the antenna with the BS as class 2 and the LMU with a separate antenna as class 3.  Add the following table to TS 36.111 clause 5 to define the test ports based on the LMU class.

Table 5.1 Test port assignment

| LMU class | Physical Node | RF Requirements | Test Port | Comments |
| --- | --- | --- | --- | --- |
| 1 | BS | TS 36.104 | A or B | Test port determined per TS 36.104 |
| 2 | BS | TS 36.104 | A or B | Test port determined per TS 36.104 |
|  | LMU | TS 36.111 | B | Same physical test port as used for the BS |
| 3 | LMU | TS 36.111 | A or B | Test port determined per TS 36.111 |

# Conclusion

It was agreed during RAN4#66bis to add text to TS 36.111 clause 4 that defines the LMU integrated into the BS as class 1, the LMU that shares the antenna with the BS as class 2 and the LMU with a separate antenna as class 3.  The choice to deploy a specific class of LMU is a function of several factors, including the base station equipment configuration.  For the class 2 LMU several deployment options were given as shown by Figures 2, 3 and 4 where the method of antenna sharing is dependent upon access to the base station’s receiver RF path.  In addition, by defining the test ports in the manner consistent with the requirements of TS 36.104 clauses 6 and 7 and TS 36.111 clause 5 as shown in the figures above we can draw the following conclusions:

- An antenna sharing combiner can be added to the base station’s RF path that allows for the verification of conformance of the base station to the existing requirements of 36.104 clause 7 and also clause 6 if the antenna sharing method includes the RF transmission path.

- An antenna sharing combiner can be added to the base station’s RF path that allows for the verification of conformance of the LMU to the existing requirements of 36.111 clause 5.

- Just as for the base station RF requirements in 36.104, a single set of RF requirements can be defined for the LMU in 36.111 that is applicable regardless of the LMU’s method of connection to the RF antenna.  All that is needed is to properly define the test ports used to verify conformance to the requirements.

Proposal 1:  For antenna sharing the LMU test port is the same test port as is used for the base station.

Proposal 2: The use of antenna sharing combiners in LMU deployments does not impact the LMU RF requirements in TS 36.111 clause 5.

Proposal 3:  It was agreed during RAN 4 #66bis to add text to TS 36.111 clause 4 that defines the LMU integrated into the BS as class 1, the LMU that shares the antenna with the BS as class 2 and the LMU with a separate antenna as class 3.  Add the following table to TS 36.111 clause 5 to define the test ports based on the LMU class.

Table 5.1 Test port assignment

| LMU class | Physical Node | RF Requirements | Test Port | Comments |
| --- | --- | --- | --- | --- |
| 1 | BS | TS 36.104 | A or B | The BS receiver is also the LMU’s receiver. The test port is determined per TS 36.104 |
| 2 | BS | TS 36.104 | A or B | Test port determined per TS 36.104 |
|  | LMU | TS 36.111 | B | Same physical test port as used for the BS |
| 3 | LMU | TS 36.111 | A or B | Test port determined per TS 36.111 |

# References

[1] R4-131601, “On RF aspects with NBPS”, Ericsson, April 2013.

[2] R4-131606, “RF aspects of NBPS”, Ericsson, April 2013.

[3] TS 36.104, “Base Station (BS) radio transmission and reception”

[4] TS 36.111, “Location Measurement Unit (LMU) performance specification”