3GPP TSG RAN WG4 Meeting #67	R4-132506

- Fukuoka, JAPAN, 20th – 24th May, 2013

- Agenda Item:	9.1

- Source:	LG Electronics

- Title:	TP for S-band UE Tx/Rx RF requirements for FS_LTE_1980_2170_Korea

Document for:	Approval

- Introduction

In the RAN Plenary #57 meetings, the study for the LTE S-band in Korea was approved. And the skeleton of technical report ab.cde for Rel-12 was approved in RAN4 #64bis meeting.

One of the objectives for this study item is to facilitate and harmonize the efficient use of these bands for terrestrial IMT, especially LTE, for Korea.

In this contribution, we provide the UE transmitter/Receiver RF requirements considering IMD and harmonic for S-band UE.

- References

- R4-130018, “TR 36.861 v0.0.2,” KT

- TS 36.101 v11.4.0

- R4-120891, “Information of KCC plan on 2.1GHz Spectrum in Korea,” TTA

- R4-131390, “TP for TR ab.cde : General issue for FS_LTE_1980_2170_Korea,” LG Electronics

Text proposal for TR 36.861

********************** Start of text proposal to TR 36.861 Chapter 8 ***********************

- 8	Study of E-UTRA specific issues

This section summarizes issues and additional requirements when MS UE is deployed under Korean regulations

- 8.1    UE Tx RF Requirements

8.1.1	IMD and Harmonic analysis

In this section, we analyze the harmonic and IMD products which have impact on the bands in Korea.

Table 8.1.1-1 and 8.1.1-2 give the harmonic products from S-band UE according to the non-CA UE and intra-NC CA UE. From Table 8.1.1-1 and 8.1.1-2, it can be seen that none of the harmonic products fall into the own S-band DL and the other deployed bands in Korea, which means there is no harmonic problem from S-band UE in Korea.

Table 8.1.1-1: S-band UE harmonic products for non-CA

| UE UL carriers | f1_low | f1_high | f1_low | f1_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 10MHz |  | 20MHz |  |
|  | 2000 | 2010 | 1990 | 2010 |
| 2nd order harmonics frequency range (MHz) | 4000 to 4020 |  | 3980 to 4020 |  |
| 3rd order harmonics frequency range (MHz) | 6000 to 6030 |  | 5970 to 6030 |  |

Table 8.1.1-2: S-band UE harmonic products for intra-NC CA

| UE UL carriers | f1_low | f1_high |
| --- | --- | --- |
| UL frequency (MHz) | 1980 | 2010 |
| 2nd order harmonics frequency range (MHz) | 3960 to 4020 |  |
| 3rd order harmonics frequency range (MHz) | 5940 to 6030 |  |

Table 8.1.1-3 and 8.1.1-4 show the frequency ranges of IMD products for S-band UE according to the non-CA UE and intra-NC CA UE. We can also see that there is no IMD products falling into the own S-band DL and the other deployed bands in Korea, which means there is no IMD problem from S-band UE in Korea.

Table 8.1.1-3: S-band UE IMD products for non CA

| UE UL carriers | f1_low | f1_high | f1_low | f1_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 10MHz |  | 20MHz |  |
|  | 2000 | 2010 | 1990 | 2010 |
| Two-tone 3rd order IMD products | ¦2*f1_low – f1_high¦ | ¦2*f1_high – f1_low¦ | ¦2*f1_low – f1_high¦ | ¦2*f1_high – f1_low¦ |
| IMD frequency range (MHz) | 1990 to 2020 |  | 1970 to 2030 |  |
| Two-tone 3rd order IMD products | (2*f1_low + f1_high) | (2*f1_high + f1_low) | (2*f1_low + f1_high) | (2*f1_high + f1_low) |
| IMD frequency range (MHz) | 6010 to 6020 |  | 5990 to 6010 |  |
| Two-tone 5th order IMD products | (3*f1_low – 2*f1_high) | (3*f1_high – 2*f1_low) | (3*f1_low – 2*f1_high) | (3*f1_high – 2*f1_low) |
| IMD frequency range (MHz) | 1980 to 2030 |  | 1950 to 2050 |  |
| Two-tone 7th order IMD products | (4*f1_low – 3*f1_high) | (4*f1_high -3*f1_low) | (4*f1_low – 3*f1_high) | (4*f1_high -3*f1_low) |
| IMD frequency range (MHz) | 1970 to 2040 |  | 1930 to 2070 |  |

Table 8.1.1-4: S-band UE IMD products for intra-NC CA

| UE UL carriers | f1_low | f1_high |
| --- | --- | --- |
| UL frequency (MHz) | 1980 | 2010 |
| Two-tone 3rd order IMD products | ¦2*f1_low – f1_high¦ | ¦2*f1_high – f1_low¦ |
| IMD frequency range (MHz) | 1950 to 2040 |  |
| Two-tone 3rd order IMD products | (2*f1_low + f1_high) | (2*f1_high + f1_low) |
| IMD frequency range (MHz) | 5970 to 6000 |  |
| Two-tone 5th order IMD products | (3*f1_low – 2*f1_high) | (3*f1_high – 2*f1_low) |
| IMD frequency range (MHz) | 1920 to 2070 |  |
| Two-tone 7th order IMD products | (4*f1_low – 3*f1_high) | (4*f1_high -3*f1_low) |
| IMD frequency range (MHz) | 1890 to 2100 |  |

8.1.2 	Spurious emission for UE co-existence

From the UE-to-UE coexistence analysis [1] and harmonic/IMD product analysis in this paper, we can see that there is no need to define A-MPR requirements and NS signalling to protect the bands in Korea.

As a conclusion of previous discussion, SE requirements for UE-to-UE coexistence can be defined as -50dBm/MHz for S-band UE as in Table 8.1.2-1.

Table 8.1.2-1: Spurious emission band UE co-existence of S-band UE

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Level (dBm) | Bandwidth (MHz) | Comment |
| XX | E-UTRA Band 1,3,5,8,26,XX,40 | FDL_low | - | FDL_high | -50 | 1 | 1 |
| NOTE 1: This requirement is applicable for S-band UE of Band XX in Korea |  |  |  |  |  |  |  |

8.1.3	Other expected UE Tx RF requirements

In Table 8.1.3-1, detailed descriptions on UE transmitter RF requirements were proposed to predict any potential requirements changes needed for S-band UE.

Table 8.1.3-1: Considering UE Tx RF requirements for S-band UE

| Description | Comments and Description of change |
| --- | --- |
| UE Maximum Output Power | No changes are needed for power class 3 due to the large duplex gap and Tx-Rx separation. |
| UE Maximum Output Power with additional requirements (A-MPR) | No changes are needed to protect adjacent coexistence bands due to no IMD and harmonics in Korea [1]. |
| Spurious emission | No changes are needed to protect whole frequency bands for S-band UE in Korea. |
| ACLR/SEM | No changes are needed in OOB ranges for S-band UE in Korea. |
| Additional Spectrum Emission Mask | No changes are needed to protect adjacent coexistence bands due to no IMD and harmonics in Korea [1]. |

  - UE Rx RF Requirements

    - REFSENS requirements for S-band UE

From the analysis of S-band UE co-existence and duplexer requirements, there is no co-existence impact on band 1 UE. Also S-band UE have same Tx-Rx frequency separation and total bandwidth of S-band is smaller than that of Band 1, therefore S-band UE’s REFSENS level can be easily regarded as same as that of Band 1. To verify this assumption, we performed simple RF simulation, the result of which is shown in Fig. 8.2.1-1.

Figure8.2.1-1 show Tx leakage level of 20MHz S-band UE into S-Band DL band with full RBs allocation.

leakage

Figure 8.2.1-1 Simulation results of the Tx leakage level into S-band DL

The leakage level in DL band is about -75dBm/MHz, which is further attenuated by Tx/Rx isolation level with 50dB in the DL. So the leakage level in DL is -125dBm/MHz. Eq. 8-1 shows how to calculate REFSENS in general.

- REFSENS = kTB + 10log(CH_BW) + NF+ IM + SINR +Diversity gain + DFB (dBm)    (Eq. 8-1)

= -174 dBm/Hz + 10log(20MHz) + 9dB+2dB-1dB -3dB +0dB

= -104+11-4= - 94dBm/CH_BW

Where, kTB is thermal noise level, NF is noise figure, IM is impairment margin, SINR level is for QPSK modulation, Diversity gain is the receiver diversity gain and DFB means that Additional relaxation by reflection of design challenges for FE.

Eq. 8-2 shows the impact of REFSENS in S-band DL induced by Tx leakage.

- REFSENS for S-band UE = Conventional REFSENS + Tx_leakage levels                      (Eq. 8-2)

= -94dBm/CH_BW +  -125dBm/MHz

= -107dBm/MHz + -125dBm/MHz = - 106.93dBm/MHz

= -93.97dBm/CH_BW

From Eq. 8-1 and Eq. 8-2, we can see that the REFSENS impact by Tx leakage is negligible.

As a conclusion, REFSENS requirements for S-band UE can be defined as -94dBm/20MHz in Table 8.2.1-1 with the same uplink configurations as in Band1 [2].

Table 8.2.1-1: Reference sensitivity QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| XX |  |  | -100 | -97 | -95.2 | -94 | FDD |

    - Other UE Rx RF requirements

For the Receiver requirements for S-band UE, we propose preliminary descriptions on UE receiver requirements in Table 8.2.2-1. From the above REFSENS analysis, Tx leakage is expected to be negligible since the large duplexer spacing and same Tx-Rx frequency separation as Band1. Hence we can reuse the receiver requirements in Table 8.2.2-1 for S-band UE.

Table 8.2.2-1: Considering UE Rx RF requirements for S-band UE

| Description | Comments and Comparison to 2-band Solution |
| --- | --- |
| Adjacent Channel Selectivity | No changes since there is no Tx leakage impacts on the S-band DL for the T-put requirements that can be larger than or equal to 95% with same condition in section 7.5.1 [2] |
| Blocking characteristics | No changes since there is no Tx leakage impacts on the S-band DL for the T-put requirements can be larger than or equal to 95% with same condition in section 7.6 [2] |
| Receiver inter-modulation characteristics | No changes since there is no Tx leakage impacts on the S-band DL for the T-put requirements can be larger than or equal to 95% with same condition in section 7.8.1 [2] |

********************** End of text proposal to TR 36.861 Chapter 8 ***************************