3GPP TSG-RAN WG4 Meeting #67	R4-132726

# Fukuoka, Japan, 20 - 24 May, 2013

| CR-Form-v10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 25.141 | CR | 650 | rev | - | Current version: | 11.4.0 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of BS performance requirements for HSUPA MIMO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia Siemens Networks |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | MIMO_64QAM_HSUPA-Perf |  |  |  |  |  |  | Date: |  | 2013-05-06 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-11 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change: |  | MIMO with 64QAM for HSUPA core part WI has been completed. Appropriate BS performance requirements have to be introduced to ensure correct functionality of the new feature. |  |  |  |  |  |  |  |  |
| Summary of change: |  | BS performance requirements are introduced for HSUPA MIMO transmission. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | BS performance requirements are not introduced for HSUPA MIMO transmission. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.12A; A.10; A.19; A.20; B.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of changes>

## 8.12	Demodulation of E-DPDCH in multipath fading conditions

### 8.12.1	Definition and applicability

The performance requirement of the E-DPDCH in multi path fading condition is determined by the minimum throughput, R. Test parameters are specified in Table 8.38.

For a BS supporting DC-HSUPA the requirements for FRC1, FRC2, FRC3 and FRC8 shall apply on each cell.

# Table 8.38: Test parameters for testing E-DPDCH

| Parameter | Unit | Test |  |
| --- | --- | --- | --- |
| RSN |  | {0, 1, 2, 3} |  |
| HARQ combining |  | IR |  |
| Maximum number of HARQ transmission |  | 4 |  |
| Power control |  | OFF |  |
| DPCCH slot format |  | FRC8 or BS supporting DC-HSUPA | 1 |
|  |  | Otherwise | 0 |
| E-DPCCH # code words |  | 1024, no optimization based on prior knowledge of valid code words. |  |
| Physical channels to be turned on |  | DPCCH, E-DPDCH and E-DPCCH |  |

Only tests in Pedestrian A shall be applied to Home BS.

### 8.12.2	Minimum requirement

The minimum requirement is in TS 25.104 [1] clause 8.11.

# Table 8.39: (void)

### 8.12.3	Test Purpose

The test shall verify the receiver's ability to receive the test signal under slow multipath fading propagation conditions with a throughput not below a specified limit.

### 8.12.4	Method of test

#### 8.12.4.1	Initial conditions

Test environment: 			normal; see clause 4.4.1.

RF channels to be tested: 	B, M and T; see clause 4.8

1)	For BS with Rx diversity, connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to both BS antenna connectors for diversity reception via a combining network as shown in annex B.

2)	For BS without Rx diversity, connect the BS tester generating the wanted signal, multipath fading simulator and AWGN generator to the BS antenna connector via a combining network as shown in annex B.

#### 8.12.4.2	Procedure

1)	Adjust the AWGN generator depending on the BS class under test at the BS input as follows:.

Wide Area:	-84 dBm/3.84 MHz

Medium Range:	-74 dBm/3.84 MHz

Local Area / Home BS:	-70 dBm/3.84 MHz

2)	The characteristics of the wanted signal shall be configured according to the corresponding UL reference measurement channel defined in annex A.

3)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex D.

4)	Adjust the equipment so that required EC/N0 specified in table 8.40 is achieved.

5)	For each of the reference channels in table 8.40 applicable for the base station, measure the throughput.

#### 8.12.4.3	Test requirements

The throughput measured according to clause 8.12.4.2 shall not be below the limits for the EC/N0 levels specified in table 8.40.

# Table 8.40: Test Requirement for E-DPDCH

| Fixed Reference Channel |  | Reference value, EC/N0 (dB), for R ≥ 30% and R ≥ 70% of maximum information bit rate |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Propagation conditionsPropagation conditions |  | FRC1 | FRC2 | FRC3 | FRC4 | FRC5 | FRC6 | FRC7 | FRC8 |  |
|  |  |  |  |  |  |  |  |  | Non E-DPCCH boosting | E-DPCCH Boosting |
| Pedestrian A without RX diversity | 30% | -1.8 | 1.4 | 3.0 | -6.5 | -3.8 | -0.8 | -14.4 | NA | NA |
|  | 70% | 4.3 | 7.7 | 9.7 | 0.0 | 2.7 | 5.8 | -7.8 | 16.8 | 17.5 |
| Pedestrian A with RX diversity | 30% | -5.6 | -2.5 | -0.8 | -10.0 | -7.4 | -4.4 | -17.7 | NA | NA |
|  | 70% | -0.4 | 2.8 | 4.7 | -4.6 | -2.0 | 0.8 | -12.7 | 10.7 | 11.0 |
| Pedestrian B without RX diversity* | 30% | -1.9 | 1.7 | 4.1 | -6.9 | -4.1 | -0.7 | -13.0 | NA | NA |
|  | 70% | 4.5 | NA | NA | -1.5 | 1.5 | 5.9 | -9.5 | NA | NA |
| Pedestrian B with RX diversity* | 30% | -5.5 | -2.5 | -0.4 | -10.1 | -7.5 | -4.3 | -17.4 | NA | NA |
|  | 70% | 0.3 | 4.5 | 8.8 | -5.1 | -2.3 | 1.3 | -13.2 | 13.0 | 13.7 |
| Vehicular 30 without RX diversity* | 30% | -1.9 | 1.6 | 3.8 | -6.9 | -4.0 | -0.8 | -13.7 | NA | NA |
|  | 70% | 5.5 | NA | NA | -1.1 | 2.0 | 6.4 | -9.5 | NA | NA |
| Vehicular 30 with RX diversity* | 30% | -5.5 | -2.3 | -0.3 | -10.1 | -7.4 | -4.3 | -17.0 | NA | NA |
|  | 70% | 1.2 | 5.3 | 9.4 | -4.8 | -2.0 | 1.6 | -13.1 | 13.9 | 14.2 |
| Vehicular 120 without RX diversity* | 30% | -1.5 | 1.9 | 4.2 | -6.7 | -3.6 | -0.6 | -13.4 | NA | NA |
|  | 70% | 5.7 | NA | NA | -0.7 | 2.1 | 6.7 | -9.5 | NA | NA |
| Vehicular 120 with RX diversity* | 30% | -5.1 | -2.0 | 0.1 | -9.8 | -7.0 | -3.7 | -16.4 | NA | NA |
|  | 70% | 1.3 | 5.6 | 10.1 | -4.5 | -1.7 | 1.8 | -12.6 | NA | NA |
| * Not applicable for Home BS |  |  |  |  |  |  |  |  |  |  |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause 4.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in Annex F.

## 8.12A	Demodulation of E-DPDCH and S-E-DPDCH in multipath fading conditions for UL MIMO

### 8.12A.1	Definition and applicability

The performance requirements of the E-DPDCH and S-E-DPDCH in multi path fading condition for UL MIMO are determined by the minimum throughput, R. Test parameters are specified in Table 8.38A.

# Table 8.38A: Test parameters for testing E-DPDCH and S-E-DPDCH for UL MIMO

| Parameter | Test |
| --- | --- |
| RSN | {0, 1, 2, 3} |
| HARQ combining | IR |
| Maximum number of HARQ transmission | 4 |
| Power control | OFF |
| TX weight vector selection | A fixed precoding weight vector |
| Physical channels to be turned on | DPCCH, S-DPCCH, E-DPCCH, S-E-DPCCH, E-DPDCH and S-E-DPDCH |

### 8.12A.2	Minimum requirement

# The minimum requirement is in TS 25.104 [1] clause 8.11A.

### 8.12A.3	Test Purpose

The test shall verify the receiver's ability to receive the test signal under slow multipath fading propagation conditions with a throughput not below a specified limit.

### 8.12A.4	Method of test

#### 8.12A.4.1	Initial conditions

Test environment: 			normal; see clause 4.4.1.

RF channels to be tested: 	B, M and T; see clause 4.8

For BS with Rx diversity, connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to both BS antenna connectors for diversity reception via a combining network as shown in annex B.

#### 8.12A.4.2	Procedure

1)	Adjust the AWGN generator depending on the BS class under test at the BS input as follows:.

Wide Area:	-84 dBm/3.84 MHz

Medium Range:	-74 dBm/3.84 MHz

Local Area / Home BS:	-70 dBm/3.84 MHz

2)	The characteristics of the wanted signal shall be configured according to the corresponding UL reference measurement channel defined in annex A.

3)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex D.

4)	Adjust the equipment so that required EC/N0 specified in table 8.40A is achieved.

5)	For each of the reference channels in table 8.40A applicable for the base station, measure the throughput.

#### 8.12A.4.3	Test requirements

The throughput measured according to clause 8.12A.4.2 shall not be below the limits for the EC/N0 levels specified in table 8.40A.

# Table 8.40A: Test Requirement for E-DPDCH and S-E-DPDCH for UL MIMO

| Fixed Reference Channel | Reference value, EC/N0 (dB), for R ≥ 70% of maximum information bit rate |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Propagation conditions | FRC9 |  |  | FRC10 |  |  |
|  | E-DPDCH | S-E-DPDCH | total | E-DPDCH | S-E-DPDCH | total |
| Ped A, 3 km/h | TBD | TBD | TBD | TBD | TBD | TBD |
| Ped B, 3 km/h | TBD | TBD | TBD | TBD | TBD | TBD |
| Veh A, 3 km/h | TBD | TBD | TBD | TBD | TBD | TBD |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause 4.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in Annex F.

<Next change>

# A.10	Summary of E-DPDCH Fixed reference channels

# Table A.10

| Fixed Ref Channel | TTI [ms] | NINF | SF1 | SF2 | SF3 | SF4 | NBIN | Coding rate | Max inf bit rate [kbps] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FRC1 | 2 | 2706 | 4 | 4 | 0 | 0 | 3840 | 0.705 | 1353.0 |
| FRC2 | 2 | 5412 | 2 | 2 | 0 | 0 | 7680 | 0.705 | 2706.0 |
| FRC3 | 2 | 8100 | 2 | 2 | 4 | 4 | 11520 | 0.703 | 4050.0 |
| FRC4 | 10 | 5076 | 4 | 0 | 0 | 0 | 9600 | 0.529 | 507.6 |
| FRC5 | 10 | 9780 | 4 | 4 | 0 | 0 | 19200 | 0.509 | 978.0 |
| FRC6 | 10 | 19278 | 2 | 2 | 0 | 0 | 38400 | 0.502 | 1927.8 |
| FRC7 | 10 | 690 | 16 | 0 | 0 | 0 | 2400 | 0.288 | 69.0 |
| FRC8 | 2 | 16218 | 2 | 2 | 4 | 4 | 23040 | 0.704 | 8109.0 |
| FRC9 | 2 | 16200 | 2 | 2 | 4 | 4 | 23040 | 0.703 | 8100.0 |
| FRC10 | 2 | 32436 | 2 | 2 | 4 | 4 | 46080 | 0.704 | 16218.0 |

# A.11	E-DPDCH Fixed reference channel 1 (FRC1)

# Table A.11

| Parameter | Unit | Value |
| --- | --- | --- |
| Maximum. Inf. Bit Rate | kbps | 1353.0 |
| TTI | ms | 2 |
| Number of HARQ Processes | Processes | 8 |
| Information Bit Payload (NINF) | Bits | 2706 |
| Binary Channel Bits per TTI (NBIN) (3840 / SF x TTI sum for all channels) | Bits | 3840 |
| Coding Rate (NINF/ NBIN) |  | 0.705 |
| Physical Channel Codes | SF for each physical channel | {4,4} |
| E-DPDCH testing: E-DPDCH/DPCCH power ratio E-DPCCH/DPCCH power ratio | dB dB dB dB | Diversity: 8.94 Non-diversity: 12.04 Diversity: 2.05 Non-diversity: 6.02 E-DPDCH /DPCCH power ratio is calculated for a single E-DPDCH. |
| E-DPCCH missed detection testing: E-DPDCH/DPCCH power ratio E-DPCCH/DPCCH power ratio | dB dB dB dB | Diversity: 8.94 Non-diversity: 12.04 Diversity: -1.94 Non-diversity: 0.00 |

# Figure A.11

# A.12	E-DPDCH Fixed reference channel 2 (FRC2)

# Table A.12

| Parameter | Unit | Value |
| --- | --- | --- |
| Maximum. Inf. Bit Rate | kbps | 2706.0 |
| TTI | ms | 2 |
| Number of HARQ Processes | Processes | 8 |
| Information Bit Payload (NINF) | Bits | 5412 |
| Binary Channel Bits per TTI (NBIN) (3840 / SF x TTI sum for all channels) | Bits | 7680 |
| Coding Rate (NINF/ NBIN) |  | 0.705 |
| Physical Channel Codes | SF for each physical channel | {2,2} |
| E-DPDCH testing: E-DPDCH/DPCCH power ratio E-DPCCH/DPCCH power ratio | dB dB dB dB | Diversity: 9.92 Non-diversity: 13.00 Diversity: 4.08 Non-diversity: 6.02 E-DPDCH /DPCCH power ratio is calculated for a single E-DPDCH. |

# Figure A.12

# A.13	E-DPDCH Fixed reference channel 3 (FRC3)

# Table A.13

| Parameter | Unit | Value |
| --- | --- | --- |
| Maximum. Inf. Bit Rate | kbps | 4050.0 |
| TTI | ms | 2 |
| Number of HARQ Processes | Processes | 8 |
| Information Bit Payload (NINF) | Bits | 8100 |
| Binary Channel Bits per TTI (NBIN) (3840 / SF x TTI sum for all channels) | Bits | 11520 |
| Coding Rate (NINF/ NBIN) |  | 0.703 |
| Physical Channel Codes | SF for each physical channel | {2,2,4,4} |
| E-DPDCH testing: E-DPDCH/DPCCH power ratio E-DPCCH/DPCCH power ratio | dB dB dB dB | Diversity: 6.02 Non-diversity: 8.94 Diversity: 0.0 Non-diversity: 2.05 E-DPDCH/DPCCH power ratio is calculated for a single E-DPDCH with SF 4. The power of an E-DPDCH with SF2 is twice that of an E-DPDCH with SF4. |

# Figure A.13

# A.14	E-DPDCH Fixed reference channel 4 (FRC4)

# Table A.14

| Parameter | Unit | Value |
| --- | --- | --- |
| Maximum. Inf. Bit Rate | kbps | 507.6 |
| TTI | ms | 10 |
| Number of HARQ Processes | Processes | 4 |
| Information Bit Payload (NINF) | Bits | 5076 |
| Binary Channel Bits per TTI (NBIN) (3840 / SF x TTI sum for all channels) | Bits | 9600 |
| Coding Rate (NINF/ NBIN) |  | 0.529 |
| Physical Channel Codes | SF for each physical channel | {4} |
| E-DPDCH testing: E-DPDCH/DPCCH power ratio E-DPCCH/DPCCH power ratio | dB dB dB dB | Diversity: 8.94 Non-diversity: 12.04 Diversity: -1.94 Non-diversity: 0.0 |
| E-DPCCH missed detection testing: E-DPDCH/DPCCH power ratio E-DPCCH/DPCCH power ratio | dB dB dB dB | Diversity: 8.94 Non-diversity: 12.04 Diversity: -7.96 Non-diversity: -5.46 |

# Figure A.14

# A.15	E-DPDCH Fixed reference channel 5 (FRC5)

# Table A.15

| Parameter | Unit | Value |
| --- | --- | --- |
| Maximum. Inf. Bit Rate | kbps | 978.0 |
| TTI | ms | 10 |
| Number of HARQ Processes | Processes | 4 |
| Information Bit Payload (NINF) | Bits | 9780 |
| Binary Channel Bits per TTI (NBIN) (3840 / SF x TTI sum for all channels) | Bits | 19200 |
| Coding Rate (NINF/ NBIN) |  | 0.509 |
| Physical Channel Codes | SF for each physical channel | {4,4} |
| E-DPDCH testing: E-DPDCH/DPCCH power ratio E-DPCCH/DPCCH power ratio | dB dB dB dB | Diversity: 8.94 Non-diversity: 12.04 Diversity: -1.94 Non-diversity: 0.0 E-DPDCH /DPCCH power ratio is calculated for a single E-DPDCH. |

# Figure A.15

# A.16	E-DPDCH Fixed reference channel 6 (FRC6)

# Table A.16

| Parameter | Unit | Value |
| --- | --- | --- |
| Maximum. Inf. Bit Rate | kbps | 1927.8 |
| TTI | ms | 10 |
| Number of HARQ Processes | Processes | 4 |
| Information Bit Payload (NINF) | Bits | 19278 |
| Binary Channel Bits per TTI (NBIN) (3840 / SF x TTI sum for all channels) | Bits | 38400 |
| Coding Rate (NINF/ NBIN) |  | 0.502 |
| Physical Channel Codes | SF for each physical channel | {2,2} |
| E-DPDCH testing: E-DPDCH/DPCCH power ratio E-DPCCH/DPCCH power ratio | dB dB dB dB | Diversity: 9.92 Non-diversity: 13.00 Diversity: -5.46 Non-diversity: -1.94 E-DPDCH /DPCCH power ratio is calculated for a single E-DPDCH. |

# Figure A.16

# A.17	E-DPDCH Fixed reference channel 7 (FRC7)

# Table A.17

| Parameter | Unit | Value |
| --- | --- | --- |
| Maximum. Inf. Bit Rate | kbps | 69.0 |
| TTI | ms | 10 |
| Number of HARQ Processes | Processes | 4 |
| Information Bit Payload (NINF) | Bits | 690 |
| Binary Channel Bits per TTI (NBIN) (3840 / SF x TTI sum for all channels) | Bits | 2400 |
| Coding Rate (NINF/ NBIN) |  | 0.288 |
| Physical Channel Codes | SF for each physical channel | {16} |
| E-DPDCH testing: E-DPDCH/DPCCH power ratio E-DPCCH/DPCCH power ratio | dB dB dB dB | Diversity: 6.02 Non-diversity: 8.94 Diversity: 0.0 Non-diversity: 4.08 |

# Figure A.17

# A.18	E-DPDCH Fixed reference channel 8 (FRC8)

# Table A.18

| Parameter | Unit | Value |
| --- | --- | --- |
| Modulation |  | 16QAM |
| Maximum. Inf. Bit Rate | kbps | 8109.0 |
| TTI | ms | 2 |
| Number of HARQ Processes | Processes | 8 |
| Information Bit Payload (NINF) | Bits | 16218 |
| Binary Channel Bits per TTI (NBIN) (3840 / SF x TTI sum for all channels) | Bits | 23040 |
| Coding Rate (NINF/ NBIN) |  | 0.704 |
| Physical Channel Codes | SF for each physical channel | {2,2,4,4} |
| E-DPDCH testing: E-DPDCH/DPCCH power ratio E-DPCCH/DPCCH power ratio ΔT2TP E-DPDCH/DPCCH power ratio E-DPCCH/DPCCH power ratio | dB dB dB dB dB dB dB dB dB dB | Non E-DPCCH boosting Diversity: 4.09 Non-diversity: 6.98 Diversity: -9.54 Non-diversity: -5.46 E-DPCCH Boosting Diversity: 12 Non-diversity: 15 Diversity: 19.99 Non-diversity: 22.00 Diversity: 16.03 Non-diversity: 14.09 E-DPDCH/DPCCH power ratio is calculated for a single E-DPDCH with SF 4. The power of an E-DPDCH with SF2 is twice that of an E-DPDCH with SF4. |

# Figure A.18

# A.19	E-DPDCH Fixed reference channel 9 (FRC9)

# Table A.19

| Parameter | Unit | Value |
| --- | --- | --- |
| Modulation |  | QPSK |
| Maximum. Inf. Bit Rate | kbps | 8100 |
| TTI | ms | 2 |
| Number of HARQ Processes | Processes | 8 |
| Information Bit Payload (NINF) | Bits | 16200 |
| Binary Channel Bits per TTI (NBIN) (3840 / SF x TTI sum for all channels) | Bits | 23040 |
| Coding Rate (NINF/ NBIN) |  | 0.703 |
| Physical Channel Codes | SF for each physical channel | {2,2,4,4} |
| E-DPDCH/DPCCH power ratio E-DPCCH/DPCCH power ratio S-DPCCH/DPCCH power ratio S-E-DPCCH/DPCCH power ratio S-E-DPDCH/DPCCH power ratio | dB dB dB dB dB | 6.02 -1.94 -1.94 0.00 6.02 E-DPDCH/DPCCH power ratio is calculated for a single E-DPDCH with SF 4. The power of an E-DPDCH with SF2 is twice that of an E-DPDCH with SF4. |

# Figure A.19

# A.20	E-DPDCH Fixed reference channel 10 (FRC10)

# Table A.20

| Parameter | Unit | Value |
| --- | --- | --- |
| Modulation |  | 16QAM |
| Maximum. Inf. Bit Rate | kbps | 16218 |
| TTI | ms | 2 |
| Number of HARQ Processes | Processes | 8 |
| Information Bit Payload (NINF) | Bits | 32436 |
| Binary Channel Bits per TTI (NBIN) (3840 / SF x TTI sum for all channels) | Bits | 46080 |
| Coding Rate (NINF/ NBIN) |  | 0.704 |
| Physical Channel Codes | SF for each physical channel | {2,2,4,4} |
| E-DPDCH/DPCCH power ratio E-DPCCH/DPCCH power ratio S-DPCCH/DPCCH power ratio S-E-DPCCH/DPCCH power ratio S-E-DPDCH/DPCCH power ratio | dB dB dB dB dB | 19.99 16.03 16.03 [0.00] 19.99 E-DPDCH/DPCCH power ratio is calculated for a single E-DPDCH with SF 4. The power of an E-DPDCH with SF2 is twice that of an E-DPDCH with SF4. |

<Next change>

## B.3.4	Demodulation of E-DPDCH, S-E-DPDCH and E-DPCCH signalling in multipath fading conditions

Figure B.17: Functional Set-up for Demodulation of E-DPDCH and E-DPCCH in multipath fading conditions for BS with Rx diversity

Figure B.17A: Functional Set-up for Demodulation of E-DPDCH and S-E-DPDCH in multipath fading conditions for UL MIMO

Figure B.18: Functional Set-up for Demodulation of E-DPDCH and E-DPCCH in multipath fading conditions for BS without Rx diversity

<End of changes>