TSG-RAN Working Group 4 (Radio) meeting #68		R4-133782

Barcelona, Spain, 19-23 August 2013

Source:	ETRI, LGU+, SK Telecom, and KT

Title:	Monte-Carlo analysis of coexistence of S-band with Bands 1 and 34

Agenda item:	10.1

Document for:	Discussion

Introduction

In the RAN Plenary #57 meetings, the study for the LTE Mobile S-band in Korea are approved. And the skeleton of technical report ab.cde for Rel-12 was approved in RAN4 #64bis meeting.

The objective of study item on LTE FDD in the bands 1980-2010 MHz and 2170-2200 MHz is to facilitate and harmonize the efficient use of these bands for terrestrial IMT, especially LTE, for Korea.

In this contribution, we provide the system-level simulation results considering the interference from S-band to Band 1 and 34. Note that Band 34 is designated as TDD usage only.

Simulation scenario

Based on the band allocation of 2.1GHz in Korea, described in Figure 2.1, it needs to see how the S-band affects to adjacent Band 1 and 34. Therefore, Monte-Carlo evaluation is conducted on four simulation scenarios as described in Table 2.1. Since the result of interference from S-band UE to Band 34 BS is analogous to the uplink interference result for band 1, we conduct the simulation for Band 34 downlink victim case only (UE to UE) as far as Band 34 is the victim. Moreover, when Band 34 is victim system, the 20 MHz aggressor is only assumed, because it gives the strongest interference.

Figure 2.1 Simulation scenarios

Table 2.1 Simulation scenarios

| Scenario # | Aggressor system | Victim system | Simulation frequency | Environment | ISD | Cell Range |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 10 MHz S-Band LTE-FDD | 10 MHz Band 1 LTE-FDD | 2100 MHz | Urban Area | 750 m | 500 m |
| 2 | 20 MHz S-Band LTE-FDD | 10 MHz Band 1 LTE-FDD | 2100 MHz | Urban Area | 750 m | 500 m |
| 3 | 20 MHz S-Band LTE-FDD UL | 10 MHz Band 34 LTE-TDD DL | 2100 MHz | Urban Area | 750 m | 500 m |

Assumption and Methodology

The same methodology as used in [1] for LTE coexistence study is used. Detailed assumption and ACIR model are described in the annex. Note that coordinated deployment is assumed for scenario 3 unlike the other scenarios, so that the worst case can occur in the simulation run, where two UEs are closely located in the cell edges of both systems. Also, TPC set 1 is only assumed in scenario 3, since aggressor UEs transmit at a higher power than when TPC set 2 is used.

Simulation Results

Downlink S-Band 10/20 MHz LTE-FDD aggressor – 10 MHz Band 1 LTE-FDD victim

Simulation results for downlink throughput loss of Band 1 victim are presented in Table 4.1 and Fig. 4.1.

| Table 4.1 Downlink throughput loss Offset value [dB]; Average throughput loss; 5%-ile throughput loss. 10 MHz; 20 MHz; 10 MHz; 20 MHz. -20; 13.71; 9.48; 54.08; 37.25. -15; 6.70; 4.34; 25.86; 16.78. -10; 3.14; 1.97; 11.87; 7.66. -5; 1.46; 0.93; 5.07; 2.96. 0; 0.49; 0.27; 1.71; 1.07. 5; 0.31; 0.23; 0.36; 0.11. 10; 0.11; 0.08; 0.2; 0.11. 15; 0.08; 0.07; 0.2; 0. 20; 0; 0; 0; 0. | Figure 4.1 Downlink throughput loss |
| --- | --- |

Uplink 10/20 MHz S-Band LTE-FDD aggressor – 10 MHz Band 1 LTE-FDD victim

Simulation results for uplink throughput loss of Band 1 victim are presented in Table 4.2 and Figure 4.2 for TPC set 1 and Table 4.3 and Figure 4.3 for TPC set 2, respectively.

| Table 4.2 Uplink throughput loss (TPC 1) Offset value [dB]; Average throughput loss; 5%-ile throughput loss. 10 MHz; 20 MHz; 10 MHz; 20 MHz. -20; 19.81; 22.85. -15; 10.75; 12.69; 29.56; 32.17. -10; 5.29; 6.13; 11.56; 11.12. -5; 2.38; 2.74; 3.59; 3.84. 0; 0.97; 1.12; 1.12; 1.59. 5; 0.36; 0.42; 0.37; 0.37. 10; 0.12; 0.14; 0.11; 0.05. 15; 0.11; 0.04; 0.02; 0. 20; 0; 0; 0; 0. | Figure 4.2 Uplink throughput loss (TPC 1) |
| --- | --- |

| Table 4.3 Uplink throughput loss (TPC 2) Offset value [dB]; Average throughput loss; 5%-ile throughput loss. 10 MHz; 20 MHz; 10 MHz; 20 MHz. -20; 18.89; 22.75; 52.52; 55.63. -15; 10.13; 12.05; 24.32; 27.49. -10; 4.93; 5.81; 8.81; 10.82. -5; 2.19; 2.58; 2.60; 4.08. 0; 0.89; 1.07; 0.66; 1.21. 5; 0.33; 0.41; 0.23; 0.26. 10; 0.12; 0.14; 0.09; 0.05. 15; 0.04; 0.05; 0.02; 0.04. 20; 0; 0; 0; 0. | Figure 4.3 Uplink throughput loss (TPC 2) |
| --- | --- |

Uplink 10/15/20 MHz S-Band LTE-FDD – Downlink 10 MHz Band 1 LTE-TDD victim

Simulation results for downlink throughput loss of Band 34 victim are presented in Table 4.4 and Figure 4.4.

| Table 4.4 LTE-TDD downlink throughput loss Offset value [dB]; Average throughput loss; 5%-ile throughput loss. -30; 5.42; 1.47. -25; 2.88; 0.8. -20; 1.48; 0.42. -15; 0.68; 0.21. -10; 0.34; 0.10. -5; 0.17; 0.05. 0; 0.10; 0.02. 5; 0.01; 0.01. 10; 0.01; 0. | Figure 4.4 Average LTE-TDD downlink throughput loss |
| --- | --- |

Conclusion

From the simulation result, it is identified that the average and 5-%tile throughput degradation are less than 5% for all scenarios. As far as a system capacity is concerned, it can be concluded that S-band has no coexistence issue with Band 1and 34, using a conventionally assumed UE ACLR model (ACLR1-30dB, ACLR-43dB and ACLR-50dB) and UE ACS model (ACS1: 33dB, ACS2: 34.3dB, and ACS3:46.3dB) [1]. This result is comparable to the previous studies of other bands as in [1][4][5]. The scope of this study is limited to the statistical evaluation averaging interference impact over the system and using uniform user densities. Further studies may be necessary in other perspectives e.g. RF simulation based approach for the coexistence between S-band UE and adjacent Band 1 and Band 34 UE.

References

[1]	3GPP TR 36.942. V11.0.0, E-UTRA, Radio Frequency (RF) System Scenario

[2]	3GPP TR 25.942. V11.0.0, UMTS, Radio Frequency (RF) System Scenario

[3]	J.E. Berg: “A Recursive Model For Street Microcell Path Loss Calculations”, International Symposium on Personal Indoor and Mobile indoor Communications (PIMRC) '95, p 140 - 143, Toronto.

[4]	ECC Report 119, Coexistence between mobile systems in the 2.6 GHz frequency band at the FDD/TDD boundary

[5]	ITU-R Rep. M.2030 Coexistence between IMT-2000 TDD and FDD radio interface technologies within the frequency range 2500-2690MHz operating in adjacent bands and in the same geographical area, 2003.

- Annex: Simulations assumptions and Methodology

- A.1	Assumptions

| Parameter | Assumption (common) |  |
| --- | --- | --- |
| Environment | Macro cell, Urban area, Uncoordinated deployment (BS↔UE) / Coordinated deployment (UE↔UE) |  |
| Carrier frequency | 2100 MHz |  |
| Cellular layout | Hexagonal grid, 19 cell sites, 57 sectors with BTS in the corner of the cell , |  |
|  | 65-degree sectored beam. |  |
| BTS antenna gain (include feeder loss) | 15 dBi |  |
| Antenna pattern | A() = -min[12(/),Am], where ,= 65 degree, and Am=20dB |  |
| BTS antenna height | 30 m |  |
| Inter-site distance | 750 m |  |
| Pathloss model | BS↔UE | 128.1+37.6log10(r) + 21*log10(fc/2.0) |
|  | UE↔UE | H. Xia path loss model formula [2][3]. The break distance is assumed to be 50m. |
| log-normal fade shadow | 10 dB |  |
| Shadowing correlation | Between cells: 0.5, Between sectors: 1.0 |  |
| MCL (including antenna gain) | 70 dB (BS↔UE) / 40 dB (UE↔UE) |  |
| Handover margin | 3 dB |  |
| white noise power density | -174 dBm/Hz |  |
| BTS noise figure | 5 dB |  |
| UE noise figure | 9 dB |  |
| Scheduling algorithm | Round Robin |  |
| BS max Tx power | 46 dBm |  |
| UE max Tx power | 23 dBm |  |
| UE min Tx power | -40 dBm |  |
| Power control algorithm | Fractional TPC |  |
| P0PUSCH | -101.0 dBm (TPC set1), -92.24 dBm (TCP set 2) |  |
| alpha | 1.0 (TPC set 1), 0.8 (TPC set 2) |  |
| Traffic model | full-buffer |  |
| Resource Block (RB) size | 180kHz, total: 50/100 RBs for 10/20MHz |  |
| RB number per active UEs | 15 RBs (UL), (total 90 RBs for 20MHz) 50 RBs (DL) for 10MHz |  |
| number of active UEs | 3/6 UEs for 10/20MHz (UL), 1 UE (DL) |  |
| Link simulation interface | Attenuated and truncated form of the Shannon bound in [1] |  |

Table A.1 Simulation assumptions

- A.2	ACIR Model

A.2.1	Uplink ACIR model

For uplink, it is assumed that the ACIR is dominated by the UE ACLR. As shown in Table A.2, the bandwidth for each ACIR value was assumed to be the same as the transmission bandwidth of LTE UE. Note that ACLR model (ACLR1: 30dB, ACLR2: 43dB and ACLR3: 50dB) is referred from [1].

Table A.2 Uplink ACIR models for RB offsets

| Frequency offset between aggressor (15RBs) and victim (15RBs) | ACIR value (dB) |
| --- | --- |
| 0RBs | 30 + X |
| 15RBs | 43+X |
| 30RBs | 50+X |

A.2.2	Downlink ACIR model

For the downlink, a common ACIR is assumed. The ACIR is dominated by the UE ACS and obtained by averaging ACS values which fall on the aggressor’s channel bandwidth. Table A.3 represents the average ACIR. Note that ACS model (ACS1: 33dB, ACS2: 34.3dB, and ACS3:46.3dB) is referred from [1].

Table A.3 ACS and downlink ACIR model

|  | Aggressor system |  |  |
| --- | --- | --- | --- |
|  | 10 MHz LTE | 15 MHz LTE | 20 MHz LTE |
| ACS | ACS1 = 33 dB, ACS2 = 34.3 dB, ACS3 = 46.3 dB [1] |  |  |
| Average ACIR | 33.6 dB | 35.2 dB | 36.4 |

A.2.3	UE-to-UE ACIR model

For the UE-UE interference analysis, ACIR is dominated by neither ACLR nor ACS; Any of ACLR and ACS of UE are not much bigger than the other. Therefore, it is required to taking into account both of aggressor UE’s ACLR and victim UE’s ACS in calculating ACIR. As for the victim’s ACS, depending on the aggressor UE’s relative position to the victim in the frequency channel, different ACS values are applied. For some aggressor UEs such as UE2 and UE4 in Fig. A.1, the ACS is averaged by two ACSs. Consequently, Table A.4 represents ACS values according to the position of aggressor UE in the frequency channel.

Figure A.1 TDD downlink ACS model when interfered by FDD uplink

Table A.4 TDD downlink ACS value when interfered by FDD uplink

|  | UE1 | UE2 | UE3 | UE4 | UE5 |
| --- | --- | --- | --- | --- | --- |
| ACS | 33.6 | 33.7 | 34.3 | 35.7 | 46.3 |

For ACLR, a common ACLR is obtained by averaging ACLRs which fall on the channel of the victim, as described in Fig. A.2 and Fig. A.3. The average ACLRs are listed in Table A.5 according to the aggressor UE’s position in the channel. Consequently ACIR values are calculated by 1/(1/ACLR + 1/ACS) and depicted in Table A.6.

Figure A.2 Uplink ACLR models from aggressor UE1

Figure A.3 Uplink ACLR models from aggressor UE2

Table A.5 Average ACLR value depending on an aggressor UE

|  | UE1 | UE2 | UE3 |
| --- | --- | --- | --- |
| ACLR average | 37.1 | 47.6 | 50 |

Table A.6 Average ACIR value depending on an aggressor UE

|  | UE1 | UE2 | UE3 | UE4 | UE5 |
| --- | --- | --- | --- | --- | --- |
| ACIR | 32 | 33.5 | 34.2 | 35.5 | 44.7 |