3GPP TSG-RAN WG4 Meeting #68  	                                    R4-133835

Barcenola, Spain, 19th – 23rd Aug, 2013

Agenda Item:	8.1.1

Source:	Samsung

Title:	Performance Evaluation of PUSCH for Phase-II Cases

Document for:	Discussion

# Introduction

In RAN4 #66bis meeting, the simulation assumption for PUSCH in multipath fading propagation conditions, 2Tx 8Rx have been decided [1]. This contribution presents the ideal and impairment simulation results accordingly.

# Simulation Assumption for PUSCH with 2Tx

Table 1	Simulation assumptions for PUSCH, 2Tx 8Rx

| Parameters | Values |
| --- | --- |
| Propagation conditions | EPA |
| Antenna configuration | 2x8 |
| Correlation matrix for low correlation [6] |  |
| Noise Model | AWGN |
| Channel bandwidth | 1.4MHz, 3MHz, 5MHz, 10 MHz, 15 MHz, 20 MHz |
| Channel estimation | Practical channel and noise estimation |
| Cyclic prefix | Normal |
| Number of codewords | 2 |
| Number of layers | 2 |
| Resource allocation | Based on FRC |
| Modulation scheme and code rate | QPSK 1/3, 16QAM 3/4 |
| Maximum number of HARQ transmissions | 4 |
| Redundancy version sequence | 0, 2, 3, 1 |
| HARQ combining | Incremental redundancy |

# Ideal Performance of PUSCH with 2Tx 8Rx

Table 2 to Table 7 give the ideal performance requirements for PUSCH with 2Tx 8Rx under 1.4MHz, 3MHz, 5MHz, 10MHz, 15MHz and 20MHz configurations respectively.

# Table 2 Ideal Minimum requirements for PUSCH, 1.4 MHz Channel Bandwidth, 2Tx

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | 8 | Normal | EPA 5Hz Low | A3-2 | 70% | -4.9 |
|  |  |  |  | A4-3 | 70% | 6.5 |

Table 3 Ideal Minimum requirements for PUSCH, 3 MHz Channel Bandwidth, 2Tx

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | 8 | Normal | EPA 5Hz Low | A3-3 | 70% | -5.2 |
|  |  |  |  | A4-4 | 70% | 6.6 |

Table 4 Ideal Minimum requirements for PUSCH, 5 MHz Channel Bandwidth, 2Tx

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | 8 | Normal | EPA 5Hz Low | A3-4 | 70% | -5.9 |
|  |  |  |  | A4-5 | 70% | 6.7 |

Table 5 Ideal Minimum requirements for PUSCH, 10 MHz Channel Bandwidth, 2Tx

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | 8 | Normal | EPA 5Hz Low | A3-5 | 70% | -5.3 |
|  |  |  |  | A4-6 | 70% | 6.7 |

Table 6 Ideal Minimum requirements for PUSCH, 15 MHz Channel Bandwidth, 2Tx

| Number of TX antennas | Number of RX antennas |  | Cyclic prefix | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 8 | Normal |  | EPA 5Hz Low | A3-6 | 70% | -5.7 |
|  |  |  |  |  | A4-7 | 70% | 7.2 |

Table 7 Ideal Minimum requirements for PUSCH, 20 MHz Channel Bandwidth, 2Tx

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | 8 | Normal | EPA 5Hz Low | A3-7 | 70% | -5.2 |
|  |  |  |  | A4-8 | 70% | 7.6 |

# Practical Performance of PUSCH with 2Tx 8Rx

Table 2 to Table 7 give the impairment performance requirements for PUSCH with 2Tx 8Rx under 1.4MHz, 3MHz, 5MHz, 10MHz, 15MHz and 20MHz configurations respectively.

# Table 8 Practical Minimum requirements for PUSCH, 1.4 MHz Channel Bandwidth, 2Tx

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | 8 | Normal | EPA 5Hz Low | A3-2 | 70% | -2.9 |
|  |  |  |  | A4-3 | 70% | 8.5 |

Table 9 Practical Minimum requirements for PUSCH, 3 MHz Channel Bandwidth, 2Tx

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | 8 | Normal | EPA 5Hz Low | A3-3 | 70% | -3.2 |
|  |  |  |  | A4-4 | 70% | 8.6 |

Table 10 Practical Minimum requirements for PUSCH, 5 MHz Channel Bandwidth, 2Tx

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | 8 | Normal | EPA 5Hz Low | A3-4 | 70% | -3.9 |
|  |  |  |  | A4-5 | 70% | 8.7 |

Table 11 Practical Minimum requirements for PUSCH, 10 MHz Channel Bandwidth, 2Tx

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | 8 | Normal | EPA 5Hz Low | A3-5 | 70% | -3.3 |
|  |  |  |  | A4-6 | 70% | 8.7 |

Table 12 Practical Minimum requirements for PUSCH, 15 MHz Channel Bandwidth, 2Tx

| Number of TX antennas | Number of RX antennas |  | Cyclic prefix | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 8 | Normal |  | EPA 5Hz Low | A3-6 | 70% | -3.7 |
|  |  |  |  |  | A4-7 | 70% | 9.2 |

Table 13 Practical Minimum requirements for PUSCH, 20 MHz Channel Bandwidth, 2Tx

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | 8 | Normal | EPA 5Hz Low | A3-7 | 70% | -3.2 |
|  |  |  |  | A4-8 | 70% | 9.6 |

# Reference

1.	R4-131776, Simulation assumptions for UL 8 antenna receiver. China Telecom, 3GPP TSG-RAN WG4 meeting #66bis, Chicago, IL, US, Apr 15 - 19 , 2013.