3GPP TSG-RAN WG4 Meeting #68  	                                    R4-133835

Barcenola, Spain, 19th – 23rd Aug, 2013

Agenda Item:	8.1.1

Source:	Samsung

Title:	Performance Evaluation of PRACH with 1Tx 8Rx

Document for:	Discussion

# Introduction

In RAN4 #66bis meeting, the simulation assumption for PRACH detection, 1Tx 8Rx have been decided [1]. This contribution presents the ideal and impairment simulation results accordingly.

# Simulation Assumption for PRACH with 1Tx 8Rx

Table 1 provides simulation assumptions for PUSCH, 2Tx 8Rx Table 6~8 provide the simulation assumptions for PRACH detection requirements. The test cases are given by Table 19 and Table 20 in [2].

The false alarm probability shall be less than or equal to 0.1%, and the probability of detection shall be equal to or exceed 99%.

The requirements for Burst format 4 are optional and only valid for base stations supporting TDD. The requirements for high speed mode are only valid for the base stations supporting high speed mode.

As described in TS 36.141[3], the timing offset base value is set to 50% of Ncs. This offset is increased within the loop, by adding in each step a value of 0.1us, until the end of the tested range, which is 0.9us. Then the loop is being reset and the timing offset is set again to 50% of Ncs. The timing offset scheme is presented in Figure 1.

Table 1	Simulation assumptions for PRACH detection, 1Tx 8Rx

| Parameters | Values |
| --- | --- |
| Propagation conditions | AWGN, ETU |
| Antenna configuration | 1x8 |
| Correlation matrix for low correlation |  |
| Noise Model | AWGN |
| Burst format | Normal Mode: format 0, 1, 2, 3, 4 High speed Mode: format 0, 1, 2, 3 |

Table 2	Test preambles for Normal Mode [4]

| Burst format | Ncs | Logical sequence index | v |
| --- | --- | --- | --- |
| 0 | 13 | 22 | 32 |
| 1 | 167 | 22 | 2 |
| 2 | 167 | 22 | 0 |
| 3 | 0 | 22 | 0 |
| 4 | 10 | 0 | 0 |

Table 3	Test preambles for High speed Mode [4]

| Burst format | Ncs | Logical sequence index | v |
| --- | --- | --- | --- |
| 0 | 15 | 384 | 0 |
| 1 | 202 | 384 | 0 |
| 2 | 202 | 384 | 0 |
| 3 | 237 | 384 | 0 |

Figure 1 Timing offset scheme [3]

# Ideal Performance of PRACH with 1Tx 8Rx

Table 4 provides the ideal results for 8Rx PRACH detection performance requirement, which are for normal mode.

Table 4 Ideal PRACH missed detection requirements for Normal Mode

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Frequency offset | SNR [5] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 | Burst format 1 | Burst format 2 | Burst format 3 | Burst format 4 |
| 1 | 8 | AWGN | 0 | -20.8 | -20.6 | -22.5 | -22.2 | -13.2 |
|  |  | ETU 70 Low* | 270 Hz | -17.0 | -16.5 | -18.6 | -18.4 | -9.7 |
| Note*: Not applicable for Local Area BS and Home BS. |  |  |  |  |  |  |  |  |

Table 5 provides the ideal results for 8Rx PRACH detection performance requirements, which are for high speed mode.

Table 5 Ideal PRACH missed detection requirements for High speed Mode

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Frequency offset | SNR [5] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 | Burst format 1 | Burst format 2 | Burst format 3 |
| 1 | 8 | AWGN | 0 | -20.4 | -20.2 | -22.4 | -22.3 |
|  |  | ETU 70 Low | 270 Hz | -16.4 | -16.0 | -18.3 | -18.2 |
|  |  | AWGN | 625 Hz | -18.6 | -17.7 | -19.9 | -19.9 |
|  |  | AWGN | 1340 Hz | -20.6 | -20.1 | -22.5 | -22.4 |

# Practical Performance of PRACH with 1Tx 8Rx

Table 6 provides the impairment results for 8Rx PRACH detection performance requirement, which are for normal mode.

Table 6 Practical PRACH missed detection requirements for Normal Mode

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Frequency offset | SNR [5] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 | Burst format 1 | Burst format 2 | Burst format 3 | Burst format 4 |
| 1 | 8 | AWGN | 0 | -18.8 | -18.6 | -20.5 | -20.2 | -11.2 |
|  |  | ETU 70 Low* | 270 Hz | -15.0 | -14.5 | -16.6 | -16.4 | -7.7 |
| Note*: Not applicable for Local Area BS and Home BS. |  |  |  |  |  |  |  |  |

Table 7 provides the impairment results for 8Rx PRACH detection performance requirements, which are for high speed mode.

Table 7 Practical PRACH missed detection requirements for High speed Mode

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Frequency offset | SNR [5] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 | Burst format 1 | Burst format 2 | Burst format 3 |
| 1 | 8 | AWGN | 0 | -18.4 | -18.2 | -20.4 | -20.3 |
|  |  | ETU 70 Low | 270 Hz | -14.4 | -14.0 | -16.3 | -16.2 |
|  |  | AWGN | 625 Hz | -16.6 | -15.7 | -17.9 | -17.9 |
|  |  | AWGN | 1340 Hz | -18.6 | -18.1 | -20.5 | -20.4 |

# Reference

1.	R4-131776, Simulation assumptions for UL 8 antenna receiver. China Telecom, 3GPP TSG-RAN WG4 meeting #66bis, Chicago, IL, US, Apr 15 - 19 , 2013.

2.	R4-130851, Test case List of 8Rx BS demodulation performance. Huawei, Nokia Siemens Networks, China Telecom, ZTE, Alcatel-Lucent, Ericsson. 3GPP TSG-RAN WG4 meeting #66, St Julian's, Malta., 28th January – 1st February 2013.

3.	3GPP TS 36.141 V11.3.0(2012-12): 3rd Generation Partnership Project;Technical Specification Group Radio Access Network;Evolved Universal Terrestrial Radio Access (E-UTRA);  Evolved Universal Terrestrial Radio Access (E-UTRA): Base Station (BS) conformance testing (Release 11).

4.	3GPP TS 36.104 V10.4.0 (2011-09): 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception  (Release 10)

5.	R. Tian, B.K.L., J. Medbo. Impact of Rician fading and cross-polarization ratio on the orthogonality of dual-polarized wireless channels. in European Conference on Antennas and Propagation (EuCAP), Prague, Czech Republic. 2012.