3GPP TSG-RAN4 Meeting #68	 R4-133874

# Barcelona, Spain, 19-23 August, 2013

Source: 	SONY Mobile Communications Japan, Inc

Title: 	TRP/TRS relation to conducted RF performance at Carrier Aggregation.

Agenda Item: 	8.3.1

Document for:	Discussion

# 1.		Introduction R4-125719

This document describes the relationship between radiated performance parameters TRP/TRS to radio conducted transmitter output power and receiver sensitivity. Since previous discussions, [ref 1], suggests specification relaxation of conducted RF-parameters when means for inter and intra band CA is introduced into the RF-circuits of mobile devices but without discussing corresponding compensation of TRP/TRS specification limits it is here highlighted the need for such a discussion.

2.		Relation of TRP and TRS to conducted output power and receiver sensitivity respectively.

Total Radiated Power is by definition the total emitted power into a full sphere surrounding the device under test. Mathematically the antenna efficiency multiplied by the feed power accepted by the antenna constitutes TRP. Similarly Total Radiated Sensitivity is defined as antenna efficiency multiplied by the sensitivity of the receiver. See [ref 2] for definitions and a description on how these parameters are measured. Hence there is a linear relation between TRP and TRS to conducted power and sensitivity respectively.

_________________________________________________________________________________

3.		Proposal.

The conducted RF performance will suffer from the introduction of required additional circuit components in typical mobile device implementations when inter and intra band Carrier Aggregation is introduced. Then as a corresponding consequence the TRP and TRS also will suffer. It is therefore suggested that any amount of relaxation considered for conducted RF performance in specification [ref 1] must also be reflected in the E-UTRA sections 6.1, 6.2, 6.3, 7.1, 7.2, and 7.3 of coming releases of the specification 3GPP TS 37.144 [ref 3].

_________________________________________________________________________________________

4.	References

[1]	R4-131686

[2]	Fujimoto H. Mobile Antenna Systems Handbook, 3:rd ed. sec. 5.2.1.2 and 5.2.5.4

ISBN-13-978-1-59693-126-8

[3]	3GPP TS 37.144