3GPP TSG-RAN WG4 #68	R4-133990

Barcelona, Spain, 19th – 23th, Aug 2013

Source: 	Ericsson

Title:	TR 36.863 Vx.0.2: Draft TP on link level assumptions for CRS-IM performance in homogeneous network

Agenda Item:	10.7

Document for:	Approval

# Discussion

In RAN4#67, link level simulation assumptions are agreed in [1]. In this contribution, we propose the draft TP for the link level simulation assumptions for CRS-IM performance in homogeneous network.

# Reference

[1]	R4-133130, “Link level interference modelling and simulation assumptions”, Ericsson, RAN4#67

# Text Proposal

< Start of draft text proposal for TR36.863 (TR for CRS-IM)>

<Start of the first change>

# 7	Link performance characterization

## 7.1	General

The purpose of this clause is to analyse the performance gain from link-level simulation results. Emphasis is on results that were based on agreed set of simulation assumption. This clause introduces an evaluation framework for link-level simulations. As described in clause 6, interference modelling was performed by interested companies in order to reach an agreement of link-level simulation assumptions.

## 7.2	Detail of simulation assumption for link-level simulation

The link-level assumptions are summarized in Table 7.2-1.

Table 7.2-1: Link level simulation assumptions for CRS-IM investigations in homogeneous deployments

| Parameter |  |  |  | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Carrier frequency |  |  |  | 2 GHz |  |  |  |  |  |
| System bandwidth |  |  |  | 10 MHz |  |  |  |  |  |
| Duplex mode |  |  |  | FDD |  |  |  |  |  |
| Transmission mode in serving cell |  |  |  | Option A (baseline): TM2 Option B (interested companies): TM4 |  |  |  |  |  |
| Transmission mode in interfering cells |  |  |  | Use TM3 for TM2 serving, and use TM4 for TM4 serving |  |  |  |  |  |
| MIMO configuration |  |  |  | 2x2, low correlation |  |  |  |  |  |
| Channel model and Doppler frequency for target and interfering cells |  |  |  | EVA5 |  |  |  |  |  |
|  |  |  |  | Use different channel seed for between cells |  |  |  |  |  |
| Number of explicitly modeled interfering cells |  |  |  | Option A (baseline): 2 interfering cells Option B (interested companies): 3 or more interfering cells are investigated in order to check whether 2 cells model sufficiently well non-full-buffer interference |  |  |  |  |  |
| Signal level for serving cell CRS (Es/Noc) |  |  |  | Range of Es/Noc: TBD |  |  |  |  |  |
| Signal level for interfering cells CRS (interference over Noc) |  |  |  | 1st interfering cell |  |  | 2nd interfering cell |  | 3rd, 4th etc interferers (option B) |
|  |  |  |  | INR1=TBD |  |  | INR2=TBD |  | INR3, INR4 = TBD |
| Other cells interference |  |  |  | AWGN with 1 Noc level |  |  |  |  |  |
| Network synchronization in time |  |  |  | All cells are synchronous |  |  |  |  |  |
|  |  |  |  | Time-delay wrt. serving cell |  |  |  |  |  |
|  |  |  |  | 1st interfering cell |  | 2nd interfering cell |  |  | 3rd, 4th interfering cells (option B) |
|  |  |  |  | [3 us] |  | [-1 us] |  |  | FFS |
| Network synchronization in frequency |  |  |  | Frequency shift wrt. serving cell |  |  |  |  |  |
|  |  |  |  | 1st interfering cell | 2nd interfering cell |  |  |  | 3rd, 4th interfering cells (option B) |
|  |  |  |  | 300 Hz | -100 Hz |  |  |  | FFS |
| CRS configuration |  |  |  | 2 CRS ports per cell with planning, non-colliding CRS between explicitly modeled serving and the first two interfering cells |  |  |  |  |  |
| Downlink power allocation (cf. Chapter 8 of TS36.101) | A |  |  | -3 dB in all modeled cells |  |  |  |  |  |
|  | B |  |  | -3 dB in all modeled cells (PB=1) |  |  |  |  |  |
|  |  |  |  | 0 dB in all modeled cells |  |  |  |  |  |
| CSI reference signals |  |  |  | N/A |  |  |  |  |  |
| CSI-RS periodicity and subframe offset (TCSI-RS / ICSI-RS) |  |  |  | N/A |  |  |  |  |  |
| CSI reference signal configuration |  |  |  | N/A |  |  |  |  |  |
| Subframes for demodulation |  |  |  | All subframes scheduled for demodulation except subframe #5 |  |  |  |  |  |
| HARQ |  |  |  | 8 HARQ processes and max 4 transmissions |  |  |  |  |  |
| Feedback mode |  |  |  | PUCCH 1-0 for TM2 and PUCCH 1-1 for TM4 |  |  |  |  |  |
| Feedback periodicity & delay for target signal |  |  |  | Feedback periodicity |  |  |  | Feedback delay |  |
|  |  |  |  | 5 milliseconds |  |  |  | 8 milliseconds |  |
| Channel and interference estimation at UE |  |  |  | Practical and realizable channel and interference covariance estimates with no a-priori knowledge of the channel state information |  |  |  |  |  |
| Time/frequency tracking at the UE |  |  |  | Practical algorithms should be used |  |  |  |  |  |
| Physical channels transmitted in serving cell |  |  |  | PSS/SSS/PBCH |  |  |  |  |  |
| PCFICH |  |  |  | CFI = 2 in all cells |  |  |  |  |  |
| PCFICH/PDCCH detection |  |  |  | Not considered |  |  |  |  |  |
| Physical channels transmitted in interfering cells |  |  |  | PDCCH PDSCH PSS/SSS/PBCH |  |  |  |  |  |
| Desired PDSCH parameterization |  | Resource allocation |  | 50 PRB |  |  |  |  |  |
|  |  | Rank |  | Rank-1 |  |  |  |  |  |
|  |  | PMI |  | TM2: N/A |  |  |  | TM4: Follow wideband PMI |  |
|  |  | Modulation |  | Option 1 (baseline): Inner- and outer-link adaptation targeting 10% BLER for the 1st transmission Option 2 (interested companies): Fixed MCS (MCS set TBD) (Note : The baseline assumption does not preclude the use of fixed MCS for any future performance requirement which may be developed for homogeneous CRS-IM) |  |  |  |  |  |
|  |  | Code rate |  |  |  |  |  |  |  |
|  |  | Channel coding, rate matching |  | As specified in TS36.212 |  |  |  |  |  |
|  |  | CRC |  |  |  |  |  |  |  |
| Interfering PDSCH parameterization |  | Resource allocation |  | Random full band (50PRB) on/off model, proportional to the average resource utilization in the interfering cells; ON/OFF pattern depends on the Possion distribution |  |  |  |  |  |
|  |  | Rank |  | Randomly changing rank per allocated subband from subframe to subframe: 80% rank-1, 20% rank-2 |  |  |  |  |  |
|  |  | PMI |  | TM3: N/A |  |  |  | TM4: Random PMI per allocated subband |  |
|  |  | Modulation |  | Randomly modulated 16QAM symbols over allocated interfering resources |  |  |  |  |  |
|  |  | Code rate |  | - |  |  |  |  |  |
|  |  | Channel coding, rate matching |  | - |  |  |  |  |  |
|  |  | CRC |  |  |  |  |  |  |  |
| Non-full buffer interference |  | Model |  | Interfering PDSCH transmissions in interfering cells are randomly & independently active over the full band with an activity in time domain equal on average to the targeted resource utilization |  |  |  |  |  |
|  |  | Average resource utilization |  | {0%, 10%, 20%, 30%, 40%, 50%} |  |  |  |  |  |
| Tx EVM |  |  |  | 6% in both alignment and impairment simulations |  |  |  |  |  |
| Noc at antenna port |  |  |  | [-98 dBm] |  |  |  |  |  |
| Cyclic prefix |  |  |  | Normal |  |  |  |  |  |
| Simulation length |  |  |  | 10000 sub-frames at minimum |  |  |  |  |  |
| Simulation output |  |  |  | PDSCH throughput vs. serving cell Es/Noc |  |  |  |  |  |
| UE receiver |  |  | Detector | MMSE-IRC with CRS based interference covariance estimation as defined in TR36.829 |  |  |  |  |  |
|  |  |  | CRS-IM | Without CRS-IM |  |  |  |  |  |
|  |  |  |  | With CRS-IM for the 1st strongest interfering cell |  |  |  |  |  |
|  |  |  |  | With CRS-IM for the 1st and 2nd strongest interfering cells |  |  |  |  |  |

<End of the first change>