3GPP TSG-RAN WG4 Meeting #68  	                                    R4-133835

Barcenola, Spain, 19th – 23rd Aug, 2013

Agenda Item:	8.1.1

Source:	Samsung

Title:	Performance Evaluation of PUCCH for Phase-II Cases

Document for:	Discussion

# Introduction

In RAN4 #66bis meeting, the simulation assumption for PUCCH format 1b with channel selection, PUCCH format 3 ACK missed detection, PUCCH format 3 NAK to ACK under 1Tx 8Rx and PUCCH format under 2Tx 8Rx 1a have been decided [1]. This contribution presents the ideal and impairment simulation results accordingly.

# PUCCH Ideal Performance Requirements

## Ideal ACK missed detection requirements for single user PUCCH format 1a

Table 1 provides the ideal simulation result for 2Tx 8Rx PUCCH format 1a.

Table 1 Ideal Minimum requirements for single user PUCCH format 1a, 2Tx

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex B) | Channel Bandwidth / SNR [2] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| 2 | 8 | Normal | EPA 5 Low | -12.8 | -13.0 | -13.2 | -13.3 | -13.3 | -13.3 |
|  |  |  | EVA 70 Low | -13.0 | -13.2 | -13.3 | -13.2 | -13.2 | -13.1 |

## Ideal ACK missed detection requirements for PUCCH format 1b with channel selection

Table 2 provides the ideal simulation result for 8Rx PUCCH format 1b with channel selection.

Table 2 Ideal Minimum requirements for PUCCH format 1b with Channel Selection

| Number of Tx antennas | Number of RX antennas | Cyclic Prefix | Propagation Conditions and correlation matrix (Annex B) | Channel Bandwidth / SNR [2] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 10 MHz | 15MHz | 20MHz |
| 1 | 8 | Normal | EPA 5 Low | -13.3 | -13.3 | -13.3 |
|  |  |  | EVA70 Low | -13.1 | -13.2 | -13.2 |

## Ideal Performance for PUCCH Format 3

### Ideal ACK missed detection requirements for PUCCH format 3

Table 3 and Table 4 provide the ideal simulation result for 8Rx PUCCH format 3 ACK missed detection performance.

Table 3 Ideal Minimum requirements for PUCCH format 3, 4AN bits

| Number of Tx antennas | Number of RX antennas | Cyclic Prefix | Propagation Conditions and correlation matrix (Annex B) | Channel Bandwidth / SNR [2] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 10 MHz | 15MHz | 20MHz |
| 1 | 8 | Normal | EPA 5 Low | -12.1 | -12.1 | -12.1 |
|  |  |  | EVA70 Low | -12.1 | -12.1 | -12.0 |

Table 4 Ideal Minimum requirements for PUCCH format 3, 16AN bits

| Number of Tx antennas | Number of RX antennas | Cyclic Prefix | Propagation Conditions and correlation matrix (Annex B) | Channel Bandwidth / SNR [2] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 10 MHz | 15MHz | 20MHz |
| 1 | 8 | Normal | EPA 5 Low | -9.8 | -10.0 | -9.9 |
|  |  |  | EVA70 Low | -9.8 | -9.9 | -9.7 |

### Ideal NAK to ACK requirements for PUCCH format 3

Table 5 provides the ideal simulation result for 8Rx PUCCH format 3 NAK to ACK performance.

Table 5 Ideal Minimum requirements for PUCCH format 3, 16AN bits

| Number of Tx antennas | Number of RX antennas | Cyclic Prefix | Propagation Conditions and correlation matrix (Annex B) | Channel Bandwidth / SNR [2] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 10 MHz | 15MHz | 20MHz |
| 1 | 8 | Normal | EPA 5 Low | -8.1 | -8.2 | -8.1 |
|  |  |  | EVA70 Low | -8.1 | -8.1 | -8.0 |

# PUCCH Impairment Performance Requirements

## Practical ACK missed detection requirements for single user PUCCH format 1a

Table 6 provides the impairment result for 2Tx 8Rx PUCCH format 1a.

Table 6 Practical Minimum requirements for single user PUCCH format 1a, 2Tx

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex B) | Channel Bandwidth / SNR [2] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| 2 | 8 | Normal | EPA 5 Low | -10.8 | -11.0 | -11.2 | -11.3 | -11.3 | -11.3 |
|  |  |  | EVA 70 Low | -11.0 | -11.2 | -11.3 | -11.2 | -11.2 | -11.1 |

## Practical ACK missed detection requirements for PUCCH format 1b with channel selection

Table 7 provides the impairment result for 8Rx PUCCH format 1b with channel selection.

Table 7 Practical Minimum requirements for PUCCH format 1b with Channel Selection

| Number of Tx antennas | Number of RX antennas | Cyclic Prefix | Propagation Conditions and correlation matrix (Annex B) | Channel Bandwidth / SNR [2] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 10 MHz | 15MHz | 20MHz |
| 1 | 8 | Normal | EPA 5 Low | -11.3 | -11.3 | -11.3 |
|  |  |  | EVA70 Low | -11.1 | -11.2 | -11.2 |

## Practical Performance for PUCCH Format 3

### Practical ACK missed detection requirements for PUCCH format 3

Table 8 and Table 9 provide the impairment result for 8Rx PUCCH format 3 ACK missed detection performance.

Table 8 Practical Minimum requirements for PUCCH format 3, 4AN bits

| Number of Tx antennas | Number of RX antennas | Cyclic Prefix | Propagation Conditions and correlation matrix (Annex B) | Channel Bandwidth / SNR [2] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 10 MHz | 15MHz | 20MHz |
| 1 | 8 | Normal | EPA 5 Low | -10.1 | -10.1 | -10.1 |
|  |  |  | EVA70 Low | -10.1 | -10.1 | -10.0 |

Table 9 Practical Minimum requirements for PUCCH format 3, 16AN bits

| Number of Tx antennas | Number of RX antennas | Cyclic Prefix | Propagation Conditions and correlation matrix (Annex B) | Channel Bandwidth / SNR [2] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 10 MHz | 15MHz | 20MHz |
| 1 | 8 | Normal | EPA 5 Low | -7.8 | -8.0 | -7.9 |
|  |  |  | EVA70 Low | -7.8 | -7.9 | -7.7 |

### Practical NAK to ACK requirements for PUCCH format 3

Table 10 provides the impairment result for 8Rx PUCCH format 3 NAK to ACK performance.

Table 10 Practical Minimum requirements for PUCCH format 3, 16AN bits

| Number of Tx antennas | Number of RX antennas | Cyclic Prefix | Propagation Conditions and correlation matrix (Annex B) | Channel Bandwidth / SNR[2] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 10 MHz | 15MHz | 20MHz |
| 1 | 8 | Normal | EPA 5 Low | -6.1 | -6.2 | -6.1 |
|  |  |  | EVA70 Low | -6.1 | -6.1 | -6.0 |

# Reference

1.	R4-131776, Simulation assumptions for UL 8 antenna receiver. China Telecom, 3GPP TSG-RAN WG4 meeting #66bis, Chicago, IL, US, Apr 15 - 19 , 2013.