3GPP TSG-RAN WG4 #67	R4-133996

Barcelona, Spain, 19th – 23th, Aug 2013

Source: 	Ericsson

Title:	TR 36.863 Vx.0.2: Draft TP for interference modelling of CRS-IM performance in homogeneous network

Agenda Item:	10.7

Document for:	Approval

# Discussion

In RAN4#66bis, the method to derive side condition for link level is agreed, and the method is further refined in RAN4#67 [1]. In this contribution, we propose the draft TP to capture the current agreements on the side condition determination methodology.

# Reference

[1]	R4-133114, WF on CRS-IM in homogeneous network, Ericsson, ST Ericsson, Renesas, Huawei, RAN4#67

# Text Proposal

< Start of draft text proposal for TR36.863 (TR for CRS-IM)>

<Start of the first change>

6	Interference modelling

6.1	General

In this clause, we define the interference models/profiles that we developed in order to assess the link level performance of CRS-IM receivers. Clause 6.2 defines a number of statistical measures that were defined during the study, and which provide useful insight into understanding the complex interference environment. System level simulations were conducted to generate results for the statistical measures defined in clause 6.2. Based on these simulation results interference profiles were developed, which were used in the link level performance and contributed to account performance characterization described in clause 8.

6.2 Statistical measurements

Network interference statistics are computed using the following defined measures. Noc is defined as

Noc=M+1NBSIorj+σ2,

where Iorj is the received power spectral density of the j-th strongest base station for serving cell CRS REs (average power obtained within the RE and normalized to the subcarrier spacing). Specifically,  Ior1 denotes the received power spectral density of serving cell, Iorj(j=2,⋯M) denotes the received power spectral density of the jth dominant cells. is the power spectral density of thermal noise (average power per RE normalised), and NBS is the total number of base stations considered including the serving cell, M is the number of dominant interfere cell(s) which can be mitigated by UE. Noc models the interference from all cells that don’t include the mitigated dominant interfere cell(s).

The interference profiles for the dominant cells are defined by IorjNoc (j=2, ⋯,M), and the serving cell signal profiles are characterized by  Ior1Noc.

Geometry G is defined as

G=Ior1Ioc=Ior1j=2j=MIorj+Noc

Here, all the interfering BSs are assumed with full load.

6.2.1 Methodology for statistical measurements

This clause led to the development of a method for calculatingIorjNoc (j=1, ⋯,M).  IorjNoc (j=1, ⋯,M) are derived based on the following steps:

- Step 1: Decide envisioned G value (G) based on the specified percentile of G distribution.

- Step 2: Select UEs. From a sample of randomly dropped UEs, select those with G close to G with a tolerance of +/- 0.2 dB. IorjNoc (j=1, ⋯,M) values are logged for those UEs, and multiple realizations are performed in order to obtain a significant number of samples.

- Step 3: Decide IorjNoc (j=1, ⋯,M): After saving the condition IorjNoc (j=1, ⋯,M) values from all samples, the Ior2 values are sorted in ascending order, after this, the data set is binned in 5-percentile bands, a mean of IorjNoc (j=1, ⋯,M)  inside the 5-percentile band is taken, yielding one characteristic set of  IorjNoc (j=1, ⋯,M) values per each 5-percentile. At the end of the process, 20 characteristic set of  IorjNoc (j=1, ⋯,M) values are obtained.

- Note 1: the “mean” is performed in the linear domain.

To account for the partial loading on the non-dominant interferers, two methods are developed to decide the Noc.  In the first method, Noc is the Noc averaged across time with target resource utilization assume partial loading and no CRS-IM is assumed. In the second method, Noc is obtained based on

Noc=M+1NBSαIorj+j∈BS with the same CRS shift as serving cellIorj+σ2,

where α is the resource utilization.

<End of the first change>

< End of draft text proposal for TR36.863 (TR for CRS-IM)>