3GPP TSG-RAN WG4 #68	R1-134002

Barcelona, Spain, 19th – 23th, Aug 2013

Source: 	Ericsson, ST-Ericsson

Title:	Preliminary link level simulation results and discussion for RI test

Agenda Item:	7.8.2.2

Document for:	Discussion

# Introduction

In RAN4#66bis meeting, one WF paper on FeICIC CSI tests is agreed in [2].  In RAN4#67, the WF is refined in [1]. For RI test, the group agrees to investigate:

- Test methods:

  - Introduce Test-1 and Test-2

  - Whether to introduce Test-3 is FFS.

In this paper, we provide link level simulation results for Test-1 and Test 2.

# Preliminary link level simulation for RI test

In the link level simulation, simulation parameters are shown in Table 1 of Appendix A. In eICIC, the minimum performance requirement is defined as

a)   The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 1 shall be  1

In Figure 1, both γ1 and γ2 are provided for 2x2 low correlation channels.  From Figure 1, we can see that the current requirement for eICIC is met with FeICIC setup. It seems reasonable to reuse eICIC requirement for FeICIC.

Figure 1: γ1 and γ2 value for 2x2 low correlation

# Conclusion

In this paper, link level simulation for RI test 1 and test 2 are provided. We hope the group can consider these results for RI requirements definition.

# Appendix A

Table 1: RI Test (FDD)

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2&Cell3 |  |
| Bandwidth |  | MHz | 10 |  |  |
| PDSCH transmission mode |  |  | 3 | Note 10 |  |
| Downlink power allocation |  | dB | -3 |  |  |
|  |  | dB | -3 |  |  |
|  |  | dB | 0 |  |  |
| Propagation condition and antenna configuration |  |  | 2 x 2 EPA5 |  |  |
| CodeBookSubsetRestriction bitmap |  |  | 01 for fixed RI = 1 10 for fixed RI = 2 11 for UE reported RI | N/A |  |
| Antenna correlation |  |  | Low |  |  |
| RI configuration |  |  | Fixed RI=1 and follow RI | N/A |  |
|  |  | dB | 0 | 12 dB for cell 2 10 dB for cell 3 |  |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN |  |
| Cell Id |  |  | 0 | 126 for cell 2 1 for cell 3 |  |
| Time Offset between Cells |  | s | 3 us for cell 2 -1 us for cell 3 |  |  |
| Frequency offset |  | Hz | 300 Hz for cell 2 -100 Hz for cell 3 |  |  |
| ABS Pattern (Note 6) |  |  | N/A | 10000000 10000000 10000000 10000000 10000000 |  |
| RLM/RRM Measurement Subframe Pattern (Note 7) |  |  | 10000000 10000000 10000000 10000000 10000000 | N/A |  |
| CSI Subframe Sets (Note 8) | CCSI,0 |  | 10000000 10000000 10000000 10000000 10000000 | N/A |  |
|  | CCSI,1 |  | 01111111 01111111 01111111 01111111 01111111 |  |  |
| Number of control OFDM Symbols |  |  | 2 | 2 |  |
| Maximum number of HARQ transmissions |  |  | 1 |  |  |
| Reporting mode |  |  | PUCCH 1-0 |  |  |
| Physical channel for CQI reporting |  |  | PUCCH Format 2 |  |  |
| PUCCH Report Type for CQI |  |  | 4 |  |  |
| Physical channel for RI reporting |  |  | PUCCH Format 2 |  |  |
| PUCCH Report Type for RI |  |  | 3 |  |  |
| Reporting periodicity |  | ms | Npd= 10 |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 11 |  |  |
| ri-ConfigurationInd |  |  | 5 |  |  |
| cqi-pmi-ConfigurationIndex2 |  |  | 10 |  |  |
| ri-ConfigurationInd2 |  |  | 2 |  |  |
| Cyclic prefix |  |  | Normal |  | Normal |

# Reference

R4-133037, WF on FeICIC CSI tests, Qualcomm, May 2013

R4-131898, WF on FeICIC demod and CSI, Qualcomm, Huawei, HiSilicon, NEC, May 2013