3GPP TSG-RAN WG4 #68bis	R1-135323

Riga, Latvia, 7th – 11th, Oct. 2013

Source: 	Ericsson

Title:	TR 36.863 V0.3.1: Draft TP on final summary for CRS-IM performance in homogeneous network

Agenda Item:	10.5

Document for:	Approval

# Discussion

In RAN4#68 meeting, link level simulation results are submitted by several interested companies [2]~[7]. These results are summarized in [1]. Additionally, system level results are also proposed in [9] by some interested companies. In last meeting, several TPs have been approved. Based on the approved TPs, available system level and link results, TP on final summary are proposed for this SI in this paper.

# Reference

R4-134481, Summary of link level simulation and side condition for CRS-IM, Ericsson, ST Ericsson, Noted

R4-133997, Link level results for CRS-IM, Ericsson, ST Ericsson, Noted

R4-133144, CRS IC Performance in Homogeneous Network, NEC, Noted

R4-133263, Link level simulation results for CRS-IM in homogeneous network, Huawei, HiSilicon, Noted

R4-133306, Preliminary link level simulaion results, ZTE, Noted

R4-133468, Link-level simulation results of CRS-IM, NSN, Nokia Corporation, Noted

R4-133853, Performance evaluation of CRS-IC and considerations on interference modelling, Renesas Mobile Europe Ltd, Noted

R4-133935, Link level performance  of CRS-IC for Homogeneous network, LG Electronics, Noted

R4-133998, Summary of system level simulation and side condition for CRS-IM, Ericsson, ST Ericsson, Noted

# Text Proposal

# <Start of the first change>

# 9	Summary

In this technical report we have documented the work that was accomplished by RAN4 as part of the feasibility study on CRS interference mitigation for LTE homogeneous network. Receiver structures based on MMSE-IRC detector combined with a CRS-IM receiver were defined. This type of receiver attempts to mitigate the CRS interference along with the data/control interference that arises from users served by neighbouring cells. Interference models/profiles were developed for this interference in terms of the number of dominant interfering cells, and their powers relative to the total other cell interference power excluding the considered dominant interference cells. This ratio is referred to as  IorjNoc . For the purpose of this study it was determined that two dominant interfering cells should be taken into account in the interference models. The interference profiles were defined for a wide range of operating conditions when the interfering cells have low traffic loading. Throughput estimates were then developed using link level simulations, which included the models for the serving and interfering cells based on the results of interference modelling. Based on the link simulation results based on weighted IorjNoc , approximately 8 -102 % gain for CRS based transmission (serving cell: TM2) was observed when the average resource utilization of aggressor cells was 10% for cell-edge UEs.  For the conditional median IorjNoc , the gain was about 40% with the same resource utilization. When the average resource utilization of aggressor cells is 50%, approximately 4%-59% gain can be achieved with the same transmission configuration. For the conditional median  IorjNoc , the gain was about 16% with the same resource utilization. System level results were also provided. The system study concluded that there is indeed an increase in user perceived throughput (UPT) when the traffic load is low at the interfering cells. With regard to UE implementation issues, it was felt that the receiver is based upon well-defined receivers used for meeting existing MMSE-IRC and FeICIC performance requirements. The reuse of these receivers’ structure minimizes the UE complexity.

Based on the observations cited above, RAN4 has concluded that significant throughput gain can be obtained for cell-edge UE by CRS-IM receiver in LTE homogeneous network under low traffic loading.

<End of the third change>