3GPP TSG-RAN WG4 Meeting #68bis 	R4-135528

Riga, Latvia, October 7-11, 2013

| CR-Form-v10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 1946 | rev |  | Current version: | 11.6.0 |  |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR Reducing MPR for CA with Non-Contiguous Resource Allocations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Motorola Mobility |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA-Core |  |  |  |  |  |  | Date: |  | 2013-09-30 |
| Category: | C |  |  |  |  |  |  | Release: |  | Rel-11 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change: |  | The allowed MPR for two non-contiguous resource allocations is larger than necessary to meet emissions requirements for some smaller allocations for which the fifth harmonics do not reach the the outermost 5 MHz of the SEM. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following changes are made - In Section 3.2, notation is introduced to denote the lowest and highest frequencies of the non-contiguous resource allocation. Notation is also introduced to denote the center frequency of the aggregated channel. - In Section 6.2.3, the MPR is limited to the minimum of MA and 4.5 dB for non-contiguous resource allocations for which the fifth-order IM’sdo not reach the outermost 5 MHz of the SEM. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The allowed MPR for contiguous CA non-contiguous resource allocations will be larger than necessary to meet emissions requirements for some smaller allocations for which the fifth-order do not reach the outermost 5 MHz of the SEM. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 6.2.3A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  | TS 36.521-1 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## << Start of changes >>

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

BWChannel	Channel bandwidth

BWChannel,block 	Sub-block bandwidth, expressed in MHz. BWChannel,block= Fedge,block,high- Fedge,block,low.

BWChannel_CA 	Aggregated channel bandwidth, expressed in MHz.

BWGB	Virtual guard band to facilitate transmitter (receiver) filtering above / below edge CCs.

Transmitted energy per RE for reference symbols during the useful part of the symbol, i.e. excluding the cyclic prefix, (average power normalized to the subcarrier spacing) at the eNode B transmit antenna connector

The averaged received energy per RE of the wanted signal during the useful part of the symbol, i.e. excluding the cyclic prefix, at the UE antenna connector; average power is computed within a set of REs used for the transmission of physical channels (including user specific RSs when present), divided by the number of REs within the set, and normalized to the subcarrier spacing

F					Frequency

Fagg_alloc_low	Aggregated Transmission Bandwidth Configuration. The lowest frequency of the simultaneously transmitted resource blocks.

Fagg_alloc_high	Aggregated Transmission Bandwidth Configuration. The highest frequency of the simultaneously transmitted resource blocks.

FInterferer (offset)	Frequency offset of the interferer

FInterferer			Frequency of the interferer

FC	Frequency of the carrier centre frequency

FC_agg	Aggregated Transmission Bandwidth Configuration.  The center frequency of the aggregated carriers.

FC,block, high		Center frequency of the highest transmitted/received carrier in a sub-block.

FC,block, low		Center frequency of the lowest transmitted/received carrier in a sub-block.

FCA_low 	The centre frequency of the lowest carrier, expressed in MHz.

FCA_high 	The centre frequency of the highest carrier, expressed in MHz.

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

Fedge,block,low 	The lower sub-block edge, where Fedge,block,low = FC,block,low - Foffset.

Fedge,block,high 	The upper sub-block edge, where Fedge,block,high = FC,block,high + Foffset.

Fedge_low 	The lower edge of aggregated channel bandwidth, expressed in MHz.

Fedge_high 	The higher edge of aggregated channel bandwidth, expressed in MHz.

Foffset 	Frequency offset from FC_high to the higher edge or FC_low to the lower edge.

Foffset,block,low	Separation between lower edge of a sub-block and the center of the lowest component carrier within the sub-block

Foffset,block,high	Separation between higher edge of a sub-block and the center of the highest component carrier within the sub-block

FOOB	The boundary between the E-UTRA out of band emission and spurious emission domains.

The power spectral density of the total input signal (power averaged over the useful part of the symbols within the transmission bandwidth configuration, divided by the total number of RE for this configuration and normalised to the subcarrier spacing) at the UE antenna connector, including the own-cell downlink signal

The total transmitted power spectral density of the own-cell downlink signal (power averaged over the useful part of the symbols within the transmission bandwidth configuration, divided by the total number of RE for this configuration and normalised to the subcarrier spacing) at the eNode B transmit antenna connector

The total received power spectral density of the own-cell downlink signal (power averaged over the useful part of the symbols within the transmission bandwidth configuration, divided by the total number of RE for this configuration and normalised to the subcarrier spacing) at the UE antenna connector

The received power spectral density of the total noise and interference for a certain RE (average power obtained within the RE and normalized to the subcarrier spacing) as measured at the UE antenna connector

LCRB	Transmission bandwidth which represents the length of a contiguous resource block allocation expressed in units of resources blocks

Ncp	Cyclic prefix length

NDL 	Downlink EARFCN

The power spectral density of a white noise source (average power per RE normalised to the subcarrier spacing), simulating interference from cells that are not defined in a test procedure, as measured at the UE antenna connector

The power spectral density of a white noise source (average power per RE normalized to the subcarrier spacing), simulating interference in non-CRS symbols in ABS subframe from cells that are not defined in a test procedure, as measured at the UE antenna connector.

The power spectral density of a white noise source (average power per RE normalized to the subcarrier spacing), simulating interference in CRS symbols in ABS subframe from all cells that are not defined in a test procedure, as measured at the UE antenna connector.

The power spectral density of a white noise source (average power per RE normalised to the subcarrier spacing), simulating interference in non-ABS subframe from cells that are not defined in a test procedure, as measured at the UE antenna connector

The power spectral density (average power per RE normalised to the subcarrier spacing) of the summation of the received power spectral densities of the strongest interfering cells explicitly defined in a test procedure plus, as measured at the UE antenna connector. The respective power spectral density of each interfering cell relative to is defined by its associated DIP value.

NOffs-DL 	Offset used for calculating downlink EARFCN

NOffs-UL 	Offset used for calculating uplink EARFCN

The power spectral density of a white noise source (average power per RE normalised to the subcarrier spacing) simulating eNode B transmitter impairments as measured at the eNode B transmit antenna connector

NRB	Transmission bandwidth configuration, expressed in units of resource blocks

NRB_agg	Aggregated Transmission Bandwidth Configuration The number of the aggregated RBs within the fully allocated Aggregated Channel bandwidth.

NRB_alloc	Total number of simultaneously transmitted resource blocks in Channel bandwidth or Aggregated Channel Bandwidth.

NUL 	Uplink EARFCN.

Rav		Minimum average throughput per RB.

PCMAX	The configured maximum UE output power.

PCMAX, c	The configured maximum UE output power for serving cell c.

PEMAX 	Maximum allowed UE output power signalled by higher layers. Same as IE P-Max, defined in [7].

PEMAX, c	Maximum allowed UE output power signalled by higher layers for serving cell c. Same as IE
P-Max, defined in [7].

PInterferer	Modulated mean power of the interferer

PPowerClass	PPowerClass is the nominal UE power (i.e., no tolerance).

PUMAX	The measured configured maximum UE output power.

RBstart 	Indicates the lowest RB index of transmitted resource blocks.

RBend	Indicates the highest RB index of transmitted resource blocks.

ΔfOOB	Δ Frequency of Out Of Band emission.

ΔRIB,c	Allowed reference sensitivity relaxation due to support for inter-band CA operation, for serving cell c.

ΔTIB,c	Allowed maximum configured output power relaxation due to support for inter-band CA operation, for serving cell c.

TC	Allowed operating band edge transmission power relaxation.

TC,c 	Allowed operating band edge transmission power relaxation for serving cell c.

Test specific auxiliary variable used for the purpose of downlink power allocation, defined in Annex C.3.2.

Wgap	Sub-block gap size

## << Unchanged sections omitted >>

### 6.2.3A	UE Maximum Output power for modulation / channel bandwidth for CA

For inter-band carrier aggregation with uplink assigned to one E-UTRA band (Table 5.6A-1), the requirements in subclause 6.2.3 apply.

For intra-band contiguous carrier aggregation the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2.2A-1due to higher order modulation and contiguously aggregated transmit bandwidth configuration (resource blocks) is specified in Table 6.2.3A-1. In case the modulation format is different on different component carriers then the MPR is determined by the rules applied to higher order of those modulations.

Table 6.2.3A-1: Maximum Power Reduction (MPR) for Power Class 3

| Modulation | CA bandwidth Class C |  |  |  | MPR (dB) |
| --- | --- | --- | --- | --- | --- |
|  | 50 RB + 100 RB | 75 RB + 75 RB | 75 RB+100 RB | 100 RB + 100 RB |  |
| QPSK | > 12 and ≤ 50 | > 16 and ≤ 75 | > 16 and ≤ 75 | > 18 and ≤ 100 | ≤ 1 |
| QPSK | > 50 | > 75 | > 75 | > 100 | ≤ 2 |
| 16 QAM | ≤ 12 | ≤ 16 | ≤ 16 | ≤ 18 | ≤ 1 |
| 16 QAM | > 12 and ≤ 50 | > 16 and ≤ 75 | > 16 and ≤ 75 | > 18 and ≤ 100 | ≤ 2 |
| 16 QAM | > 50 | > 75 | > 75 | > 100 | ≤ 3 |

For intra-band contiguous carrier aggregation bandwidth class C with non-contiguous resource allocation, the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2.2A-1 is specified as follows

MPR = CEIL {MA, 0.5}

Where MA is defined as follows

MA = 	8.2						; 0 ≤ A < 0.025

9.2 - 40A 				; 0.025	≤ A < 0.05

8 – 16A				; 0.05	≤ A < 0.25

4.83 – 3.33A			; 0.25 ≤ A ≤ 0.4,

3.83 – 0.83A			; 0.4 ≤ A ≤ 1,

Where

A = NRB_alloc / NRB_agg.

CEIL{MA, 0.5} means rounding upwards to closest 0.5dB, i.e. MPR[3.0, 3.5, 4.0, 4.5, 5.0, 5.5, 6.0, 6.5, 7.0, 7.5, 8.0, 8.5]

If the non-contiguous resource allocation is such that

the allowed Maximum Power Reduction (MPR) for the maximum output power in table 6.2.2A-1, is specified as follows

MPR = CEIL {min(MA, 4.5), 0.5}.

For the UE maximum output power modified by MPR, the power limits specified in subclause 6.2.5A apply.

For intra-band non-contiguous carrier aggregation with one uplink carrier on the PCC, the requirements in subclause 6.2.3 apply.

## << End of changes >>