3GPP TSG-RAN WG4 Meeting #68bis 	R4-135698

# Riga, Latvia, 7 – 11 October 2013

| CR-Form-v10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 1918 | rev | 1 | Current version: | 12.1.0 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | The Pcmax clauses restructured and removal of addition of ΔTc to P-MPR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Motorola Mobility |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA |  |  |  |  |  |  | Date: |  | 2013-10-07 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change: |  | Definitions are repeated unnecessarily in the clause specifying configured maximum output power. The notation of the configured maximum output power per cell and the total configured output power are not aligned with the corresponding notions in 36.213. The relaxations are handled inconsistently for P-MPR. The relaxation ΔTC is added to P-MPR when computing Pcmax, but the relaxation ΔTIB is not. The relaxations should both be added, or neither should be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Some of the changes below were already approved in the mirror CR 1822 against version 12.0.0, but the CR was erroneously based on version 11.5.0 and subsequently not implemented. The same changes as TS 36.101 CR 1822 but based on version 12.1.0. The formulas for Pcmax,c and Pcmax are modified in 6.2.5 and 6.2.5A, respectively, so that the relaxation ΔTC is not added to P-MPR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguous specifications: the notation of the configured maximum output power per cell and the total configured output power would not be aligned with the corresponding notions of 36.213. The relaxations ΔTC and ΔTIB are handled inconsistently with respect to P-MPR in the Pcmax computation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.5, 6.2.5A, 6.2.5B |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< start of changes >

### 6.2.5		Configured transmitted power

The UE is allowed to set its configured maximum output power PCMAX,c for serving cell c. The configured maximum output power PCMAX,c is set within the following bounds:

PCMAX_L,c ≤  PCMAX,c  ≤  PCMAX_H,c

with

PCMAX_L,c = MIN {PEMAX,c – TC,c,  PPowerClass – MAX(MPRc + A-MPRc + ΔTIB,c + TC,c, P-MPRc)}

PCMAX_H,c = MIN {PEMAX,c,  PPowerClass}

where

-		PEMAX,c is the value given by IE P-Max for serving cell c, defined in [7];

-	PPowerClass is the maximum UE power specified in Table 6.2.2-1 without taking into account the tolerance specified in the Table 6.2.2-1;

-	MPRc and A-MPRc for serving cell c are specified in subclause 6.2.3 and subclause 6.2.4, respectively;

-	TIB,c is the additional tolerance for serving cell c as specified in Table 6.2.5-2; TIB,c = 0 dB otherwise;

-	TC,c = 1.5 dB when Note 2 in Table 6.2.2-1 applies;

-	TC,c = 0 dB when Note 2 in Table 6.2.2-1 does not apply.

P-MPRc is the allowed maximum output power reduction for

a)	ensuring compliance with applicable electromagnetic energy absorption requirements and addressing unwanted emissions / self desense requirements in case of simultaneous transmissions on multiple RAT(s) for scenarios not in scope of 3GPP RAN specifications;

b)	ensuring compliance with applicable electromagnetic energy absorption requirements in case of proximity detection is used to address such requirements that require a lower maximum output power.

The UE shall apply P-MPRc for serving cell c only for the above cases. For UE conducted conformance testing P-MPR shall be 0 dB

NOTE 1: P-MPRc was introduced in the PCMAX,c equation such that the UE can report to the eNB the available maximum output transmit power. This information can be used by the eNB for scheduling decisions.

NOTE 2: P-MPRc may impact the maximum uplink performance for the selected UL transmission path.

The measured configured maximum output power PUMAX,c shall be within the following bounds:

PCMAX_L,c  –  MAX{TL, T(PCMAX_L,c)}  ≤  PUMAX,c  ≤  PCMAX_H,c  +  T(PCMAX_H,c)

where T(PCMAX,c) is defined by the tolerance table below and applies to PCMAX_L,c and PCMAX_H,c separately, while TL is the absolute value of the lower tolerance in Table 6.2.2-1 for the applicable operating band.

Table 6.2.5-1: PCMAX,c tolerance

| PCMAX,c (dBm) | Tolerance T(PCMAX,c) (dB) |
| --- | --- |
| 21 ≤ PCMAX,c ≤ 23 | 2.0 |
| 20 ≤ PCMAX,c < 21 | 2.5 |
| 19 ≤ PCMAX,c < 20 | 3.5 |
| 18 ≤ PCMAX,c < 19 | 4.0 |
| 13 ≤ PCMAX,c < 18 | 5.0 |
| 8 ≤ PCMAX,c < 13 | 6.0 |
| -40 ≤ PCMAX,c < 8 | 7.0 |

For the UE which supports inter-band carrier aggregation configurations with uplink assigned to one E-UTRA band the ΔTIB,c is defined for applicable bands in Table 6.2.5-2.

Table 6.2.5-2: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1A-5A | 1 | 0.3 |
|  | 5 | 0.3 |
| CA_1A-8A | 1 | 0.3 |
|  | 8 | 0.3 |
| CA_1A-18A | 1 | 0.3 |
|  | 18 | 0.3 |
| CA_1A-19A | 1 | 0.3 |
|  | 19 | 0.3 |
| CA_1A-21A | 1 | 0.3 |
|  | 21 | 0.3 |
| CA_1A-26A | 1 | 0.3 |
|  | 26 | 0.3 |
| CA_2A-4A | 2 | [0.5] |
|  | 4 | 0.5 |
| CA_2A-5A | 2 | 0.3 |
|  | 5 | 0.3 |
| CA_2A-13A | 2 | 0.3 |
|  | 13 | 0.3 |
| CA_2A-17A | 2 | 0.3 |
|  | 17 | 0.8 |
| CA_2A-29A | 2 | 0.3 |
| CA_3A-5A | 3 | 0.3 |
|  | 5 | 0.3 |
| CA_3A-7A | 3 | 0.5 |
|  | 7 | 0.5 |
| CA_3A-8A | 3 | 0.3 |
|  | 8 | 0.3 |
| CA_3A-19A | 3 | 0.3 |
|  | 19 | 0.3 |
| CA_3A-20A | 3 | 0.3 |
|  | 20 | 0.3 |
| CA_3A-26A | 3 | 0.3 |
|  | 26 | 0.3 |
| CA_3A-28A |  |  |  | 3 | 0.3 |
|  |  |  |  | 28 | 0.3 |
| CA_4A-5A |  |  |  | 4 | 0.3 |
|  |  |  |  | 5 | 0.3 |
| CA_4A-7A |  |  |  | 4 | 0.5 |
|  |  |  |  | 7 | 0.5 |
| CA_4A-12A |  |  |  | 4 | 0.3 |
|  |  |  |  | 12 | 0.8 |
| CA_4A-13A |  |  |  | 4 | 0.3 |
|  |  |  |  | 13 | 0.3 |
| CA_4A-17A |  |  |  | 4 | 0.3 |
|  |  |  |  | 17 | 0.8 |
| CA_4A-29A | 4 | 0.3 |
| CA_5A-12A |  |  |  | 5 | 0.8 |
|  |  |  |  | 12 | 0.4 |
| CA_5A-17A |  |  |  | 5 | 0.8 |
|  |  |  |  | 17 | 0.4 |
| CA_7A-20A |  |  |  | 7 | 0.3 |
|  |  |  |  | 20 | 0.3 |
| CA_8A-20A |  |  |  | 8 | 0.4 |
|  |  |  |  | 20 | 0.4 |
| CA_11A-18A |  |  |  | 11 | 0.3 |
|  |  |  |  | 18 | 0.3 |
| CA_19A-21A |  |  |  | 19 | 0.3 |
|  |  |  |  | 21 | 0.4 |
| CA_23A-29A | 23 | 0.3 |
| NOTE 1: The above additional tolerances are only applicable for the E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations NOTE 2: The above additional tolerances also apply in non-aggregated operation for the supported E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations NOTE 3: In case the UE supports more than one of the above inter-band carrier aggregation configurations and a E-UTRA operating band belongs to more than one inter-band carrier aggregation configurations then: - When the E-UTRA operating band frequency range is ≤ 1GHz, the applicable additional tolerance shall be the average of the tolerances in Table 6.2.5A-3, truncated to one decimal place for that operating band among the supported CA configurations. In case there is a harmonic relation between low band UL and high band DL, then the maximum tolerance among the different supported carrier aggregation configurations involving such band shall be applied - When the E-UTRA operating band frequency range is >1GHz, the applicable additional tolerance shall be the maximum tolerance in Table 6.2.5A-3 that applies for that operating band among the supported CA configurations |  |  |

NOTE:	The above additional tolerances do not apply to supported UTRA operating bands with frequency range below 1 GHz that correspond to the E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations when such bands are belonging only to band combination(s) where one band is <1GHz and another band is >1.7GHz and there is no harmonic relationship between the low band UL and high band DL. Otherwise the above additional tolerances also apply to supported UTRA operating bands that correspond to the E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations.

NOTE:	To meet the TIB,c requirements for CA_3A-7A with state-of-the-art technology, an increase in power consumption of the UE may be required. It is also expected that as the state-of-the-art technology evolves in the future, this possible power consumption increase can be reduced or eliminated.

### 6.2.5A	Configured transmitted power for CA

For uplink carrier aggregation the UE is allowed to set its configured maximum output power PCMAX,c  for serving cell c and its total configured maximum output power PCMAX.

The configured maximum output power PCMAX,c  on serving cell c shall be set as specified in subclause 6.2.5.

For uplink inter-band carrier aggregateion, MPR c and A-MPR c apply per serving cell c and are specified in subclause 6.2.3 and subclause 6.2.4, respectively. P-MPR c accounts for power management for serving cell c. PCMAX,c  is calculated under the assumption that the transmit power is increased independently on all component carriers.

For uplink intra-band contiguous carrier aggregation, MPR c = MPR and A-MPR c = A-MPR with MPR and A-MPR specified in subclause 6.2.3A and subclause 6.2.4A respectively. There is one power management term for the UE, denoted P-MPR, and P-MPR c = P-MPR. PCMAX,c  is calculated under the assumption that the transmit power is increased by the same amount in dB on all component carriers.

The total configured maximum output power PCMAX shall be set within the following bounds:

PCMAX_L ≤  PCMAX  ≤  PCMAX_H

For uplink inter-band carrier aggregation with up to one serving cell c per operating band,

PCMAX_L = MIN {10log10∑ MIN [ pEMAX,c/ (tC,c),  pPowerClass/(mprc·a-mprc·tC,c ·tIB,c) , pPowerClass/pmprc], PPowerClass}

PCMAX_H = MIN{10 log10 ∑ pEMAX,c , PPowerClass}

where

-	pEMAX,c is the linear value of PEMAX, c which is given by IE P-Max for serving cell c in [7];

-	PPowerClass is the maximum UE power specified in Table 6.2.2A-1 without taking into account the tolerance specified in the Table 6.2.2A-1; pPowerClass is the linear value of PPowerClass;

-	mpr c and a-mpr c are the linear values of MPR c and A-MPR c as specified in subclause 6.2.3 and subclause 6.2.4, respectively;

-	pmprc is the linear value of P-MPR c;

-	tC,c is the linear value of TC,ctC,c = 1.41 when Note 2 in Table 6.2.2-1 applies for a serving cell c, otherwise tC,c = 1;

-	tIB,c  is the linear value of the inter-band relaxation term TIB,c of the serving cell c as specified in Table 6.2.5-2; otherwise tIB,c.

For uplink intra-band contiguous carrier aggregation,

PCMAX_L  = MIN{10 log10 ∑ pEMAX,c  - TC , PPowerClass – MAX(MPR + A-MPR + ΔTIB,c + TC, P-MPR ) }

PCMAX_H  = MIN{10 log10 ∑ pEMAX,c , PPowerClass}

where

-	pEMAX,c is the linear value of PEMAX, c which is given by IE P-Max for serving cell c in [7];

-	PPowerClass is the maximum UE power specified in Table 6.2.2A-1 without taking into account the tolerance specified in the Table 6.2.2A-1;

-	MPR and A-MPR are specified in subclause 6.2.3A and subclause 6.2.4A respectively;

-	TIB,c is the additional tolerance for serving cell c as specified in Table 6.2.5-2;

-	P-MPR is the power management term for the UE;

-	TC is the highest value TC,c among all serving cells c in the subframe over both timeslots. TC,c = 1.5 dB when Note 2 in Table 6.2.2A-1 applies to the serving cell c, otherwise TC,c = 0 dB.

The measured maximum output power PUMAX over all serving cells shall be within the following range:

PCMAX_L  –  T(PCMAX_L)  ≤  PUMAX  ≤  PCMAX_H  +  T(PCMAX_H)

PUMAX = 10 log10 ∑ pUMAX,c

where pUMAX,c  denotes the measured maximum output power for serving cell c expressed in linear scale.

The tolerance T(PCMAX) is defined by the table below and applies to PCMAX_L and PCMAX_H separately.

Table 6.2.5A-2: PCMAX tolerance

| PCMAX (dBm) | Tolerance T(PCMAX) Intra-band with two active UL serving cells (dB) | Tolerance T(PCMAX) Inter-band with two active UL serving cells (dB) |
| --- | --- | --- |
| 21 ≤ PCMAX ≤ 23 | 2.0 | 2.0 |
| 20 ≤ PCMAX < 21 | [2.5] | TBD |
| 19 ≤ PCMAX < 20 | [3.5] | TBD |
| 18 ≤ PCMAX < 19 | [4.0] | TBD |
| 13 ≤ PCMAX < 18 | [5.0] | TBD |
| 8 ≤ PCMAX < 13 | [6.0] | TBD |
| -40 ≤ PCMAX < 8 | [7.0] | TBD |

### 6.2.5B	Configured transmitted power for UL-MIMO

For UE supporting UL-MIMO, the transmitted power is configured per each UE.

The definitions of configured maximum output power PCMAX,c, the lower bound PCMAX_L,c, and the higher bound PCMAX_H,c specified in subclause 6.2.5 shall apply to UE supporting UL-MIMO, where

-	PPowerClass and TC,c are specified in subclause 6.2.2B;

-	MPRc is specified in subclause 6.2.3B;

-	A-MPRc is specified in subclause 6.2.4B.

The measured configured maximum output power PUMAX,c for serving cell c shall be within the following bounds:

PCMAX_L,c  –  MAX{TL, T LOW(PCMAX_L,c)}  ≤  PUMAX,c  ≤  PCMAX_H,c  +  T HIGH(PCMAX_H,c)

where TLOW(PCMAX_L,c) and THIGH(PCMAX_H,c) are defined as the tolerance and applies to PCMAX_L,c and PCMAX_H,c separately, while TL is the absolute value of the lower tolerance in Table 6.2.2B-1 for the applicable operating band.

For UE with two transmit antenna connectors in closed-loop spatial amultiplexing scheme, the tolerance is specified in Table 6.2.5B-1. The requirements shall be met with UL-MIMO configurations specified in Table 6.2.2B-2.

Table 6.2.5B-1: PCMAX,c tolerance in closed-loop spatial multiplexing scheme

| PCMAX,c (dBm) | Tolerance TLOW(PCMAX_L,c) (dB) | Tolerance THIGH(PCMAX_H,c) (dB) |
| --- | --- | --- |
| PCMAX,c = 23 | 3.0 | 2.0 |
| [22] ≤ PCMAX,c < [23] | [5.0] | [2.0] |
| [21] ≤ PCMAX,c < [22] | [5.0] | [3.0] |
| [20] ≤ PCMAX,c < [21] | [6.0] | [4.0] |
| [16] ≤ PCMAX,c < [20] | [5.0] |  |
| [11] ≤ PCMAX,c < [16] | [6.0] |  |
| [-40] ≤ PCMAX,c < [11] | [7.0] |  |

For single-antenna port scheme, the requirements in subclause 6.2.5 apply.

## 6.3		Output power dynamics

< end of changes >