3GPP TSG RAN WG4 Meeting #68BIS	R4-135763

Riga, Latvia, 7th – 11th October, 2013

Agenda Item:	8.24

Source:	LG Electronics, LG UPlus

Title:	TP for UE-to-UE coexistence requirement for 2UL inter-band CA

Document for:	Approval

- Introduction

In the last RAN4 #68 meetings, Way forward of UE coexistence requirements for 2ULs inter-band CA was discussed as follow.

  - WF1: The 2UL co-existence table will be specified in 36.101, however it is FFS whether the 2UL tests are needed or not.

  - WF2: It is FFS whether 2UL tables and thus tests are needed or not.

However, there were some unclear points such as inter-regions issues and WG responsibility between RAN4 and RAN5 for 2ULs UE coexistence requirements tests.

In this contribution, we propose a general test method for UE-to-UE coexistence requirements for 2ULs inter-band CA and revised Way forward to progress of 2ULs inter-band CA.

- General UE-to-UE Coexistence Requirements for Inter-regions CA Combination

From the e-mail discussion after RAN4 #68 meeting,  there was consensus on WF1 to add the inter-regions bands in the protected band lists in Table 6.6.3.2-1 of TS36.101[1] for single UL UE in Table2.

To define the protected band lists for inter-region issues, we used option 5 in [3].

- Option5) Choose a subset of protected frequency ranges/bands to protect based on spectrum allocations in the countries where the two bands are deployed

For the B1+B5, B3+B5 CA combinations, the deployed operating bands in Asia and South America are summarized in table 1.

Table 1. Deployed operating bands in Asia and South America

| Operating bands | Band 1 | Band 3 | Band 5 | Band 7 | Band8 | Band26 | Band27 | Band28 | Band38 | Band40 | Band42 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidths Country | 2100MHz | 1800MHz | 850MHz | 2600MHz | 900MHz | Upper 850MHz | Lower 850MHz | 700MHz | 2600MHz | 2300MHz | 3500MHz |
| South Korea | X | X | X | X | X | X |  | X |  | X |  |
| Australia | X | X | X |  | X | F | F | X |  |  |  |
| Brazil | X | X | X | X | X | F | F | F | X |  | X |
| Costa Rica | X | X | X |  |  |  |  |  |  |  |  |
| Hong Kong | X | X | X |  | X |  |  |  |  |  |  |
| Israel | X | X | X |  |  | F | F |  |  |  |  |
| New Zealand | X | X | X |  | X | F | F | F |  |  |  |
| Note: X means the operating band deployed in the country, F means the operating band could be deployed in the country in the future. |  |  |  |  |  |  |  |  |  |  |  |

For the B4+B7 CA combinations, the deployed operating bands in South America are summarized in table 2.

Table 2. Deployed operating bands in North/South America

| Operating bands | Band 2 | Band 4 | Band 5 | Band 7 | Band10 | Band12 | Band13 | Band14 | Band17 | Band27 | Band28 | Band29 | Band38 | Band42 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidths Country | 1900 MHz | 2100 MHz | 850 MHz | 2600 MHz | 2100 MHz | Lower 700MHz | Mid 700MHz | Upper 700MHz | Lower 700MHz | 800 MHz |  |  | 2600 MHz | 3500 MHz |
| Canada | X | X | X | X | F | X | X | X | X |  |  | X | X |  |
| Mexico | X | X | X | F | F |  |  |  |  | X | F |  | F | X |
| Chile | X | X | X | X | F |  |  |  |  | X | F |  | X |  |
| Colombia | X | X | X | X | F |  |  |  |  | X | F |  | X |  |
| Argentina | X | X | X | X | F |  |  |  |  | X | F |  | X | X |
| Note: X means the operating band deployed in the country, F means the operating band could be deployed in the country in the future. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

From the table1 and 2, we can define the protect band lists for 2ULs inter-band CA UE in inter-regions.

Table 2: Proposed protected band lists for 2ULs inter-band CA in single UL coexistence requirements

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | Note |
| 1 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 38, 40, 41, 42, 43, 44 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | Frequency range | 1880 |  | 1895 | -40 | 1 | 15,27 |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 15, 26, 27 |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 15, 26, 27 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 6, 8, 15 |
|  | Frequency range | 1839.9 | - | 1879.9 | -50 | 1 | 15 |
| 2 | E-UTRA Band 4, 5, 10, 12, 13, 14, 17, 22, 23, 24, 26, 27, 28, 29, 30, 41, 42 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 3 | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 33, 34, 38, 41, 43, 44 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 13 |
|  | E-UTRA Band 22, 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 13 |
| 4 | E-UTRA Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 22, 23, 24, 25, 26, 27, 28, 29, 30, 41, 43 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 5 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 22, 23, 24, 25, 28, 29, 30, 31, 38, 40, 42, 43 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
| 6 | E-UTRA Band 1, 9, 11, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 860 | - | 875 | -37 | 1 |  |
|  | Frequency range | 875 | - | 895 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1919.6 | -41 | 0.3 | 7 |
|  |  | 1884.5 | - | 1915.7 |  |  | 8 |
| 7 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 12,13, 14, 17, 20, 22, 27, 28, 29, 30, 31, 33, 34, 42, 43 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 15, 21, 26 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 15, 21, 26 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 15, 21 |
| 8 | E-UTRA Band 1, 20, 28, 31, 33, 34, 38, 39, 40 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA band 7 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 22, 41, 42, 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 23 |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 15, 23 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 23 |
| 9 | E-UTRA Band 1, 11, 18, 19, 21, 26, 28, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1839.9 | - | 1879.9 | -50 | 1 |  |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
| 10 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 23, 24, 25, 26, 27, 28, 29, 30, 41, 43 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 22, 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 11 | E-UTRA Band 1, 11, 18, 19, 21, 28, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1839.9 | - | 1879.9 | -50 | 1 |  |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
| 12 | E-UTRA Band 2, 5, 13, 14, 17, 23, 24, 25, 26, 27, 30, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 10 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| 13 | E-UTRA Band 2, 4, 5, 10, 12, 13, 17, 23, 25, 26, 27, 29, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 15 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 11, 15 |
|  | E-UTRA Band 14 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 24, 30 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 14 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 23, 24, 25, 26, 27, 29, 30, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 12, 15 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 11, 12, 15 |
| 17 | E-UTRA Band 2, 5, 13, 14, 17, 23, 24, 25, 26, 27, 30, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 10 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| 18 | E-UTRA Band 1, 11, 21, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 860 | - | 890 | -40 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 758 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 15 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1839.9 | - | 1879.9 | -50 | 1 |  |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
| 19 | E-UTRA Band 1, 11, 21, 28, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 9, 15 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1839.9 | - | 1879.9 | -50 | 1 |  |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
| 20 | E-UTRA Band 1, 3, 7, 8, 20, 22, 33, 34, 43 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 38, 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 21 | E-UTRA Band 11 | FDL_low | - | FDL_high | -35 | 1 | 10, 15 |
|  | E-UTRA Band 1, 18, 19, 28, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 21 | FDL_low | - | FDL_high | -50 | 1 | 10 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1839.9 | - | 1879.9 | -50 | 1 |  |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
| 22 | E-UTRA Band 1, 3, 7, 8, 20, 26, 27, 28, 33, 34, 38, 39, 40, 43 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 3510 | - | 3525 | -40 | 1 | 15 |
|  | Frequency range | 3525 | - | 3590 | -50 | 1 |  |
| 23 | E-UTRA Band 4, 5, 10, 12, 13, 14, 17, 23, 24, 26, 27, 29, 30, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2 | FDL_low | - | FDL_high | -50 | 1 | 14, 15 |
|  | Frequency range | 1998 | - | 1999 | -21 | 1 | 14, 15 |
|  | Frequency range | 1997 | - | 1998 | -27 | 1 | 14, 15 |
|  | Frequency range | 1996 | - | 1997 | -32 | 1 | 14, 15 |
|  | Frequency range | 1995 | - | 1996 | -37 | 1 | 14, 15 |
|  | Frequency range | 1990 | - | 1995 | -40 | 1 | 14, 15 |
|  | Frequency range | 1990 |  | 1999 | -40 | 1 | 15, 28 |
|  | Frequency range | 1999 |  | 2000 | -40 | Note 29 | 15, 28 |
| 24 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 23, 24, 25, 26, 29, 30, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
| 25 | E-UTRA Band 4, 5, 10,12, 13, 14, 17, 22, 23, 24, 26, 27, 28, 29, 30, 41, 42 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 25 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 26 | E-UTRA Band 1, 2, 3, 4, 5, 10, 11, 12, 13, 14, 17, 18,19, 21, 22, 23, 24, 25, 26, 29, 30, 31, 34, 40, 42, 43 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  |  | 799 | - | 803 | -40 | 1 | 15 |
|  | Frequency range | 851 | - | 859 | -53 | 0.00625 | 20 |
|  | E-UTRA Band 27 | FDL_low | - | 859 | -32 | 1 | 20 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1839.9 | - | 1879.9 | -50 | 1 |  |
| 27 | E-UTRA Band 1, 2, 3, 4, 5, 7, 10, 12, 13, 14, 17, 22, 23, 25, 26, 27, 29, 30, 31, 38, 41, 42, 43 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 |  |
|  | E-UTRA Band 28 | 790 | - | FDL_high | -32 | 1 | 16 |
|  |  | FDL_low | - | 790 | -50 | 1 |  |
| 28 | E-UTRA Band 2, 3, 5, 7, 8, 18, 19, 25, 26, 27, 31, 34, 38, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 4, 10, 22, 42, 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 19, 24 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 19, 25 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 15 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 15 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 19 |
|  | Frequency range | 1839.9 | - | 1879.9 | -50 | 1 |  |
| 30 | E-UTRA Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 23, 24, 25, 26, 27, 29, 30, 38, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2200 | - | 2288 | -40 | 1 | 15, 31 |
|  | Frequency range | 2288 | - | 2292 | -37 | 1 | 15, 31 |
|  | Frequency range | 2292 | - | 2296 | -31 | 1 | 15, 31 |
|  | Frequency range | 2296 | - | 2300 | -25 | 1 | 15, 31 |
|  | Frequency range | 2320 | - | 2324 | -25 | 1 | 15, 31 |
|  | Frequency range | 2324 | - | 2328 | -31 | 1 | 15, 31 |
|  | Frequency range | 2328 | - | 2332 | -37 | 1 | 15, 31 |
|  | Frequency range | 2332 | - | 2395 | -40 | 1 | 15, 31 |
| 31 | E-UTRA Band 1, 5, 7, 8, 26, 27, 28, 38, 42 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| … |  |  |  |  |  |  |  |
| 33 | E-UTRA Band 1, 7, 8, 20, 22, 34, 38, 39, 40, 42, 43 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| 34 | E-UTRA Band 1, 3, 7, 8, 11, 18, 19, 20, 21, 22, 26, 28, 33, 38,39, 40, 41, 42, 43, 44 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 1839.9 | - | 1879.9 | -50 | 1 | 5 |
| 35 |  |  |  |  |  |  |  |
| 36 |  |  |  |  |  |  |  |
| 37 |  |  | - |  |  |  |  |
| 38 | E-UTRA Band 1, 2, 3, 4, 5, 8, 10, 12, 13, 14, 17, 20, 22, 27, 28, 29, 30, 31, 33, 34, 42, 43 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2620 | - | 2645 | -15.5 | 5 | 15, 22, 26 |
|  | Frequency range | 2645 | - | 2690 | -40 | 1 | 15, 22 |
| 39 | E-UTRA Band 22, 34, 40, 41, 42, 44 | FDL_low | - | FDL_high | -50 | 1 |  |
| 40 | E-UTRA Band 1, 3, 5, 22, 26, 27, 33, 34, 39, 41, 42, 43, 44 | FDL_low | - | FDL_high | -50 | 1 |  |
| 41 | E-UTRA Band 1, 2, 3, 4, 5, 8, 10, 12, 13 , 14, 17, 23, 24, 25, 26, 27, 28, 29, 30, 34, 39, 40, 42, 44 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 9, 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 30 |
|  | Frequency range | 1839.9 |  | 1879.9 | -50 | 1 | 30 |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 8, 30 |
| 42 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 20, 25, 26, 27, 28, 31, 33, 34, 38, 40, 41, 44 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 43 | FDL_low | - | FDL_high | -50 | 1 | 3 |
| 43 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 20, 25, 26, 27, 28, 33, 34, 38, 40 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA Band 22 | FDL_low | - | FDL_high | [-50] | [1] | 3 |
| 44 | E-UTRA Band 3, 5, 8, 34, 39, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 40, 42 | FDL_low | - | FDL_high |  | -50 | 2 |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd, 3rd or 4th harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 3: To meet these requirements some restriction will be needed for either the operating band or protected band NOTE 4: N/A NOTE 5: For non synchronised TDD operation to meet these requirements some restriction will be needed for either the operating band or protected band NOTE 6: Applicable when NS_05 in section 6.6.3.3.1 is signalled by the network. NOTE 7: Applicable when co-existence with PHS system operating in 1884.5 -1919.6MHz. NOTE 8: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. NOTE 9: Applicable when NS_08 in subclause 6.6.3.3.3 is signalled by the network NOTE 10: Applicable when NS_09 in subclause 6.6.3.3.4 is signalled by the network NOTE 11: Whether the applicable frequency range should be 793-805MHz instead of 799-805MHz is TBD NOTE 12: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB NOTE 13: This requirement applies for 5, 10, 15 and 20 MHz E-UTRA channel bandwidth allocated within 1744.9MHz and 1784.9MHz. NOTE 14: To meet this requirement NS_11 value shall be signalled when operating in 2000-2020 MHz NOTE 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 16: Applicable when NS_16 in subclause 6.6.3.3.9 is signalled by the network. NOTE 17: N/A NOTE 18: N/A NOTE 19: Applicable when the assigned E-UTRA carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 20: Applicable when NS_15 in subclause 6.6.3.3.8 is signalled by the network. NOTE 21: This requirement is applicable for an uplink transmission bandwidth less than or equal to 54 RB for carriers of 15 MHz bandwidth when carrier center frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier center frequency is within the range 2552 - 2560 MHz. No other restrictions apply for carriers with bandwidths confined in 2500-2570 MHz. NOTE 22: This requirement is applicable for an uplink transmission bandwidth less than or equal to 54 RB for carriers of 15 MHz bandwidth when carrier center frequency is within the range 2605.5 - 2607.5 MHz and for carriers of 20 MHz bandwidth when carrier center frequency is within the range 2597 – 2605 MHz. No other restrictions apply for carriers with bandwidths confined in 2570-2615 MHz. For assigned carriers with bandwidths overlapping the frequency range 2615-2620 MHz the requirements apply with the maximum output power configured to +20 dBm in the IE P-Max. NOTE 23 For carriers of 5 MHz channel bandwidth with carrier center frequencies (Fc) in the range 902.5MHz ≤ Fc < 907.5 MHz, the requirement applies for uplink transmission bandwidths less than or equal to 20 RB. No restrictions apply in the range 907.5 MHz ≤ Fc ≤ 912.5 MHz. For carriers of 10 MHz channel bandwidth, the requirement only applies for Fc = 910 MHz and uplink transmission bandwidths less than or equal to 32 RB with RBstart > 3. NOTE 24: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 25: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 26: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 27: This requirement is applicable for an uplink transmission bandwidth less than or equal to 54 RB for carriers of 15 MHz bandwidth when carrier center frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier center frequency is within the range 1930 - 1938 MHz. This requirement is applicable without any other uplink transmission bandwidth restriction for channel bandwidths within the range 1920 - 1980 MHz. NOTE 28: Applicable when NS_20 is signalled by the network. NOTE 29: The measurement bandwidth is 1% of the applicable E-UTRA channel bandwidth (Table 5.6-1). NOTE 30: This requirement applies when the E-UTRA carrier is confined within 2545-2575 MHz and the channel bandwidth is 10 or 20 MHz NOTE 31: Applicable when NS_21 is signalled by the network. |  |  |  |  |  |  |  |

- Way Forward on UE Coexistence Requirements for Dual UL CA

After RAN4 #68 meeting in Barcelona, we had E-mail discussion to solve the unclear issues in Way forward at last RAN4 meeting with some interested companies.

Until now, RAN4 has made general and some specific test requirements using general simulation assumptions and general architecture. This means RAN4 has not considered all the possible UE architecture and implementable structures of DUT since standards precedes the implementations. In some possible DUTs and architectures, some uncertain problems and issues can happen, but this should be resolved through implementation in UE side.

Therefore, we had analyzed IMD levels of 2ULs inter-band CA in general architecture.

From the analysis and measurements, RAN4 has seen that there was no IMD issue for 2ULs inter-band CA combinations.  But some companies expressed concern that the generated IMD products can exceed the general coexistence level of -50dBm/MHz in some specific frequency ranges.

Therefore, compromised WF that has been proposed in last RAN4 meeting as follow,

  - WF1: The 2UL co-existence table will be specified in 36.101, however it is FFS whether the 2UL tests are needed or not.

  - WF2: It is FFS whether 2UL tables and thus tests are needed or not.

WF1 has been agreed to put the protect bands in table 6.6.3.2-1 of TS36.101 as a result of e-mail discussion. For WF2, however, there was some ambiguity in interpreting the responsibility and inter-regions issues for 2ULs CA tests.

So, we captured these agreed Way Forwards for 2ULs inter-band CA in ad-hoc meeting and e-mail discussion.

Way forward proposal:

- Create separate UE to UE co-ex table for 2 UL inter-band CA containing all 2 UL inter-band CA configurations and the bands being protected.

- Protected bands are a subset of protected frequency ranges/bands to protect based on spectrum allocations in the countries where the two bands are deployed.

- Add a informative note below the table in the TR.

- Send an LS to RAN5 from this meeting to inform RAN5 about the issue described in the note

NOTE: The requirements in 2 UL inter-band CA UE to UE co-existence table could be verified by measuring spurious emissions [at least] at the specific frequencies where second and third order intermodulation products generated by the two transmitted carriers can occur; in that case, the requirements for remaining applicable frequencies in 2 UL inter-band CA UE to UE co-existence table would be considered to be verified by the measurements verifying the 1 UL inter-band CA UE to UE co-existence requirements.

- Conclusion

In this contribution, we propose the protected band lists for 2ULs inter-band CA in single UL coexistence requirements as shown in table2. And also we propose Way Forward as follow

Way forward proposal:

- Create separate UE to UE co-ex table for 2 UL inter-band CA containing all 2 UL inter-band CA configurations and the bands being protected.

- Protected bands are a subset of protected frequency ranges/bands to protect based on spectrum allocations in the countries where the two bands are deployed.

- Add a informative note below the table in the TR.

- Send an LS to RAN5 from this meeting to inform RAN5 about the issue described in the note

NOTE: The requirements in 2 UL inter-band CA UE to UE co-existence table could be verified by measuring spurious emissions [at least] at the specific frequencies where second and third order intermodulation products generated by the two transmitted carriers can occur; in that case, the requirements for remaining applicable frequencies in 2 UL inter-band CA UE to UE co-existence table would be considered to be verified by the measurements verifying the 1 UL inter-band CA UE to UE co-existence requirements.

- Reference

- TS 36.101 v11.6.0

- R4-133062, “Dual uplink CA intermodulation analysis,” Renesas Mobile

- R4-131762, “Dual Uplink CA UE Emissions Discussion,” NII Holdings

- R4-132774, “2UL CA UE coexistence,” Qualcomm

- R4-134389, “Passive IMD measurements levels of inter-band CA_1A-5A UE with 2ULs,” LG Electronics

- R4-131986, “WF on the protected Bands for inter-band CA with 2ULs,” LG U+, NII, KT, Nokia

- TR 36.860 v0.3.0

- R4-131416, “Protected Band for B1+B5 2 UP LINK CA,” LG U+

- R4-131612, “Uplink inter-band CA: specification of spurious emissions,” Ericsson,ST-Ericsson

Text proposal for TR 36.860

********************** Start of text proposal to TR 36.860 ***********************

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 30.007: “Guideline on WI/SI for new Operating Bands”

[3]	3GPP TR 36.850: “Inter-band Carrier Aggregation Technical Report (Release 11)

[4]	3GPP TR 36.851: “Inter-band Carrier Aggregation Technical Report (Release 12)

[5]	3GPP TS 36.101: “E-UTRA UE radio transmission and reception (Release 12)

[6]	R4-131762, “Dual Uplink CA UE Emissions Discussion,” NII Holdings

[7]	R4-132713, “Handling of potential intermodulation problems to other systems than own bands,” Renesas Mobile Europe Ltd.

[8]	R4-133060, “General coexistence analysis table for inter-band CA with 2UL, ZTE, Renesas Europe, LG Electronics

[9]	R4-133062, “Dual uplink CA intermodulation analysis,” Renesas Mobile Europe Ltd.

[10]	R4-134389, “Passive IMD measurements levels of inter-band CA_1A-5A UE with 2ULs,” LG Electronics

[11]	R4-134418, “TP for general frequency bands for WiFi, GNSS and Digital TV for 2ULs inter-band CA,” LG Electronics

< ommited of  no changes >

5.2		UE specific

Most dual-uplink inter-band CA combinations are from the same regions as shown in table 5.2-1. But some inter-band CA combinations are from the inter-regions such as B1+B5, B3+B5 and B4+B7 combinations.

Table 5.2-1: Dual-uplink inter-band CA combination lists

| Class |  | Band Combination | Frequency | Deployment Regions |
| --- | --- | --- | --- | --- |
| Dual-uplink Inter-band CA | A1 | B1+5 | 2.1G+800M | Inter-regions (Region 1+2+3) |
|  |  | B1+B19 | 2.1G+800M | Region 3 |
|  |  | B3+20 | 1.8G+800M | Region 1 |
|  | A2 | B3+8 | 1.8G+900M | Region 3 |
|  |  | B4+12 | 2.1G+700M | Region 2 |
|  |  | B4+17 | 2.1G+700M | Region 2 |
|  | A3 | B1+7 | 2.1G+2.6G | Region 1 |
|  |  | B3+B7 | 1.7G+2.6G | Region 1 |
|  |  | B4+B7 | 1.7G+2.6G | Inter-regions (Region 1+2+3) |
|  |  | B5+B12 | 800M+700M | Region 2 |
|  |  | B5+B17 | 800M+700M | Region 2 |
|  | A4 | B3+5 | 1.8G+800M | Inter-regions (Region 1+2+3) |
|  |  | B2+B4 | 1.9G+2.1G | Region 2 |
|  |  | B7+B20 | 2.6G+800M | Region 1 |
|  |  | B3+B26 | 1.8G+800M | Region 3 |
|  | A5 | B1+B21 | 2.1G+1.4G | Region 3 |

For the 1UL/2DL inter-band CA UE, the aggressor signal is generated in 1 UL band same as rel-8/9 UE. So it is possible to follow the conventional UE-to-UE coexistence requirements that are described in table 6.6.3.2-1[5]. However, the frequency range of emission level for IMD and Harmonic from 2UL inter-band CA needs to be analyzed and measured to determine the adjacent UE coexistence requirements.

From the e-mail discussion after RAN4 #68 meeting,  there was consensus to add the inter-regions bands in the protected band lists in table 6.6.3.2-1 [5] for single UL UE as follow.

Table 5.2-2: Proposed protected band lists for 2ULs inter-band CA in single UL coexistence requirements

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | Note |
| 1 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 38, 40, 41, 42, 43, 44 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | Frequency range | 1880 |  | 1895 | -40 | 1 | 15,27 |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 15, 26, 27 |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 15, 26, 27 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 6, 8, 15 |
|  | Frequency range | 1839.9 | - | 1879.9 | -50 | 1 | 15 |
| 2 | E-UTRA Band 4, 5, 10, 12, 13, 14, 17, 22, 23, 24, 26, 27, 28, 29, 30, 41, 42 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 3 | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 33, 34, 38, 41, 43, 44 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 13 |
|  | E-UTRA Band 22, 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 13 |
| 4 | E-UTRA Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 22, 23, 24, 25, 26, 27, 28, 29, 30, 41, 43 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 5 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 22, 23, 24, 25, 28, 29, 30, 31, 38, 40, 42, 43 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
| 6 | E-UTRA Band 1, 9, 11, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 860 | - | 875 | -37 | 1 |  |
|  | Frequency range | 875 | - | 895 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1919.6 | -41 | 0.3 | 7 |
|  |  | 1884.5 | - | 1915.7 |  |  | 8 |
| 7 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 20, 22, 27, 28, 29, 30, 31, 33, 34, 42, 43 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 15, 21, 26 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 15, 21, 26 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 15, 21 |
| 8 | E-UTRA Band 1, 20, 28, 31, 33, 34, 38, 39, 40 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA band 7 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 22, 41, 42, 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 23 |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 15, 23 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 23 |
| 9 | E-UTRA Band 1, 11, 18, 19, 21, 26, 28, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1839.9 | - | 1879.9 | -50 | 1 |  |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
| 10 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 23, 24, 25, 26, 27, 28, 29, 30, 41, 43 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 22, 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 11 | E-UTRA Band 1, 11, 18, 19, 21, 28, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1839.9 | - | 1879.9 | -50 | 1 |  |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
| 12 | E-UTRA Band 2, 5, 13, 14, 17, 23, 24, 25, 26, 27, 30, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 10 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| 13 | E-UTRA Band 2, 4, 5, 10, 12, 13, 17, 23, 25, 26, 27, 29, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 15 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 11, 15 |
|  | E-UTRA Band 14 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 24, 30 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 14 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 23, 24, 25, 26, 27, 29, 30, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 12, 15 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 11, 12, 15 |
| 17 | E-UTRA Band 2, 5, 13, 14, 17, 23, 24, 25, 26, 27, 30, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 10 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| 18 | E-UTRA Band 1, 11, 21, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 860 | - | 890 | -40 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 758 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 15 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1839.9 | - | 1879.9 | -50 | 1 |  |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
| 19 | E-UTRA Band 1, 11, 21, 28, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 9, 15 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1839.9 | - | 1879.9 | -50 | 1 |  |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
| 20 | E-UTRA Band 1, 3, 7, 8, 20, 22, 33, 34, 43 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 38, 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 21 | E-UTRA Band 11 | FDL_low | - | FDL_high | -35 | 1 | 10, 15 |
|  | E-UTRA Band 1, 18, 19, 28, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 21 | FDL_low | - | FDL_high | -50 | 1 | 10 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1839.9 | - | 1879.9 | -50 | 1 |  |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
| 22 | E-UTRA Band 1, 3, 7, 8, 20, 26, 27, 28, 33, 34, 38, 39, 40, 43 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 3510 | - | 3525 | -40 | 1 | 15 |
|  | Frequency range | 3525 | - | 3590 | -50 | 1 |  |
| 23 | E-UTRA Band 4, 5, 10, 12, 13, 14, 17, 23, 24, 26, 27, 29, 30, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2 | FDL_low | - | FDL_high | -50 | 1 | 14, 15 |
|  | Frequency range | 1998 | - | 1999 | -21 | 1 | 14, 15 |
|  | Frequency range | 1997 | - | 1998 | -27 | 1 | 14, 15 |
|  | Frequency range | 1996 | - | 1997 | -32 | 1 | 14, 15 |
|  | Frequency range | 1995 | - | 1996 | -37 | 1 | 14, 15 |
|  | Frequency range | 1990 | - | 1995 | -40 | 1 | 14, 15 |
|  | Frequency range | 1990 |  | 1999 | -40 | 1 | 15, 28 |
|  | Frequency range | 1999 |  | 2000 | -40 | Note 29 | 15, 28 |
| 24 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 23, 24, 25, 26, 29, 30, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
| 25 | E-UTRA Band 4, 5, 10,12, 13, 14, 17, 22, 23, 24, 26, 27, 28, 29, 30, 41, 42 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 25 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 26 | E-UTRA Band 1, 2, 3, 4, 5, 10, 11, 12, 13, 14, 17, 18,19, 21, 22, 23, 24, 25, 26, 29, 30, 31, 34, 40, 42, 43 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  |  | 799 | - | 803 | -40 | 1 | 15 |
|  | Frequency range | 851 | - | 859 | -53 | 0.00625 | 20 |
|  | E-UTRA Band 27 | FDL_low | - | 859 | -32 | 1 | 20 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1839.9 | - | 1879.9 | -50 | 1 |  |
| 27 | E-UTRA Band 1, 2, 3, 4, 5, 7, 10, 12, 13, 14, 17, 22, 23, 25, 26, 27, 29, 30, 31, 38, 41, 42, 43 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 |  |
|  | E-UTRA Band 28 | 790 | - | FDL_high | -32 | 1 | 16 |
|  |  | FDL_low | - | 790 | -50 | 1 |  |
| 28 | E-UTRA Band 2, 3, 5, 7, 8, 18, 19, 25, 26, 27, 31, 34, 38, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 4, 10, 22, 42, 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 19, 24 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 19, 25 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 15 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 15 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 19 |
|  | Frequency range | 1839.9 | - | 1879.9 | -50 | 1 |  |
| 30 | E-UTRA Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 23, 24, 25, 26, 27, 29, 30, 38, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2200 | - | 2288 | -40 | 1 | 15, 31 |
|  | Frequency range | 2288 | - | 2292 | -37 | 1 | 15, 31 |
|  | Frequency range | 2292 | - | 2296 | -31 | 1 | 15, 31 |
|  | Frequency range | 2296 | - | 2300 | -25 | 1 | 15, 31 |
|  | Frequency range | 2320 | - | 2324 | -25 | 1 | 15, 31 |
|  | Frequency range | 2324 | - | 2328 | -31 | 1 | 15, 31 |
|  | Frequency range | 2328 | - | 2332 | -37 | 1 | 15, 31 |
|  | Frequency range | 2332 | - | 2395 | -40 | 1 | 15, 31 |
| 31 | E-UTRA Band 1, 5, 7, 8, 26, 27, 28, 38, 42 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| … |  |  |  |  |  |  |  |
| 33 | E-UTRA Band 1, 7, 8, 20, 22, 34, 38, 39, 40, 42, 43 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| 34 | E-UTRA Band 1, 3, 7, 8, 11, 18, 19, 20, 21, 22, 26, 28, 33, 38,39, 40, 41, 42, 43, 44 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 1839.9 | - | 1879.9 | -50 | 1 | 5 |
| 35 |  |  |  |  |  |  |  |
| 36 |  |  |  |  |  |  |  |
| 37 |  |  | - |  |  |  |  |
| 38 | E-UTRA Band 1, 2, 3, 4, 5, 8, 10, 12, 13, 14, 17, 20, 22, 27, 28, 29, 30, 31, 33, 34, 42, 43 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2620 | - | 2645 | -15.5 | 5 | 15, 22, 26 |
|  | Frequency range | 2645 | - | 2690 | -40 | 1 | 15, 22 |
| 39 | E-UTRA Band 22, 34, 40, 41, 42, 44 | FDL_low | - | FDL_high | -50 | 1 |  |
| 40 | E-UTRA Band 1, 3, 5, 22, 26, 27, 33, 34, 39, 41, 42, 43, 44 | FDL_low | - | FDL_high | -50 | 1 |  |
| 41 | E-UTRA Band 1, 2, 3, 4, 5, 8, 10, 12, 13 , 14, 17, 23, 24, 25, 26, 27, 28, 29, 30, 34, 39, 40, 42, 44 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 9, 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 30 |
|  | Frequency range | 1839.9 |  | 1879.9 | -50 | 1 | 30 |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 8, 30 |
| 42 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 20, 25, 26, 27, 28, 31, 33, 34, 38, 40, 41, 44 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 43 | FDL_low | - | FDL_high | -50 | 1 | 3 |
| 43 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 20, 25, 26, 27, 28, 33, 34, 38, 40 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA Band 22 | FDL_low | - | FDL_high | [-50] | [1] | 3 |
| 44 | E-UTRA Band 3, 5, 8, 34, 39, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 40, 42 | FDL_low | - | FDL_high |  | -50 | 2 |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd, 3rd or 4th harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 3: To meet these requirements some restriction will be needed for either the operating band or protected band NOTE 4: N/A NOTE 5: For non synchronised TDD operation to meet these requirements some restriction will be needed for either the operating band or protected band NOTE 6: Applicable when NS_05 in section 6.6.3.3.1 is signalled by the network. NOTE 7: Applicable when co-existence with PHS system operating in 1884.5 -1919.6MHz. NOTE 8: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. NOTE 9: Applicable when NS_08 in subclause 6.6.3.3.3 is signalled by the network NOTE 10: Applicable when NS_09 in subclause 6.6.3.3.4 is signalled by the network NOTE 11: Whether the applicable frequency range should be 793-805MHz instead of 799-805MHz is TBD NOTE 12: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB NOTE 13: This requirement applies for 5, 10, 15 and 20 MHz E-UTRA channel bandwidth allocated within 1744.9MHz and 1784.9MHz. NOTE 14: To meet this requirement NS_11 value shall be signalled when operating in 2000-2020 MHz NOTE 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 16: Applicable when NS_16 in subclause 6.6.3.3.9 is signalled by the network. NOTE 17: N/A NOTE 18: N/A NOTE 19: Applicable when the assigned E-UTRA carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 20: Applicable when NS_15 in subclause 6.6.3.3.8 is signalled by the network. NOTE 21: This requirement is applicable for an uplink transmission bandwidth less than or equal to 54 RB for carriers of 15 MHz bandwidth when carrier center frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier center frequency is within the range 2552 - 2560 MHz. No other restrictions apply for carriers with bandwidths confined in 2500-2570 MHz. NOTE 22: This requirement is applicable for an uplink transmission bandwidth less than or equal to 54 RB for carriers of 15 MHz bandwidth when carrier center frequency is within the range 2605.5 - 2607.5 MHz and for carriers of 20 MHz bandwidth when carrier center frequency is within the range 2597 – 2605 MHz. No other restrictions apply for carriers with bandwidths confined in 2570-2615 MHz. For assigned carriers with bandwidths overlapping the frequency range 2615-2620 MHz the requirements apply with the maximum output power configured to +20 dBm in the IE P-Max. NOTE 23 For carriers of 5 MHz channel bandwidth with carrier center frequencies (Fc) in the range 902.5MHz ≤ Fc < 907.5 MHz, the requirement applies for uplink transmission bandwidths less than or equal to 20 RB. No restrictions apply in the range 907.5 MHz ≤ Fc ≤ 912.5 MHz. For carriers of 10 MHz channel bandwidth, the requirement only applies for Fc = 910 MHz and uplink transmission bandwidths less than or equal to 32 RB with RBstart > 3. NOTE 24: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 25: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 26: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 27: This requirement is applicable for an uplink transmission bandwidth less than or equal to 54 RB for carriers of 15 MHz bandwidth when carrier center frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier center frequency is within the range 1930 - 1938 MHz. This requirement is applicable without any other uplink transmission bandwidth restriction for channel bandwidths within the range 1920 - 1980 MHz. NOTE 28: Applicable when NS_20 is signalled by the network. NOTE 29: The measurement bandwidth is 1% of the applicable E-UTRA channel bandwidth (Table 5.6-1). NOTE 30: This requirement applies when the E-UTRA carrier is confined within 2545-2575 MHz and the channel bandwidth is 10 or 20 MHz NOTE 31: Applicable when NS_21 is signalled by the network. |  |  |  |  |  |  |  |

To define the protected band lists for inter-region issues, we used option 5 in [6].

- Option5) Choose a subset of protected frequency ranges/bands to protect based on spectrum allocations in the countries where the two bands are deployed

For the B1+B5, B3+B5 CA combinations, the deployed operating bands in Asia and South America are summarized in table 5.2-3.

Table 5.2-3. Deployed operating bands in Asia and South America

| Operating bands | Band 1 | Band 3 | Band 5 | Band 7 | Band8 | Band26 | Band27 | Band28 | Band38 | Band40 | Band42 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidths Country | 2100MHz | 1800MHz | 850MHz | 2600MHz | 900MHz | Upper 850MHz | Lower 850MHz | 700MHz | 2600MHz | 2300MHz | 3500MHz |
| South Korea | X | X | X | X | X | X |  | X |  | X |  |
| Australia | X | X | X |  | X | F | F | X |  |  |  |
| Brazil | X | X | X | X | X | F | F | F | X |  | X |
| Costa Rica | X | X | X |  |  |  |  |  |  |  |  |
| Hong Kong | X | X | X |  | X |  |  |  |  |  |  |
| Israel | X | X | X |  |  | F | F |  |  |  |  |
| New Zealand | X | X | X |  | X | F | F | F |  |  |  |
| Note: X means the operating band deployed in the country, F means the operating band could be deployed in the country in the future. |  |  |  |  |  |  |  |  |  |  |  |

For the B4+B7 CA combinations, the deployed operating bands in South America are summarized in table 5.2-4.

Table 2. Deployed operating bands in North/South America

| Operating bands | Band 2 | Band 4 | Band 5 | Band 7 | Band10 | Band12 | Band13 | Band14 | Band17 | Band27 | Band28 | Band29 | Band38 | Band42 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidths Country | 1900 MHz | 2100 MHz | 850 MHz | 2600 MHz | 2100 MHz | Lower 700MHz | Mid 700MHz | Upper 700MHz | Lower 700MHz | 800 MHz |  |  | 2600 MHz | 3500 MHz |
| Canada | X | X | X | X | F | X | X | X | X |  |  | X | X |  |
| Mexico | X | X | X | F | F |  |  |  |  | X | F |  | F | X |
| Chile | X | X | X | X | F |  |  |  |  | X | F |  | X |  |
| Colombia | X | X | X | X | F |  |  |  |  | X | F |  | X |  |
| Argentina | X | X | X | X | F |  |  |  |  | X | F |  | X | X |
| Note: X means the operating band deployed in the country, F means the operating band could be deployed in the country in the future. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

From the table5.2-3 and 5.2-4, we can define the protect band lists for 2ULs inter-band CA UE in inter-regions as shown in table 5.2-2.

********************** Omitted of No changes to TR 36.860 ***********************

### 5.2.5		Dual Uplink CA UE Coexistence requirements

For inter-band carrier aggregation with the uplink assigned to two E-UTRA bands the requirements in Table 5.2.5-1 applies for CA_1A-5A UE as an example.

NOTE: The requirements in 2 UL inter-band CA UE to UE co-existence table could be verified by measuring spurious emissions [at least] at the specific frequencies where second and third order intermodulation products generated by the two transmitted carriers can occur; in that case, the requirements for remaining applicable frequencies in 2 UL inter-band CA UE to UE co-existence table would be considered to be verified by the measurements verifying the 1 UL inter-band CA UE to UE co-existence requirements.

Table 5.2.5-1: Requirements

| E-UTRA CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | Note |
| CA_1A-5A | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 21, 22, 28, 31, 38, 40, 42, 43, 44 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3,34 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA band 26 | FDL_low | - | FDL_high | -27 | 1 |  |
|  | E-UTRA band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 15,27 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 15, 26, 27 |
|  | Frequency range | 1900 | - | 1915 | -15.5 | 5 | 15, 26, 27 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 15, 26, 27 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 6, 8, 15 |
|  | Frequency range | 1839.9 | - | 1879.9 | -50 | 1 | 15 |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd, 3rd or 4th harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 6: Applicable when NS_05 in section 6.6.3.3.1 is signalled by the network. NOTE 8: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. NOTE 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 26: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 27: This requirement is applicable for an uplink transmission bandwidth less than or equal to 54 RB for carriers of 15 MHz bandwidth when carrier center frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier center frequency is within the range 1930 - 1938 MHz. This requirement is applicable without any other uplink transmission bandwidth restriction for channel bandwidths within the range 1920 - 1980 MHz. NOTE XX: CA 1A-5A UE does not need to provide protection for Band 20 and 27 beyond the SEM limits. |  |  |  |  |  |  |  |

********************** End of text proposal to TR 36.860 ***********************