3GPP TSG-RAN WG4 Meeting #68bis	R4-134966

Riga, Latvia, October 7th – 11th, 2013

Agenda item:	8.24

Source: 	MediaTek Inc.

Title: 	More on UE inter-band 2UL CA analysis: A4 scope

Document for:	Discussion

## 1	Introduction

UE inter-band CA class A4 is currently defined as “low-low, low-high, or high-high band combination with intermodulation problem (low order IM)” [1]. Up till now only IMD2 and IMD3 from 2UL with direct frequency overlap on DL carriers was considered. In RAN4 #68 meeting, it was brought up that IMD3 impinging upon the 3rd order harmonic of DL LO may cause desensitization and should be considered [2]. In this contribution, we provide further analysis on low-order IMD overlapping with 3rd order harmonic of DL LO and share our view on A4 scope.

## 2	Discussion

Switching mixers have been commonly employed in radio frequency conversion circuits which are known to have rich odd harmonic contents in LO. Consequently, a direct conversion radio receiver may suffer from sensitivity degradation by the interference located at odd harmonics of Rx LO due to harmonic mixing in frequency conversion process. Among all LO harmonics, the 3rd order one is of particular concern as its magnitude is strongest and only 9 dB below the main tone.

Owing to this concern, in last RAN4 meeting, it was proposed that in inter-band 2UL CA desensitization analysis, IMD3 impinging upon the 3rd order harmonic of DL LO should be considered [2]. In fact, for the analysis to be more inclusive, we think IMD2 should also be taken into consideration. In the following sections, we provide both frequency analysis as well as desensitization level analysis on this particular IMD issue to conclude our view on A4 scope.

#### Frequency analysis

### IMD2 overlapping with 3rd order harmonic of DL LO

### This condition will only take place under,

- High-low or mid-low band combinations

- IMD2 is generated by frequency sum of 2 UL carriers.

- Low band DL is the victim band.

And that can also be described by the following equation,

f1 + f2 = 3*(f2 + f2_dup) or f1 = 2*f2 + 3*f2_dup

where f1 is high/mid band UL carrier frequency, f2 is low band UL carrier frequency, and f2_dup is low band duplex gap. Based on this equation, we can investigate all high-low and mid-low band CA combinations to determine which CA band would encounter this issue. The result is summarized in Table 2.1.1-1 where the band combinations having this issue are highlighted in yellow.

| B1-Bx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| B1 UL Range -> |  |  |  | f1L | f1H |
|  |  |  |  | 1920 | 1980 |
|  | f2L | f2H | f2_dup | 2*f2L+3*f2_dup | 2*f2H+3*f2_dup |
| B5 | 824 | 849 | 45 | 1783 | 1833 |
| B8 | 880 | 915 | 45 | 1895 | 1965 |
| B18 | 815 | 830 | 45 | 1765 | 1795 |
| B19 | 830 | 845 | 45 | 1795 | 1825 |
| B26 | 814 | 849 | 45 | 1763 | 1833 |
| B2-Bx |  |  |  |  |  |
| B2 UL Range -> |  |  |  | f1L | f1H |
|  |  |  |  | 1850 | 1910 |
|  | f2L | f2H | f2_dup | 2*f2L+3*f2_dup | 2*f2H+3*f2_dup |
| B5 | 824 | 849 | 45 | 1783 | 1833 |
| B12 | 699 | 716 | 30 | 1488 | 1522 |
| B13 | 777 | 787 | -31 | 1461 | 1481 |
| B17 | 704 | 716 | 30 | 1498 | 1522 |
| B3-Bx |  |  |  |  |  |
| B3 UL Range -> |  |  |  | f1L | f1H |
|  |  |  |  | 1710 | 1785 |
|  | f2L | f2H | f2_dup | 2*f2L+3*f2_dup | 2*f2H+3*f2_dup |
| B5 | 824 | 849 | 45 | 1783 | 1833 |
| B8 | 880 | 915 | 45 | 1895 | 1965 |
| B19 | 830 | 845 | 45 | 1795 | 1825 |
| B20 | 832 | 862 | -41 | 1541 | 1601 |
| B26 | 814 | 849 | 45 | 1763 | 1833 |
| B28 | 703 | 748 | 55 | 1571 | 1661 |
| B4-Bx |  |  |  |  |  |
| B4 UL Range -> |  |  |  | f1L | f1H |
|  |  |  |  | 1710 | 1755 |
|  | f2L | f2H | f2_dup | 2*f2L+3*f2_dup | 2*f2H+3*f2_dup |
| B5 | 824 | 849 | 45 | 1783 | 1833 |
| B12 | 699 | 716 | 30 | 1488 | 1522 |
| B13 | 777 | 787 | -31 | 1461 | 1481 |
| B17 | 704 | 716 | 30 | 1498 | 1522 |
| B11-B18 CA |  |  |  |  |  |
| B11 UL Range -> |  |  |  | f1L | f1H |
|  |  |  |  | 1427.9 | 1447.9 |
|  | f2L | f2H | f2_dup | 2*f2L+3*f2_dup | 2*f2H+3*f2_dup |
| B18 | 815 | 830 | 45 | 1765 | 1795 |
| B12-B25 CA |  |  |  |  |  |
| B25 UL Range -> |  |  |  | f1L | f1H |
|  |  |  |  | 1850 | 1915 |
|  | f2L | f2H | f2_dup | 2*f2L+3*f2_dup | 2*f2H+3*f2_dup |
| B12 | 699 | 716 | 30 | 1488 | 1522 |
| B19-B21 CA |  |  |  |  |  |
| B21 UL Range -> |  |  |  | f1L | f1H |
|  |  |  |  | 1447.9 | 1462.9 |
|  | f2L | f2H | f2_dup | 2*f2L+3*f2_dup | 2*f2H+3*f2_dup |
| B19 | 830 | 845 | 45 | 1795 | 1825 |

Table 2.1.1-1 Analysis table for IMD2 overlapping with 3rd order harmonic of DL LO

### IMD3 overlapping with 3rd order harmonic of DL LO

This condition can be met in high-high or low-low band combinations when,

2*f1 + f2 = 3*(f1 + f1_dup) or f2 = f1 + 3*f1_dup

2*f2 + f1 = 3*(f1 + f1_dup) or f2 = f1 + (3/2)*f1_dup

2*f1 + f2 = 3*(f2 + f2_dup) or f1 = f2 + (3/2)*f2_dup

2*f2 + f1 = 3*(f2 + f2_dup) or f1 = f2 + 3*f2_dup

where f1 is first CA band UL carrier frequency, f2 is second CA band UL carrier frequency, f1_dup is first CA band duplex gap, and f2_dup is second CA band duplex gap. Based on these equations, we can investigate all high-high and low-low band CA combinations to determine which CA band would encounter this issue. The result is summarized in Table 2.1.2-1, where none of the band combinations is found to have this issue.

|  | fL | fH | fdup | fL + (3/2)*fdup | fH+(3/2)*fdup | fL+3*fdup | fH+3*fdup |
| --- | --- | --- | --- | --- | --- | --- | --- |
| B2 | 1850 | 1910 | 80 | 1970 | 2030 | 2090 | 2150 |
| B4 | 1710 | 1755 | 400 | 2310 | 2355 | 2910 | 2955 |
| B3 | 1710 | 1785 | 95 | 1852.5 | 1927.5 | 1995 | 2070 |
| B7 | 2500 | 2570 | 120 | 2680 | 2750 | 2860 | 2930 |
| B4 | 1710 | 1755 | 400 | 2310 | 2355 | 2910 | 2955 |
| B7 | 2500 | 2570 | 120 | 2680 | 2750 | 2860 | 2930 |
| B5 | 824 | 849 | 45 | 891.5 | 916.5 | 959 | 984 |
| B12 | 699 | 716 | 30 | 744 | 761 | 789 | 806 |
| B5 | 824 | 849 | 45 | 891.5 | 916.5 | 959 | 984 |
| B17 | 704 | 716 | 30 | 749 | 761 | 794 | 806 |
| B8 | 880 | 915 | 45 | 947.5 | 982.5 | 1015 | 1050 |
| B20 | 832 | 862 | -41 | 770.5 | 800.5 | 709 | 739 |

Table 2.1.2-1 Analysis table for IMD3 overlapping with 3rd order harmonic of DL LO (H-H, L-L)

The same condition can also be met under,

- High-low or mid-low band combinations

- IMD3 is generated by 2 times high/mid band UL carrier frequency minus low band UL carrier frequency.

- Low band DL is the victim band.

And that can also be described by the following equation,

2*f1 - f2 = 3*(f2 + f2_dup) or f1 = 2*f2 + (3/2)*f2_dup

where f1 is high/mid band UL carrier frequency, f2 is low band UL carrier frequency, and f2_dup is low band duplex gap. Based on this equation, we can investigate all high-low and mid-low band CA combinations to determine which CA band would encounter this issue. The result is summarized in Table 2.1.2-2 where the band combinations having this issue are highlighted in yellow.

| B1-Bx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| B1 UL Range -> |  |  |  | f1L | f1H |
|  |  |  |  | 1920 | 1980 |
|  | f2L | f2H | f2_dup | 2*f2L+(3/2)*f2_dup | 2*f2H+(3/2)*f2_dup |
| B5 | 824 | 849 | 45 | 1715.5 | 1765.5 |
| B8 | 880 | 915 | 45 | 1827.5 | 1897.5 |
| B18 | 815 | 830 | 45 | 1697.5 | 1727.5 |
| B19 | 830 | 845 | 45 | 1727.5 | 1757.5 |
| B26 | 814 | 849 | 45 | 1695.5 | 1765.5 |
| B2-Bx |  |  |  |  |  |
| B2 UL Range -> |  |  |  | f1L | f1H |
|  |  |  |  | 1850 | 1910 |
|  | f2L | f2H | f2_dup | 2*f2L+(3/2)*f2_dup | 2*f2H+(3/2)*f2_dup |
| B5 | 824 | 849 | 45 | 1715.5 | 1765.5 |
| B12 | 699 | 716 | 30 | 1443 | 1477 |
| B13 | 777 | 787 | -31 | 1507.5 | 1527.5 |
| B17 | 704 | 716 | 30 | 1453 | 1477 |
| B3-Bx |  |  |  |  |  |
| B3 UL Range -> |  |  |  | f1L | f1H |
|  |  |  |  | 1710 | 1785 |
|  | f2L | f2H | f2_dup | 2*f2L+(3/2)*f2_dup | 2*f2H+(3/2)*f2_dup |
| B5 | 824 | 849 | 45 | 1715.5 | 1765.5 |
| B8 | 880 | 915 | 45 | 1827.5 | 1897.5 |
| B19 | 830 | 845 | 45 | 1727.5 | 1757.5 |
| B20 | 832 | 862 | -41 | 1602.5 | 1662.5 |
| B26 | 814 | 849 | 45 | 1695.5 | 1765.5 |
| B28 | 703 | 748 | 55 | 1488.5 | 1578.5 |
| B4-Bx |  |  |  |  |  |
| B4 UL Range -> |  |  |  | f1L | f1H |
|  |  |  |  | 1710 | 1755 |
|  | f2L | f2H | f2_dup | 2*f2L+(3/2)*f2_dup | 2*f2H+(3/2)*f2_dup |
| B5 | 824 | 849 | 45 | 1715.5 | 1765.5 |
| B12 | 699 | 716 | 30 | 1443 | 1477 |
| B13 | 777 | 787 | -31 | 1507.5 | 1527.5 |
| B17 | 704 | 716 | 30 | 1453 | 1477 |
| B11-B18 CA |  |  |  |  |  |
| B11 UL Range -> |  |  |  | f1L | f1H |
|  |  |  |  | 1427.9 | 1447.9 |
|  | f2L | f2H | f2_dup | 2*f2L+(3/2)*f2_dup | 2*f2H+(3/2)*f2_dup |
| B18 | 815 | 830 | 45 | 1697.5 | 1727.5 |
| B12-B25 CA |  |  |  |  |  |
| B25 UL Range -> |  |  |  | f1L | f1H |
|  |  |  |  | 1850 | 1915 |
|  | f2L | f2H | f2_dup | 2*f2L+(3/2)*f2_dup | 2*f2H+(3/2)*f2_dup |
| B12 | 699 | 716 | 30 | 1443 | 1477 |
| B19-B21 CA |  |  |  |  |  |
| B21 UL Range -> |  |  |  | f1L | f1H |
|  |  |  |  | 1447.9 | 1462.9 |
|  | f2L | f2H | f2_dup | 2*f2L+(3/2)*f2_dup | 2*f2H+(3/2)*f2_dup |
| B19 | 830 | 845 | 45 | 1727.5 | 1757.5 |

Table 2.1.2-2 Analysis table for IMD3 overlapping with 3rd order harmonic of DL LO (H-L, M-L)

Table 2.1-1 further summarizes all the CA combinations with this potential IMD issue and their band class based on current definition. It is interesting to see that except B1-B8, all other band combinations already belong to class A4.

|  | Class | fRX | 3*fRX |
| --- | --- | --- | --- |
| B1-B8 | A1 | None | IMD2 |
| B3-B5 | A4 | IMD2 | IMD2, IMD3 |
| B3-B19 | A4 | IMD2 | IMD3 |
| B3-B26 | A4 | IMD2 | IMD2, IMD3 |
| B4-B5 | A4 | IMD2 | IMD3 |

Table 2.1-1 Summary of CA band combinations with IMD overlapping with 3rd order harmonic of DL LO

#### Desensitization level analysis

Compared to IMD falling directly onto DL carrier as in current A4 definition, the desensitization level of IMD locating at 3rd order harmonic of DL LO shall be much lower as,

- Receive band filter in duplexer will provide substantial attenuation to the IMD.

- PA forward or reverse mixing IMD would be lower outside of operation band.

- Transceiver RF front-end may also provide some rejection to far-out interference.

- DL mixer LO 3rd order harmonic is at least 9 dB lower than fundamental.

If based on the following assumptions,

- Duplexer receive filter out-of-band rejection in is 45 dB.

- DL mixer LO 3rd order harmonic is 9 dB lower than fundamental.

- UE desensitization noise requirement is -110 dBm [3].

The required IMD power level to cause desensitization would be above -56 dBm.

According to the measurement data in Nokia’s contribution in RAN4 #66bis for IMD generated through PA reverse mixing [4], we can deduce,

- IMD2 resulting from f1+f2 is lower than -62 dBm.

- IMD3 resulting from 2*f1+f2 or 2*f2+f1 is lower than -48 dBm.

- IMD3 resulting from 2*f1-f2 shall be relatively weak (proven by measurement and theory) and can be left out in the analysis.

Also according to the measurement data in LGE’s and MediaTek’s contributions in RAN4 #68 for IMD generated by passive devices [5-6], the IMD power level was also relatively weak and can be left out in the analysis.

Based on the above analysis, we can conclude,

- IMD2 located at 3rd order harmonic of DL LO can be excluded from A4 consideration (like B1-B8).

- IMD3 resulting from 2*f1-f2 can be excluded from A4 consideration.

- Further study is required for IMD3 resulting from 2*f1+f2 or 2*f2+f1 to determine if it needs to be included in A4 consideration, though none of all current CA combinations has this issue.

## 3	Conclusion

In this paper, we provide both qualitative and quantitative analyses on inter-band 2UL CA with lower order IMD overlapping with 3rd order harmonic of DL LO. Based on frequency analysis, it is identified that there are 5 CA band combinations subjected to this potential issue, where 4 of them have already been classified as A4. Based on desensitization level analysis, we further draw the following conclusions,

- IMD2 located at 3rd order harmonic of DL LO can be excluded from A4 consideration (like B1-B8).

- IMD3 resulting from 2*f1-f2 can be excluded from A4 consideration.

- Further study is required for IMD3 resulting from 2*f1+f2 or 2*f2+f1 to determine if it needs to be included in A4 consideration, though none of all current CA combinations has this issue.

## 4	References

- 3GPP TR 36.850 V11.1.0 (2013-07)

- R4-134090, “2UL inter-band CA intermodulation products” Qualcomm Incorporated, TSG-RAN WG4 Meeting #68, Barcelona, Spain, August 19th – 23rd, 2013

- R4-134965, “UE self-desensitization noise density requirement”, MediaTek Inc., TSG-RAN WG4 Meeting #68bis, Riga, Latvia, October 7th – 11th, 2013

- R4-131208, “Interband CA 2 UL IMD measurements”, Nokia Corporation, TSG-RAN WG4 Meeting #66bis, Chicago, USA, April 15th – 19th, 2013

- R4-133793, “Passive IMD measurements levels of inter-band CA_1A-5A UE with 2ULs”, LG Electronics, TSG-RAN WG4 Meeting #68, Barcelona, Spain, August 19th – 23rd, 2013

- R4-133544, “UE Antenna switch IMD measurement for 2UL inter-band CA consideration”, MediaTek Inc., TSG-RAN WG4 Meeting #68, Barcelona, Spain, August 19th – 23rd, 2013