3GPP TSG RAN WG4 Meeting #68BIS	R4-135175

Riga, Latvia, 7th – 11th October, 2013

Agenda Item:	8.23

Source:	LG Electronics, LG UPlus

Title:	TP for UE-to-UE coexistence requirement for 2UL inter-band CA

Document for:	Approval

- Introduction

In the last RAN4 #68 meetings, Way forward of UE coexistence requirements for 2ULs inter-band CA was discussed as follow.

  - WF1: The 2UL co-existence table will be specified in 36.101, however it is FFS whether the 2UL tests are needed or not.

  - WF2: It is FFS whether 2UL tables and thus tests are needed or not.

However, there were some unclear points such as inter-regions issues and WG responsibility between RAN4 and RAN5 for 2ULs UE coexistence requirements tests.

In this contribution, we propose a general test method for UE-to-UE coexistence requirements for 2ULs inter-band CA and revised Way forward to progress of 2ULs inter-band CA.

- General UE-to-UE Coexistence Requirements for Inter-regions CA Combination

From the e-mail discussion after RAN4 #68 meeting,  there was consensus on WF1 to add the inter-regions bands in the protected band lists in Table 6.6.3.2-1 of TS36.101[1] for single UL UE in Table2.

To define the protected band lists for inter-region issues, we used option 5 in [3].

- Option5) Choose a subset of protected frequency ranges/bands to protect based on spectrum allocations in the countries where the two bands are deployed

For the B1+B5, B3+B5 CA combinations, the deployed operating bands in Asia and South America are summarized in table 1.

Table 1. Deployed operating bands in Asia and South America

| Operating bands | Band 1 | Band 3 | Band 5 | Band 7 | Band8 | Band26 | Band27 | Band28 | Band38 | Band40 | Band42 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidths Country | 2100MHz | 1800MHz | 850MHz | 2600MHz | 900MHz | Upper 850MHz | Lower 850MHz | 700MHz | 2600MHz | 2300MHz | 3500MHz |
| South Korea | X | X | X | X | X | X |  | X |  | X |  |
| Australia | X | X | X |  | X |  | X | X |  |  |  |
| Brazil | X | X | X | X | X | X | X | X | X |  | X |
| Costa Rica | X | X | X |  |  |  |  |  |  |  |  |
| Hong Kong | X | X | X |  | X |  |  |  |  |  |  |
| Israel | X | X | X |  |  |  |  |  |  |  |  |
| New Zealand | X | X | X |  | X |  |  |  |  |  |  |

From the table1, we can define the protect band lists for inter-band CA 1A-5A UE.

Table 2: Proposed protected band lists for 2ULs inter-band CA in single UL coexistence requirements

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | Note |
| 1 | E-UTRA Band 1, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 38, 40, 41, 42, 43, 44 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 5 | FDL_low | - | FDL_high | -50 | 1 | 32 |
|  | E-UTRA Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | Frequency range | 1880 |  | 1895 | -40 | 1 | 15,27 |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 15, 26, 27 |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 15, 26, 27 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 6, 8, 15 |
|  | Frequency range | 1839.9 | - | 1879.9 | -50 | 1 | 15 |
| 2 | E-UTRA Band 4, 5, 10, 12, 13, 14, 17, 22, 23, 24, 26, 27, 28, 29, 30, 41, 42 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 3 | E-UTRA Band 1, 7, 8, 20, 26, 27, 28, 31, 33, 34, 38, 41, 43, 44 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 5 | FDL_low | - | FDL_high | -50 | 1 | 32 |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 13 |
|  | E-UTRA Band 22, 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 13 |
| 4 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 22, 23, 24, 25, 26, 27, 28, 29, 30, 41, 43 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 7 | FDL_low | - | FDL_high | -50 | 1 | 32 |
|  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 5 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 22, 23, 24, 25, 28, 29, 30, 31, 42, 43 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 3, 7, 8, 38, 40 | FDL_low | - | FDL_high | -50 | 1 | 32 |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
| 6 | E-UTRA Band 1, 9, 11, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 860 | - | 875 | -37 | 1 |  |
|  | Frequency range | 875 | - | 895 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1919.6 | -41 | 0.3 | 7 |
|  |  | 1884.5 | - | 1915.7 |  |  | 8 |
| 7 | E-UTRA Band 1, 3, 7, 8, 20, 22, 27, 28, 29, 30, 31, 33, 34, 42, 43 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4 | FDL_low | - | FDL_high | -50 | 1 | 32 |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 15, 21, 26 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 15, 21, 26 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 15, 21 |
| NOTE 32: Applicable when UE supports 2ULs inter-band CA |  |  |  |  |  |  |  |

For the B4+B7 CA combinations, RAN4 needs to summarize the deployed operating bands in North & South America, Asia and Europe. It will be FFS.

- Way Forward on UE Coexistence Requirements for Dual UL CA

After RAN4 #68 meeting in Barcelona, we had E-mail discussion to solve the unclear issues in Way forward at last RAN4 meeting with some interested companies.

Until now, RAN4 has made general and some specific test requirements using general simulation assumptions and general architecture. This means RAN4 has not considered all the possible UE architecture and implementable structures of DUT since standards precedes the implementations. In some possible DUTs and architectures, some uncertain problems and issues can happen, but this should be resolved through implementation in UE side.

Therefore, we had analyzed IMD levels of 2ULs inter-band CA in general architecture.

From the analysis and measurements, RAN4 has seen that there was no IMD issue for 2ULs inter-band CA combinations.  But some companies expressed concern that the generated IMD products can exceed the general coexistence level of -50dBm/MHz in some specific frequency ranges.

Therefore, compromised WF that has been proposed in last RAN4 meeting as follow,

  - WF1: The 2UL co-existence table will be specified in 36.101, however it is FFS whether the 2UL tests are needed or not.

  - WF2: It is FFS whether 2UL tables and thus tests are needed or not.

WF1 has been agreed to put the protect bands in table 6.6.3.2-1 of TS36.101 as a result of e-mail discussion. For WF2, however, there was some ambiguity in interpreting the responsibility and inter-regions issues for 2ULs CA tests.

So, we proposed two options for Way Forward to progress the 2ULs inter-band CA.

Way Forward:

Option1: The 2ULs co-existence requirements table will be specified in TS36.101 in Rel-12 for some exception cases.

For the option1, RAN4 can determine some exception cases among 2ULs inter-band CA combinations and include this exceptional case into the UE coexistence table. Some exception cases could be needed for specific FE architectures, specific DUT and specific RF components.

Interested companies can provide this example as an exception case to add test requirements. And RAN5 would make detailed test methods for UE coexistence requirements in these specific cases.

Option2: The 2ULs co-existence requirements table will be specified in TS36.101 in Rel-12.

For the option2, RAN4 only adds 2ULs co-existence requirements table in TS36.101. And RAN5 decide whether or not to test 2ULs UE coexistence in TS35.521-1. And RAN5 will make detailed test methods for UE coexistence requirements in some specific bands. RAN5 should verify the duplicated IMD impacts and down-select the necessary test cases among all 2ULs CA combinations, which seems not desirable.

Among two options, we have preference on the option1 in WF.

- Conclusion

In this contribution, we propose the protected band lists for 2ULs inter-band CA in single UL coexistence requirements as shown in table2. And also we propose Way Forward as follow

Way Forward:

Option1: The 2ULs co-existence requirements table will be specified in TS36.101 in Rel-12 for some exception cases.

Option2: The 2ULs co-existence requirements table will be specified in TS36.101 in Rel-12.

- Reference

- TS 36.101 v11.6.0

- R4-133062, “Dual uplink CA intermodulation analysis,” Renesas Mobile

- R4-131762, “Dual Uplink CA UE Emissions Discussion,” NII Holdings

- R4-132774, “2UL CA UE coexistence,” Qualcomm

- R4-134389, “Passive IMD measurements levels of inter-band CA_1A-5A UE with 2ULs,” LG Electronics

- R4-131986, “WF on the protected Bands for inter-band CA with 2ULs,” LG U+, NII, KT, Nokia

- TR 36.860 v0.3.0

- R4-131416, “Protected Band for B1+B5 2 UP LINK CA,” LG U+

- R4-131612, “Uplink inter-band CA: specification of spurious emissions,” Ericsson,ST-Ericsson

Text proposal for TR 36.860

********************** Start of text proposal to TR 36.860 ***********************

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 30.007: “Guideline on WI/SI for new Operating Bands”

[3]	3GPP TR 36.850: “Inter-band Carrier Aggregation Technical Report (Release 11)

[4]	3GPP TR 36.851: “Inter-band Carrier Aggregation Technical Report (Release 12)

[5]	3GPP TS 36.101: “ E-UTRA UE radio transmission and reception (Release 12)

[6]	R4-131762, “Dual Uplink CA UE Emissions Discussion,” NII Holdings

[7]	R4-132713, “Handling of potential intermodulation problems to other systems than own bands,” Renesas Mobile Europe Ltd.

[8]	R4-133060, “General coexistence analysis table for inter-band CA with 2UL, ZTE, Renesas Europe, LG Electronics

[9]	R4-133062, “Dual uplink CA intermodulation analysis,” Renesas Mobile Europe Ltd.

[10]	R4-134389, “Passive IMD measurements levels of inter-band CA_1A-5A UE with 2ULs,” LG Electronics

[11]	R4-134418, “TP for general frequency bands for WiFi, GNSS and Digital TV for 2ULs inter-band CA,” LG Electronics

< ommited of  no changes >

5.2		UE specific

Most dual-uplink inter-band CA combinations are from the same regions as shown in table 5.2-1. But some inter-band CA combinations are from the inter-regions such as B1+B5, B3+B5, B4+B7 and B39+B41 combinations.

Table 5.2-1: Dual-uplink inter-band CA combination lists

| Class |  | Band Combination | Frequency | Deployment Regions |
| --- | --- | --- | --- | --- |
| Dual-uplink Inter-band CA | A1 (HW) | B1+5 | 2.1G+800M | Inter-regions (Region 1+2+3) |
|  |  | B1+B19 | 2.1G+800M | Region 3 |
|  |  | B3+20 | 1.8G+800M | Region 1 |
|  | A2 (QC) | B3+8 | 1.8G+900M | Region 3 |
|  |  | B4+12 | 2.1G+700M | Region 2 |
|  |  | B4+17 | 2.1G+700M | Region 2 |
|  | A3 (Ericsson) | B1+7 | 2.1G+2.6G | Region 1 |
|  |  | B3+B7 | 1.7G+2.6G | Region 1 |
|  |  | B4+B7 | 1.7G+2.6G | Inter-regions (Region 1+2+3) |
|  |  | B5+B12 | 800M+700M | Region 2 |
|  |  | B5+B17 | 800M+700M | Region 2 |
|  |  | B39+B41 | 1.9G+2.6G | Inter-regions (Region 2+3) already protect between them |
|  | A4 (Nokia) | B3+5 | 1.8G+800M | Inter-regions (Region 1+2+3) |
|  |  | B2+B4 | 1.9G + 2.1G | Region 2 |
|  |  | B7+B20 | 2.6G+800M | Region 1 |
|  | A5 (Ren.) | B1+B21 | 2.1G+1.4G | Region 3 |

For the 1UL/2DL inter-band CA UE, the aggressor signal is generated in 1 UL band same as rel-8/9 UE. So it is possible to follow the conventional UE-to-UE coexistence requirements that are described in table 6.6.3.2-1[5]. However, the frequency range of emission level for IMD and Harmonic from 2UL inter-band CA needs to be analyzed and measured to determine the adjacent UE coexistence requirements.

From the e-mail discussion after RAN4 #68 meeting,  there was consensus to add the inter-regions bands in the protected band lists in table 6.6.3.2-1 [5] for single UL UE as follow.

Table 5.2-2: Proposed protected band lists for 2ULs inter-band CA in single UL coexistence requirements

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | Note |
| 1 | E-UTRA Band 1, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 38, 40, 41, 42, 43, 44 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 5 | FDL_low | - | FDL_high | -50 | 1 | 32 |
|  | E-UTRA Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | Frequency range | 1880 |  | 1895 | -40 | 1 | 15,27 |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 15, 26, 27 |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 15, 26, 27 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 6, 8, 15 |
|  | Frequency range | 1839.9 | - | 1879.9 | -50 | 1 | 15 |
| 2 | E-UTRA Band 4, 5, 10, 12, 13, 14, 17, 22, 23, 24, 26, 27, 28, 29, 30, 41, 42 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 3 | E-UTRA Band 1, 7, 8, 20, 26, 27, 28, 31, 33, 34, 38, 41, 43, 44 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 5 | FDL_low | - | FDL_high | -50 | 1 | 32 |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 13 |
|  | E-UTRA Band 22, 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 13 |
| 4 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 22, 23, 24, 25, 26, 27, 28, 29, 30, 41, 43 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 7 | FDL_low | - | FDL_high | -50 | 1 | 32 |
|  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 5 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 22, 23, 24, 25, 28, 29, 30, 31, 42, 43 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1,3, 7, 8, 38, 40 | FDL_low | - | FDL_high | -50 | 1 | 32 |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
| 6 | E-UTRA Band 1, 9, 11, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 860 | - | 875 | -37 | 1 |  |
|  | Frequency range | 875 | - | 895 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1919.6 | -41 | 0.3 | 7 |
|  |  | 1884.5 | - | 1915.7 |  |  | 8 |
| 7 | E-UTRA Band 1, 3, 7, 8, 20, 22, 27, 28, 29, 30, 31, 33, 34, 42, 43 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4 | FDL_low | - | FDL_high | -50 | 1 | 32 |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 15, 21, 26 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 15, 21, 26 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 15, 21 |
| NOTE 32: Applicable when UE supports 2ULs inter-band CA |  |  |  |  |  |  |  |

To define the protected band lists for inter-region issues, we used option 5 in [6].

- Option5) Choose a subset of protected frequency ranges/bands to protect based on spectrum allocations in the countries where the two bands are deployed

For the B1+B5, B3+B5 CA combinations, the deployed operating bands in Asia and South America are summarized in table 5.2-3.

Table 5.2-3. Deployed operating bands in Asia and South America

| Operating bands | Band 1 | Band 3 | Band 5 | Band 7 | Band8 | Band26 | Band27 | Band28 | Band38 | Band40 | Band42 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidths Country | 2100MHz | 1800MHz | 850MHz | 2600MHz | 900MHz | Upper 850MHz | Lower 850MHz | 700MHz | 2600MHz | 2300MHz | 3500MHz |
| South Korea | X | X | X | X | X | X |  | X |  | X |  |
| Australia | X | X | X |  | X |  | X | X |  |  |  |
| Brazil | X | X | X | X | X | X | X | X | X |  | X |
| Costa Rica | X | X | X |  |  |  |  |  |  |  |  |
| Hong Kong | X | X | X |  | X |  |  |  |  |  |  |
| Israel | X | X | X |  |  |  |  |  |  |  |  |
| New Zealand | X | X | X |  | X |  |  |  |  |  |  |

From the table 5.2-3, we can define the protect band lists for inter-band CA 1A-5A UE.

For the B4+B7 CA combinations, RAN4 needs to summarize the deployed operating bands in North & South America, Asia and Europe. It will be FFS.

********************** End of text proposal to TR 36.860  ***********************