3GPP TSG RAN WG4 Meeting #68BIS	R4-135192

- Riga, Latvia, 7th – 11th October, 2013

- Agenda Item:	10.1

- Source:	LG Electronics

- Title:	TP for S-band UE Tx/Rx RF requirements for standalone Band

Document for:	Approval

- Introduction

In the RAN Plenary #57 meetings, the study for the LTE S-band in Korea was approved. And the UE-to-UE coexistence analysis and requirements was approved [1] for TR 36.861 v0.2.0 in RAN4 #67 meeting.

One of the objectives for this study item is to facilitate and harmonize the efficient use of these bands for terrestrial IMT, especially LTE, for Korea.

TDD Band 34 is considered the coexistence band for S-band UE but SI rapporteur request is not approved in RAN plenary and RAN WG4.

In this contribution, we provide the UE transmitter/Receiver RF requirements based on the deployments scenarios of new standalone S-band.

- References

- R4-132400, “TR 36.861 v0.2.0,” SKT

- R4-133145, “Coexistence with Band 34,” KT, SK Telecom, LGU+, ETRI

- TS 36.101 v11.6.0

- R4-135186, “UE coexistence analysis to protect Band 34,” LG Electronics

- R4-131390, “TP for TR ab.cde : General issue for FS_LTE_1980_2170_Korea,” LG Electronics

Text proposal for TR 36.861

********************** Start of text proposal to TR 36.861 chapter 8 ***********************

- 8	Study of E-UTRA specific issues

This section summarizes issues and additional requirements when S-band UE is deployed under Korean regulations.

- 8.1    UE Tx RF Requirements

- 8.1.1	New separate band (1980-2010 MHz and 2170-2200 MHz)

- 8.1.1.1 IMD and Harmonic analysis

In this section, we analyze the harmonic and IMD products which have impact on the bands in Korea.

Table 8.1.1.1-1 gives the harmonic products from S-band UE according to the non-CA UE. From Table 8.1.1.1-1, it can be seen that none of the harmonic products fall into the own S-band DL and the other deployed bands in Korea, which means there is no harmonic problem from S-band UE in Korea.

Table 8.1.1.1-1: S-band UE harmonic products

| UE UL carriers | f1_low | f1_high |
| --- | --- | --- |
| UL frequency (MHz) | 30MHz |  |
|  | 1980 | 2010 |
| 2nd order harmonics frequency range (MHz) | 3960 to 4020 |  |
| 3rd order harmonics frequency range (MHz) | 5940 to 6030 |  |

Table 8.1.1.1-2 shows the frequency ranges of IMD products for S-band UE according to multi-clustered transmission of non-CA UE. We can also see that there is no IMD products falling into the own S-band DL and the other deployed bands in Korea, which means there is no IMD problem from S-band UE in Korea.

Table 8.1.1.1-2: S-band UE IMD products

| UE UL carriers | f1_low | f1_high | f1_low | f1_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 10MHz |  | 20MHz |  |
|  | 1980 or 2000 | 1990 or 2010 | 1980 or 1990 | 2000 or 2010 |
| Two-tone 3rd order IMD products | ¦2*f1_low – f1_high¦ | ¦2*f1_high – f1_low¦ | ¦2*f1_low – f1_high¦ | ¦2*f1_high – f1_low¦ |
| IMD frequency range (MHz) | 1970 to 2000 or 1990 to 2020 |  | 1960 to 2020 or 1970 to 2030 |  |
| Two-tone 3rd order IMD products | (2*f1_low + f1_high) | (2*f1_high + f1_low) | (2*f1_low + f1_high) | (2*f1_high + f1_low) |
| IMD frequency range (MHz) | 5950 to 5960 or 6010 to 6020 |  | 5960 to 5980 or 5990 to 6010 |  |
| Two-tone 5th order IMD products | (3*f1_low – 2*f1_high) | (3*f1_high – 2*f1_low) | (3*f1_low – 2*f1_high) | (3*f1_high – 2*f1_low) |
| IMD frequency range (MHz) | 1960 to 2010 or 1980 to 2030 |  | 1940 to 2040 or 1950 to 2050 |  |
| Two-tone 7th order IMD products | (4*f1_low – 3*f1_high) | (4*f1_high -3*f1_low) | (4*f1_low – 3*f1_high) | (4*f1_high -3*f1_low) |
| IMD frequency range (MHz) | 1950 to 2020 or 1970 to 2040 |  | 1920 or 2060 or 1930 to 2070 |  |

8.1.1.2 	Spurious emission for UE co-existence

For the separate S-band, RAN4 consider same duplexer characteristics compare to Band 1 duplexer to make UE transmitter requirements.

Case1) Protection Bands without Band 34:

From the UE-to-UE coexistence analysis [5] and harmonic/IMD product analysis in this paper, we can see that there is no need to define A-MPR requirements and NS signalling to protect the bands.

As a conclusion, SE requirements for UE-to-UE coexistence can be defined as -50dBm/MHz for S-band UE as in Table 8.1.1.2-1.

Table 8.1.1.2-1: Spurious emission band UE co-existence of S-band UE

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Level (dBm) | Bandwidth (MHz) | Comment |
| XX | E-UTRA Band 1,3,5,8,26,XX,40 | FDL_low | - | FDL_high | -50 | 1 | 1 |
| NOTE 1: This requirement is applicable for S-band UE of Band XX. |  |  |  |  |  |  |  |

Case2) Protection Bands with Band 34:

When RAN4 consider that Band 34 can be deployed in same country and region, the SE requirements for UE-to-UE coexistence can be defined as in Table 8.1.1.2-2, which is exploiting the conventional TDD/FDD coexistence requirements. It should be verified the number of RB restriction and guard band to protect legacy Band 34 UE in the future.

Table 8.1.1.2-2: Spurious emission band UE co-existence of S-band UE

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Level (dBm) | Bandwidth (MHz) | Comment |
| XX | E-UTRA Band 1,3,5,8,26,XX,40 | FDL_low | - | FDL_high | -50 | 1 | 1 |
|  | Frequency range | 2010 | - | 2015 | +1.6 | 5 | 2, 3, 4 |
|  | Frequency range | 2015 | - | 2025 | -15.5 | 5 | 2, 3, 4 |
| NOTE 1: This requirement is applicable for S-band UE of Band XX. NOTE 2: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 3: This requirement is applicable for an uplink transmission bandwidth less than or equal to [54] RB for carriers of 15 MHz bandwidth when carrier center frequency is within the range [ ] – [ ] MHz and for carriers of 20 MHz bandwidth when carrier center frequency is within the range [ ] – [ ] MHz. No other restrictions apply for carriers with bandwidths confined in 1980-2010 MHz. NOTE 4: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |  |  |  |  |  |

If RAN4 decides to make new SE requirements for S-band UE coexistence instead of exploiting the conventional FDD/TDD coexistence requirements, then RAN4 should define network signalling message such as NS_XX to apply A-MPR mask and RB restrictions for S-band UE. The SE requirements for a new UE-to-UE coexistence will be FFS.

8.1.1.3	Other expected UE Tx RF requirements

In this section, the detailed descriptions on UE Tx RF requirements are proposed to describe any potential requirements changes needed for S-band UE.

Case1) Protection Bands without Band 34:

For this case, there is no need to define A-MPR requirements and NS signalling to protect the bands. So we proposed the detailed descriptions in table 8.1.1.3-1.

Table 8.1.1.3-1: Considering UE Tx RF requirements for S-band UE

| Description | Comments and Description of change |
| --- | --- |
| UE Maximum Output Power | No changes are needed for power class 3 due to the large duplex gap and Tx-Rx separation. |
| UE Maximum Output Power with additional requirements (A-MPR) | No changes are needed to protect adjacent coexistence bands due to no IMD and harmonics [1]. |
| Spurious emission/ additional Spurious emission | No changes are needed to protect whole frequency bands for S-band UE. |
| ACLR/SEM | No changes are needed in OOB ranges for S-band UE. |
| Additional Spectrum Emission Mask | No changes are needed to protect adjacent coexistence bands due to no IMD and harmonics [1]. |

Case2) Protection Bands with Band 34:

When RAN4 decides to protect band 34 exploiting the conventional FDD/TDD coexistence requirements, RAN4 can reuse the UE transmitter RF requirements as in table 8.1.1.3-1.

But, if RAN4 decide to make new SE requirements for S-band UE coexistence instead of exploiting the conventional FDD/TDD coexistence requirements, then RAN4 should define network signalling message such as NS_XX to apply A-MPR mask and RB restrictions for S-band UE. The other transmitter requirements for S-band UE will be FFS.

8.1.2	New Extended band (1920-2010 MHz and 2110-2200 MHz)

< <text will be added>.

  - UE Rx RF Requirements

    - New separate band (1980-2010 MHz and 2170-2200 MHz)

- 8.2.1.1 REFSENS requirements for S-band UE

Standalone S-band has same Tx-Rx frequency separation and total bandwidth of S-band is smaller than that of Band 1. Therefore S-band UE’s REFSENS level can be easily regarded as the same as that of Band 1. RF simulations are performed to verify the REFSENS requirements.

Case1) Protection Bands without Band 34:

Figure 8.2.1.1-1 show the simulation results of emission level in S-band DL lower bands. The red line indicates the leakage level with full RBs allocation in S-Band DL lower band.

- 20MHz S-band UE (fc=2000MHz)                      (b) 10MHz S-band UE (fc=2005MHz)

Figure 8.2.1.1-1 Simulation results of the Tx leakage level into S-band DL

The leakage level in DL band is about -75dBm/MHz, which is further attenuated by Duplexer Tx/Rx isolation level with 50dB. So the leakage level in DL band is -125dBm/MHz. Eq. 8-1 shows the impact of REFSENS in S-band DL induced by Tx leakage.

- REFSENS for S-band UE = Conventional REFSENS + Tx_leakage levels                          (Eq. 8-1)

= -94dBm/CH_BW +  -125dBm/MHz

= -107dBm/MHz + -125dBm/MHz = - 106.93dBm/MHz

= -93.93dBm/CH_BW

From Eq. 8-1, we can see that the REFSENS in own Rx band by Tx leakage is negligible.

As a conclusion, REFSENS requirements for S-band UE can be defined as -94dBm/20MHz in Table 8.2.1.1-1 with the same uplink configurations as in Band1 [2].

Table 8.2.1.1-1: Reference sensitivity QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| XX |  |  | -100 | -97 | -95.2 | -94 | FDD |

Case2) Protection Bands with Band 34:

The REFSENS level of own S-band is expected as same in Eq.8-1, but RAN4 should study how much impact there is when the S-band Tx leakage falls into the B34 lower band.

The S-band Tx leakage levels in B34 lower frequency ranges are about -16~ -33dBm/MHz according to the channel BW and frequency shift levels as shown in Figure 8.2.1.1-2. The emission level is calculated in Eq. 8-2 and the estimated REFSENS level is calculated in Eq.8-3

- Emission level in TDD B34 UE = Tx leakage level - MCL -  penetration loss                    (Eq.8-2)

= -16dBm/MHz - 54.44dB - 0dB = -70.44dBm/MHz

- REFSENS for band 34 UE = Conventional REFSENS + Emission levels in B34 UE         (Eq. 8-3)

= -94dBm/CH_BW +  -70.44dBm/MHz

= -107dBm/MHz + -70.44dBm/MHz = - 70.44dBm/MHz

= -57.439dBm/CH_BW

Figure 8.2.1.1-2 show the simulation results of emission level in band 34 lower bands. The brown line indicates the leakage level with full RBs allocation in Band 34.

From these simulation results, the REFSENS of Band 34 will be desensitized with 36.5dB. Hence it would not be possible to protect Band 34 without A-MPR, RB restriction and guard bands when band 34 and S-band is deployed in same regions and/or country.

- 0MHz shift (fc=2000MHz, CH_BW=20MHz)

- 5MHz shift (fc=1995MHz, CH_BW=20MHz)       (c) 10MHz shift (fc=1990MHz, CH BW=20MHz)

- 0MHz shift (fc=2005MHz, CH_BW=10MHz)

(e)  5MHz shift (fc=2000MHz, CH BW=10MHz)       (f) 10MHz shift (fc=1995MHz, CH BW=10MHz)

Figure 8.2.1.1-2 Tx leakage level in B34 lower edge frequency

As a conclusion, REFSENS level for extended S-band UE is estimated as conventional levels in TS36.101. However, the REFSENS of the Band 34 UE will be desensitized with 36.5dB by S-band UE. Therefore RAN4 should define NS_XX message to protect B34 UE from S-band UE. Network signalling message can be controlled the maximum output power of aggressor UE according to the RB starting position and number of contiguous RB allocation size.

- 8.2.1.2 Other UE Rx RF requirements

For the receiver requirements for S-band UE, we propose preliminary descriptions on UE receiver requirements according to the protect band list with/without B34.

Case1) Protection Bands without Band 34:

From the above REFSENS analysis, Tx leakage is expected to be negligible since the large duplexer spacing and same Tx-Rx frequency separation as Band1. Hence we can reuse the receiver requirements in Table 8.2.1.2-1 for S-band UE.

Table 8.2.1.2-1: Considering UE Rx RF requirements for S-band UE

| Description | Comments and Comparison to 2-band Solution |
| --- | --- |
| Adjacent Channel Selectivity | No changes since there is no Tx leakage impacts on the S-band DL for the T-put requirements that can be larger than or equal to 95% with same condition in section 7.5.1 [2] |
| Blocking characteristics | No changes since there is no Tx leakage impacts on the S-band DL for the T-put requirements can be larger than or equal to 95% with same condition in section 7.6 [2] |
| Receiver inter-modulation characteristics | No changes since there is no Tx leakage impacts on the S-band DL for the T-put requirements can be larger than or equal to 95% with same condition in section 7.8.1 [2] |

Case2) Protection Bands with Band 34:

When RAN4 decide to protect band 34 using the conventional FDD/TDD coexistence requirements, then RAN4 can reuse the UE receiver RF requirements as in table 8.2.2.2-1.

But, if RAN4 decide to make new SE requirements for S-band UE coexistence, then RAN4 should define network signalling message as NS_XX to apply A-MPR mask and RB restrictions for S-band UE.

8.2.2	New Extended band (1920-2010 MHz and 2110-2200 MHz)

< <text will be added>.

********************** End of text proposal to TR 36.861 Chapter 8 ***************************