3GPP TSG RAN WG4 Meeting #6868BIS	R4-135195

Riga, Latvia, 7th – 11th October, 2013

- Agenda Item:	10.1

- Source:	LG Electronics

- Title:	TP for S-band UE Tx/Rx RF requirements for the extended Band

Document for:	Approval

- Introduction

In the RAN Plenary #57 meetings, the study for the LTE S-band in Korea was approved. And the UE-to-UE coexistence analysis and requirements was approved [1] for TR 36.861 v0.2.0 in RAN4 #67 meeting.

One of the objectives for this study item is to facilitate and harmonize the efficient use of these bands for terrestrial IMT, especially LTE, for Korea.

TDD Band 34 is considered the coexistence band for S-band UE but SI rapporteur request is not approved in RAN plenary and RAN WG4.

In this contribution, we provide the UE transmitter/Receiver RF requirements based on the deployments scenarios of new extended S-band.

- References

- R4-132400, “TR 36.861 v0.2.0,” SKT

- R4-133145, “Coexistence with Band 34,” KT, SK Telecom, LGU+, ETRI

- TS 36.101 v11.6.0

- R4-135186, “UE coexistence analysis to protect Band 34,” LG Electronics

- R4-131390, “TP for TR ab.cde : General issue for FS_LTE_1980_2170_Korea,” LG Electronics

Text proposal for TR 36.861

********************** Start of text proposal to TR 36.861 Chapter 8 ***********************

- 8.1    UE Tx RF Requirements

- 8.1.1	New separate band (1980-2010 MHz and 2170-2200 MHz)

< <text will be added>.

- 8.1.2	New Extended band (1920-2010 MHz and 2110-2200 MHz)

- 8.1.2.1 IMD and Harmonic analysis

In this section, we analyze the harmonic and IMD products which have impact on the bands in Korea.

Table 8.1.2.1-1 gives the harmonic products from S-band UE according to multi-clustered transmission of non-CA UE. From Table 8.1.2.1-1, it can be seen that none of the harmonic products fall into the own S-band DL and the other deployed bands in Korea, which means there is no harmonic problem from S-band UE in Korea.

Table 8.1.2.1-1: S-band UE harmonic products

| UE UL carriers | f1_low | f1_high |
| --- | --- | --- |
| UL frequency (MHz) | 90MHz |  |
|  | 1920 | 2010 |
| 2nd order harmonics frequency range (MHz) | 3840 to 4020 |  |
| 3rd order harmonics frequency range (MHz) | 5760 to 6030 |  |

Table 8.1.2.1-2 show the frequency ranges of IMD products for S-band UE according to multi-clustered transmission of non-CA UE. We can see that the 7th order IMD products falling into the Band 3 DL band but the 7th order IMD level is less than -50dBm/MHz, so it can not have impact on the UE to UE co-existence requirements.  Therefore there is no IMD problem from S-band UE in Korea.

Table 8.1.2.1-2: S-band UE IMD products

| UE UL carriers | f1_low | f1_high | f1_low | f1_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 10MHz |  | 20MHz |  |
|  | 1920 or 2000 | 1930 or 2010 | 1920 or 1990 | 1940 or 2010 |
| Two-tone 3rd order IMD products | ¦2*f1_low – f1_high¦ | ¦2*f1_high – f1_low¦ | ¦2*f1_low – f1_high¦ | ¦2*f1_high – f1_low¦ |
| IMD frequency range (MHz) | 1910 to 1940 or 1990 to 2020 |  | 1900 to 1960 or 1970 to 2030 |  |
| Two-tone 3rd order IMD products | (2*f1_low + f1_high) | (2*f1_high + f1_low) | (2*f1_low + f1_high) | (2*f1_high + f1_low) |
| IMD frequency range (MHz) | 5770 to 5780 or 6010 to 6020 |  | 5780 to 5800 or 5990 to 6010 |  |
| Two-tone 5th order IMD products | (3*f1_low – 2*f1_high) | (3*f1_high – 2*f1_low) | (3*f1_low – 2*f1_high) | (3*f1_high – 2*f1_low) |
| IMD frequency range (MHz) | 1900 to 1950 or 1980 to 2030 |  | 1880 to 1980 or 1950 to 2050 |  |
| Two-tone 7th order IMD products | (4*f1_low – 3*f1_high) | (4*f1_high -3*f1_low) | (4*f1_low – 3*f1_high) | (4*f1_high -3*f1_low) |
| IMD frequency range (MHz) | 1890 to 1960 or 1970 to 2040 |  | 1860 to 2000 or 1930 to 2070 |  |

- 8.1.2.2 	Spurious emission for UE co-existence

For the coexistence requirements, a single/dual duplexer approaches can be considered, but the requirements should be defined as single requirements and should be met regardless of duplexer configuration.

Case1) Protection Bands without Band 34:

Even though the duplexer gap is decreased from 130MHz to 100MHz and pass band is increased from 60MHz to 90MHz, the single duplexer can keep the conventional duplexer filter characteristics when we consider the protect band list without Band 34. So the UE-to-UE coexistence analysis [5] is applicable to the extended S-band UE. We can also see that there is no need to define A-MPR/RB restriction and NS signalling to protect the bands.

As a conclusion of previous discussion, SE requirements for UE-to-UE coexistence can be defined as -50dBm/MHz for the extended S-band UE as in Table 8.1.2.2-1.

Table 8.1.2.2-1: Spurious emission band UE co-existence of the extended S-band UE

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Level (dBm) | Bandwidth (MHz) | Comment |
| XX | E-UTRA Band 1,3,5,8,26,XX,40 | FDL_low | - | FDL_high | -50 | 1 | 1 |
| NOTE 1: This requirement is applicable for S-band UE of Band XX. |  |  |  |  |  |  |  |

Case2) Protection Bands with Band 34:

In this case, RAN4 can consider dual duplexer approach to protect band 34 because dual duplexer approach has more emission rejection capability than the single duplexer approach when we consider protecting Band1 based on the current UE coexistence requirement of -50dBm/MHz. Therefore RAN4 can assume dual-duplexer filter for the extended S-bands.

For the extended bands operation, SE requirements for UE-to-UE coexistence can be defined exploiting the conventional TDD/FDD coexistence requirements as in Table 8.1.2.2-2.

Table 8.1.2.2-2: Spurious emission band UE co-existence of the extended S-band UE

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Level (dBm) | Bandwidth (MHz) | Comment |
| XX | E-UTRA Band 1,3,5,8,26,XX,40 | FDL_low | - | FDL_high | -50 | 1 | 1 |
|  | Frequency range | 2010 | - | 2015 | +1.6 | 5 | 2, 3, 4 |
|  | Frequency range | 2015 | - | 2025 | -15.5 | 5 | 2, 3, 4 |
| NOTE 1: This requirement is applicable for S-band UE of Band XX. NOTE 2: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 3: This requirement is applicable for an uplink transmission bandwidth less than or equal to [54] RB for carriers of 15 MHz bandwidth when carrier center frequency is within the range [ ] – [ ] MHz and for carriers of 20 MHz bandwidth when carrier center frequency is within the range [ ] – [ ] MHz. No other restrictions apply for carriers with bandwidths confined in 1920-2010 MHz. NOTE 4: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |  |  |  |  |  |

If RAN4 decides to make new SE requirements for S-band UE coexistence instead of exploiting the conventional FDD/TDD coexistence requirements, then RAN4 should define network signalling message as NS_XX to apply A-MPR mask and RB restrictions for S-band UE with dual-duplexer. The SE requirements for a new UE-to-UE coexistence will be FFS.

- 8.1.2.3	Other expected UE Tx RF requirements

For the other Tx requirements, a single/dual duplexer approaches can be considered. But the requirements should be defined as single requirements and should be met regardless of duplexer configurations. In this section, the detailed descriptions on UE Tx RF requirements are proposed to describe any potential requirements changes needed for the extended S-band UE

Case1) Protection Bands without Band 34:

For this case, there is no need to define A-MPR requirements and NS signalling to protect the bands even if we consider the extended bands. So we propose the detailed descriptions in Table 8.1.2.3-1.

Table 8.1.2.3-1: Considering UE Tx RF requirements for S-band UE

| Description | Comments and Description of change |
| --- | --- |
| UE Maximum Output Power | Additional insertion loss will be added to calculate the UE MOP due to the decreased duplex gap and increased Pass band. |
| UE Maximum Output Power with additional requirements (A-MPR) | No changes are needed to protect adjacent coexistence bands due to no IMD and harmonics [1]. |
| Spurious emission/ additional Spurious emission | No changes are needed to protect whole frequency bands for S-band UE. |
| ACLR/SEM | No changes are needed in OOB ranges for S-band UE. |
| Additional Spectrum Emission Mask | No changes are needed to protect adjacent coexistence bands due to no IMD and harmonics [1]. |

Case2) Protection Bands with Band 34:

When RAN4 decide to protect band 34 using the conventional FDD/TDD coexistence requirements, RAN4 can reuse the UE transmitter RF requirements as in Table 8.1.2.3-1.

But, if RAN4 decide to make new SE requirements for S-band UE coexistence, then RAN4 should define network signalling message as NS_XX to apply A-MPR mask and RB restrictions for S-band UE with dual-duplexer. The other transmitter requirements for S-band UE will be FFS.

  - UE Rx RF Requirements

    - New separate band (1980-2010 MHz and 2170-2200 MHz)

< <text will be added>.

    - New Extended band (1920-2010 MHz and 2110-2200 MHz)

- 8.2.2.1 REFSENS requirements for S-band UE

For the UE REFSENS requirements, a single/dual duplexer approaches are considered, but the requirements should be defined as single requirements and should be met regardless of duplexer configurations.

Case1) Protection Bands without Band 34:

Figure 8.2.2.1-1 show the simulation results of emission levels in S-band DL lower bands. The red line indicates the leakage level with full RBs allocation in extended S-band DL lower band.

- 20MHz S-band UE (fc=2000MHz)                      (b) 10MHz S-band UE (fc=2005MHz)

Figure 8.2.2.1-1 Simulation results of the Tx leakage level into S-band DL

The leakage level in DL band is about -65dBm/MHz, which would be further attenuated by Duplexer Tx/Rx isolation level with 50dB. So the leakage level in DL band is -115dBm/MHz. Eq. 8-4 shows the impact of REFSENS on S-band DL induced by Tx leakage.

- REFSENS for S-band UE = Conventional REFSENS + Tx_leakage levels                          (Eq. 8-4)

= -94dBm/CH_BW +  -115dBm/MHz

= -107dBm/MHz + -115dBm/MHz = - 106.36dBm/MHz

= -93.36dBm/CH_BW

From Eq. 8-4, we can see that the REFSENS in own Rx band by Tx leakage is negligible.

However, the insertion loss may be increased for the single duplexer compared to conventional Band 1 Duplexer, while we can keep the same insertion loss for dual duplexer approach.

As a conclusion, REFSENS requirements for S-band UE can be relaxed by the factor of additional insertion loss due to the introduction of new single duplexer of the extended S-band UE.

Case2) Protection Bands with Band 34:

Single duplexer approach is not sufficient to protect band 1 and band 34 based on general UE coexistence requirements of -50dBm/MHz[6] in extended S-band. This is because the new filter does not guarantee the same performances compare to band1 filter due to the decreased duplex gap and increased pass band, and also there is no guard band to protect band 34.

Therefore, RAN4 can use dual-duplexer filter which could not offer any attenuation at the lower edge of band 34.

The REFSENS level of own S-band is expected as same in Eq.8-4, but as shown in section 8.2.1.1, the REFSENS of Band 34 will be desensitized with 36.5dB. Hence it would not be possible to protect Band 34 without A-MPR, RB restriction and guard bands.

As a conclusion, REFSENS level for extended S-band UE is estimated as conventional levels in TS36.101. However, the REFSENS of the Band 34 UE will be desensitized with 36.5dB by S-band UE. Therefore RAN4 should consider the network signalling message as NS_XX to apply A-MPR and RB restriction to protect Band 34.

- 8.2.2.2 Other UE Rx RF requirements

For the receiver requirements for S-band UE, we propose preliminary descriptions on UE receiver requirements according to the protect band list with/without B34.

Case1) Protection Bands without Band 34:

From the above REFSENS analysis, Tx leakage is expected to be negligible since the large duplexer spacing and same Tx-Rx frequency separation as Band1.Also we can consider single duplexer and dual duplexer when RAN4 do not consider the B34 in protect band lists for S-band. Hence we can reuse the receiver requirements in Table 8.2.2.2-1 for S-band UE.

Table 8.2.2.2-1: Considering UE Rx RF requirements for S-band UE

| Description | Comments and Comparison to 2-band Solution |
| --- | --- |
| Adjacent Channel Selectivity | No changes since there is no Tx leakage impacts on the S-band DL for the T-put requirements that can be larger than or equal to 95% with same condition in section 7.5.1 [2] |
| Blocking characteristics | No changes since there is no Tx leakage impacts on the S-band DL for the T-put requirements can be larger than or equal to 95% with same condition in section 7.6 [2] |
| Receiver inter-modulation characteristics | No changes since there is no Tx leakage impacts on the S-band DL for the T-put requirements can be larger than or equal to 95% with same condition in section 7.8.1 [2] |

Case2) Protection Bands with Band 34:

When RAN4 decide to protect band 34 using the conventional FDD/TDD coexistence requirements, then RAN4 can reuse the UE receiver RF requirements as in Table 8.2.2.2-1.

But, if RAN4 decide to make new SE requirements for S-band UE coexistence, then RAN4 should define network signalling message as NS_XX to apply A-MPR mask and RB restrictions for S-band UE with dual-duplexer.

********************** End of text proposal to TR 36.861 Chapter 8 ***************************