3GPP TSG-RAN WG4 #68	R4-135352

Riga, Latvia, 7th – 11th, Oct. 2013

Source: 	Ericsson

Title:		TR 36.863 V0.3.1: Draft TP on System link level results for CRS-IM performance in homogeneous network from interested companies

Agenda Item:	10.5.1

Document for:	Approval

# Discussion

In RAN4#66bis, system level simulation assumptions are agreed in [1]. After RAN4#67, different companies propose system level simulation based on the agreed system level simulation assumption and the agreed method. Based on the agreed system level side condition, different companies proposed link level simulation.   In this contribution, we propose the draft TP to capture the link level throughput gain for CRS-IM performance in homogeneous network from various interested companies.

# Reference

[1]	R4-131957, “System level simulation assumptions for LTE UE CRS-IM SI”, Ericsson, RAN4#66bis

# Text Proposal

< Start of draft text proposal for TR36.863 (TR for CRS-IM)>

<Start of the first change>

# System performance characterization

## General

The purpose of this clause is to analyse the performance gain from system-level simulation results based on the agreed set of simulation assumption.

## Simulation results

This section presents the evaluation results for system level simulation generated by various interested companies to show the contribution.

Table 8.2-1: System simulation results (R4-132603, Ericsson)

|  |  | User Throughput |  | Relative gain in user throughput |
| --- | --- | --- | --- | --- |
| Resource Utilization | Performance metrics | No CRS IC (M=0) | CRS IC from the two strongest interferers (M=2) | CRS IC from the two strongest interferer (M=2) |  |
| About 10% | 5%-ile | 10.1 Mbps | 13.9 Mbps | 37% |  |
|  | 50%-ile | 20.2 Mbps | 23.2 Mbps | 14% |  |
| About ~20% | 5%-ile | 8.1Mbps | 11.2 Mbps | 36% |  |
|  | 50%-ile | 16.8Mbps | 19.7Mbps | 17% |  |
| About ~30% | 5%-ile | 6.5Mbps | 9Mbps | 38% |  |
|  | 50%-ile | 14.1Mbps | 16.9Mbps | 19% |  |
| About ~40% | 5%-ile | 5.0Mbps | 7.1Mbps | 41% |  |
|  | 50%-ile | 11.6Mbps | 14.3Mbps | 22% |  |
| About ~50% | 5%-ile | 3.7Mbps | 5.4Mbps | 46% |  |
|  | 50%-ile | 9.2Mbps | 11.8Mbps | 28% |  |
| Note: The transmission mode is TM4 |  |  |  |  |  |  |

Table 8.2-3: System simulation results (R4-132384, Huawei)

|  |  | Relative gain in user throughput |  |
| --- | --- | --- | --- |
| Resource Utilization | Performance metrics | CRS IC from the two strongest interferer (M=2) | CRS IC from all interferers (M=∞) |
| About ~20% | 5%-ile | 10.2% | 14.6% |
|  | 50%-ile | 8.3% | 13.7% |
|  | 95%-ile | 0% | 0% |
| About ~50% | 5%-ile | 14.4% | 14.4% |
|  | 50%-ile | 7.4% | 7.9% |
|  | 95%-ile | 1.9% | 1.9% |
| About ~70% | 5%-ile | 2.5% | 3.5% |
|  | 50%-ile | 2.5% | 3.0% |
|  | 95%-ile | 1.3% | 3.4% |

Table 8.2-3: System simulation results (R4-133469, NSN)

|  |  |  | User Throughput |  |  |  |  |  | Relative gain in user throughput |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Resource Utilization |  | Performance metrics |  | No CRS IC (M=0) |  | CRS IC from the two strongest interferers (M=2) |  | CRS IC from all interferers (M=∞) |  | CRS IC from the two strongest interferer (M=2) |  | CRS IC from all interferers (M=∞) |  |
| About 5% |  | 5%-ile |  | 18.7Mbps |  | 19.8Mbps |  | 20.6Mbps |  | 5.9% |  | 10.2% |  |
|  |  | 50%-ile |  | 38.0Mbps |  | 41.1Mbps |  | 45.6Mbps |  | 8.1% |  | 20% |  |
| About ~20% | 5%-ile |  | 12.7Mbps |  | 13.5Mbps |  | 14.3Mbps |  | 6.3% |  | 12.6% |  |
|  | 50%-ile |  | 30.3Mbps |  | 31.5Mbps |  | 33.9Mbps |  | 3.9% |  | 11.9% |  |
| About ~50% |  | 5%-ile |  | 6.4Mbps |  | 6.7Mbps |  | 7.2Mbps |  | 4.7% |  | 12.5% |  |
|  |  | 50%-ile |  | 18.6Mbps |  | 19.4Mbps |  | 20.8Mbps |  | 4.3% |  | 11.8% |  |
| About ~70% |  | 5%-ile |  | 2.7Mbps |  | 2.8Mbps |  | 3.0Mbps |  | 3.7% |  | 11.1% |  |
|  |  | 50%-ile |  | 8.7Mbps |  | 9.0Mbps |  | 9.9Mbps |  | 3.4% |  | 13.8% |  |

<End of the first change>

< End of draft text proposal for TR36.863 (TR for CRS-IM)>