3GPP TSG RAN WG4 Meeting #68BIS	R4-135729

Riga, Latvia, 7th – 11th October, 2013

Agenda Item:	8.25

Source:	LG Electronics

Title:	TP for IMD/harmonics analysis for WLAN, Bluetooth and GNSS for inter-band CA 1A-5A UE with 2ULs

Document for:	Approval

- Introduction

In the RAN4 #68 meetings in Barcelona, RAN4 had agreed that the IMD/harmonics table for the impact of LTE-A dual uplink inter-band CA on GNSS and the connectivity radios (WLAN, BT etc in ISM bands) [1].

In this contribution, we analysed inter-modulation problems between CA_1A-5A UE and the connectivity radios (WLAN, BT etc. in ISM bands) and positioning systems (GPS, Galileo, Glonass, Compass, etc).

- References

[1] TR 36.860 v0.3.0

[2] TS 36.101 v11.6.0

[3] R4-134418, “TP for general frequency bands for WiFi, Bluetooth and GNSS for 2ULs inter-band CA,” LG Electronics

Text proposal for TR 36.860

********************** Start of text proposal to TR 36.860 Chapter 6.2.1 ***********************

6.2.1	Inter-region OOBE requirements for Carrier Aggregation of Band 1 and Band 5

Table 6.2.1-1: 2UL inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) band |  |  |  | Downlink (DL) band |  |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | Channel BW (MHz) | BS transmit / UE receive |  |  | Channel BW (MHz) |  |
|  |  | FUL_low – FUL_high |  |  |  | FUL_low – FUL_high |  |  |  |  |
| CA_1-5 | 1 | 1920 MHz | – | 1980 MHz | 10 | 2110 MHz | – | 2170 MHz | 10 | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 10 | 869 MHz | – | 894 MHz | 10 |  |

      - List of specific combination issues

        - Channel bandwidths per operating band for CA

Table 6.2.1.1.1-1: 2UL inter-band CA operating bands

| E-UTRA band / channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Band | E-UTRA Bands | 1.4MHz | 3MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Bandwidth combination set |
| CA_1A-5A | 1 |  |  |  | Yes |  |  | 0 |
|  | 5 |  |  |  | Yes |  |  |  |

        - Co-existence studies for CA_1A-5A with 2ULs

For the analysis of 2UL inter-band CA 1A-5A UE, we should study and analyze the inter-modulation products and harmonics products when both Band 1 and Band 5 with 2ULs are transmitting simultaneously.

The combination of Band 1 and Band 5 has harmonics frequencies that are far away from the components receive and transmit frequencies, in the UL and in the DL of the CA combination, as shown in table 6.2.1.1.2-1. So there was no harmonic issue except Band 41. In the conventional SE for UE-to-UE coexistence requirements, the 3rd harmonics of Band 5 UE already fall into B41 DL band, but exceptions for SE measurements are permitted if there is at least one individual RB within the transmission bandwidth for which the 2nd, 3rd or 4th harmonic totally or partially overlaps the measurement bandwidth (MBW) [3].

Also, the inter-modulation products for CA_1A-5A UE with 2 ULs as shown in table 6.2.1.1.2-1. None of the IMD products fall into the own CA_1A-5A UE receive bands.  But, 2-tone 3rd IMD products may have impact on the Band 22 DL, Band 42 DL and 43 DL bands. But Band 22, Band 42 and Band 43 are far away from Band1 and Band 5, so we can assume that the UE front-end filter can effectively attenuate these IMD products.

Table 6.2.1.1.2-1: 2 UL B1 + B5 harmonic products and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 |  | 824 | 849 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |  |
| 2nd harmonics frequency limits (MHz) | 3840 to 3960 |  |  | 1648 to 1698 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low |  | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 to 5940 |  |  | 2472 to 2547 |  |  |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1071 to 1156 |  |  | 2744 to 2829 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 2991 to 3136 |  |  | 222 to 332 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 4664 to 4809 |  |  | 3568 to 3678 |  |  |

For the connectivity radios (WLAN, BT etc. in ISM bands) and positioning systems (GPS, Galileo, Glonass, Compass, etc), RAN4 analyzed the impact of the IMD/Harmonic from dual-uplink CA transmission in Table 6.2.1.1.2-2. From the analysis table, WLAN system has some harmonics problems in 5GHz and 2.4GHz by 3rd harmonics from each Band 1 and Band5, respectively.

Table 6.2.1.1.2-2: The IMD/Harmonic analysis to protect ISM bands and positioning systems

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | Band5 3rd harmonics |
|  | 2400 | - | 2494 | Yes | Asia | Band5 3rd harmonics |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | Band1 3rd harmonics |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | Yes | Asia | Band1 3rd harmonics |

        - ∆TIB and ∆RIB values

******************** End of text proposal to TR 36.860 Chapter 6.2.1 ************************