3GPP TSG-RAN WG4 #69	R4-136106

San Francisco, USA, 11 - 15 Nov, 2013

Source: 	Huawei, HiSilicon

Title: 	Simulation results of DL COMP CSI tests

Agenda Item:	7.7.2

Document for:	Discussion

# Introduction

In this contribution, we provide the simulation and impairment results of static and fading CQI tests for DL CoMP based on the agreed simulation assumptions [1].

# Simulation results

## 2.1 Static CQI definition test

In the following, the simulation results for static CQI definition test are provided in Table 1.

Table 1 Simulation results of static CQI test

| TP2 SNR (dB) |  | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 6 | 7 | 14 | 15 |
| The time ratio of reported CQI1 within the set {median CQI1-1, median CQI1+1} |  | 100% | 100% | 100% | 100% |
| Median CQI-1 | BLER 0 | 0 | 0 | 0 | 0 |
|  | BLER 1 | 0 | 0 | 0 | 0 |
| Median CQI+1 | BLER 0 | 0.4213 | 1 | 0.1113 | 0.4678 |
|  | BLER 1 | 0.4204 | 1 | 0.1112 | 0.476 |

Based on the simulation results, we have the following proposal.

Proposal 1:

Reuse Rel-10 performance requirement for DL CoMP static CQI test

## 2.2 CQI fading test

Based on the detailed simulation assumptions in the framework document [1], simulation results are obtained and provided in Table 2 and Table 3. About the simulation results for the CSI processes:

- Ratio 1: a sub-band differential CQI offset level of 0 shall be reported at least % of the time but less than % for each sub-band for CSI process 1, 2, or 3

- Ratio 2: a CQI index not in the set {median CQI -1, median CQI +1} shall be reported at least % of the time for CSI process 0

- CQI difference: the difference of the median CQIs of the reported wideband CQI between CSI process 0 and configurated CSI processes

Table 2 Throughput results of the agreed test cases

| Test cases | Best CQI |  | wideband CQI median (Mbps) |  |
| --- | --- | --- | --- | --- |
|  | Throughput (Mbps) | BLER |  |  |
| TP1=10dB,TP2=7dB | 0.8779 | 0.0601 | 0.5162 | 1.7006 |
| TP1=11dB,TP2=8dB | 0.8963 | 0.058 | 0.5263 | 1.7029 |
| TP1=14dB,TP2=9dB | 1.1369 | 0.0449 | 0.6193 | 1.8356 |
| TP1=15dB,TP2=10dB | 1.1487 | 0.0442 | 0.6227 | 1.8447 |

Table 3 Reporting distribution of the agreed test cases

| CSI process index |  | CSI process 0 | CSI process 1 | CSI process 2 | CSI process 3 |
| --- | --- | --- | --- | --- | --- |
| TP1=10dB, TP2=7dB | Ratio 1 [%] | N/A | 0.1472 | 0.2778 | 0.2213 |
|  | Ratio 2 [%] | 0.3235 | N/A | N/A | N/A |
|  |  | N/A | N/A | 1.7006 | N/A |
|  | CQI difference: CSI process 0 | N/A | 3 | 4 | 6 |
| TP1=11dB, TP2=8dB | Ratio 1 [%] | N/A | 0.16 | 0.2811 | 0.2226 |
|  | Ratio 2 [%] | 0.2685 | N/A | N/A | N/A |
|  |  | N/A | N/A | 1.7029 | N/A |
|  | CQI difference: CSI process 0 | N/A | 4 | 4 | 7 |
| TP1=14dB, TP2=9dB | Ratio 1 [%] | N/A | 0.1653 | 0.2725 | 0.2342 |
|  | Ratio 2 [%] | 0.331 | N/A | N/A | N/A |
|  |  | N/A | N/A | 1.8356 | N/A |
|  | CQI difference: CSI process 0 | N/A | 5 | 5 | 9 |
| TP1=15dB, TP2=10dB | Ratio 1 [%] | N/A | 0.158 | 0.2712 | 0.2299 |
|  | Ratio 2 [%] | 0.2185 | N/A | N/A | N/A |
|  |  | N/A | N/A | 1.8447 | N/A |
|  | CQI difference: CSI process 0 | N/A | 6 | 6 | 9 |

Based on the simulation results, we have the following proposal.

Proposal 2:

The proposed performance requirements for DL CoMP CQI fading tests are:

Table 9.3.6.1-2 Minimum requirement (FDD)

|  | CSI process 0 | CSI process 1 | CSI process 2 | CSI process 3 |
| --- | --- | --- | --- | --- |
|  [%] | N/A | 2 | 2 | 2 |
|  [%] | N/A | 40 | 40 | 40 |
|  [%] | 15 | N/A | N/A | N/A |
|  | N/A | N/A | 1.1 | N/A |
| UE Category | 1-8 |  |  |  |

Table 9.3.6.1-3 Minimum median CQI difference between configured CSI processes (FDD)

|  | CSI process 1 | CSI process 2 | CSI process 3 |
| --- | --- | --- | --- |
| CSI process 0 | 2 | 3 | 5 |
| UE Category | 1-8 |  |  |

# 3 Conclusions

In this contribution, simulation results based on agreed assumptions in framework are provided. The following is the summary of our proposals,

For CQI definition test:

Proposal 1:

Reuse Rel-10 performance requirement for DL CoMP static CQI test

For CQI fading test:

Proposal 2:

The proposed performance requirements for DL CoMP CQI fading tests are:

Table 9.3.6.1-2 Minimum requirement (FDD)

|  | CSI process 0 | CSI process 1 | CSI process 2 | CSI process 3 |
| --- | --- | --- | --- | --- |
|  [%] | N/A | 2 | 2 | 2 |
|  [%] | N/A | 40 | 40 | 40 |
|  [%] | 15 | N/A | N/A | N/A |
|  | N/A | N/A | 1.1 | N/A |
| UE Category | 1-8 |  |  |  |

Table 9.3.6.1-3 Minimum median CQI difference between configured CSI processes (FDD)

|  | CSI process 1 | CSI process 2 | CSI process 3 |
| --- | --- | --- | --- |
| CSI process 0 | 2 | 3 | 5 |
| UE Category | 1-8 |  |  |

# References

- R4-134989, “Framework document for downlink CoMP CSI test (Version 5)”, Huawei, HiSilicon, Samsung, Ericsson.