3GPP TSG RAN WG4 Meeting #69	R4-136349

San Francisco, CA, U.S.A, 11th – 15th Nov., 2013

- Agenda Item:	10.1

- Source:	LG Electronics

- Title:	TP for S-band UE Tx/Rx RF requirements for standalone Band

Document for:	Approval

- Introduction

In the RAN Plenary #57 meetings, the study for the LTE S-band in Korea was approved. And the UE-to-UE coexistence simulation results to protect band 34 was captured [1] for TR 36.861 v0.4.0 in last RAN4 meeting.

In this contribution, we provide the UE transmitter/Receiver RF requirements based on the deployments scenarios assuming new standalone S-band.

- References

- R4-133145, “Coexistence with Band 34,” KT, SK Telecom, LGU+, ETRI

- TS 36.101 v11.6.0

- R4-135186, “UE coexistence analysis to protect Band 34,” LG Electronics

- R4-131390, “TP for TR ab.cde : General issue for FS_LTE_1980_2170_Korea,” LG Electronics

Text proposal for TR 36.861

********************** Start of text proposal to TR 36.861 chapter 8 ***********************

- 8	Study of E-UTRA specific issues

This section summarizes issues and additional requirements when S-band UE is deployed under Korean regulations.

- 8.1    UE Tx RF Requirements

- 8.1.1	New separate band (1980-2010 MHz and 2170-2200 MHz)

8.1.1.1 	Spurious emission for UE co-existence

For the standalone S-band, RAN4 assumes the same duplexer characteristics compared to the Band 1 duplexer to specify UE RF transmitter requirements.

When Band 34 is allocated in the same country and region where S-band UE is deployed, RAN4 can define network signalling such as NS_XX to apply A-MPR mask with guard band to protect band 34 for S-band UE. SE requirements for UE-to-UE coexistence can be defined as in Table 8.1.1.1-1. The SE requirements level and detail test methodology to protect band 34 are for further study.

Table 8.1.1.1-1: Spurious emission band UE co-existence of the standalone S-band UE

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Level (dBm) | Bandwidth (MHz) | Comment |
| XX | E-UTRA Band 1,3,5,8,26,XX,40 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 34 | 2010 | - | 2025 | TBD | TBD | 1 |
|  |  |  | - |  |  |  |  |
| NOTE 1: This requirement is only applicable for S-band UE, when Band34 and S-band deployed in same region. |  |  |  |  |  |  |  |

8.1.1.2	Other expected UE Tx RF requirements

In this section, the detailed descriptions on UE Tx RF requirements other than spurious emission band UE co-existence are proposed to describe any potential requirement changes needed for S-band UE.

When RAN4 decides to protect the band 34 for operating S-band UE in the same geographical area, the other expected UE transmitter RF requirement changes are estimated and summarized in Table 8.1.1.2-1.

Table 8.1.1.2-1:  Expected UE Tx RF Requirements for S-band UE

| Description | Comments and Description of change |
| --- | --- |
| UE Maximum Output Power | No change the max power and tolerance for power class 3. Not allowed additional insertion loss. |
| UE Maximum Output Power with additional requirements (A-MPR) | Additional-MPR for NS_XX can be defined to protect Band 34. |
| Spurious emission/ additional Spurious emission | Additional SE can be defined to protect Band 34. |
| ACLR/SEM | No changes are needed in OOB ranges for S-band UE. |
| Additional Spectrum Emission Mask | No changes are needed to protect adjacent coexistence bands. |

In this case RAN4 can define a network signalling such as NS_XX to apply A-MPR mask and Guard band to protect Band 34.

8.1.2	New Extended band

< <text will be added>.

  - UE Rx RF Requirements

    - New separate band (1980-2010 MHz and 2170-2200 MHz)

- 8.2.1.1 REFSENS requirements for S-band UE

Standalone S-band has the same Tx-Rx frequency separation and the total bandwidth of S-band is smaller than that of Band 1. Therefore S-band UE’s REFSENS level can be easily as assumed to be the same as that of Band 1. RF simulations are provided to verify this assumption.

Figure 8.2.1.1-1 shows the simulation results of emission level in S-band DL lower bands. The red line indicates the leakage level with full RBs allocation in S-Band DL lower band.

- 20MHz S-band UE (fc=2000MHz)                      (b) 10MHz S-band UE (fc=2005MHz)

Figure 8.2.1.1-1 Simulation results of the Tx leakage level into S-band DL

The leakage level in DL band is about -75dBm/MHz, which is further attenuated by Duplexer Tx/Rx isolation level, which is assumed to be 50dB. So the leakage level in DL band is estimated as -125dBm/MHz, which is 18 dB lower than REFSENS. Eq. 8-1 shows the impact of REFSENS in S-band DL induced by S-band Tx leakage.

- REFSENS for S-band UE = Conventional REFSENS + Tx_leakage levels                          (Eq. 8-1)

= -94dBm/CH_BW +  -125dBm/MHz

= -107dBm/MHz + -125dBm/MHz = - 106.93dBm/MHz

= -93.93dBm/CH_BW

From Eq. 8-1, we can see that the REFSENS in own Rx band by Tx leakage is negligible.

As a conclusion, REFSENS requirements for S-band UE can be defined as -94dBm/20MHz in Table 8.2.1.1-1 with the same uplink configurations as in Band1 [2].

Table 8.2.1.1-1: Reference Sensitivity for QPSK

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| XX |  |  | -100 | -97 | -95.2 | -94 | FDD |

However, RAN4 should study how much impact there is when the S-band Tx leakage falls into the B34 lower parts.

The S-band Tx leakage levels in B34 lower frequency ranges are about -16~ -33dBm/MHz according to the channel BW and frequency shift levels as shown in Figure 8.2.1.1-2. The emission level is calculated in Eq. 8-2 and the estimated REFSENS level in Band 34 is calculated in Eq.8-3

- Emission level into TDD B34 UE = Tx leakage level - MCL -  penetration loss                    (Eq.8-2)

= -16dBm/MHz - 54.44dB - 0dB = -70.44dBm/MHz

- REFSENS for band 34 UE = Conventional REFSENS + Emission levels into B34 UE     (Eq. 8-3)

= -94dBm/CH_BW +  -70.44dBm/MHz

= -107dBm/MHz + -70.44dBm/MHz = - 70.44dBm/MHz

= -57.439dBm/CH_BW

Figure 8.2.1.1-2 shows the simulation results of emission level into band 34 lower parts. The brown line indicates the leakage level with full RBs allocation in Band 34.

From these simulation results, the REFSENS of Band 34 will be desensitized with 36.5dB. Hence it would not be possible to protect Band 34 without A-MPR and guard bands when band 34 and S-band is deployed in the same geographical region and/or country.

- 0MHz shift (fc=2000MHz, CH_BW=20MHz)

- 5MHz shift (fc=1995MHz, CH_BW=20MHz)       (c) 10MHz shift (fc=1990MHz, CH BW=20MHz)

- 0MHz shift (fc=2005MHz, CH_BW=10MHz)

(e)  5MHz shift (fc=2000MHz, CH BW=10MHz)       (f) 10MHz shift (fc=1995MHz, CH BW=10MHz)

Figure 8.2.1.1-2 Tx leakage level in B34 lower edge frequency

As a conclusion, REFSENS level for extended S-band UE is estimated to be the same as conventional levels in TS36.101. However, the REFSENS of the Band 34 UE will be degraded by 36.5dB by S-band UE emissions. Therefore RAN4 should define NS_XX message to protect B34 UE from S-band UE. Network signalling can control the maximum output power of aggressor UE according to the RB starting position and number of contiguous RB allocation.

- 8.2.1.2 Other UE Rx RF requirements

For the receiver requirements other than REFSENS for S-band UE, requirement changes are not expected since there are no critical impacts to determine the UE reception requirements. So we propose as in Table 8.2.1.2-1.

Table 8.2.1.2-1: Expected UE Rx RF requirements for S-band UE

| Description | Comments and Description of change |
| --- | --- |
| Adjacent Channel Selectivity | No changes since there is no Tx leakage impacts on the S-band DL for the T-put requirements that can be larger than or equal to 95% with same condition in section 7.5.1 [2] |
| Blocking characteristics | No changes since there is no Tx leakage impacts on the S-band DL for the T-put requirements can be larger than or equal to 95% with same condition in section 7.6 [2] |
| Receiver inter-modulation characteristics | No changes since there is no Tx leakage impacts on the S-band DL for the T-put requirements can be larger than or equal to 95% with same condition in section 7.8.1 [2] |

8.2.2	New Extended band

< <text will be added>.

********************** End of text proposal to TR 36.861 Chapter 8 ***************************