TSG-RAN WG4 Meeting #69	R4- 137064

San Francisco, USA, November 11th – 16rd, 2013

| Title: | AAS radiated performance comparison between MIMO MOTA test methodologies |
| --- | --- |
| Source: | Motorola Mobility LLC, Intel, Aalborg University, ETS-Lindgren |
| Agenda Item: | 8.3 |
| Work Item: | HSPA_LTE_measRP_MIMO-Perf |
| Release: | 12 |
| Document for: | Discussion |

# 1. Abstract

This contribution describes the radiated performance of an active antenna system (AAS) UE test fixture, featuring a tunable antenna with four switching states, performs analysis of absolute data throughput, comparing results between anechoic chamber boundary array and reverberation chamber.

# 2. Introduction

In the scope of the work done in 3GPP RAN4 on MIMO OTA, a number of companies have raised the question of the ability of test methodologies to discriminate various antenna designs. The CTIA reference antennas [1] have provided a baseline for the work in 3GPP and CTIA to advance but do not cover the vast and diverse number of antenna design techniques utilized in the industry to improve channel capacity and ultimately throughput. In [2] and [3] it was clearly shown that reverberation chamber based methodologies are incapable of distinguishing antenna designs with pure polarization diversity. A Device Under Test (DUT) utilizing polarization diversity would then be seen only through its antenna efficiency and the only benefit of the utilization of polarization diversity on the DUT would be potentially lower correlation between antennas. This however does not allow for antenna designers to optimize for power collection from realistic environments since any such optimization will not be seen in a reverberation chamber as shown in [2] and [3]. This is due to the fundamental limitation of the reverberation chamber to produce any other propagation conditions but those with environment XPR = 0, at least not as promoted and used to date. It must be noted here that environment XPR reported in the peer reviewed literature varies significantly and the propagation and antenna experts from COST IC1004 summarized a strong technical objection to an environment with XPR = 0 in [4].

Another aspect of antenna design not addressed with the CTIA reference antennas from [1] is the actively tunable and Adaptive Antenna System (AAS). AASs come in various flavors with many different techniques to tune impedance, pattern, polarization etc. and of course any combination of those together. Every tuning component comes at a dollar cost but also at the expense of total antenna efficiency since the tunable components are typically utilized after the calibrated RF connector. It is very important that antenna design freedom and innovation is not stifled by the evaluation methodology’s incapability to demonstrate the benefits of a particular innovation. While a reverberation chamber would probably demonstrate 0.5 dB lost in total antenna efficiency due to an antenna tuner for example, the benefit from that tuner in a realistic propagation environment with non-zero XPR or directive incoming power might be completely ignored. Again the propagation and antenna experts from COST IC 1004 sound the alarm in [4]. To put this argument into perspective consider the case of beam forming and a very small sample of a vast pool of peer reviewed literature dealing with this issue.

# 3. Antenna Design

The sections below describe a simple beam forming design. We have not optimized for efficiency or produced an antenna ready for integration in real devices but simply continued on the original CTIA reference antenna track of demonstrating antennas principles.

## 3.1 Main Antenna

Secondary PIFA Antenna 1

Main antenna

The main antenna is shown in Fig. 3.1-1 extends few millimeters out of the PCB. The main antenna is passive broadband top loaded monopole. It covers both TX and RX of the European LTE Band 7. The center RX frequency is 2655 MHz and the center TX frequency is 2535 MHz.

SW1  SW2

SPDT switches emulating sensors

Battery and DC/DC converter

Secondary PIFA Antenna 2

(a)

Main antenna

RF switch

(b)

Fig. 3.1-1, AAS prototype back view (a), front view (b)

## 3.2 AAS for Diversity

The diversity antenna covers only the RX LTE Band 7 with a center frequency of 2655 MHz. It consists of four PIFA elements – two on the front and 2 on the back of the platform PCB – see Fig. 3.1-1 (a,b). The elements are fed via two 3dB, 90 degrees couplers implemented as microstrip lines on the PCB as shown in Fig. 3.1-1(a,b).

The front two elements shown in Fig. 3.1-1 (b) top left corner form a beam forming pair combined at the coupler. The same is done for the back two elements combined at the second coupler. The front pair covers roughly the front hemisphere of the spherical coordinate system and the back pair covers the other hemisphere. By selectively feeding the couplers’ ports the beam can be steered not only between hemispheres but within a single quadrant of one hemisphere.

### 3.3 RF Switching and power supply

The RF switching is done with an internally terminated RF switch from Hittite Microwave Corporation [5]. The switch requires a power supply and control bits for the four different states. This is done using a DC-DC converter based on a single chip solution from Texas Instruments [6]. The battery supply is delivered from a small single Li-Io cell. The circuit is placed on the back side of the RF enclosure.

The RF switch circuit is shown in Fig. 3.1-1. The two SPDT switches SW1 and SW2 are used to manually operate the beam direction by switching the main RF switch between the four states. Clearly on a real product all of this would be optimized for performance, power consumption, sensor array and a smart algorithm would control the switching.

The truth table for U2 can be found in [7]. The same RF port notation is used here.

# 4. Measurement Results

4.1 Antenna FoM measurement results

The Table 4.1-1 summarizes the antenna FoM measurement results, antenna 1 called “secondary” is the one being adapted, and antenna 2 is the main antenna.

Table 4.1-1 MIMO B7 antennas FoM based on 3D isotropic measurements @ 2655MHz

As indicated in Table 4.1-1, the antenna system has a polarization ratio near 0 dB, which is characteristic to antennas at high frequencies in small form factors. The envelope correlation is smaller than 0.1 guaranteeing uncorrelated antennas; at least at uniform incoming power; however the branch imbalance is near 3dB.

The following Figs. 4.1-1~7, shows the antenna’s return loss and radiation patterns in 3D and polar format, in different switch states. As indicated in Table 1, the main antenna maintains the same total efficiency and radiation patterns across all four switch states. The difference in performance < 0.1dB, is within the anechoic chamber measurement uncertainty.

Fig. 4.1-1, Main antenna 3D free space radiation pattern, and return loss.

Fig. 4.1-2, Secondary antenna 3D radiation pattern, and return loss at switch state 00.

Fig. 4.1-3, Secondary antenna 3D radiation pattern, and return loss at switch state 01.

Fig. 4.1-4, Secondary antenna 3D radiation pattern, and return loss at switch state 11.

Fig. 4.1-5, Secondary antenna 3D radiation pattern, and return loss at switch state 10.

- (b)                                         (c )                                      (d)

Fig. 4.1-6, Main antenna polar plot at elevation 90º, (a) switch state 00, (b) 01, (c) 11, (d) 10

- (b)

(c)                                                                              (d)

Fig. 4.1-7, Secondary antenna polar plot at elevation 90º, (a) switch state 00, (b) 01, (c) 11, (d) 10

4.2 MIMO OTA absolute data throughput measurement results

The OTA measurement results are based in two test methodologies considered valid, the anechoic chamber multi-cluster boundary array, and the reverberation chamber without channel emulation assistance. As indicated in the Table 4.1-1, there is a branch imbalance between antennas averaging 3.3 dB, during these measurements a 3 dB series attenuator was placed between the main antenna and the DUT RF port, thus partially equalizing both antennas’ gain. Ideally for each tuner position a specific attenuation value should be used to equalize all antennas by the least efficient, i.e. tuner position 01 secondary antenna. However attenuators in sub-multiple values of 1dB that could be accommodated inside the RF enclosure were not readily available. For this experiment due the lack of precise equalization of total efficiency and branch imbalance, a variation of approximately 1 dB is shown within all four switching stages.

4.2.1 Anechoic chamber setup.

The OTA test system consisted of an ETS-Lindgren AMS-8700 boundary array with eight active dual polarized antennas at a radius of 1.95 meters driven by two Spirent VR5 8 output channel emulators for 16 total output channels used to generate the applied channel model and resulting signal levels within the test volume.  Two ETS-Lindgren 8-channel power amplifiers were used to amplify the outputs of the channel emulators to produce the required signal levels within the test volume.  The reported measurements were captured using a Rohde & Schwarz CMW-500 as the eNodeB emulator/communication tester.  The two outputs were each split and fed into the two VR5s.  A separate circularly polarized conical log spiral antenna was used to provide the uplinik from the DUT.  The uplink path was then fed through a pre-amplifier to provide additional downlink isolation prior to feeding the signal to the eNodeB input.  The Fig. 4.2.1-1 contains a system schematic for the test setup, while Table 4.2.1-1 contains the detailed equipment list.

Fig 4.2.1-1 ETS-Lindgren AMS-8700 Boundary Array Schematic Diagram

Table 4.2.1-1, Anechoic chamber test equipment list

| Equipment | Manufacturer | Model | Firmware Revision |
| --- | --- | --- | --- |
| MIMO Boundary Array System | ETS-Lindgren | AMS-8700 | NA |
| Channel Emulator (2X) | Spirent | VR5 | NA |
| 8-Channel Power Amplifier (2X) | ETS-Lindgren | P/N 117907 | NA |
| eNodeB Emulator/ Communication Tester | R&S | CMW-500 | 3.2.20 |
| Power Splitters (2X) | Mini Circuits | ZFSC-2-2500-S+ | NA |
| Uplink Preamp | ETS-Lindgren | NA | NA |

4.2.2 Reverberation chamber setup

An ETS-Lindgren AMS-7000 wireless OTA reverb test system was used to perform the average isotropic (uniform probability distribution) testing.  The system consists of a compact reverberation chamber (2.00 x 1.20 x 1.50 m) with two independent stirring paddles and a DUT turntable having a lowest operating frequency of ~700 MHz, connected to a Rohde & Schwarz CMW-500 as the eNodeB emulator/communication tester. The cell was selectively loaded to produce an RMS delay spread of 80 ns for the NIST model. Tests were performed using continuous stirring of all positioners for an integral number of rotations of all positioners at a fixed ratio and timed such that one long throughput measurement was performed per revolution of the slowest positioner, thus producing one average throughput measurement per power level.

Fig 4.2.2-1, ETS-Lindgren Reverberation chamber schematic diagram

4.3 Absolute data throughput, equipment settings

The table 4.3-1 bellow presents the complete list of equipment settings used during these measurements. Allowing any lab to reproduce the data presented in this work.

Table 4.3-1 Anechoic and reverberation chamber test setup settings

| Test settings | Anechoic chamber settings | Reverberation chamber settings |
| --- | --- | --- |
| ID | AC AAS v1 | RC AAS v1 |
| Lab | ETS-Lindgren – Cedar Park | ETS-Lindgren – Cedar Park |
| Date | 11/05/2013 | 11/05/2013 |
| Methodology | Anechoic chamber Multi-cluster, 8DP | Reverberation chamber |
| eNodeB emulator | R&S CMW500 | R&S CMW500 |
| eNodeB emulator version | Installed SW V 3.2.20 | Installed SW V 3.2.20 |
| eNodeB ant configuration | Sec 7.2 in 37.977 | Sec 7.2 in 37.977 |
| eNodeB PHY configuration | Sec 7.1 in 37.977 | Sec 7.1 in 37.977 |
| Band | 7 | 7 |
| DL channel | 5230 | 5230 |
| UL channel | 23230 | 23230 |
| RMC | R35 | R35 |
| Number of sub-frames | 5000/position/0.5 – 1dB resolution | 20000/rotation/0.5 - 1dB resolution |
| Channel emulator | Spirent VR5 | NA |
| Channel emulator version | 2.5.341.3 | NA |
| Channel model configuration | Sec 8.2 in 37.977 | NA |
| Channel model | SCME Umi | NIST, 80 ns delay spread |
| Emulated vehicle speed | 30 km/h | NA |
| UE mfg | Samsung | Samsung |
| UE model | Galaxy Express, GT-18730 | Galaxy Express, GT-18730 |
| UE ID | IMEI: 355052/05/009958/0 | IMEI: 355052/05/009958/0 |
| Transmission Mode | TM3 | TM3 |
| Max theoretical throughput | 35.424 Mb/s | 35.424 Mb/s |
| Num theta pos. | Sec 9.3.1.3 in 37.977 | NA |
| Theta pos. | Sec 9.3.1.3 in 37.977 | NA |
| Num phi pos. | Sec 9.3.1.3 in 37.977 | NA |
| Phi pos. | Sec 9.3.1.3 in 37.977 | NA |
| Test plan version | Lab “E1” 001 | Lab “E2” 001 |
| Comments | XPR = 9dB , 3dB attenuation in main antenna | 3dB attenuation in main antenna |

4.4 Absolute data throughput radiated

4.4.1 Anechoic chamber multi-cluster boundary array absolute data throughput results

The measurements defined in 4.4.1-1 ~ 4.4.1-4, show the absolute data throughput results with the DUT at each switch state. The measurement shown in 4.4.1-6 is a compilation of the previous measurements indicating the DUT performance as the AAS was implemented, where compares the average throughput for each of the individual switch states to the average throughput determine by choosing the optimum switch state for each test position, thus replicating the effect of an intelligent algorithm adapting the antenna to the current orientation in the environment.

Fig. 4.4.1-1, Absolute data throughput, 12 DUT rotations individual curves, and average of 12 individual curves. SCME Umi, 30kph, XPR=9dB, switch state 00.

Fig. 4.4.1-2, Absolute data throughput, 12 DUT rotations individual curves, and average of 12 individual curves. SCME Umi, 30kph, XPR=9dB, switch state 01.

Fig. 4.4.1-3, Absolute data throughput, 12 DUT rotations individual curves, and average of 12 individual curves. SCME Umi, 30kph, XPR=9dB, switch state 11.

Fig. 4.4.1-4, Absolute data throughput, 12 DUT rotations individual curves, and average of 12 individual curves. SCME Umi, 30kph, XPR=9dB, switch state 10.

Fig. 4.4.1-5, Sensitivity vs. Angle at 30Mbp

Fig. 4.4.1-6, Absolute data throughput, optimal selection of 12 DUT rotations individual curves, and average of 12 optimal individual curves. SCME Umi, 30kph, XPR=9dB, AAS virtually implemented.

4.4.2 Reverberation chamber absolute data throughput results

The measurement results on 4.4.2-1, shows the absolute data throughput results with the DUT at each switch state measured at reverberation chamber as defined in the Table 4.3-1.

Fig. 4.4.2-1, Absolute data throughput, NIST (80ηs RMS Delay Spread) Isotropic channel model, switch state 00 (red curve), 01 (purple), 11 (blue), 10 (green).

# Conclusions

In this paper a simplified however realistic implementation of AAS in B7 was presented. While the sensor and action taking functions are not part of this prototype implementation, the antenna pattern rotation implementation is feasible to be implemented in commercial devices. From this experiment some preliminary conclusions can be drawn.

- The AAS antenna prototype radiation pattern measurements agree with design proposal and simulation results;

- The variation in absolute data throughput performance in each switch state, doesn’t agree between AC and RC, while the AC test methodology  has the switch state “10” as the best performer; which agrees with results shown in table 4.1-1, the RC method has the same switch state as the worse performance, once again contradicting the concept of “ranking” between test methodologies;

- The simplified but realistic AAS demonstrated that while being measured in a channel model with spatial characteristics, there’s an improvement in throughput vs. Sensitivity of approximately 2dB @ 70% data throughput;

- There’s no known method to provide such AAS performance evaluation in RC, due to:

  - Lack of spatial information in the channel model;

  - Uncertainty of AoA; or multiple AoAs; for each isotropic state;

  - Uncertainty of magnitude and direction of multiple AoA in the same isotropic state;

  - Lack of realistic transition on AoA between consecutive isotropic states;

  - Dependency of RC unique features, such as:

    - Synchronism between stirrer(s) and turn table positions;

    - Relative position and shape of stirrer(s), antennas and turn table

    - RC size;

    - Loading.

# References

[1]	I. Szini, G. F. Pedersen, J. Estrada, A. Scannavini, and L. J. Foged, “Design and verification of MIMO 2x2 reference antennas,” in Antennas and Propagation Society International Symposium (APSURSI), 2012 IEEE, 2012, pp. 1–2.

[2]	Motorola Mobility LLC, ETS-Lindgren, Spirent Communications, “R4-134154- DUT Polarization discrimination, among MIMO OTA test methods,” presented at the R4-68, Barcelona, 2013.

[3]	Motorola Mobility LLC, “R4-135424 - Update results on DUT Polarization discrimination among MIMO OTA test,” presented at the R4-68_bis, Riga, Latvia, 2013.

[4]	Aalborg University, Motorola Mobility LLC, Anite Telecoms ltd, “R4-135433 - COST IC1004 TWGO comments on proposal for additional channel models for MIMO OTA performance,” presented at the R4-68_bis, Riga, Latvia, 2013.

[5]	Hittite Microwave Corporation, “HMC241LP3 / HMC241LP3E - SP4T Non-Reflective Switch SMT, DC - 4 GHz.” Hittite Microwave Corporation.

[6]	Texas Instruments, “TPS61240 - 5V, 400mA, 4MHz Step-Up DC/DC Converter.” Texas Instruments.

[7]	Intel Corporation, Motorola Mobility LLC, Aalborg University, “R4-136185 - MIMO AAS spatially filtered channel model simulations,” presented at the R4-69, San Francisco, USA, 2013.