3GPP TSG RAN WG4 Meeting #69	R4-137072

San Francisco, CA, U.S., 11th – 15th Nov., 2013

- Agenda Item:	8.27

- Source:	LG Electronics

- Title:	Consideration on GNSS, WLAN protection for 2ULs inter-band CA

Document for:	Discussion and Approval

- Introduction

In the last RAN4 #68bis meetings, there was proposal that for self-band, GPS, WLAN, or other receiver desense, harmonics of the Tx fundamental, IM2, IM3, and IM5 products should be taken into consideration when they land within the receive bandwidth [1]. In this contribution, we provide our view on the self-desense for WLAN from harmonics and IMD of the 2UL CA, especially for class A1. The coexistence with GPS can be discussed further but not the scope of this contribution.

- View on Harmonics into ISM bands

Currently, RAN4 has agreed to analyze the impact of the IMD/harmonic problems onto GNSS and ISM bands. As was described in [1], the two different kinds of co-existence can exist, i.e. inter-device and intra-device coexistence.

The inter-device co-existence, which describes the situation where the aggressor and victim UEs are separated geographically, the emission into victim device is far less severe compared to the intra-device case considering path loss [1]. The in-device interference, however, can be more harmful due to the direct path from Tx Harmonics and IMD into Rx bands.

In RAN2 and RAN4, the study on in-device coexistence between LTE and ISM has been carried out. The result of the study is that there should be a kind of network signalling to apply TDM solution between LTE and ISM, which means that eNB informs the DRX configuration for LTE and ISM, so that there should be no interference between the two system. The other alternative is FDM approach, in which case the operating frequency of the LTE signal moves away from the ISM band. These solutions, however, can be applied after there is some desense problems in LTE or GPS/WLAN.

Table 2-1 is the IMD/Harmonics analysis for WLAN/GPS from CA_1A-5A UE.

Table 2-1: The IMD/Harmonic analysis to protect ISM bands and positioning systems

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | Band5 3rd harmonics |
|  | 2400 | - | 2494 | Yes | Asia | Band5 3rd harmonics |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | Band1 3rd harmonics |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | Yes | Asia | Band1 3rd harmonics IMD 5 |

The problem is how to solve the identified harmonic and IMD problem of ISM bands in 3GPP RAN4.

We need to remind that the harmonics from UL transmission into ISM bands have existed since LTE transmission was introduced from Rel-8. And this issue was also existent in Rel-10/11 CA for 1UL/2DL. In those cases, a lot of implementation skills have been applied in UE side to protect ISM bands from the harmonic self-interference.

One typical solution could be a harmonic filter in the TX path of RFIC. But the coupling problems by radiated RF signal in RF path also had impact on the WLAN receive band. So the separation distance between aggressor Tx block and victim Rx block in the surface of UE should be considered. Alternative solution is to re-tune the WLAN frequency where the harmonic does not exist.

Based on these observations, we can see that the current devices on the market have already considered the in-device interference from harmonics into WLAN bands in a way of implementation.

If RAN4 agrees to define a new solution other than implementation for the harmonic problem, this should be discussed from the legacy LTE UE and LTE-A 1UL inter-band CA UE. We also propose the IMD into ISM band should not be related to the insertion loss for filtering solutions.

Therefore we propose way forwards on IMD/Harmonics into ISM band as follow

Proposal 1: For harmonics and IMD problems in ISM bands, RAN4 should not consider an additional insertion loss for filtering solutions compared to 1UL inter-band CA.

Proposal 2: The insertion loss for CA_1A-5A 2UL should be the same as that for CA_1A-5A 1UL.

- Conclusion and Way Forwards

In this contribution, we show our view to protect ISM bands from self-interference. The followings are way forward proposals in this paper.

Proposal 1: For harmonics and IMD problems in ISM bands, RAN4 should not consider an additional insertion loss for filtering solutions compared to 1UL inter-band CA.

Proposal 2: The insertion loss for CA_1A-5A 2UL should be the same as that for CA_1A-5A 1UL.

With this proposal, we have the following text proposal which says the delta TIB and delta RIB for 2UL CA_1A-5A should be the same as those for 1UL CA_1A-5A.

- References

- R4-134747, “Mapping of 2UL intermodulation products,” Qualcomm Incorporated

- R4-135791, “TP for UE coexistence requirements for 2ULs inter-band CA,” LG Electronics

- TR 36.860 v0.4.0

- R4-131416, “Protected Band for B1+B5 2 UP LINK CA,” LG U+

- TS 36.101 v11.6.0

Text proposal for TR 36.860

********************** Start of text proposal to TR 36.860 Chapter 6.2.1 ***********************

- 6.2.1	Inter-region OOBE requirements for Carrier Aggregation of Band 1 and Band 5

Table 6.2.1-1: 2UL inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) band |  |  |  | Downlink (DL) band |  |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | Channel BW (MHz) | BS transmit / UE receive |  |  | Channel BW (MHz) |  |
|  |  | FUL_low – FUL_high |  |  |  | FUL_low – FUL_high |  |  |  |  |
| CA_1-5 | 1 | 1920 MHz | – | 1980 MHz | 10 | 2110 MHz | – | 2170 MHz | 10 | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 10 | 869 MHz | – | 894 MHz | 10 |  |

      - List of specific combination issues

        - Channel bandwidths per operating band for CA

Table 6.2.1.1.1-1: 2UL inter-band CA operating bands

| E-UTRA band / channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Band | E-UTRA Bands | 1.4MHz | 3MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Bandwidth combination set |
| CA_1A-5A | 1 |  |  |  | Yes |  |  | 0 |
|  | 5 |  |  |  | Yes |  |  |  |

        - Co-existence studies for CA_1A-5A with 2ULs

For the analysis of 2UL inter-band CA 1A-5A UE, we should study and analyze the inter-modulation products and harmonics products when both Band 1 and Band 5 with 2ULs are transmitting simultaneously.

The combination of Band 1 and Band 5 has harmonics frequencies that are far away from the components receive and transmit frequencies, in the UL and in the DL of the CA combination, as shown in table 6.2.1.1.2-1. So there was no harmonic issue except Band 41. In the conventional SE for UE-to-UE coexistence requirements, the 3rd harmonics of Band 5 UE already fall into B41 DL band, but exceptions for SE measurements are permitted if there is at least one individual RB within the transmission bandwidth for which the 2nd, 3rd or 4th harmonic totally or partially overlaps the measurement bandwidth (MBW) [3].

Also, the inter-modulation products for CA_1A-5A UE with 2 ULs as shown in table 6.2.1.1.2-1. None of the IMD products fall into the own CA_1A-5A UE receive bands.  But, 2-tone 3rd IMD products may have impact on the Band 22 DL, Band 42 DL and 43 DL bands. But Band 22, Band 42 and Band 43 are far away from Band1 and Band 5, so we can assume that the UE front-end filter can effectively attenuate these IMD products.

Table 6.2.1.1.2-1: 2 UL B1 + B5 harmonic products and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 |  | 824 | 849 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |  |
| 2nd harmonics frequency limits (MHz) | 3840 to 3960 |  |  | 1648 to 1698 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low |  | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 to 5940 |  |  | 2472 to 2547 |  |  |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1071 to 1156 |  |  | 2744 to 2829 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 2991 to 3136 |  |  | 222 to 332 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 4664 to 4809 |  |  | 3568 to 3678 |  |  |

For the connectivity radios (WLAN, BT etc. in ISM bands) and positioning systems (GPS, Galileo, Glonass, Compass, etc), RAN4 analyzed the impact of the IMD/Harmonic from dual-uplink CA transmission in Table 6.2.1.1.2-2. From the analysis table, WLAN system has some harmonics problems in 5GHz and 2.4GHz by 3rd harmonics from each Band 1 and Band5, respectively.

Table 6.2.1.1.2-2: The IMD/Harmonic analysis to protect ISM bands and positioning systems

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | Band5 3rd harmonics |
|  | 2400 | - | 2494 | Yes | Asia | Band5 3rd harmonics |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | Band1 3rd harmonics |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | Yes | Asia | Band1 3rd harmonics |

        - ∆TIB and ∆RIB values

For the class A1 UE with 2ULs simultaneous transmission, additional insertion loss is introduced by diplexer, which can impact to the maximum output power and reference sensitivity level. Generated IMD products and harmonics from 2UL inter-band CA 1A-5A UE do not impact to its own DL frequency bands. The insertion loss of the diplexer has been reported by the RF filter vendors as following Table 6.2.1.1.3-1.

Table 6.2.1.1.3-1: Reported ILs for CA_1A-5A diplexer

| E-UTRA bands | IL (dB) | IL (dB) | IL (dB) | IL (dB) | IL (dB) |
| --- | --- | --- | --- | --- | --- |
| 1 | 0.38 | 0.66 | 0.53 | 0.41 | 0.45 |
| 5 | 0.36 | 0.49 | 0.45 | 0.35 | 0.3 |

086

For the reported IL values there was no or only marginal difference for the Tx/Rx paths reported. Table 6.2.1.1.3-2 shows the average IL

Table 6.2.1.1.3-2: Average Tx and Rx ILs for CA_1A-5A

| Inter-band CA Configuration | E-UTRA Band | Tx IL [dB] | Rx IL [dB] |
| --- | --- | --- | --- |
|  | 1 | 0.49 | 0.49 |
|  | 5 | 0.39 | 0.39 |

Therefore, for the UE which supports CA_1A-5A the ΔTIB,c is defined for applicable bands in table 6.2.1.1.3-3.

Table 6.2.1.1.3-3: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1A-5A | 1 | 0.3 |
|  | 5 | 0.3 |
| NOTE: The values in the table reflect what can be achieved with the present state of the art technology. They shall be reconsidered when the state of the art technology progresses. |  |  |

For the UE which supports CA_1A-5A the ΔRIB is defined for applicable bands in table 6.2.1.1.3-4.

Table 6.2.1.1.3-4: ΔRIB

| Inter-band CA Configuration | E-UTRA Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_1A-5A | 1 | 0 |
|  | 5 | 0 |

********************** End of text proposal to TR 36.860 chapter 6.2.1 ***********************