3GPP TSG-RAN WG4 (Radio) Meeting #70	R4-140450

Prague, Czech, 10 – 14 February 2014

Agenda Item:	7.24.2

Source: 	Alcatel-Lucent, KDDI

Title: 	Text Proposal on Coexistence Studies of Harmonics and Intermodulation Products caused by LTE Advanced Carrier Aggregation of Band Combination (1 + 28)

Document for:	Approval

# 1.	Introduction

The WI proposal to support the LTE-Advanced inter-band Carrier Aggregation of Band 1 and Band 28  was approved in RAN#62 [1]. One objective of the WI is to specify the band-combinations specific Radio Frequency (RF) requirements for inter-band CA of this combination.

The impact of Harmonics and InterModulation Distortion (IMD) products caused by LTE Advanced Base Station (BS) supporting CA of this band combination to the receiver of own or different BS was investigated in [2]. In this paper, we provide a text proposal to record the findings in the Inter-band Carrier Aggregation Technical Report [3].

# 2.	Text Proposal

<New section>

##### 6.2.X.1.2.1	Co-existence studies for 1 UL/2 DL

The 2nd and 3rd order harmonics and IMD products caused in the BS by transmitting of Band 1 and Band 28 DL carriers can be calculated as shown in Table 6.2.X.1.2.1-1 below:

Table 6.2.X.1.2.1-1: Band 1 and Band 28 DL harmonics and IMD products

| BS DL carriers | f1-low | f1-high | f2-low | f2-high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 758 | 803 | 2110 | 2170 |
| 2nd harmonics frequency limits (MHz) | 1516 | 1606 | 4220 | 4340 |
| 3rd harmonics frequency limits (MHz) | 2274 | 2409 | 6330 | 6510 |
| 2nd order IMD products | (f2-low – f1-high) | (f2-high – f1-low) | (f2-low + f1-low) | (f2-high + f1-high) |
| IMD frequency limits (MHz) | 1307 | 1412 | 2868 | 2973 |
| 3rd order IMD products | (f2-low – 2*f1-high) | (f2-high – 2*f1-low) | (2*f2-low – f1-high) | (2* f2-high – f1-low) |
| IMD frequency limits (MHz) | 504 | 654 | 3417 | 3582 |
| 3rd order IMD products | (2*f1-low + f2-low) | (2*f1-high + f2-high) | (2*f2-low + f1-low) | (2*f2-high + f1-high) |
| IMD frequency limits (MHz) | 3626 | 3776 | 4978 | 5143 |
| 3rd order IMD products | (f1-low – f2-high + f2-low) | (f1-high + f2-high – f2-low) | (f2-low – f1-high + f1-low) | (f2-high + f1-high – f1-low) |
| IMD frequency limits (MHz) | 698 | 863 | 2065 | 2215 |
| 3rd order IMD products (with maximum channel bandwidth) | (f1-low – f2-BWmax) | (f1-high + f2- BWmax) | (f2-low – f1- BWmax) | (f2-high + f1- BWmax) |
| IMD frequency limits (MHz) | 738 | 823 | 2090 | 2190 |

It can be seen from Table 6.2.X.1.2.1-1 that that the 3rd harmonics of BS transmitting in Bands 28 may fall into the BS receive band of Bands 30 and 40, while the 3rd IMD products supporting CA of Band 1 and Band 28 may fall into the BS receive band of Bands 5, 6, 12, 13, 14, 17, 18, 19, 20, 22, 26, 27, 28, 42, 43 and 44. Note that the calculation in Table 6.2.X.1.2.1-1 (except the last row) assumes the BS is transmitting with the whole 60 MHz DL frequency of Band 1 and the whole 45 MHz DL frequency of Band 28. If the BS is only transmitting an up to 20 MHz DL in Band 1 and an up to 20 MHz DL in Band 28 as stated in the WIDS, then the 3rd IMD products may only fall into the BS receive band of the Bands 13, 14, 18, 22, 26, 27, 28, 42, 43 and 44 as shown in the last row in Table 6.2.X.1.2.1-1. Moreover, the 3rd IMD products may only fall into the BS receive band of certain frequency range within Bands 18, 26, 27 and 28 under the transmit configurations shown, respectively, in Tables 6.2.X.1.2.1-2, 6.2.X.1.2.1-3, 6.2.X.1.2.1-4 and 6.2.X.1.2.1-5 below. Note that the available frequency range for mobile cellular communications in Japan is 718 – 748 MHz for UL and 773 – 803 MHz for DL.

Table 6.2.X.1.2.1-2: Band (1 + 28) BS transmit configurations with 3rd IMD within Band 18 BS receive band

| Band 28 DL channel bandwidth (MHz) | Higher edge of Band 28 DL frequency block (MHz) | Band 1 DL channel bandwidth (MHz) | Higher edge of IMD frequency limits (MHz) |
| --- | --- | --- | --- |
| 5, 10, 15 or 20 | 800.1 – 803 | 15 | 815.1 – 818 |
| 5, 10, 15 or 20 | 795.1 – 803 | 20 | 815.1 – 823 |

Table 6.2.X.1.2.1-3: Band (1 + 28) BS transmit configurations with 3rd IMD within Band 26 BS receive band

| Band 28 DL channel bandwidth (MHz) | Lower edge of Band 28 DL frequency block (MHz) | Band 1 DL channel bandwidth (MHz) | Higher edge of IMD frequency limits (MHz) |
| --- | --- | --- | --- |
| 5, 10, 15 or 20 | 799.1 – 803 | 15 | 814.1 – 818 |
| 5, 10, 15 or 20 | 794.1 – 803 | 20 | 814.1 – 823 |

Table 6.2.X.1.2.1-4: Band (1 + 28) BS transmit configurations with 3rd IMD within Band 27 BS receive band

| Band 28 DL channel bandwidth (MHz) | Higher edge of Band 28 DL frequency block (MHz) | Band 1 DL channel bandwidth (MHz) | Higher edge of IMD frequency limits (MHz) |
| --- | --- | --- | --- |
| 5, 10, 15 or 20 | 792.1 – 803 | 15 | 807.1 – 818 |
| 5, 10, 15 or 20 | 787.1 – 803 | 20 | 807.1 – 823 |

Table 6.2.X.1.2.1-5: Band (1 + 28) BS transmit configurations with 3rd IMD within Band 28 BS receive band

| Band 28 DL channel bandwidth (MHz) | Lower edge of Band 28 DL frequency block (MHz) | Band 1 DL channel bandwidth (MHz) | Lower edge of IMD frequency limits (MHz) |
| --- | --- | --- | --- |
| 5, 10, 15 or 20 | 758 – 762.9 | 15 | 743 – 747.9 |
| 5, 10, 15 or 20 | 758 – 767.9 | 20 | 738 – 747.9 |

It should be noted that Bands 13, 14, 30, 40 and 43 are not intended for use in the same geographical area as Bands 1 and 28. Moreover, co-location of Band (1 + 28) transmitter and Band 44 transceiver implies FDD/TDD co-location on adjacent frequencies which requires the use of certain site-engineering solutions to avoid mutual interference. Therefore, the focus here will be on the IMD falling into Bands 18, 22, 26, 27, 28 and 42.

With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, we expect the IMD interference generated within the Band 22 or 42 receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Bands 1 and 28 BS transmitters do not share the same antenna with Band 22 or 42 BS receiver.

On the other hand, it is recommended that Bands 1 and 28 BS transmitters should not share the same antenna with Band 22 or 42 BS receiver, or Band 18, 26, 27 or 28 BS receiver for the affected frequency ranges if the aforementioned BS transmit configurations are used, in order to prevent BS receiver desensitization, unless the antenna path meets very stringent 3rd order PIM specification so that the PIM will not cause Band 18, 22, 26, 27, 28 or 42 BS receiver desensitization.

<End of section>

# References

[1]	RP-132028, “New Work Item Proposal: LTE-Advanced inter-band Carrier Aggregation of Band 1 and Band 28”, KDDI.

[2]	R4-140449, “Harmonics and Intermodulation Products caused by LTE Advanced Carrier Aggregation of Band Combination (1 + 28)”, Alcatel-Lucent, KDDI.

[3]	R4-140843, “TR 36.851 V0.9.0: Rel-12 Inter-band Carrier Aggregation”, Ericsson.