3GPP TSG-RAN WG4 Meeting #70	 R4-140912

Prague, Feb 10th – 14th, 2014

Source: 	Sporton International Inc.

Title: 					Creation of a section in TR 25.914 for PHABLET

Agenda Item: 	7.1.4

Document for:	Approval

1	Introduction

Phablet is a device type that supports the same functionality as a smartphone (UE/MS) but with a wide form factor.

It can be used as a tablet but as a mobile voice speech as well.

This document is a proposal for introduction of device type that is wider than 72 mm and is not in Tablet category and supporting voice function into TR 25.914.

2	Summary

Phablet devices are considered as mobile but wider than 72 mm, the usage case is relatively depending on customers And it might be useful to not consider them as part of UE/MS since they don't feat with existing artificial hands for testing.

Therefore it is a need to have a test methodology and requirements defined in TR 25.914

3	Proposal

It is proposed to approve the creation of new section for phablet device type.

4	References

[1] TR 25.914: "Measurements of radio performances for UMTS terminals in speech mode"

<< start change >>

5.3.1	Notebook

A notebook PC is a portable personal computer combining the computer, keyboard and display in one form factor. Typically the keyboard is built into the base and the display is hinged along the back edge of the base. The largest single dimension for a notebook is limited to 0.42 m.

As notebooks are not body worn equipment nor recommended for use placed directly on the lap, the notebook shall be tested in a free space configuration without head and hand phantoms.

When the notebook is placed in a measurement chamber the display shall be configured according to Table 5.3.1-1.

Table 5.3.1-1: Display Settings

| Parameter | Value | Note |
| --- | --- | --- |
| Display lid angle | 110 +/- 5 degrees | The lid angle is defined as the angle between the front of the display to the leveled base. |
| LCD Backlight | 50% |  |
| Ambient sensor | Disabled |  |

A typical notebook PC is equipped with several radio access technologies. During the measurement the DUT shall be configured according to Table 5.3.1-2.

Table 5.3.1-2: Embedded radio transmitters

| Parameter | Value | Note |
| --- | --- | --- |
| WWAN | Enabled | This is the DUT tranceiver |
| Other transceivers | Disabled | UWB, WLAN, Bluetooth™ |

The notebook power management shall be configured according to Table 5.3.1-3.

Table 5.3.1-3: Power management

| Parameter | Value | Note |
| --- | --- | --- |
| Screensaver | Disabled |  |
| Turn OFF display | Never |  |
| Turn OFF Hard drive | Never |  |
| System Hibernate | Never |  |
| Sysem Standby | Never |  |
| Dynamic control of clock frequencies | Disabled |  |
| Power source | Standard battery |  |

If the notebook is equipped with retractable antennas the device shall be tested with the antennas in a configuration recommended by the manufacturers.

5.3.2	Tablet

5.3.3	Phablet

<< end of changes >>