3GPP TSG RAN WG4 Meeting #70	                     R4-141181

Prague, Czech Republic, 10th – 14th Feb., 2014

Agenda Item:	7.23.1

Source:	LG Electronics, LG UPlus

Title:	TP for TR 36.851: B1+B5 Harmonic and IMD analysis for 3DLs CA_1A_5A_7A band combination UE

Document for:	Approval

- Introduction

In the last RAN4 #69AH UE RF meetings, there are some clarifications for additional bandwidth combination set to support 3DLs inter-band CA [1]. The previous B1+B5 band combination UE only support 10MHz+10MHz bandwidth combination set, therefore RAN4 do not guarantee the fallback operation from 3DLs CA to 2DLs CA.

In this paper, we analyzed the harmonic and IMD products for B1+B5 UE to support additional bandwidth combination set for B1+B5+B7 3DLs inter-band CA UE.

- Reference

- R4-69AH-0023, “Carrier aggregation bandwidth support,” Qualcomm Incorporated

- TR 36.851 v0.8.0

Text proposal for TR 36.851

******************** Start of text proposal to TR 36.851 Chapter 6.1.X **********************

6.1.X	LTE Advanced Carrier Aggregation of Band 1 and Band 5

CA_1-5 is designed to operate in the operating bands in table 6.1.X-1.

Table 6.1.X-1: Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) band |  |  |  | Downlink (DL) band |  |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | Channel BW (MHz) | BS transmit / UE receive |  |  | Channel BW (MHz) |  |
|  |  | FUL_low – FUL_high |  |  |  | FDL_low – FDL_high |  |  |  |  |
| CA_1-5 | 1 | 1920 MHz | – | 1980 MHz | 5, 10, 15, 20 (Note 1) | 2110 MHz | – | 2170 MHz | 5, 10, 15, 20 | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 5,10 (Note 1) | 869 MHz | – | 894 MHz | 5, 10 |  |
| NOTE 1: The WI considers only one uplink component carrier to be used in any of the two frequency bands at any time |  |  |  |  |  |  |  |  |  |  |

6.1.X.1		List of specific combination issues

6.1.X.1.1	Channel bandwidths per operating band for CA

Table 6.1.X.1.1-1: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| CA operating / channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| CA_1A-5A | 1 |  |  | Yes | Yes | Yes | Yes |
|  | 5 |  |  | Yes | Yes |  |  |

##### 6.1.X.1.2	Co-existence studies for CA_1-5

Table 6.1.X.1.2-1 summarizes frequency ranges where harmonics occur due to Band 1 or Band 5 for both UL and DL. It can be seen that the harmonic frequencies of Band 1 and Band 5 in DL are away from the BS receive bands of interest in the UL. For the UE aspect, the UL harmonic frequencies of Band 1 and Band 5 does not locate within the UE receive bands of interest in the DL. Therefore we can conclude that there is no issue on harmonic interference.

Table 6.1.X.1.2-1: Impact of UL/DL Harmonic Interference

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 1 | 1920 | 1980 | 2110 | 2170 | 3840 | 3960 | 5760 | 5940 | 4220 | 4340 | 6330 | 6510 |
| 5 | 824 | 849 | 869 | 894 | 1648 | 1698 | 2472 | 2547 | 1738 | 1788 | 2607 | 2682 |

The 2nd DL harmonics of Band 5 carriers may fall into the BS receive band of Bands 3, 4, 9 and 10. And also 3rd DL harmonics of Band 5 carriers may fall into the BS receiver band of Bands 38 and 41.

The 2nd and 3rd order harmonics and IMD products caused in the BS by transmitting of Band 1 and Band 5 DL carriers can be calculated as shown in table 6.1.X.1.2-2 below:

Table 6.1.X.1.2-2: Band 1 and Band 5 DL harmonics and IMD products

| BS DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 869 | 894 | 2110 | 2170 |
| 2nd order harmonics frequency range (MHz) | 1738 | 1788 | 4220 | 4340 |
| 3rd order harmonics frequency range (MHz) | 2607 | 2682 | 6330 | 6510 |
| 2nd order IMD products | (f2-low – f1-high) | (f2-high – f1-low) | (f2_low + f1_low) | (f2_high + f1_high) |
| IMD frequency limits (MHz) | 1216 | 1301 | 2979 | 3064 |
| 3rd order IMD products | (f2_low – 2*f1_high) | (f2_high – 2*f1_low) | (2*f2_low – f1_high) | (2*f2_high – f1_low) |
| IMD frequency limits (MHz) | 322 | 432 | 3326 | 3471 |
| 3rd order IMD products | (2*f1_low + f2_low) | (2*f1_high + f2_high) | (2*f2_low + f1_low) | (2*f2_high + f1_high) |
| IMD frequency limits (MHz) | 3848 | 3958 | 5089 | 5234 |
| 3rd order IMD products | (f1_low – f2_high + f2_low) | (f1_high + f2_high – f2_low) | (f2_low – f1_high + f1_low) | (f2_high + f1_high – f1_low) |
| IMD frequency limits (MHz) | 809 | 954 | 2085 | 2195 |
| 3rd order IMD products (with maximum channel bandwidth) | (f1_low – f2_BWmax) | (f1_high + f2_BWmax) | (f2_low – f1_BWmax) | (f2_high + f1_BWmax) |
| IMD frequency limits (MHz) | 849 | 914 | 2100 | 2180 |

It can be seen from Table 6.1.X.1.2-2 that the 3rd IMD products caused by BS supporting carrier aggregation of Band 1 and Band 5 may fall into the BS receive bands of Band 5, 8, 18, 19, 20, 22, 26, 27 and 42. However, when the impact of maximum bandwidth is considered, the 3rd order IMD products do not fall into the BS receive of Band 5, 18, 19, 26 and 27.

It should be noted that Bands 4, 9, 10, 20, and 22 are not intended for use in the same geographical area as Band 1 and 5. Consequently, the focus here will be on the harmonics and IMD products falling into Bands 3, 8, 38, 41 and 42.

With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the harmonics and IMD interference generated within the Bands 3, 8, 38, 41 and 42 receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization.

Therefore, it is recommended that Band 1 and 5 BS transmitters should not share the same antenna with Band 3, 8, 38, 41 or 42 BS receivers for the affected frequency ranges.

6.1.X.1.3		ΔTIB,c and ΔRIB (1 UL)

For the UE which supports CA_1A-5A with 2DLs and 1UL, the ΔTIB,c is defined for applicable bands in table 6.1.X.1.3-1.

Table 6.1.X.1.3-1: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1A-5A | 1 | 0.3 |
|  | 5 | 0.3 |

For the UE which supports CA_1A-5A with 2DLs and 1UL, the ΔRIB is defined for applicable bands in table 6.1.X.1.3-2.

Table 6.1.X.1.3-2: ΔRIB

| Inter-band CA Configuration | E-UTRA Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_1A-5A | 1 | 0 |
|  | 5 | 0 |

<End of TP>