3GPP TSG-RAN WG4 Meeting #70                                                                       R4-140348

# Prague, Czech Republic, 10 - 14 Feb, 2014

Source:	LG Electronics

Title:	Discussion on DL CoMP with CRS-IC

Agenda item:	6.4

Document for:	Discussion

Introduction

In previous RAN4 meeting, DL CoMP test 2-C which is Scenario 3 with non-colliding CRS was discussed and there was no consensus for following options [1].

  - Option 1: Define Test 2-C in Rel-11; Study and define comprehensive CRS-IC performance in Rel-12

  - Option 2: No test case in Rel-11; Study and define comprehensive CRS-IC performance in Rel-12

In this contribution, we provide system level simulation performance for DL CoMP with CRS-IC.

Discussion on DL CoMP test with CRS-IC

Rel-11 DL CoMP test cases are defined to verify the PDSCH demodulation performance for TM10 UE under the assumption of non-quasi-colocated antennas. However, CRS-IC is additional feature to improve PDSCH performance, and it is not a mandatory feature for DL CoMP. CRS-IC feature was already verified in R-11 FeICIC WI. Therefore, CRS-IC feature to verify PDSCH demodulation performance for Rel-1l DL CoMP is unnecessary, and it is better to leave UE implementation issue since CRS-IC is not a mandatory feature for CoMP. In following subsection, performance of DL CoMP with and without CRS-IC is evaluated in system level perspective.

Performance analysis

Figure 1 and Figure 2 show CDF of 10%-tile of UE throughput based on system level simulator. 10%-tile of UE throughput can represent the performance range which can obtain benefit through CoMP operation. Simulation assumption is listed in Table 2. Table 1 shows 10%-tile UE average throughput performance gain of DL CoMP without CRS-IC and w/ serving cell CRS-IC in comparison with that of non-CoMP according to power difference between transmission points. 10%-tile UE average throughput of DL CoMP without CRS-IC is improved by 17.01~25.1% in comparison with that of non-CoMP under 4dB and 6dB power difference, respectively. And the difference of performance gain between CoMP w/o CRS-IC and w/ serving cell CRS-IC is within 2%. It implies that serving cell CRS-IC for DL CoMP brings minor performance improvement in system perspective.

- Observation: Serving cell CRS-IC for DL CoMP brings minor performance improvement in system perspective.

Figure 1 CDF of 10%-tile of UE throughput with 4dB power difference

Figure 2 CDF of 10%-tile of UE throughput with 6dB power difference

Table 1 10%-tile UE average throughput performance gain (%) based on Non-CoMP performance

| Power Diff | 4dB | 6dB |
| --- | --- | --- |
| No CRS-IC | 17.01 | 25.10 |
| Serving CRS-IC | 18.96 | 27.15 |

Table 2 Simulation assumption

| Parameter | Value |
| --- | --- |
| Cellular layout | 3-sectorized Hexagonal grid with 19 cells wrap-around |
| System frequency | 2 GHz carrier, 10 MHz bandwidth |
| Indoor/outdoor UE ratio | 50% indoor UE, 50% outdoor UE (homogeneous network) 80% indoor UE, 20% outdoor UE (heterogeneous network) |
| Number of small per sector | 4 |
| Number of UEs per sector | 10 |
| Traffic model | Full buffer |
| Scheduling algorithm | Proportional Fair |
| Transmission mode | Transmission mode 10 with SU -MIMO |
| Antenna configuration | 2x2 antenna |
| Hybrid ARQ | Incremental Redundancy (IR), Maximum four transmissions, Initial transmission target FER: 10% |
| Hybrid ARQ round trip delay for UE | 8 subframes (8 ms) |
| Channel Estimation | Non Ideal |

Conclusion

In this contribution, we provide simulation results for DL CoMP with and without CRS-IC. From simulation results, our observation is as follows:

- Observation: Serving cell CRS-IC for DL CoMP brings minor performance improvement in system perspective.

We propose

- Proposal: For Rel-11 DL CoMP test, serving cell CRS-IC is unnecessary feature.

Reference

- RP-131465, “Status report for WI Perf. part: Coordinated Multi-Point Operation for LTE – Downlink,” Samsung